3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220981

- 17-26 Jan 2022

- Online

Agenda item:	10.2.4

Source: 	ZTE, China Telecom

Title: 	(TP for SON BL CR 38.413) Inter-system Load Balancing

Document for:	Agreement

Introduction

This contribution provides a TP for BL CR 38.413 [1] on the inter-system load balancing, i.e. removing some “FFS” and introducing the PRB usage in the Inter-system Resource Status Report IE.

Text Proposal for 38.413

First Change

9.3.3.yy1	Inter-system Resource Status Request

This IE contains information on the requested Inter-system Load Reporting reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting System |  |  |  |  |
| > E-UTRAN |  |  |  |  |
| >>E-UTRAN Cell To Report List |  | 1 |  |  |
| >>>E-UTRAN Cell To Report Item |  | 1 .. < maxnoofReportedCells> |  |  |
| >>> Cell ID | M |  | E-UTRAN CGI 9.3.1.9 | Contains the E-UTRAN CGI IE. |
| > NG-RAN |  |  |  |  |
| >>NG-RAN Cell To Report List |  | 1 |  |  |
| >>>NG-RAN Cell To Report Item |  | 1 .. <maxnoofReportedCells > |  |  |
| >>>> Cell ID | M |  | NG-RAN CGI 9.3.1.73 | Contains the NG-RAN CGI IE. |
| Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the receiving node is requested to report. First Bit =Number of Active UEs, Second Bit =RRC connections, Third Bit =PRB. Other bits shall be ignored by the receiving node. |
| CHOICE report type | M |  |  |  |
| >Event based reporting |  |  |  |  |
| >>Inter-system Resource Threshold Low | M |  | INTEGER (0..100) | Inter-system Resource Status reporting is enabled when Composite Available Capacity is above this threshold or is disabled when Composite Available Capacity is below this threshold. The reporting node sends a report when the cell Composite Available Capacity becomes greater than or equal to the threshold. |
| >>Inter-system Resource Threshold High | M |  | INTEGER (0..100) | Inter-system Resource Status reporting is enabled when Composite Available Capacity is below this threshold or is disabled when Composite Available Capacity is above this threshold. The reporting node sends a report when the cell Composite Available Capacity becomes smaller than or equal to the threshold. |
| >>Number Of Measurement Reporting Levels | O |  | ENUMERATED (2, 3, 4, 5, 10, ...) | The reporting node divides the cell load scale into the indicated number of reporting levels, evenly distributed on a linear scale between the Inter-system Resource Threshold Low and the Inter-system Resource Threshold High. The reporting node sends a report each time the cell load changes from one reporting level to another. |
| >Periodic Reporting |  |  |  |  |
| >>Reporting Periodicity | M |  | ENUMERATED(single, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting. If the value is “single” there is only one report. |

| Range bound | Explanation |
| --- | --- |
| maxnoofReportedCells | Maximum no. of cells that can be reported. Value is 256 . |

9.3.3.yy2	Inter-system Resource Status Report

This IE contains the Inter-system load report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting System |  |  |  |  |
| > E-UTRAN |  |  |  |  |
| >>E-UTRAN Cell To Report List |  | 1 |  |  |
| >>>E-UTRAN Cell To Report Item |  | 1 .. < maxnoofReportedCells > |  |  |
| >>>>Cell ID | M |  | E-UTRAN CGI 9.3.1.9 | Contains the E-UTRAN CGI IE. |
| >>>>Composite Available Capacity Group | M |  |  | Contains the Composite Available Capacity Group IE as defined in TS 36.423[40]. |
| >>>>Number of Active UEs | O |  |  | Contains the Number of Active UEs IE as defined in TS 36.314. |
| >>>>Radio Resource Status | O |  |  | Contains the Radio Resource Status IE as defined in TS 36.423[40]. |
| >NG-RAN |  |  |  |  |
| >>NG-RAN Cell To Report List |  | 1 |  |  |
| >>> NG-RAN Cell To Report Item |  | 1 .. < maxnoofReportedCells > |  |  |
| >>>>Cell ID | M |  | NG-RAN CGI 9.3.1.73 | Contains the NG-RAN CGI IE. |
| >>>>Composite Available Capacity Group | M |  |  | Contains the Composite Available Capacity Group IE as defined in TS 36.423[40]. |
| >>>>Number of Active UEs | O |  |  | Contains the Number of Active UEs IE as defined in TS 38.423[24]. |
| >>>>RRC Connections | O |  |  | Contains the RRC Connections IE as defined in TS 38.423[24]. |
| >>>>Radio Resource Status | O |  |  | Contains the Radio Resource Status IE as defined in TS 36.423[40]. |

End of Changes

References

- R3-216254 BLCR to 38.413_Addition of SON features enhancement  Ericsson