3GPP TSG-RAN WG3 #114bis-e	R3-221020

E-meeting, 17th – 26th January 2022

Source:	CATT (moderator)

Title:	CB: # SONMDT5_RACHOpt

Agenda Item:	10.2.1.7

Document for:	Approval

- Introduction

CB: # SONMDT5_RACHOpt

- Check TPs to reflect agreements in last meeting

- Define new IE structures to deliver neighbour cell information necessary for PRACH coordination? Introduce the NR cell PRACH configurations in the NR neighbour Information IE?

- Whether gNB-DU should be capable to request the gNB-CU to provide PRACH related information or not? A gNB-DU indicates to the gNB-CU the occurrence of PRACH reconfiguration or PRACH configuration due to conflict/potential conflict for cases when the RACH procedure is not known to the gNB-CU? SN indicates the availability of RA Report of a set of UEs specified by their XnAP UE ID?

- Capture agreements and provide TPs if agreeable

(CATT - moderator)

The deadline for the first phase is 00:00 UTC on 21st January (Friday).

- For the Chairman’s Notes

First phase:

For agreement:

Proposal 1: The outer list (i.e. list of served cells) is an optional IE with criticality “ignore” directly within the two F1AP messages delivered from the gNB-CU toward the gNB-DU.

Proposal 2: Neighbour cell list included in each item of the served cell list should be optional

Proposal 3: For the content of the inner list, i.e. the list of neighbour cells, follows the encoding in R3-220530

Proposal 4: For the ANS.1 coding, use sequence type as in R3-220530

Proposal 5: Agree that TP for X2 AP follows the same principle as the TP for F1AP

For online discussion:

It is FFS on whether the gNB-DU “shall” store or “may” store the received PRACH configurations of neighbour cells.

For second round discussion:

TP for 38.473 Revised from R3-220530

TP for 36.423 Revised from R3-220379

To be continued:

FFS on whether some enhancement are needed so that the gNB-DU can provide some information toward the gNB-CU, and if so, what enhancement.

Postpone to Rel-18 the discussion of whether/how the SN can pull RA report from the MN, i.e. indicating that RA procedure happens at SCG toward the MN.

- Discussion (first phase)

  - Neighbour PRACH Info

In this part we discuss only the IE structure containing the neighbour PRACH info delivered either from the gNB-CU toward the gNB-DU or from the eNB toward the en-gNB. Whether/how the gNB-DU provides some information toward the gNB-CU is discussed in Section 3.2.

    - F1AP

This topic has been discussed for a long time (2 years or so) and all of the companies raised their own TPs ([1][3][5][6]) on this topic according to what we agreed last meeting, two of which ([3][5]) contained the corresponding ASN.1.

For this part we will first check the IE structure from the outermost to the innermost: the presence of the outer list, the content of the outer list item, and finally the content of the inner list item. Some other miscellaneous issue are checked later.

Presence of the outer list (i.e. list of served cells)

The tabular in [3][6] and the ASN.1 in [3][5] proposed that the served cell list should be an optional IE, with criticality “ignore”, within the two F1AP messages delivered from the gNB-CU toward the gNB-DU. The tabular in [1][5] didn’t propose so, but the moderator thinks they were just typos and were not intended.

Questions 1.1-1: Do you agree to confirm that the outer list (i.e. list of served cells) is an optional IE with criticality “ignore” directly within the two F1AP messages delivered from the gNB-CU toward the gNB-DU?

| Company | Comment |
| --- | --- |
| CATT | Agree. |
| Huawei | Agree |
| Ericsson | Agree. Just to clarify, while the List of outer Cell IDs is optionally present, the CGIs of the outer cells shall be mandatory. This is how we encoded it in [5] |
| Samsung | Agree. |
| Nokia | Agree. |
| CMCC | Agree |
| ZTE | Agree. |

Moderator’s summary: 7 companies provided feedback and all agreed that the list of served cells is an optional IE with criticality “ignore”.

Proposal 1: The outer list (i.e. list of served cells) is an optional IE with criticality “ignore” directly within the two F1AP messages delivered from the gNB-CU toward the gNB-DU.

Content of the outer list item

All companies agreed that each item of the served cell list should contain an NRCGI of served cell (which has been agreed in previous meetings) and the inner list, i.e. the list of neighbour cell, but view were split on the presence of the neighbour cell list.

- The tabular in [1] and the ASN.1 in [5] proposed that the neighbour list is a mandatory IE.

- The tabular in [3][5][6] and the ASN.1 in [3] seemingly proposed that the neighbour cell list is an optional IE. One reason of setting it optional is provided in [2]:

One approach is to define that list dedicated for NR PRACH coordination, while another approach is to define it suitable to be expanded for other cross-link interferences (at least for NR-E-UTRA PRACH coordination maybe). We general prefer the latter approach as it does not cost many more bits. As the result the NR neighbour cell list should be optional present (as one day in the future we may add an E-UTRA neighbour cell list into it).

Questions 1.1-2: What is your view on whether the neighbour cell list should be mandatory or optional?

| Company | Comment |
| --- | --- |
| CATT | Both options are acceptable for us, but we prefer setting it optional. |
| Huawei | Either way is fine. |
| Ericsson | We are fine in both ways. Another reason to have the list optional could be to send the outer list of cells in potential conflict to the gN-DU and let the gNB-DU resolve the issue by its own internal means, without neighbour RACH information needed. |
| Samsung | Optional for future proof. |
| Nokia | Both options are acceptable to us, but maybe optional is better. |
| CMCC | Slightly prefer optional |
| ZTE | we prefer setting it optional. |

Moderator’s summary: 7 companies provided feedback and all agreed that the list of neighbour cell is optionally present.

Proposal 2: Neighbour cell list included in each item of the served cell list should be optional

Content of the inner list item

Views were split on the content in each item of the inner list, i.e. the list of neighbour cells. Three companies ([1][5][6]) provided one option (there is some small difference between [1] and [5][6] anyhow), whereas one company ([3]) provided another.

In addition, the moderator noticed that some company made some IEs other than the NR CGI as mandatory IE, but according to the principle of “Approach 2bis”, every IE except the NR CGI should be optional in order to avoid duplication. The moderator decided not to discuss this topic here.

Questions 1.1-3: Which option do you prefer for the content of the inner list, i.e. the list of neighbour cells?

| Company | Comment |
| --- | --- |
| CATT | The option in [3]. [1][5][6] didn’t provide enough information as what we agreed in Q2 2020 (see in Section 10.2.3.1 of the chairman notes / meeting reports): RAN3#107bis-e: Introduce NR PRACH Configuration list per UL/SUL for a cell. Reuse current NR ARFCN IE instead of introducing new IE. Introduce frequencyShift7p5khz per-UL/SUL No need to include the freqBandIndicatorNR Introduce scs-SpecificCarrierList for UL (DL is FFS) (Note: whether it should be included in serving cell information or PRACH configuration is FFS) Introduce an optional IE into the Served Cell Information NR structure to indicate the SSB Positions In Burst Not introduce any cause IE for random access Agree exchanging NR PRACH coordination over X2AP Aeparate the discussion on SCS-SpecificCarrier for DL with PRACH configuration i.e. remove the SCS-SpecificCarrier for DL in the TP and discuss this issue as a correction RAN3#108-e: Position to include scs-SpecificCarrierList should be per UL/SUL introduce a new TDD pattern for RACH optimization Not included Root Sequence Index BFR for PRACH Optimization in Rel-16. Due to lack of time in Rel-16 whether to introduce Root Sequence Index BFR is proposed to be discussed in Rel-17. To use one newly-defined IE “PRACH Frequency Start from Carrier” to indicate the difference between the “frequency-domain start point” of the RACH occasion and the “frequency-domain start point” of associated carrier To use ssb-perRACH-Occasion to indicate the mapping between RACH resources and SSB to move Zero Correlation Zone Config IE out of FreqDomainLength IE PRACH conflict detection in CU-DU split case is postponed to rel-17 PRACH configuration per served cell is signalled from DU to CU and over Xn agree exchange NR PRACH coordination over X2AP signal the PRACH configuration of Served Cells over X2AP, the number of PRACH configurations can be limited to 16 X2AP signalling of PRACH configurations of neighbour cells is postponed to Rel-17 The IEs highlighted in blue is encoded within the NR PRACH Configuration list (and thus included in both options), whereas the IEs highlighted in green is encoded out of that list (so we have to include all of them here in the neighbour cell item, which is only obeyed in [3]). |
| Huawei | We are ok with the encoding in [3]. |
| Ericsson | We are ok with the encoding in [3] |
| Samsung | The option in [3] |
| Nokia | We are ok with the encoding in [3]. |
| CMCC | OK with [3] |
| ZTE | We are ok with the encoding in [3] |

Moderator’s summary: 7 companies provided feedback and all agreed that the content listed in [3] should be included in the items of neighbour cell list.

Proposal 3: For the content of the inner list, i.e. the list of neighbour cells, follows the encoding in R3-220530

ASN.1 type

The TPs provided by two companies respectively used different ASN.1 type:

- The TP in [3] used plain ASN.1 types, e.g. SEQUENCE. As the result the F1AP PDU contained neither IE ID nor criticality in this part.

- The TP in [5] used ASN.1 types dedicated for F1AP, e.g. “F1AP-PROTOCOL-IES”. As the result the F1AP PDU contained both an IE ID and a criticality value for each IE in this part.

Questions 1.1-4: Which option do you prefer for ASN.1 coding?

| Company | Comment |
| --- | --- |
| CATT | Plain ASN.1 types. It costs far less bits (ordinarily 4 octets for each IE). And there are a few ASN.1 coding errors in [5]… |
| Huawei | Normally, sequence type is the right implementation? |
| Ericsson | Both types are ok for us |
| Nokia | We have a preference to plain ASN.1 types. |
| ZTE | Plain ASN.1 types. |

Moderator’s summary: 5 companies provided feedback and all agreed that plain ASN.1 type (i.e. the manner without criticality in coding) should be used.

Proposal 4: For the ANS.1 coding, use sequence type as in R3-220530

Storage in gNB-DU?

Views are split on whether the gNB-DU may store the information.

- The TP in [1][3] proposed that the gNB-DU “may store or update” the received neighbour PRACH information besides using it for PRACH conflict resolution.

- The TP in [5][6] didn’t propose so.

Questions 1.1-5: May a gNB-DU store the received neighbour PRACH information?

| Company | Comment |
| --- | --- |
| CATT | We prefer yes. As pointed out in [2], this can be very useful for gNB-DU sharing scenario. Nevertheless it is not precluded that a gNB-DU not doing so. |
| Huawei | Agree. The DU shall store the received PRCH configurations sent from CU, like other parameters from CU. |
| Ericsson | The gNB-DU shall not be mandated to store the neighbour RACH information. Memory capabilities on the gNB-DU may be limited and there might be lots of neighbour RACH configurations to store at the gNB-DU. Storing such information can of course be left to implementation. Technically there is no need to store the information, because when the gNB-CU signals to the gNB-DU a list of cells in potential conflict, the gNB-DU reacts to that by correcting the cell´s RACH configuration. After such correction, neighbour RACH configurations are not needed anymore. If a new conflict is detected by the gNB-CU, the gNB-CU will send again the CGI of the cell in conflict and neighbour PRACH information. The following formulation gives freedom to the gNB-DU to store or not store the information: If the GNB-CU CONFIGURATION UPDATE message contains the RACH Conflict Assistance Information List IE, the gNB-DU may use this information to resolve RACH conflicts. |
| Samsung | This can be left for implementation but using “may” is also fine for us. |
| Nokia | Yes. Our understanding is that there is an F1AP requirement that a DU shall store updated configuration data, including PRACH configurations. |
| CMCC | Yes |
| ZTE | Yes, Agree with Nokia and Huawei. |

Moderator’s summary: 7 companies provided feedback. 3 companies thought the gNB-DU “shall” store, 2 companies thought it “may”, and 1 company thought it should be up to implementation. 1 company’s opinion was not clear.

Proposal 5:FFS on whether the gNB-DU “shall” store or “may” store the received PRACH configurations of neighbour cells. This may be discussed online or in the second phase.

    - X2AP

Two companies proposed TP for X2AP ([1][4]) and their content were exactly the same (except the point that one is provided with ASN.1 and the other is not).

Questions 1.2-1: Do you agree with the TP for X2AP raised in [1] and [4]?

| Company | Comment |
| --- | --- |
| CATT | Yes. |
| Huawei | Yes. |
| Ericsson | Yes |
| Samsung | Yes |
| Nokia | Yes. |
| CMCC | Yes |
| ZTE | Yes |

Moderator’s summary: 7 companies provided feedback and all agree with the TP provided in [1][4].

Proposal 6: Agree to capture TP for X2AP BLCR.

  - Information provided from the gNB-DU toward the gNB-CU

One company proposed that the gNB-DU may provide some information toward the gNB-CU due to “the selection of a new PRACH configuration by the gNB-DU may trigger a new conflict with a neighbour not signalled by the gNB-CU” [5]. The information proposed included:

1) reconfigure its local PRACH configuration for the corresponding cell(s) e.g., changing the PRACH sweeping pattern or 2) Signal to the gNB-CU that more neighbour PRACH configuration is needed in order to avoid other possible conflicts If action 1) is carried out, the gNB-DU shall inform the gNB-CU of a PRACH sweeping pattern change so that the gNB-CU can update its neighbour cell relations. This is because a change of sweeping pattern implies that the neighbour relations of certain SSB indexes with other neighbour cells has changed. If action 2) is carried out, the gNB-DU shall inform the gNB-CU that more neighbours´PRACH information is needed.

On the other hand, one company expressed their opposing [6]:

In addition, it was discussed whether in addition to a push mechanism, a pull mechanism could also be supported where a gNB-DU requests a gNB-CU for more neighbour PRACH Configurations. In our thinking such pull mechanism is not needed since a gNB-DU has enough PRACH Configurations provided by its gNB-CU to resolve PRACH Configuration conflicts locally.

Questions 2-1: What is your opinion on whether some enhancement are needed so that the gNB-DU can provide some information toward the gNB-CU, and if so, what enhancement.

| Company | Comment |
| --- | --- |
| CATT | Not needed. The prerequisite in [5] does not stand. Please see what we agreed last meeting: It should be possible for the gNB-CU to provide the gNB-DU with information indicating the CGI of the cells potentially in conflict and the neighbouring relation between these cells and their neighbour cells, along with the PRACH configurations of those neighbour cells, so as to prevent the gNB-DU from reconfiguring one of its cells from conflicting with one neighbour toward conflicting with another neighbour. How/whether gNB-CU do the filter is up to implementation. So there is no such case of “the selection of a new PRACH configuration by the gNB-DU may trigger a new conflict with a neighbour”, we have prevented this. If some company thinks the cap of 32 neighbours is not enough to cover all neighbour cells (more strictly, all homogeneous cells and larger cells), we should raise the value to e.g. 64 rather than revert what we have agreed. Due to the lack of time we think we should stop discussing this in Rel-17. Maybe if can be left to Rel-18. |
| Huawei | No strong view. |
| Ericsson | We support the proposals. In reply to CATT, the gNB-CU should always try to minimise the list of neighbour RACH information signalled to the gNB-DU, to avoid oversized messages. This is a principle we discussed for many meetings and in the end we embraced with a list of 32 cells in conflict. One possible implementation may be that the gNB-CU signal to the gNB-DU the CGI of the cell in conflict, its neighbours and its neighbours of neighbours. However, there might be unpredictable channel propagation effects by which a cell far away still interferes on RACH resources. These phenomenon are very common in radio propagation and are not a corner case. This is why it is useful for the gNB-DU to ask for more neighbour information to the gNB-CU. Equally, if the gNB-DU decides to solve the issue by beam sweeping order change, it is useful that the gNB-DU signal such cell configuration change to the gNB-CU. The gNB-CU shall at least be aware that the SSB sweeping order is not the same anymore for one of the DU´s cells, otherwise the gNB-CU has wrong cell configuration information. |
| Samsung | Agree with CATT. |
| Nokia | Not needed. We agree with the view of CATT in this. |
| CMCC | No strong view |
| ZTE | Not needed. Agree with CATT. |

Moderator’s summary: 7 companies provided feedback. 4 companies thought they were not needed. 1 company though it should be supported. 2 companies had no strong opinion.

Proposal 7: It is still FFS on whether some enhancement are needed so that the gNB-DU can provide some information toward the gNB-CU, and if so, what enhancement.

  - SCG RA report

This issue is the continuation of CB # 11_SCGRACHReport last meeting (R3-216204). There is one leftover issue on whether we should enhance the interface so that an SN can pull RA report from the MN.

In this meeting one company proposed so [5], whereas other companies didn’t express their opinion.

| Company | Comment |
| --- | --- |
| CATT | Postpone to Rel-18. We have agreed that the “pull” mechanism of RA report over F1AP is postponed to Rel-18. We think the case on X2AP/XnAP should be aligned with it. |
| Huawei | If the pull mechanism in F1AP is postpone to Rel-18, its better to postpone the X2/XNAP case to Rel-18 as well. |
| Ericsson | Support the proposal. RAN2 has finalised all the agreements needed on this topic. Given that the problem and the solution are very simple, we do not see why we should postpone the issue. Postponing it would imply that SCG RACH reports in Rel17 would be lost, which is inconsistent with the RAN2 agreements that the UE generates these reports and makes them available to the MN, if the MN is aware that they are available. Of course, without the solutions proposed, the MN will not be aware of SCG RACH report availability. |
| Samsung | Agree with CATT |
| Nokia | RAN2 hasn’t yet concluded on important aspects on SN related RACH information, including the detailed split of MN and SN related contents in the same or different UE variables. So we propose to postpone this topic to Rel. 18. |
| CMCC | Fine to postpone this to Rel-18 |
| ZTE | Agree with CATT. since pull mechanism in F1AP is postpone to Rel-18, the case on X2AP/XnAP should be aligned with it. |

Moderator’s summary: 7 companies provided feedback. 6 companies thought they should be postponed to Rel-18 whereas one company thought it should be supported in Rel-17.

Proposal 8: Postpone to Rel-18 the discussion of whether/how the SN can pull RA report from the MN, i.e. indicating that RA procedure happens at SCG toward the MN.

In addition, one company proposed adding UE AP ID(s) into the Access and Mobility Indication message. But the moderator noticed that this topic was already discussed in last meeting and the reference ID was already added into the SON BL CRs (R3-220056 and R3-220058), e.g.:

#### 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | EACH | ignore |
| >> Successful HO Report Container | O |  | OCTET STRING | FFS on the definition | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. FFS |

Therefore this proposal is invalid and should not be discussed.

- Conclusion, recommendations [if needed]

- Reference

[1] R3-220379; (TP for SON BL CR for TS 38.473, TS 36.423): Left overs on RACH Optimization Enhancements; Huawei.

[2] R3-220529; Discussion on Rel-16 leftover issues for PRACH coordination; CATT, CMCC, China Telecom.

[3] R3-220530; (TP on SON for 38.473) TP on PRACH coordination for F1AP; CATT, CMCC, China Telecom.

[4] R3-220531; (TP on SON for 36.423) TP on PRACH coordination for X2AP; CATT, CMCC, China Telecom.

[5] R3-220579; (TP for SON BL CR for TS 38.473 and for TS38.423) RACH conflict resolution and reporting procedures; Ericsson.

[6] R3-220630; (TP for TS 38.473) Discussion on Leftover Issues of RACH Optimization; Nokia, Nokia Shanghai Bell.