3GPP TSG-RAN WG3 #114bis-e                                                                 R3-221147

17-26 Jan 2022

Online

Agenda Item:	30.3

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: #18_UEPowerSaving

Document for:	Approval

# Introduction

CB: # 18_UEPowerSaving

- Check the LS from RAN2 and focus on WID scope

- Identify RAN3 related issues and discuss the corresponding solutions

- Capture agreements and open issues

- Provide stage2/3 BL CRs if agreeable, split work

(ZTE - moderator)

Summary of offline disc R3-221010

# For the Chairman’s Notes

RAN3 BL CR allocated by rapporteur as below. (whether 38.470,38.410  will be impacted will be checked in second round.)

| Specification | Baseline CR rapporteurs |
| --- | --- |
| TS 38.413 | Nokia |
| TS 38.423 | Huawei |
| TS 38.473 | ZTE |
| TS 38.300 | Ericsson |

the following agreements achieved:

- proposal 1: The CN provides subgroup ID included in subgroup information.

- proposal 2: Include UE paging subgroup information into the NG Paging message for CN paging.

- proposal 3: Introduce UE subgroup information into NGAP Core Network Assistance Information.

- proposal 4: Introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE.

- proposal 5: Adopt the terminology "PEIPS Assistance Information" as the IE name to align with SA2 terminology, containing a CN subgroup ID.

- proposal 6: The CN assigned subgroup ID is encoded as 0..7. (note: further checking maybe needed)

- proposal 7: Add some clarification on stage 2 behaviour related PEIPS assistance information for RRC INACTIVE for TS38.300.

the following WA achieved:

Proposal 8: WA: NG-RAN node can know the total number of subgroups supported by CN via OAM, there is no RAN3 signaling impact.

- to be continued:

- RAN3 needs to wait RAN'2 further progress for Xn impact.

- It is FFS for F1 signaling impact of UEID-based subgrouping capability.

# Discussion- Second round

Question 1:  Companies are invited to provide their views on  whether TS38.470 will be impacted. if yes, please provide comments in the following table.

| Companies | Yes/No | Comments |
| --- | --- | --- |

Question 2:  Companies are invited to provide their views on  whether TS38.410 will be impacted. if yes, please provide comments in the following table.

| Companies | Yes/No | Comments |
| --- | --- | --- |

In this meeting , RAN3 receives the LS[17] from RAN2, and RAN2 request RAN3 to provide further information on the following issue once concluded: "Signalling between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE, e.g., to convey Subgroup ID, UE capability, etc". Therefore, RAN3 needs to reply to RAN2 about the progress of RAN3.

Question 3:  Does company think there is any addition issues needed to be included in LS?

| Companies | Yes/No | Comments |
| --- | --- | --- |

BL CR checking:

Provide revised version for BL CR for checking, based on the agreements achieved in first round discussion.

BL CR to TS38.300  Support for UE Power Saving Enhancements in R3-220286 rev in xxxx  [Ericsson]

BL CR to TS38.413  Support for UE Power Saving Enhancements  in  R3-220366 rev in xxxx  [Nokia]

BL CR to TS38.473  Support for UE Power Saving Enhancements  in R3-220899 rev in xxxx  [ZTE]

38.470,38.410 CR (if needed)

Reply LS to RAN2.

# Discussion-First round

## Work plan

In RAN#86 meeting, the RAN2-led WI of UE power saving enhancements in NR was agreed [1], and then in RAN#88 meeting, the WID was revised [2]. Furthermore, the revised WID “UE Power Saving Enhancements” [3] was approved in RAN#93-e, and RAN allocated TU(s) to RAN3. RAN3 shall discuss the specification impact to support the UE power saving.

Regarding the related work plan in R3-220767 [4] provided by ZTE and MediaTek, please provide comments in the following table, if any,

| Companies | Comments |
| --- | --- |
| Nokia | OK. |
| Samsung | OK |
| Ericsson | OK |
| CATT | OK |
| Huawei | OK |

After the coordination with companies, the list of RAN3 BL CR moderators is proposed as below:

| Specification | Baseline CR rapporteurs |
| --- | --- |
| TS 38.413 | Nokia |
| TS 38.423 | Huawei |
| TS 38.473 | ZTE |
| TS 38.300 | Ericsson |

Please provide comments for split of BL CRs  in the following table, if any.

| Companies | Comments |
| --- | --- |
| Nokia | OK. |
| Samsung | OK. In addition, it can be foreseen that TS38.470 will be impacted to describe the PEI related operation for Paging function, e.g., section 5.2.5. If possible, Samsung would like to act as Rapp. for this BL CR. |
| Ericsson | OK |
| CATT | OK. I consider whether TS38.410 will also be impacted. The current 38.410 only include the paging area. We are ok to discuss it. If possible, CATT would like to contribute the BL CR |
| Qualcomm | OK |
| Huawei | OK |

Moderator’s summary:

For chairman notes:

RAN3 BL CR allocated by rapporteur as below. (whether 38.470,38.410  will be impacted will be checked in second round.)

| Specification | Baseline CR rapporteurs |
| --- | --- |
| TS 38.413 | Nokia |
| TS 38.423 | Huawei |
| TS 38.473 | ZTE |
| TS 38.300 | Ericsson |

## CN-assigned subgrouping

In the past RAN3 meeting,  RAN3 has discussed some ePowSav related issues triggered by RAN2's LS. the later discussion can be found in Summary of Offline Discussion [5] in previous meeting. we have seen that most companies are mostly aligned on the RAN3 impacts during the offline discussion, and agreed to take the following proposals in [5] into consideration. In addition, we also have seen that most of the contributions submitted at this meeting reflect these proposals.

Proposal 1: The CN provides subgroup ID included in subgroup information.

Proposal 2:  Include UE paging subgroup information into the NG Paging message for CN paging.

Proposal 3: Introduce UE subgroup information into NGAP Core Network Assistance Information.

Proposal 4: Introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE.

Question 1: Does Company agree to turn the above proposals(1-4) into agreements?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes |  |
| Qualcomm | Yes |  |
| Huawei | Yes |  |

Moderator’s summary:

All companies agree to turn the above proposals(1-4) into agreements.

For chairman notes:

To agree the following proposals.

- Proposal 1: The CN provides subgroup ID included in subgroup information.

- Proposal 2: Include UE paging subgroup information into the NG Paging message for CN paging.

- Proposal 3: Introduce UE subgroup information into NGAP Core Network Assistance Information.

- Proposal 4:  Introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE.

In Q1, the proposed paging subgroup information includes paging subgroup ID provided by CN. In the contributions of companies, there are different IE names for such paging subgroup information in related message, e.,g "PEI Assistance Information", "PEIPS(PEI with Paging Subgrouping) Assistance Information", "Paging Subgroup Information", or "Subgroup ID". Moderator found that more companies(3) adopted the "PEIPS Assistance Information" terminology, so Moderator proposes to use  "PEIPS Assistance Information" as the IE name for the corresponding paging subgroup information.

Question 2: In order to align the IE name, does Company agree to adopt the terminology "PEIPS Assistance Information" as the IE name for the corresponding paging subgroup information？

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia | Yes + comments | We support PEIPS Assistance Information to align with SA2 terminology. However, in our view this assistance information contains today a “CN subgroup ID” but could potentially be extended with other IE. |
| Samsung | Yes | Share view with Nok. This IE should be extendible. |
| Ericsson | Yes | Align with SA2’s terminology and definition |
| CATT | Yes | Agree with Nok. |
| Qualcomm | Yes |  |
| Huawei | Yes |  |

Moderator’s summary:

All companies agree to adopt the terminology "PEIPS Assistance Information" as the IE name for the corresponding paging subgroup information.

For chairman notes:

Proposal 5:  Adopt the terminology "PEIPS Assistance Information" as the IE name to align with SA2 terminology, containing a CN subgroup ID.

In [6], the paging subgroup ID provided by CN is encoded as 0..7, and in other contributions[7-11], the paging subgroup ID is encoded as 1..8.

Question 3: Companies are invited to provide their views on how to encode the paging subgroup ID provided by CN ？

| Companies | encoded as 0..7 or 1..8 | Comments |
| --- | --- | --- |
| ZTE | 0..7 | In the RAN2's 38.304 running CR, the CN assigned subgroup ID is defined between 0 to 7 . A UE supporting CN assigned subgrouping in RRC_IDLE or RRC_INACTIVE state can be assigned a subgroup ID (between 0 to 7 by AMF through NAS signalling. |
| Nokia | Both | No strong view. |
| Samsung | Both | No strong view. |
| Ericsson | both | As long as it’s up to 8 |
| CATT | Both | No strong view. |
| Qualcomm | Both | But can later check based on other specifications – better to align to avoid future confusion |
| Huawei | 0..7 | It seems better to align with RAN2 (but it seems only RAN2 Stage 2 has this number). |

Moderator’s summary:

All companies can accept to encode the CN assigned subgroup ID  as 0..7, and one company suggest to later check based on other specifications.

For chairman notes:

Proposal 6:  The CN assigned subgroup ID is encoded as 0..7. (note: further checking maybe needed)

RAN2 had not reached a conclusion on whether UEs only monitor PEI in the last used cell until the last meeting. Some companies think such a restriction would eliminate the power saving benefits when there are many UEs moving from last served cell. If PEI is not restricted in the last used cell, Xn paging impact on subgrouping exists. [12][13] propose to introduce Paging Subgroup Information into Xn PAGING message, with the editor's note that it needs to be confirmed in RAN2.  However, other companies propose that RAN3 needs to wait RAN'2 further progress.

Question 4: Companies are invited to provide their views on whether RAN3 needs to wait RAN'2 further progress for Xn impact?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | Limiting the PEI only used in last used cell will eliminate the power saving benefits. so, We think RAN3 can consider XnAP CR[12] as the WA, and ask RAN2 whether it is feasibility in the reply LS. |
| Nokia | Yes | Need to wait RAN2 decision. |
| Samsung | Yes | Wait for RAN2 decision. In case that RAN2 decides to go for limiting PEI in last serving cell, RAN3 can further discuss the potential impact. |
| Ericsson | Yes | Same as it happened in the past when 3GPP discussed Rel-15 WUS limitation for last visited cell ID, we must wait for RAN2’s conclusions |
| CATT | Yes | We need to discuss the XnAP CR after RAN2 has agreement. |
| Qualcomm | Yes | For sure we should wait for RAN2, but in the absence of any decision, then by default it should not be restricted |
| Huawei | Can be as WA till RAN2 final conclusion. | Agree with ZTE that the PEI should not be limited to the last serving cell. And we agree with Qualcomm by default there is no limitation. Also we agree with RAN3 to consider the XnAP CR as WA, until final RAN2 final conclusion. |

Moderator’s summary:

majority of views support to wait RAN2 decision, while two companies prefer to  consider the XnAP CR as WA, until final RAN2 conclusion.   Moderator think we can follow the majority.

For chairman notes:

RAN3 needs to wait RAN'2 further progress for Xn impact.

In [14], it is proposed to add some clarification on stage 2 behaviour related to the PEIPS assistance data sent from 5G CN to NG-RAN for RRC INACTIVE configuration with Paging Early Indication for Paging with Subgrouping.

Question 5: Does Company agree to add some clarification on stage 2 behaviour related PEIPS assistance information for RRC INACTIVE for TS38.300?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | This CR is fine for us. |
| Nokia | OK |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes |  |
| Qualcomm | Yes |  |
| Huawei | OK |  |

Moderator’s summary:

all company agree to add some clarification on stage 2 behaviour related PEIPS assistance information for RRC INACTIVE for TS38.300.

For chairman notes:

Proposal 7: add some clarification on stage 2 behaviour related PEIPS assistance information for RRC INACTIVE for TS38.300.

## UEID-based subgrouping

The UE Radio Capability for Paging information is present in XnAP RAN paging message, while such information is not presented in the current F1 paging message, [6][11][10] propose to introduce UEID-based subgrouping support indication IE into F1 PAGING message, to indicate the gNB-DU whether the UE supports the UE-ID based subgrouping or not. However, other companies think the UE's paging Subgrouping Support capability can be included in the UE Radio Capability for Paging IE, the DU can be aware of UE Radio Capability for Paging via UE context setup/modification procedure, so there is no F1 signaling impact.

Question 6: Companies are invited to provide their views on whether F1 signaling is needed to signal the UEID-based subgrouping support indication of the UE from CU to DU?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes, but | We think limiting the PEI only used in last used cell will eliminate the power saving benefits. If PEI is not restricted in the last used cell, CU may need to signal the UEID-based subgrouping support indication of the UE to other DUs, RAN3 can consider it as the WA, and ask RAN2 whether it is feasibility in the reply LS. |
| Nokia | depends | Agreed that the support of the UE of the UE-ID based subgrouping needs to be sent from CU to DU, however it is FFS if it can be sent as UEID Support or whether the UE Radio Capability for Paging is sent. |
| Samsung | Yes | The intention of such F1AP enhancement is to let gNB-DU know the capability of supporting UEID-based subgrouping. However, the existing F1AP PAGING message does not contain any capability information. Thus, this enhancement is needed. |
| Ericsson | No | The DU can understand the UE-ID support via UE context setup/modification procedure. We can keep it as FFS |
| CATT | Tend to No | Agree to introduce the capability of UEID based subgrouping from CU to DU. Slight prefer “the DU can be aware of UE Radio Capability for Paging via UE context setup/modification procedure”. Because this is about capability. It does not need to be included in each paging message. |
| Qualcomm | Depends | Agree with Nokia: UE support of the UE-ID based subgrouping needs to be known at the DU, however it is FFS if it can be sent as UEID Support or whether the UE Radio Capability for Paging is sent |
| Huawei | Agree | Agree with Samsung. And we think the new UE-ID based subgrouping support indication is sufficient, which is just for UE-ID based PEI in paging message. If we consider the UE Radio Capability for Paging, it is too heavy and no clear benefits compared with the simple support indication. |

Moderator’s summary:

(3/7) of companies support to introduce UEID support indication in F1 paging (one company think If PEI is not restricted in the last used cell, CU may need to signal it to other DUs);  (3/7) of companies state to  keep it as FFS; (2/7) of companies say no F1 impact.

Anyway, all companies agree  that UEID support needs to be known at the DU, however it is FFS if it can be sent as UEID Support or whether the UE Radio Capability for Paging is sent, or already supported via UE context setup/modification procedure. we can keep it as FFS.

For chairman notes:

It is FFS for F1 signaling impact of UEID-based subgrouping capability.

RAN2 agreed both UE ID-based and CN-assigned  subgrouping can be supported simultaneously in a cell, and no overlapping rule is applied, which means CN-assigned subgroups shall not be remapped to UEID-based subgroup. CN-based subgrouping will have priority in utilizing PEI indication capacity, which means UEID-based subgrouping can only utilize the rest of capacity.  According to RAN2's agreement, a new parameter Nsg-UEID is introduce to SIB to indicate the total number of UE ID-based subgroups supported by the cell.  However, the gNB needs be aware of the total number of subgroups supported by CN, then it can configure the Nsg-UEID per cell.

[6] thinks there is no RAN3 impact on co-exist of CN-assigned and UEID-based subgrouping at this stage, e.g, The gNB or DU can know the total number of CN-assigned subgroups via OAM, and [15][16] assume there is RAN3 impact.

[16] proposes that AMF can send in the NG Setup Response message the number of CN subgroups used by AMF. [15] thinks there are  two options, one is  CN is notified with the max number of subgroups of RAN side; the other is RAN is notified with the max number of subgroups of CN side.

Question 7: Companies are invited to provide their views on whether there is any RAN3 impact for  UE ID-based and CN-assigned  subgrouping supported simultaneously in a cell?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | If the total number of groups supported by CN is static, e.g, RAN2 assumes that all the cells within the registration area supports the same number of CN assigned subgroups, there is no need for CN to signal such number to gNB. In addition, RAN2 has achieved the following agreement “The total number of CN-assigned subgroups that is used is not fixed and can be configured up to 8 (e.g. by OAM). No impact on signalling is assumed.” So, The gNB or DU can know the total number of CN-assigned subgroups via OAM, No RAN3 impact here at this stage. |
| Nokia | Yes | The gNBs need to know the number of CN subgroups used by AMF. This can be signaled from AMF to gNB in NG setup response. Also, tdoc [16] explains that some form of re-mapping is inevitable because if for example there are 6 CN subgroups sued by AMF but the cell has only 4 L1 bits used for CN subgrouping feature, then there needs remapping from 6 values onto 4 values. Some rules need to be set like e.g. CN subgroups beyond 4 maps on the last 4th L1 bit. |
| Samsung | Yes | The mismatch between CN-based total number and UEID-based total number is inevitable due to the following reasons: - Different OAMs configures different total number: the total number of CN-based subgrouping is configured by OAM of CN, while the total number of UEID-based subgrouping is configured by OAM of gNB-DU. In the real deployment, OAM of CN may be different from that of gNB-DU, so, it is difficult to ensure the configuration of total number is aligned between CN and RAN. - UEID-based total number is cell-specific, while CN-based total number is registration area specific. Thus, a solution to resolve such mismatch is needed. As we proposed, one solution is to inform the CN-based total number to RAN, while another solution is to inform the UEID-based total number to CN. We are open for the discussion. |
| Ericsson | No | Agree with ZTE, RAN2 have agreed that up to the maximum 8 groups can be configured by OAM in order to be used in the deployment. Such number would/should not change at all, otherwise there might be problems during a change. On Nokia’s statement: "The gNBs need to know the number of CN subgroups used by AMF. This can be signaled from AMF to gNB in NG setup response." We did not support this for Group WUS in Rel-16, and at that time we had 20 codepoints (VS 8 for PEIPS). |
| CATT | No | GNB knows the total number of CN-assigned subgroups via OAM. GNB-DU obtains the total number of UE ID based subgroup ID also via OAM. Therefore, there is no such thing as the number of subgroup ID reserved for CN being smaller than the subgroup ID that CN can allocate i.e., no remapping is needed. If the CN based subgroup ID is 6, the gNB cannot broadcast UE based subgroup ID more than 2. |
| Qualcomm | No | This feature requires anyway coordination of the number of subgroups based on configuration – we will not have random numbers in different AMFs and RAN nodes in a TA (nor there is any need for it). See ZTE and CATT’s response. |
| Huawei | see comments | On the total number of groups supported by CN, we agree with ZTE, that there is no RAN3 signaling impact. Also we notice that RAN2 is discussing the RAN sharing case, where the CN controlled subgrouping is supported/allowed by the gNB for only one (set of) PLMN(s)/CN(s) in the shared cell, which might have RAN3 impact. So we may suggest further discussing this based on RAN2 latest agreement: - RAN2 assumes that there is no particular impact to Uu signalling to support RAN sharing. It is further assumed that Core Networks must have consistent policy if subgrouping is used by multiple Core Networks. |

Moderator’s summary:

On the total number of groups supported by CN, 5 companies state there is no RAN3 impact, while 2 companies disagree. In addition, one company suggest to discuss RAN3 impact for the RAN sharing case based on RAN2 agreement.  Moderator think the RAN Sharing scenario need further clarification.

Since there is a RAN2 agreement, "The total number of CN-assigned subgroups that is used is not fixed and can be configured up to 8 (e.g. by OAM).", Moderator think  we can follow the majority of views, take the following as WA, NG-RAN node can know the total number of subgroups supported by CN via OAM, there is no RAN3 signaling impact.

For chairman notes:

Proposal 8: WA: NG-RAN node can know the total number of subgroups supported by CN via OAM, there is no RAN3 signaling impact.

Furthermore, [16] thinks "CN nodes are best placed to evaluate the percentage of UEs supporting CN subgrouping because this is a NAS capability. For example, AMF could easily build statistics on percentage of UEs supporting CN subgrouping per TA by receiving in every Register Update the RA of the UE and its NAS capability.Similarly, the gNB is better placed to have statistics on average number of UEs supporting the UE-ID based method on a per cell basis. With these two pieces of information the gNB can set appropriate N1 and N2 values in every cell". (N can be split between N1 bits used for CN subgroup notification and N2 bits used UE-ID based notification with N1 + N2 = N).

so, [16] puts foward the following proposal :

The AMF may send to the gNB in the NGAP SETUP RESPONSE message or the AMF CONFIGURATION UPDATE message for assistance information on a per TA basis the average fraction of UEs supporting CN subgrouping, either as a percentage or as a number of equivalent L1 bits out of 8 (e. g. 6/8).

Question 8: Does Company agree with the above proposal?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | Not needed, gNB can configure the number of UEID-based subgroups by itself. How to remap the subgroup to L1 bits is out of RAN3 scope. Anyway, if needed, it shall be discussed by RAN2/SA2 firstly. |
| Nokia | Yes | In order to set a correct number of bits for UE-ID based method which is broadcast (Nsg-UEID), it is useful if the gNB can estimate the percentage of UEs supporting CN subgrouping in a cell compared to UEs supporting UE-ID based method. Given that the first one is a NAS capability sent by UE to AMF, only the AMF has this knowledge. The AMF could send to gNB the average percentage of UEs supporting CN subgrouping per TA as assistance information for NG-RAN. |
| Samsung | Wait for Q7 | This is the following-up step after making decision of Q7. So, we would like to see the conclusion of Q7 first, and then discuss the solution of Q7. |
| Ericsson | No | Agree with ZTE. Also, we do not send the number/percentage of UEs in cell-specific signalling messages such as during NG Setup. This will create very frequent signalling when a UE performs, e.g., mobility or a new UE arrives in the NG-RAN; the percentage/number of UEs has to be dynamically updated over NG. This is not a good design. |
| CATT | No | GNB can set the N2 well based on the total number of UE ID subgroup ID and CN based subgroup ID via the same or different OAM. N2=N-N1=8-6, if gNB supports 4 UE based subgroup ID, it only can broadcast 2. |
| Qualcomm | No | This is presumably to aid selecting number of groups etc. But this is not a continuously adaptive quantity. Operators can configure this for their known UE population, and adjust occasionally based on changes, stats etc. |
| Huawei | No | On this particular issue, agree with “No” camp. |

Moderator’s summary:

Moderator think there is no consensus. Majority of views do not support this.

# Conclusion, Recommendations [if needed]

If needed

# References

- RP-193239, New WID: UE Power Saving Enhancements, MediaTek Inc., RAN#86 meeting, Dec. 2019

- RP-200938, Revised WID: UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88 meeting, July 2020

- RP-212630, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., ZTE

- R3-220767,	Work plan for UE power saving enhancements WI, ZTE,MediaTek Inc

- R3-215812, Summary of Offline Discussion on CB: # 9_UEPowerSaving, ZTE

- R3-220897, Discussion on UE paging subgroup , ZTE Corporation, MediaTek

- R3-220236,	Support of paging subgrouping for UE power saving (NGAP), CATT

- R3-220308,	Addition of PEIPS Assistance Information, Ericsson, Qualcomm Incorporated

- R3-220366,	Support of UE Power Saving Enhancements, Nokia, Nokia Shanghai Bell

- R3-220565, CR to support UE power saving over F1, Samsung

- R3-220675, (TP for TS 38.413 and TS 38.473) Supporting UE Power Saving Enhancements, Huawei

- R3-220676, Supporting UE Power Saving Enhancement , Huawei

- R3-220900	CR to TS38.423 for UE paging subgroup, ZTE Corporation, MediaTek

- R3-220286	Addition of PEIPS Assistance Information, Qualcomm Incorporated, Ericsson

- R3-220564	Discussion on UE power saving, Samsung

- R3-220365	Support of UE Power Saving Enhancements , Nokia, Nokia Shanghai Bell

- R3-220102        LS on paging subgrouping and PEI, RAN2