3GPP TSG-RAN WG3 #114bis-e	R3-22xxxx

17-26 Jan 2022

Agenda Item:	8.1

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on CB: # 3_IABKeyDerivation

Document for:	Approval

# Introduction

CB: # 3_IABKeyDerivation

- IAB-donor-CU-CP derives and forwards KIAB to the IAB-donor-CU-UP, the KIAB is to be used between the IAB-node and the IAB-donor-CU-UP? Introduce a new IAB specific E1 procedure to enable the IAB-donor-CU-CP sending the KIAB to the IAB-donor-CU-UP? Introduce a list of IP address pair for IAB-donor-CU-UP and IAB-DU, with each pair for one KIAB ? Huawei, Lenovo, Motorola Mobility, Samsung

- Capture agreements and provide CRs if agreeable

(HW - moderator)

Summary of offline disc R3-220990

# For the Chairman’s Notes

Propose the following:

For chairlady to copy

Detailed discussions

# Discussion [if needed]

According to the incoming LS [1], SA3 made clear answer that, to support the IPsec protection for F1-U interface, IAB-donor-CU-CP should provide the PSK (KIAB) to IAB-donor-CU-UP via E1 interface when dynamic PSK for IKEv2 PSK authentication is used.

The draft CR [3][4] tries to propose stage 3 signaling design for this KIAB transfer from CP to UP, which mainly include the following:

- A new dedicated UE associated signaling/procedure “IAB PSK Notification” ;

- A list of IP address pair with each pair being associated with a PSK

- The existing AP ID is used to represent the IAB-MT

Companies are invited to provide comments on the details for stage 2&3 CR [3][4], e.g. whether a dedicated procedure is needed, whether a list of IP address pair is needed, etc.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | We think a dedicated signaling would be clearer, and a list of IP address pair is needed, according to SA3 spec that the IP addresses of IAB-donor-CU and IAB-DU are used as parameters for deriving KIAB, thus when the IAB-donor-CU-UP receives the KIAB, it shall identify the mapping between KIAB and IP address pair. The signalling should be UE associated since it is associated with an IAB-MT. We also think both stage 2&3 CRs are needed, and open to further refinements if any. |
| Ericsson | OK, but we have some issues | We prefer a new IAB-specific procedure. Can the proponents clarify why does IAB-Donor-CU-UP IP Address have to be signalled? Does it not know it’s own IP address? How does the CU-CP know it, especially if no Bearer Context is established, for IAB-MTs? |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-220124, Reply LS on support of Pre-shared key derivation for IAB-donor-CU-UP (SA3)	LS in

- R3-220797, Further discussion on Security handling for CP-UP separation of IAB-donor (Huawei)	discussion

- R3-220798, CR to 38.463 on support of Pre-shared key derivation for IAB-donor-CU-UP (Huawei, Lenovo, Motorola Mobility, Samsung)	CR0673r, TS 38.463 v16.8.0, Rel-16, Cat. B

- R3-220799, CR to 38.460 on support of Pre-shared key derivation for IAB-donor-CU-UP (Huawei, Lenovo, Motorola Mobility, Samsung)	CR0053r, TS 38.460 v16.4.0, Rel-16, Cat. B