3GPP TSG-RAN WG3 #114bis-e														R3-220793

17-26 Jan 2022

Agenda item:	22.2.5.

Source:	Samsung, Qualcomm Incorporated, ZTE, Huawei, Lenovo, Nokia, Nokia Shanghai Bell

Title:				(TP for BL CR for 38.473) Multicast Group Paging procedure in F1

Document for:	Discussions & Approval

# Introduction

In the last RAN3 #114 e-meeting, we have deeply discussed the Multicast Group Paging (refer to R3-216212), then we have achieved the following progress.

RAN3 shall support Option 2 (i.e., Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s)). It is proposed to include “UE Identity Index value” IE (i.e., 5G-S-TMSI mod 1024 defined in 9.3.3.23 in TS38.413) in the Multicast group paging message. It is proposed to include “Paging DRX” IE in the Multicast group paging message. FFS: Paging DRX is UE specific or Session specific. FFS: paging area shall include per list of UEs to be paged. It is proposed to include “MBS Service Area” IE in the Multicast group paging message, the detail is FFS. The IE“UE RAN Paging Identity” is not included in RAN Multicast Group Paging message. The set of TPs are technically correct

During the offline and online discussion, the set of TPs are technically correct. However, the corresponding CRs as below are noted.

R3-214753 rev [in R3-215972] – agreed 38.413 ZTE R3-215271 rev [in R3-215973] – agreed 38.423 Lenovo R3-215514 rev [in R3-215974] – agreed 38.473 Samsung R3-215296 rev [in R3-215975] – agreed 38.470 LG R3-214755 rev in R3-215976 rev [in R3-216213] – agreed LS to RAN2, CC:SA2

In the subsequent SA2 #148 e-meeting, the MBS counting issue was not discussed in SA2. So that, in this RAN3 e-meeting, the corresponding TP to TS38.473 is resubmitted for agreement. The DRX value and the Paging priority are marked as FFS.

# 2	Text Proposal

/*--------------------------------Start of the First Change----------------------------*/

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |

/*--------------------------------Omit unchanged part---------------------------*/

## 8.x	NR MBS Procedures

- 8.x.y	Multicast Group Paging

- 8.x.y.1	General

The purpose of the Multicast Group Paging procedure is used to provide the paging information to enable the gNB-DU to multicast group page UEs which have joined an MBS Session about its activation. The procedure uses non-UE associated signalling.

Editor’s Note:	 procedure text is FFS

- 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: Multicast Group Paging

The gNB-CU initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the gNB-DU.

At the reception of the MULTICAST GROUP PAGING message, the gNB-DU shall perform paging of the MBS Session identified by the MBS Session ID IE.

If the Paging DRX IE is included in the MULTICAST GROUP PAGING message gNB-DU shall use it according to TS 38.304 [24].

- 8.x.y.3	Abnormal Conditions

Void.

/*--------------------------------Omit unchanged part---------------------------*/

#### 9.2.13.x	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to multicast group page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | FFS |  | YES | reject |
| UE Identity List for Paging |  | 0..1 |  |  | YES | reject |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >> UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| >>Paging DRX | M |  | 9.3.1.40 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for paging. Value is FFS. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

/*--------------------------------End of the Changes-----------------------------*/