3GPP TSG-RAN WG3 #114bis-e	        R3-221159

Jan. 17~26, 2022

Online

Agenda item:	23.3 (Specification of Control Plane procedures)

Source:	Samsung (moderator)

Title:	Summary of offline discussion on CB # SLRelay2_ControlPlane

Document for:	Approval

Introduction

CB: # SLRelay2_ControlPlane - Architecture related open issues, i.e. termination points as well as related functionalities, local ID allocation. - Procedure related open issues, i.e. remote/relay UE identification during initial access procedure, baseline flow chart for RRC establishment/resume/reestablishment. - F1AP signalling design, i.e. F1AP signalling to configure remote UE, Uu/PC5 RLC channel configuration, mapping configuration. - Capture agreements and open issues, provide CRs/TPs, if agreeable (Samsung - moderator) Summary of offline disc R3-221082

This e-mail discussion is divided into two phases:

- Phase I: View collection

Deadline: Wednesday, Jan. 19th, 2022, 11:00 UTC.

- Phase II:

Deadline: TBD

For the Chairman’s Notes(Phase-II)

The following proposals can be captured:

  - Keep the original WA, i.e., the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE ID.

  - The gNB-DU can include the local ID of remote UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

  - FFS: the local ID of remote UE can be notified to gNB-DU before initial access of remote UE

  - The control plane traffic type is indicated via SRB ID

  - Endorse R3-221163 as BL CR to TS38.401

  - Agree TP of TS38.473 in R3-221161

  - Agree LS to RAN2 in R3-221340

To be continue…:

  - Inclusion of Relay UE ID in INITIAL UL RRC MESSAGE

  - Notification of local ID of remote UE before initial access

  - Lower layer configuration for remote UE in INITIAL UL RRC MESSAGE TRANSFER message and indication of rejection to remote UE

  - Impact caused by service continuity

Discussions (Phase-II)

According to online discussion, the focuses of Phase 2 discussion are listed as below:

For 2nd Round: - Address the above WAs and FFSes to seek the potential progress on - The gNB-DU can include the local ID of remote UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE - WA: the local ID of remote UE can be notified to gNB-DU before initial access of remote - Mapping configuration at relay UE (LCID issue in Q8) - Check stage2/3 BL CRs

- Identification of remote UE and relay UE

During the online discussion, we reach the followings (for the sake of discussion, the moderator adds bullet number):

- WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE ID.

- The gNB-DU can include the local ID of remote UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

- WA: the local ID of remote UE can be notified to gNB-DU before initial access of remote UE

Q1: Can companies agree to change the above three bullets as agreement?  If better rewording is foreseen, please spell it out.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Follow majority view in phase-I |
| ZTE | See comments | For a, as we discussed during the online session, it is suggested to firstly double check with RAN2 if the local remote UE ID unique per relay UE ID exists before we turn it into agreement. For b, we think it can be an agreement. Based on the local ID of remote UE contained in the INITIAL UL RRC message, CU can associate the remote UE’s F1-C connection with the local remote UE ID which is previously allocated by CU based on the relay UE’s request. Otherwise, CU only know that a new UE is trying to set up RRC connection. CU has no idea that it is related with a remote UE and the corresponding local remote UE ID. On the other hand, the CU can not identify the remote UE based on the first RRC message (e.g. RRCSetupRequest) contained in the INITIAL UL RRC message since it does not contain the ID of remote UE. The local remote UE ID is allocated by CU after the relay UE receive the RRCSetupRequest message and then request the local ID for remote UE. For c, this WA is not necessary. DU may get the local ID of remote UE and identify that it is the first SRB0 RRC message via the Uu adaptation layer subheader of the RRC message forwarded by relay UE. Then DU may send the initial UL RRC message transfer. It is meaningless for the CU to send the local ID of remote UE to DU before initial access. |
| CATT | Ok for two WAs | I understand that the first and second bullet are aim to let the CU knows which relay UE is the incoming remote UE connected with. I agree with the first bullet. But I have some concern for the second bullet. Even gNB-DU does not include the local ID of remote UE in the INITIAL UL RRC MESSAGE. CU is able to identify the remote UE via S-TMSI which is contained in RRC message. RRCSetupRequest-IEs ::= SEQUENCE { ue-Identity InitialUE-Identity, InitialUE-Identity ::= CHOICE { ng-5G-S-TMSI-Part1 BIT STRING (SIZE (39)), CU also can relate local ID of remote UE with S-TMSI of remote UE based on the following agreement from RAN2. Relay UE uses SUI message to provide remote UE information (i.e. 5G-S-TMSI/I-RNTI) to network. When CU allocates local ID, it uses (S-TMSI of relay UE, S-TMSI of remote UE) local ID of remote UE. When remote UE access, CU identifies relay UE via the first WA, and identifies remote UE via S-TMSI of remote UE. In other word, local ID is used on the SRAP which is not even located at CU. Why CU should identify remote UE via local ID rather than S-TMSI of remote UE? I am not against the second bullet. I am trying to understand some technical issues. It is ok for me if majority companies agree with that. |
| Lenovo, Motorola Mobility |  | For wa1, second thought on “with local UE ID unique per relay UE”, does it mean the local UE ID is unique within one relay UE? Not sure how does it work if two remote UEs associated with one relay UE have the same local UE ID… Isn’t the problem about two remote UEs associated with different relay UEs have the same local UE ID? For wa2, we have some sympathy with what ZTE explained, although our intuitive thinking is yes. Maybe we can convert it to agreement if something is identified broken without it. |
| Huawei | Yes | a. The key point is that since the range is so small, it cannot be unique in a gNB. So we need an additional identifier to make it unique. b. yes. c. It is needed if we use Uu RLC channel configuration which is used for dedicated configuration. The usage of default configuration is FFS. |
| E/// | Yes for the first two | Have concern on ZTE’s comment to a. RAN2 has discussed the size of local ID of remote UE, and decided to use 8bits. So the understanding is that gNB should guarantee the uniqueness of local ID for remote UE. So it should be one relay UE with unique remote UE to work properly. We don’t see any open point to check with RAN2. For c, share the same view as ZTE. |
| Qualcomm | a – Yes b- Yes c – Maybe | a – Even though RAN2 agreed local ID of remote UE to be 8 bits, only 2^8 = 256 remote UEs can be uniquely identified if it has to be unique per gNB. If it is relay UE specific, there can be a lot more remote UEs uniquely identified by the combination of (relay UE F1AP ID, local remote UE ID). Hence, we think it is useful to include relay UE F1AP ID in INITIAL UL RRC MESSAGE TRANSFER. If no consensus, we think RAN3 can perhaps decide whether it is relay UE specific or gNB specific, instead of checking with RAN2. b- Yes c – Agree that gNB-DU can know the local ID of remote UE from the Uu SRAP subheader. But, we still think it might be better for gNB-CU to inform gNB-DU explicitly before initial access. |

---------

- 5/7 yes, 1 needs to check RAN2, and 1 is unclear

- 5/7 yes, 2 did not indicate

- 2 yes, others have concern

The moderator gives the following proposal:

  - Keep the original WA, i.e., the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE ID.

  - The gNB-DU can include the local ID of remote UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

  - FFS: the local ID of remote UE can be notified to gNB-DU before initial access of remote UE

---------

- FFS for control plane traffic type

The online discussion reach the following agreements:

The additional information, e.g., QoS profile info (e.g., 5QI, priority, PDB, PER, etc), RLC mode, and control plane traffic type (FFS: SRB ID vs. priority), can be included when configuring PC5/Uu RLC CH from gNB-CU to gNB-DU.

The FFS part for control plane traffic type, which has two options: SRB ID vs. Priority. The moderator guess the option of “priority” come from IAB design when configuring the BH RLC CHs for F1-C traffic. However, sidelink relay is different since the control plane signaling here is referring to SRBs. It seems SRB ID is a straightforward choice.

Q2: Can companies agree to use SRB ID as the indication of control plane traffic type?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| ZTE | Yes with comments | For the IAB, both the SRB ID and priority have been candidates for the QoS indication of CP traffic for BH RLC CHs and finally priority win out. Actually, both option works. We have no strong view. |
| CATT | YES | Follow the SRB design. |
| Lenovo, Motorola Mobility | Yes |  |
| Huawei | Yes | For simplicity, we can follow the IAB design.. |
| E/// | Yes |  |
| Qualcomm | Yes |  |

---------

All companies agree to use SRB ID.

The moderator gives the following proposal:

  - The control plane traffic type is indicated via SRB ID

---------

- Lower layer configuration for remote UE in INITIAL UL RRC MESSAGE TRANSFER message

The following agreement was achieved during online discussion:

the INITIAL UL RRC MESSAGE can include the DU to CU RRC Container IE including the lower layer configurations of PC5 RLC channel and SRB1 PC5 RLC channel configuration for remote UE, FFS on how to deal with the mandatory IE, i.e., CellGroupConfig, in DU to CU RRC Container ID in this case.

After some further checking, the moderator feels that there may be some problem.

The INITIAL UL RRC MESSAGE TRANSFER message contains DU to CU RRC Container IE, which includes the CellGroupConfig IE.

- 9.2.3.1	INITIAL UL RRC MESSAGE TRANSFER

This message is sent by the gNB-DU to transfer the initial layer 3 message to the gNB-CU over the F1 interface.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| <unrelated part is omitted> |  |  |  |  |  |  |
| RRC-Container | M |  | 9.3.1.6 | Includes the UL-CCCH-Message IE or UL-CCCH1-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | rejet |
| DU to CU RRC Container | O |  | OCTET STRING | CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [8]. Required at least to carry SRB1 configuration. The ReconfigurationWithSync field is not included in the CellGroupConfig IE. | YES | reject |
| <unrelated part is omitted> |  |  |  |  |  |  |

As the above agreement indicated, we assume to use DU to CU RRC Container IE to include the lower layer configuration for Sidelink. However, such assumption seems to be incorrect since RAN2 uses SL-PHY-MAC-RLC-Config-r16 IE to include the lower layer configuration of sidelink. In this sense, the INITIAL UL RRC MESSAGE TRANSFER message should include SL-PHY-MAC-RLC-Config container rather than DU to CU RRC Container IE.

Observation 1: For initial access of remote UE, the INITIAL UL RRC MESSAGE TRANSFER message should include SL-PHY-MAC-RLC-Config IE rather than the DU to CU RRC Container IE as the lower layer configuration of sidelink.

Meanwhile, the current spec. uses the presence of the DU to CU RRC Container IE to indicate the admission result at the gNB-DU, as specified below:

If the DU to CU RRC Container IE is not included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU should reject the UE under the assumption that the gNB-DU is not able to serve such UE. If the gNB-DU is able to serve the UE, the gNB-DU shall include the DU to CU RRC Container IE and the gNB-CU shall configure the UE as specified in TS 38.331 [8]. The gNB-DU shall not include the ReconfigurationWithSync field in the CellGroupConfig IE as defined in TS 38.331 [8] of the DU to CU RRC Container IE.

Observation 2: the presence of the DU to CU RRC Container IE cannot be used to indicate the admission result of gNB-DU for remote UE since this IE is not needed for remote UE.

In this sense, for remote UE, the SL-PHY-MAC-RLC-Config IE should be added in INITIAL UL RRC MESSAGE TRANSFER message to include PC5 RLC channel configuration at least for sidelink RLC for the initial access of remote UE, the presence of which can be used to indicate the admission result of remote UE at gNB-DU.

With the above, the moderator gives the following potential proposal:

Potential proposal 1:

RAN3 agrees to change the following agreement:

“the INITIAL UL RRC MESSAGE can include the DU to CU RRC Container IE including the lower layer configurations of PC5 RLC channel and SRB1 PC5 RLC channel configuration for remote UE, FFS on how to deal with the mandatory IE, i.e., CellGroupConfig, in DU to CU RRC Container ID in this case.”

to

the INITIAL UL RRC MESSAGE TRANSFER message is enhanced by including the SL-PHY-MAC-RLC-Config IE for remote UE to include PC5 RLC channel configuration at least for SRB1, and the absence of such IE indicates the admission of remote UE is rejected by gNB-DU.

Q3: Can companies agree the above Potential proposal 1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | We need SL-PHY-MAC-RLC-Config IE in INITIAL UL RRC MESSAGE TRANSFER message. |
| ZTE | See comments | The analysis from rapporteur looks reasonable. But the current DU to CU RRC information definition already include the SL-PHY-MAC-RLC-Config. Maybe only the Semantics description of DU to CU RRC information IE in INITIAL UL RRC message Transfer need to be updated. |
| Samsung2 |  | [Reply to ZTE] The INITIAL UL RRC MESSAGE TRANSFER message does not contain DU to CU RRC Information IE, instead, it contains DU to CU RRC Container IE which is CellGroupConfig. |
| CATT | Yes | Agree with moderator. SL-PHY-MAC-RLC-Config IE should be included in DU to CU RRC Container in INITIAL UL RRC MESSAGE TRANSFER. And the description of “reject” also should be updated. |
| Lenovo, Motorola Mobility | Yes | Tend to agree with moderator’s observation. |
| Huawei | See comment | Another option is maybe to modify the existing semantics and include either of them. This requires a bit investigation. But we agree a solution is needed |
| E/// | Partial yes | We agree the first part, but for the rejection there may be other way. We prefer not to mess them together. |
| Qualcomm | Need clarification | Do we have to include the PC5 RLC configuration of SRB1 in INITIAL UL RRC MESSAGE TRANSFER itself? Can’t we use UE CONTEXT SETUP RESPONSE which already includes the SL-PHY-MAC-RLC-Config IE? Also needs more thought on the admission control. Propose to move it to next meeting. |

---------

3 companies may need more investigation on including SL-PHY-MAC-RLC-Config IE, and the admission result in gNB-DU needs further investigate as well.

The moderator propose to list this issue as “to be continue…”:

---------

- Mapping configuration at relay UE (LCID issue in Q8 of Phase I)

During phase-I discussion, majority companies believe this issue should wait for RAN2 progress first. However, the moderator is wondering if RAN2 can realize this issue since their design does not take the principle of CU-DU split (i.e., logical channel ID is assigned by gNB-DU, and it is unknown to gNB-CU) into account. To resolve issue, we either follow RAN2 signaling design to enhance our specification, or tell RAN2 the issue and ask them to change their spec. Since we only have one meeting left, “waiting” may not be a good approach for the sake of progress. So, the moderator gives the following two options:

  - Option 1: follow RAN2 signaling design and enhance RAN3 spec.

  - Option 2: send LS to RAN2 and ask RAN2 update their spec.

Q4: Which option is preferred to move this issue forward?

| Company | Option 1 or 2 | Comments |
| --- | --- | --- |
| Samsung | Option 2 | Option 1 is also acceptable to us. |
| ZTE | Option 2 | It is hard for CU to know the LCID of both PC5 and Uu RLC channel and then configure the bearer mapping based on the LCID. It is suggested to send LS to RAN2 about this issue. On the other hand, it is suggested for RAN3 to discuss which ID is recommended to RAN2 for the bearer mapping configuration. |
| CATT | Option 2 | Update LCID to Uu RLC channel ID |
| Lenovo, Motorola Mobility | Option 2 | Using RLC channel ID is simpler from technical point of view. |
| Huawei | Option 2 | Wait for RAN2 |
| E/// | Neutral |  |
| Qualcomm | Check with RAN2 | We can send an LS describing the problem and request RAN2 to look into it. RAN2 should decide whether to use Option 2 or any other alternative solution. |

---------

All companies agree to send LS to RAN2.

The moderator will prepare the draft LS for checking.

---------

- TPs for TS38.401 & TS38.473

Both drafts will be submitted in https://www.3gpp.org/ftp/tsg_ran/WG3_Iu/TSGR3_114bis-e/Inbox/Drafts/CB%20%23%20SLRelay2_ControlPlane/Phase-2/TPs

The moderator suggests to make comments directly to them so that the owners for TPs can collects view directly and make the further modification. However, if companies need to use this SoD for comments provision, please provide them in below table.

Q5: Any comments for draft TPs which cannot be directly provided to the files in above link?

| Company | Comments |
| --- | --- |

For the Chairman’s Notes(Phase-I)

Proposal 1: RAN3 agree the following:

- From protocol stack point of view, the termination point of Uu adaptation layer is located at gNB-DU

- gNB-CU is responsible for the allocation of local ID of remote UE

- Over F1, the remote UE is configured via the UE-associated F1AP messages for itself

Proposal 2: RAN3 agrees the following:

  - WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

  - The gNB-DU can include the local ID of remote UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

  - WA: the local ID of remote UE can be notified to gNB-DU before initial access of remote UE

  - The gNB-CU should send the updated local ID of remote UE to gNB-DU after initial access of remote UE

Proposal 3: The stage-2 TP can include the new flow charts for RRC establishment/reestablishment/resume for sidelink relay by considering the following aspects:

  - Local ID allocation for remote UE via SUI of relay UE

  - Configuration of relay UE Uu RLC CH for relaying remote UE’s SRB message before remote UE initial access

  - INITIAL UL RRC MESSAGE & DL RRC MESSAGE enhancement (if any, related to Open issue 4)

  - Remote UE context setup, including PC5 RLC channel related configuration, mapping configuration

  - Relay UE context modification, including Uu/PC5 RLC channel related configuration

Note: the details can be addressed in phase 2 discussion

Proposal 4: the UE context management procedures should be enhanced to include the Uu RLC channel and PC5 RLC channel related information, i.e.,

- UE CONTEXT SETUP REQUEST message: Uu RLC channel to be setup list (for relay UE), PC5 RLC channel to be setup list (for relay UE and remote UE)

- UE CONTEXT SETUP RESPONSE message: Uu RLC channel setup list (for relay UE), Uu RLC channel failed to setup list (for relay UE), PC5 RLC channel setup list (for relay UE and remote UE), PC5 RLC channel failed to setup list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION REQUEST message: Uu RLC channel to be setup/modified/release list (for relay UE), PC5 RLC channel to be setup/modified/release list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION RESPONSE message: Uu RLC channel setup/modified list (for relay UE), Uu RLC channel failed to setup/modified list (for relay UE), PC5 RLC channel setup/modified list (for relay UE and remote UE), PC5 RLC channel failed to setup/modified list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION REQUIRED message: Uu RLC channel required to be modified/release list (for relay UE), PC5 RLC channel required to be modified/release list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION CONFIRM message: Uu RLC channel modified list (for relay UE), PC5 RLC channel modified list (for relay UE and remote UE)

Proposal 5: the additional information, e.g., QoS profile info (e.g., 5QI, priority, PDB, PER, etc), RLC mode, and control plane traffic type (FFS: SRB ID vs. priority), can be included when configuring PC5/Uu RLC CH from gNB-CU to gNB-DU.

Proposal 6: RAN3 agrees the following:

  - gNB-CU determines the mapping configuration

  - F1AP message(s) are used to configure the mapping between DRB/SRB and Uu RLC Channel at the gNB-DU side

  - the UE associated F1AP message(s) of remote UE are used to configure the mapping between DRB/SRB and Uu RLC Channel at the gNB-DU

  - the mapping between DRB and Uu RLC Channel is configured at the granularity of GTP-U tunnel.

Proposal 7: the following responsibility are defined for gNB-CU and gNB-DU, respectively, for sidelink relay:

gNB-CU’s responsibility:

- Local Remote UE ID allocation

- Remote UE and relay UE association and context maintenance

- Remote UE bearer mapping and multiplexing

- Relaying Uu/PC5 RLC channel management

- E2E QoS split management for relaying

- Dedicated thresholds for relay discovery

gNB-DU’s responsibility

- Uu adaptation layer (AL) support for CP/UP data

- Determine the RLC/MAC/PHY Configuration for the relaying Uu/PC5 RLC CHs of relay UE

- Dedicated resource pool for NR ProSe service (same as legacy)

Note: those aspects can be reflected by stage-3 TP.

Proposal 8:RAN3 agrees the following:

  - the UE CONTEXT SETUP REQUEST message is enhanced to include local ID of remote UE for, e.g., inter-gNB-DU mobility

  - the INITIAL UL RRC MESSAGE can include the DU to CU RRC Container IE including the lower layer configurations of PC5 RLC channel and SRB1 PC5 RLC channel configuration for remote UE, FFS on how to deal with the mandatory IE, i.e., CellGroupConfig, in DU to CU RRC Container ID in this case.

Phase-2 work:

- Address the above WAs and FFSes to seek the potential progress

- Mapping configuration at relay UE (LCID issue in Q8)

- Draft and try to agree stage-2 TP for TS38.401 (Samsung)

- Draft and try to agree stage-3 TP for TS38.473 (Huawei)

Discussions (Phase-I)

In this meeting, contributions are mainly discussing the open issues listed in last meeting. Companies seem to have consensus to the following open issues:

Open issue 1: the termination point of Uu adaptation layer from protocol stack point of view (CU vs. DU)

Open issue 3: local ID allocation (CU vs. DU)

Open issue 6: F1AP signalling to configure remote UE with following options

- Option 1: via the UE-associated F1AP messages for remote UE

- Option 2: via the UE-associated F1AP message for relay UE

Thus, the moderator gives the following set of proposals for agreements:

Potential proposal 1: RAN3 can take the following as agreements:

- From protocol stack point of view, the termination point of Uu adaptation layer is located at gNB-DU

- gNB-CU is responsible for the allocation of local ID of remote UE

- Over F1, the remote UE is configured via the UE-associated F1AP messages for itself

Q1: Can companies agree the potential proposal 1?  If better rewording is foreseen, please spell it out.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Qualcomm | Yes |  |
| Lenovo, Motorola Mobility | Yes |  |
| Nokia | Yes |  |
| Huawei | Yes |  |
| E/// | Yes |  |
| China Telecom | Yes |  |
| CATT | Yes |  |
| CMCC | Yes |  |
| ZTE | Yes |  |

----------

All companies agree the potential proposal 1.

Proposal 1: RAN3 agree the following:

- From protocol stack point of view, the termination point of Uu adaptation layer is located at gNB-DU

- gNB-CU is responsible for the allocation of local ID of remote UE

- Over F1, the remote UE is configured via the UE-associated F1AP messages for itself

-----------

Open issue 4: remote/relay UE identification during initial access procedure

- Remote UE identification: the following proposals are given in this meeting:

  - include local ID of remote UE from CU to DU via DL RRC Message [4,E///]

  - include local ID from DU to CU via INITIAL UL RRC MESSAGE [5, ZTE] [10, Samsung]

  - include the allocated local ID of remote UE to gNB-DU before initial access (e.g., when receiving SUI from the relay UE, the gNB-CU sends the allocated local ID to gNB-DU via UE CONTEXT MODIFICATION REQUEST message) [3, HW]

  - include the updated local ID of remote UE from gNB-CU to gNB-DU [10, Samsung]

- Relay UE identification: the following proposal are given in this meeting:

  - include the relay UE ID in the INITIAL UL RRC MESSAEG [1, CATT][3, HW][4, E///][8, ChinaTelecom][10, Samsung], the discussion point is which relay UE ID is included, i.e., C-RNTI vs relay UE gNB-DU F1AP UE ID

  - no need to include relay UE ID in the INITIAL UL RRC MESSAGE [5, ZTE].

To summarize the above proposals, four aspects are addressed:

- identification of relay UE during initial access procedure

Should the gNB-CU be informed the associated relay UE via INITIAL UL RRC MESSAGE? Which ID is used (C-RNTI vs. F1AP UE ID)?

- identification of remote UE during initial access procedure

Whether the notification of local ID of remote UE (from CU to DU, or from DU to CU) is needed or not?

- identification of remote UE before initial access (e.g., during local ID allocation)

Should the gNB-DU be informed the local ID of remote UEs after gNB-CU allocates it according to the received SUI?

- identification of remote UE after initial access (e.g., update of local ID of remote UE)

Should the gNB-DU be informed the updated local ID of remote UE?

Q2: please provide your view for the identification of relay/remote UE by considering the following aspects at least:

        - relay UE ID (C-RNTI vs. F1AP UE ID)  notification via INITIAL UL RRC MESSAGE

        - notification of local ID of remote UE (“from CU to DU”, or “from DU to CU”, or “not needed”) during initial access procedure

        - notification of local ID of remote UE to the gNB-DU before initial access, e.g., after gNB-CU allocates it according to the received SUI

        - notification of the updated local ID of remote UE to gNB-DU after initial access

| Company | Comments |
| --- | --- |
| Samsung | a. yes. Either C-RNTI or F1AP UE ID is fine to us b. notification of local ID from DU to CU is needed to help the gNB-CU identify the remote UE, which has been allocated local ID before. c. no d. yes |
| Qualcomm | a. Yes, prefer F1AP UE ID (since this is identification over F1 interface) b. Local ID of remote UE can be sent from gNB-DU to gNB-CU in INITIAL UL RRC MESSAGE TRANSFER c. Are we referring to sending the local ID of remote UE from gNB-CU to gNB-DU via UE CONTEXT MODIFICATION REQUEST but before the INITIAL UL RRC MESSAGE TRANSFER? If so, this should be possible if relay UE context is modified for the preparation of Uu RLC channel for SRB1 along with the notification of local ID of remote UE, before the initial access. d. Yes, updated local ID of remote UE can be sent from gNB-CU to gNB-DU after initial access via UE CONTEXT MODIFICATION REQUEST |
| Lenovo, Motorola Mobility | a. both can work, F1AP UE ID seems easier from processing point of view. b. during initial access, gNB-DU should inform gNB-CU about remote UE’s local ID c. yes. According to the RAN2 agreement, the relay UE will be configured with the remote UE local ID before remote UE’s initial access. Thus, in our understanding the remote UE local ID shall be provided to DU as part of the relay UE’s SRAP configuration before remote UE initial access. RAN2#116e Agreement: Proposal 15 (modified): Relay UE is configured by gNB with the local/temp remote UE ID to be used in adaptation layer by RRCReconfiguration message, after reporting the remote UE’s L2ID via SUI message to gNB and before forwarding the first SRB0 UL message of the remote UE. FFS if impact to the SUI contents is needed to enable this. d. yes, local ID update is possible |
| Nokia | - - a. Yes. F1AP UE ID. - b. Yes. We assume the question is only related to the F1AP INITIAL UL RRC MESSAGE TRANSFER message. That is ok. Please note, in RAN3 spec (e.g. 38.401), the UE Initial Access procedure have multiple F1AP messages to be exchanged. - - c: Yes - d: Yes |
| Huawei | a. Yes, relay UE identification in initial UL RRC message is necessary, and F1AP UE ID is better. b. yes. From gNB-DU to gNB-CU. c. yes- this is needed to support question (b) above. gNB-CU allocate the remote UE local ID when receiving the SUI from relay UE. gNB-CU has to indicate the remote UE local ID to gNB-DU, so that gNB-DU can identify the SRB0 message from the remote UE. The remote UE local ID can be indicated along with the Uu RLC channel configurations for SRB0/1. Uu RLC Channel configuration may not always be needed if default Uu RLC channel configurations is used [FFS in RAN2] for SRB0/1. d. It is agreed in RAN2 that the gNB can trigger a change, so this should be supported. The detailed solution can be investigated later. |
| E/// | - a. Yes. F1AP UE ID seems enough. - b. Yes. From DU to CU during initial access. - c: Yes - d: Yes |
| China Telecom | a. Yes. F1AP UE ID. b. Yes. From DU to CU during initial access. c. Yes. d. Yes. |
| CATT | a. yes. Both ok but slight prefer F1AP UE ID. Because relay UE ID is sent via F1AP b. CU can identify remote UE via remote UE ID included in RRC message. From this point view we think DU does not need to include remote local ID in initial access procedure. But if companies consider this is reasonable e.g., for “confirmation” purpose, it may fine with me. c. yes, DU needs to identify that this RRC message is comeing from remote UE and include the local ID into the initial UL RRC message transfer. d. yes |
| CMCC | a. Yes, C-RNTI and F1AP UE ID are both acceptable for us, considering the size, we slightly prefer C-RNTI. b. Local ID of remote UE can be included in INITIAL UL RRC MESSAGE TRANSFER from DU to CU. c. Yes, gNB-CU allocates remote UE local ID according to L2ID in SUI, and includes the remote UE local ID in RRC Reconfiguration message for relay UE for SRB0 transmission. The remote UE local ID should also be notified to gNB-DU, which may be used in SRAP header for SRB0. d. Yes. |
| ZTE | a.no. If we adopt b, it is not necessary to adopt a. Suppose DU notify the local ID of remote UE to CU, the CU may identify the remote UE based on the local ID of remote UE assigned by CU. Since this local ID of remote UE is allocated by CU upon receiving the SUI message from relay UE, CU may identify the PC5 connected relay UE. It is not necessary to include the relay UE ID in the INITIAL UL RRC MESSAGE. b. yes. Based on the local ID of remote UE contained in the INITIAL UL RRC message, CU can associate the remote UE’s F1-C connection with the local remote UE ID. Otherwise, CU only know that a new UE is trying to set up RRC connection. However, CU has no idea that it is related with a remote UE and the corresponding local remote UE ID assigned by CU. c. No. DU may get the local ID of remote UE and identify that it is the first SRB0 RRC message via the Uu adaptation layer subheader of the RRC message forwarded by relay UE. Then DU may initiate the initial UL RRC message transfer. It is meaningless for the CU to send the local ID of remote UE to DU before initial access. d. yes. If the update of remote UE’s local ID is supported, it is reasonable for CU to send the updated local ID to DU. However, RAN2 has identified a lot of exceptional cases for the local ID update. It is not clear if the local ID update is really feasible. |

-----------

- Majority (9/10) agrees to include the relay UE ID in the INITIAL UL RRC Message, and F1AP UE ID (8) gets more support than C-RNTI

- Majority (9/10) agrees to include the local ID of remote UE in the INITIAL UL RRC Message, and 1 company can follow majority view

- Majority (8/10) agrees to include the local ID of remote UE to the gNB-DU before initial access, while 2 companies disagree this.

- All companies (10) agrees to notify the updated local ID of remote UE to gNB-DU after initial access.

Considering the majority support, the moderator gives the following proposal

Proposal 2: RAN3 agrees the following:

  - WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

  - The gNB-DU can include the local ID of remote UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

  - WA: the local ID of remote UE can be notified to gNB-DU before initial access of remote UE

  - The gNB-CU should send the updated local ID of remote UE to gNB-DU after initial access of remote UE

----------

Open issue 5: baseline flow chart for RRC establishment/resume/reestablishment for sidelink relay by considering CU-DU split

In this meeting, [2, QC][3, HW][4, E///][5, ZTE][9, Lenovo][10,11, Samsung][12, CMCC]. Among those contributions, the following sidelink relay dedicated aspects are addressed for the RRC establishment/resume/reestablishment:

  - Local ID allocation for remote UE via SUI of relay UE

  - Configuration of relay UE Uu RLC CH for relaying remote UE’s SRB message before remote UE initial access

  - INITIAL UL RRC MESSAGE & DL RRC MESSAGE enhancement (if any, related to Open issue 4)

  - Remote UE context setup, including PC5 RLC channel related configuration, mapping configuration

  - Relay UE context modification, including Uu/PC5 RLC channel related configuration

  - SRAP related operation at the gNB-DU side, e.g., add/remove SRAP subheader, mapping

Before developing stage-2 TP, it is better to have a common understanding on which aspects should be reflected in the procedures. In addition, the moderator would like also to collect views on how to develop the stage-2 procedures. In this meeting, majority companies draw new figures for the procedures, while [11, Samsung] propose to add sidelink relay dedicated aspects on top of the existing procedures in 38.401 since most of the steps in the current specification can be reused for sidelink relay.

Q3:Please provide your views on baseline flow chart for RRC establishment/resume/reestablishment for sidelink relay by considering the following aspects at least:

- The sidelink relay dedicated aspects reflected in the flow chart include the following:

  - Local ID allocation for remote UE via SUI of relay UE

  - Configuration of relay UE Uu RLC CH for relaying remote UE’s SRB message before remote UE initial access

  - INITIAL UL RRC MESSAGE & DL RRC MESSAGE enhancement (if any, related to Open issue 4)

  - Remote UE context setup, including PC5 RLC channel related configuration, mapping configuration

  - Relay UE context modification, including Uu/PC5 RLC channel related configuration

  - SRAP related operation at the gNB-DU side, e.g., add/remove SRAP subheader, mapping

- The flow chart is developed on top of existing ones or by drawing new ones

NOTE: the details stage-2 TP for the flow chart can be discussed in Phase 2.

| Company | Comments |
| --- | --- |
| Samsung | We are fine to include the aspects list above. We prefer to develop stage-2 text on top of existing ones. |
| Qualcomm | We are also fine to include the above aspects. Prefer a new flowchart and new stage-2 text. Would be nice to separate the relay/remote UE and show the entire call flow for better clarity. Even IAB had dedicated flowcharts for BH RLC channel establishment. |
| Lenovo, Motorola Mobility | OK in general. We assume the first sub bullet“ Local ID allocation for remote UE via SUI of relay UE” means the local ID allocation for remote UE happens after gNB CU receives SUI from the relay UE. |
| Nokia | Agree with Lenovo. |
| Huawei | Fine with the above aspect, except the last one seems to be RAN2 issues (can refer to RAN2). A new flowchart with new stage-2 text is ok. |
| E/// | Ok to include the above except the last point. How do we capture the DU own related operation in the flow chart? We are fine to proceed with some basic flows and leave FFS if no firm agreement. |
| China Telecom | Fine to include the aspects list above. It is ok for us to develop the flowchart on top of existing ones or use a new flowchart. |
| CATT | Ok with general. For second sub bullet. Is that depends on RAN2 discussion about whether default Uu RLC configuration is adopted? The flow chart should present the SUI sends from relay UE to Gnb-DU and further sends to CU for local ID allocation. Furthermore, the PC5/Uu BH RLC channel configuration procedure should also be explicit included. Hence we prefer a new flow chart to reflect the above procedure. |
| CMCC | We are fine to include all the aspects list above. We think new flow charts can be more clearly to show details of SL relay procedure. Considering the future release, multi-connection and inter-gnb mobility scenario are introduced, separated flow charts is a better choice. |
| ZTE | The flow chart focuses on the control plane signalling. The last bullet involves the UP processing. It is suggested not to capture the last bullet in the flow chart. A note or some procedure descriptions of the UP processing is enough. |

----------

All companies agrees to capture the above listed aspects in stage-2 TP except:

- 2 companies are not fine to include the last bullet

- 1 company has concern on the second bullet which may need RAN2 progress

- 1 company mention that FFS can be put if no firm agreement.

Meanwhile, 4 companies clearly mentioned to have new flow chart for sidelink relay.

The intention of this discussion is to help our phase 2 discussion on TPs. So, the moderator would like propose:

Proposal 3: The stage-2 TP can include the new flow charts for RRC establishment/reestablishment/resume for sidelink relay by considering the following aspects:

  - Local ID allocation for remote UE via SUI of relay UE

  - Configuration of relay UE Uu RLC CH for relaying remote UE’s SRB message before remote UE initial access

  - INITIAL UL RRC MESSAGE & DL RRC MESSAGE enhancement (if any, related to Open issue 4)

  - Remote UE context setup, including PC5 RLC channel related configuration, mapping configuration

  - Relay UE context modification, including Uu/PC5 RLC channel related configuration

Note: the details can be addressed in phase 2 discussion

-----------

Open issue 7: Uu/PC5 RLC channel configuration via F1AP

[2, QC][3, HW] [6, ZTE][10, Samsung] address this issue in details for stage-3 signaling. The moderator gives the following proposal for discussion:

Potential proposal 2: the UE context management procedures should be enhanced to include the Uu RLC channel and PC5 RLC channel related information, i.e.,

- UE CONTEXT SETUP REQUEST message: Uu RLC channel to be setup list (for relay UE), PC5 RLC channel to be setup list (for relay UE and remote UE)

- UE CONTEXT SETUP RESPONSE message: Uu RLC channel setup list (for relay UE), Uu RLC channel failed to setup list (for relay UE), PC5 RLC channel setup list (for relay UE and remote UE), PC5 RLC channel failed to setup list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION REQUEST message: Uu RLC channel to be setup/modified/release list (for relay UE), PC5 RLC channel to be setup/modified/release list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION RESPONSE message: Uu RLC channel setup/modified list (for relay UE), Uu RLC channel failed to setup/modified list (for relay UE), PC5 RLC channel setup/modified list (for relay UE and remote UE), PC5 RLC channel failed to setup/modified list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION REQUIRED message: Uu RLC channel required to be modified/release list (for relay UE), PC5 RLC channel required to be modified/release list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION CONFIRM message: Uu RLC channel modified list (for relay UE), PC5 RLC channel modified list (for relay UE and remote UE)

Q4: Can companies agree the potential proposal 2?  If better rewording is foreseen, please spell it out.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Qualcomm | Yes | Don’t we need the following in UE CONTEXT MODIFICATION CONFIRM (although this is sent by gNB-CU) as well? - Uu RLC channel failed to be modified list (for relay UE) - PC5 RLC channel failed to be modified list (for relay UE and remote UE) |
| Lenovo, Motorola Mobility | Yes with comment | Ok if it is the CU configures the mapping according to Q6. We are fine to follow majority view. |
| Nokia | Yes |  |
| Huawei | Yes with comments | Uu RLC channel to be setup list (for relay UE) is not needed in UE CONTEXT SETUP REQUEST/RESPONSE since we always have to first set up the UE context for a relay separately before we add remote UEs. |
| E/// | Yes |  |
| China Telecom | Yes |  |
| CATT | Yes | Uu RLC channel to be setup list (for relay UE) may be needed if default Uu RLC is used during initial accress procedure. And in the current spec. we do not see the failed to release/modified list in UE CONTEXT MODIFICATION CONFIRM. |
| CMCC | Yes |  |
| ZTE | Yes |  |

----------

All companies agrees to the above potential proposal 2, and some comments are received as below:

- FFS-1: Whether UE CONTEXT MODIFICATION CONFIRM message needs include Uu RLC channel failed to be modified list (for relay UE), PC5 RLC channel failed to be modified list (for relay UE and remote UE)

- FFS-2:Whether UE CONTEXT SETUP REQUEST message needs include Uu RLC Channel to be setup list (for relay UE)

The moderator’s understanding is that for FFS-1, the failed list may not need since the existing spec. only contains modified list, while for FFS-2, the setup list may be needed since the Uu RLC CH (at least default Uu RLC CH) can be prepared before initial access. In this sense, the moderator gives the following proposal:

Proposal 4: the UE context management procedures should be enhanced to include the Uu RLC channel and PC5 RLC channel related information, i.e.,

- UE CONTEXT SETUP REQUEST message: Uu RLC channel to be setup list (for relay UE), PC5 RLC channel to be setup list (for relay UE and remote UE)

- UE CONTEXT SETUP RESPONSE message: Uu RLC channel setup list (for relay UE), Uu RLC channel failed to setup list (for relay UE), PC5 RLC channel setup list (for relay UE and remote UE), PC5 RLC channel failed to setup list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION REQUEST message: Uu RLC channel to be setup/modified/release list (for relay UE), PC5 RLC channel to be setup/modified/release list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION RESPONSE message: Uu RLC channel setup/modified list (for relay UE), Uu RLC channel failed to setup/modified list (for relay UE), PC5 RLC channel setup/modified list (for relay UE and remote UE), PC5 RLC channel failed to setup/modified list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION REQUIRED message: Uu RLC channel required to be modified/release list (for relay UE), PC5 RLC channel required to be modified/release list (for relay UE and remote UE)

- UE CONTEXT MODIFICATION CONFIRM message: Uu RLC channel modified list (for relay UE), PC5 RLC channel modified list (for relay UE and remote UE)

-----------

In addition, some additional information has been proposed for PC5/Uu RLC CH, which include:

- QoS profile info (e.g., 5QI, priority, PDB, PER, etc)

- RLC mode

- Control plane traffic type (e.g., SRB ID or priority)

Q5: Can companies agree to include QoS profile info., RLC mode, control plane traffic type for PC5/Uu RLC CH from gNB-CU to gNB-DU?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Qualcomm | Yes |  |
| Nokia | Yes |  |
| Huawei | Yes |  |
| E/// | Yes with comment | One clarification is needed on priority for traffic type. Assume it is not related to QoS? |
| China Telecom | Yes |  |
| CATT | Yes |  |
| CMCC | Yes |  |
| ZTE | Yes |  |

----------

All companies agrees to include the above additional information for PC5/Uu RLC CH with one comment.

The moderator’s understanding to the priority for the Control plane traffic type is not related to QoS. However, please note that which information is used to indicate the control plane traffic type (SRB ID vs. priority) is not decided yet, which can be addressed in Phase 2.

With the above summary, the moderator gives the following proposal:

Proposal 5: the additional information, e.g., QoS profile info (e.g., 5QI, priority, PDB, PER, etc), RLC mode, and control plane traffic type (FFS: SRB ID vs. priority, can be include when configuring PC5/Uu RLC CH.

-----------

Open issue 8: mapping configuration via F1AP

According to the contributions in this meeting, the following aspects are address

              - Determination of the mapping configuration

An intuitive thought is gNB-CU, which seems to be majority view. While, [9, Lenovo] has proposals for letting gNB-DU generating the mapping configuration. To have a clear discussion basis, it is better to have a common understanding on this.

              - Mapping configured via F1AP

For sidelink relay, the following types of mapping is needed for configuration:

- Type 1: mapping between DRB/SRB and Uu RLC CH at gNB-DU side

- Type 2: mapping between DRB/SRB and PC5 RLC CH at relay UE side

- Type 3: mapping between DRB/SRB and PC5 RLC CH at remote UE side

However, which configuration should be performed at the gNB-DU side is not decided yet.

              - Procedures for configuring the mapping

Two options are given in this meeting:

- Option 1 (relay UE’s F1AP message): for each Uu RLC Channel, the gNB-CU should indicate the DRB(s)/SRB(s) and the corresponding remote UE local ID, which are mapped to it

- Option 2 (remote UE’s F1AP message): for each SRB/DRB, the gNB-DU should indicate the mapped Uu RLC CH

              - Granularity to indicate the mapping between DRB and Uu RLC channel

Two options are given in this meeting:

- Option 1 (per DRB): each DRB can be indicated the mapped Uu RLC CH

- Option 2 (per tunnel): each tunnel can be indicated the mapped Uu RLC CH

Q6:Please provide your views on mapping configuration via F1AP by considering the following aspects at least:

  - Determination of the mapping configuration (gNB-CU vs. gNB-DU)

  - Mapping configured via F1AP (type 1, type 2, type 3)

  - Procedures for configuration the mapping (relay UE associated F1AP vs. remote UE associated F1AP)

  - Granularity to indicate the mapping between DRB and Uu RLC channel (per DRB vs. per GTP-U tunnel)

| Company | Comments |
| --- | --- |
| Samsung | - gNB-CU determines mapping - Only type 1 is needed when configuring the mapping via F1AP. Type 2&3 can be configured via RRC message - Remote UE associated F1AP This method can reduce the specification impact since it only needs to add the mapped Uu RLC CH for each SRB/DRB. In addition, this method may save the signaling overhead. Specifically, the Uu RLC CH may be established to aggregate multiple DRBs/SRBs. After setting up Uu RLC CH, once a new DRB/SRB is setup for remote UE, only the remote UE F1AP signaling is needed. However, if using relay UE F1AP, the remote UE F1AP signaling should be used first to set up DRB/SRB, and then the relay UE F1AP signaling should be used to configure the mapping. - The granularity is per GTP-U tunnel This is more future-proof if the PDCP duplication/multiple path is supported in the future. |
| Qualcomm | Same views as Samsung |
| Lenovo, Motorola Mobility | a. if we consider SRAP as part of gNB DU, it could be cleaner to let the gNB DU provides the configuration related to SRAP. We are fine to follow majority view here though. b. some clarification is needed. Type 1, we assume it means the mapping between remote UE SRB/DRB and relay UE Uu RLC CH. yes, coordination via F1 AP is needed. Type 2, does it mean the mapping between the relay UE Uu RLC CH and relay UE PC5 RLC CH mapping? Since relay UE SRB/DRB is not applicable here or? If so, without F1 AP configuration, does it mean the CU generates SRAP configuration for relay UE without DU involvement although SRAP locates in DU? It is also relevant to Q8.c if gNB DU has to provide LCID to gNB CU. Type 3, we assume it means the mapping between remote UE SRB/DRB and remote UE PC5 RLC CH. Similar question as for type 2, without F1 AP configuration, does it mean the CU generates SRAP configuration for relay UE without DU involvement although SRAP locates in DU? It is also relevant to Q8.c if gNB DU has to provide LCID to gNB CU. c. we assume this is only for type 1. Then, via remote UE F1AP message is preferred for type 1. d. per GPT-U tunnel seems better. |
| Nokia | Agree with Samsung. |
| Huawei | Same view as Samsung |
| E/// | Share similar view with Samsung. |
| China Telecom | Same views as Samsung. |
| CATT | Agree with Samsung. |
| CMCC | Agree with Samsung. |
| ZTE | Agree with Samsung for a and b. For c, we prefer relay UE associated F1AP. Considering that multiple remote UE’s Uu E2E bearer may be mapped to the same relay UE’s Uu RLC channel, relay UE specific signalling may support the configuration update of multiple remote UE’s Uu E2E bearer mapping. However, with remote UE specific signalling, gNB-DU need to send multiple F1AP signallings to involved remote UEs for the for bearer mapping. From the overhead point of view, relay UE specific signalling seems better than remote UE specific signalling for bearer mapping configuration. For d, per RB is preferred. We need to consider the mapping of both DRB and SRB. For SRB, only the per SRB approach can be used. It is suggested to align the mapping of DRB and SRB. |

----------

              - Majority (9/10) agree to let gNB-CU determines the mapping, while 1 company prefer to gNB-DU and can accept gNB-CU determination

              - All companies agree to use F1AP configure the mapping between DRB/SRB and Uu RLC CH at gNB-DU side

              - Majority (9/10) agree to use remote UE associated F1AP message to configure the mapping between DRB/SRB and Uu RLC CH at the gNB-DU side, 1 company prefer to use relay UE associated F1AP message.

              - Majority (9/10) agree to configure the mapping at the granularity of GTP-U tunnel, 1 company prefers to at the granularity of DRB

Considering the majority view, the moderator gives the following proposal:

Proposal 6: RAN3 agrees the following:

  - gNB-CU determines the mapping configuration

  - F1AP message(s) are used to configure the mapping between DRB/SRB and Uu RLC Channel at the gNB-DU side

  - WA: the UE associated F1AP message(s) of remote UE are used to configure the mapping between DRB/SRB and Uu RLC Channel at the gNB-DU

  - WA: the mapping between DRB and Uu RLC Channel is configured at the granularity of GTP-U tunnel.

-----------

Open issue 2: responsibilities for sidelink relay related functionalities between gNB-CU and gNB-DU

With the above discussion, this issue seems to be clear. To check companies view, the moderator use QC’s paper as the baseline for the responsibility of gNB-CU and gNB-DU by additionally including some proposals from HW’s paper. Then, the following potential proposal is given:

Potential proposal 3: the following responsibility are defined for gNB-CU and gNB-DU, respectively, for sidelink relay:

gNB-CU’s responsibility:

- Local Remote UE ID allocation

- Remote UE and relay UE association and context maintenance

- Remote UE bearer mapping and multiplexing

- Relaying Uu/PC5 RLC channel management

- E2E QoS split management for relaying

- Dedicated thresholds for relay discovery

gNB-DU’s responsibility

- Uu adaptation layer (AL) support for CP/UP data

- Determine the RLC/MAC/PHY Configuration for the relaying Uu/PC5 RLC CHs of relay UE

- Dedicated resource pool for NR ProSe service (same as legacy)

In addition, after determining the responsibility, the moderator wants to check companies view on if any stage-2 text should be added.

Q7: Can companies agree the potential proposal 3?  If company believe some stage-2 text is needed to clearly indicate the responsibility of gNB-CU and gNB-DU, please raise it out here.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | However, it seems to no need to have stage-2 text to reflect this since the stage-3 can reflect it clearly. |
| Qualcomm | Yes | Perhaps stage-2 text not needed at this time for distinguishing the roles of gNB-CU and gNB-DU. This also depends on how/where we describe the flow chart in Q3. |
| Lenovo, Motorola Mobility | Yes | Ok in general. |
| Nokia | Yes | Stage-2 text may be not needed, e.g. if the functions are described in related call flow. |
| Huawei | Yes |  |
| E/// | Yes | We would say P3 is to align companies’ view on the responsibilities, then they will be mapped to stage-3 changes. |
| China Telecom | Yes | The responsibility of gNB-CU and gNB-DU can be reflected in the flowchart and stage-3 changes. |
| CATT | Yes | Agree to reflect them in stage-3 |
| CMCC | Yes |  |
| ZTE | Yes |  |

----------

All companies agree the potential proposal 3, and 6 companies comment that no specific stage-2 text is needed for now, and stage-3 TP can reflect those responsibilities.

So, the moderator has the following proposal:

Proposal 3: the following responsibility are defined for gNB-CU and gNB-DU, respectively, for sidelink relay:

gNB-CU’s responsibility:

- Local Remote UE ID allocation

- Remote UE and relay UE association and context maintenance

- Remote UE bearer mapping and multiplexing

- Relaying Uu/PC5 RLC channel management

- E2E QoS split management for relaying

- Dedicated thresholds for relay discovery

gNB-DU’s responsibility

- Uu adaptation layer (AL) support for CP/UP data

- Determine the RLC/MAC/PHY Configuration for the relaying Uu/PC5 RLC CHs of relay UE

- Dedicated resource pool for NR ProSe service (same as legacy)

Note: those aspects can be reflected by stage-3 TP.

-----------

Other issues

In this meeting, some additional issues are raised, which include:

- Inter-gNB-DU mobility ([3, HW])

The following proposal is given:

Proposal 9: During Inter-DU Mobility procedure, local ID are allocated by the gNB-CU during the Remote UE context setup procedure, and gNB-CU/Target-DU shall associate the local ID with the F1AP UE ID of Remote UE.

According to [3], the specification impact for the above proposal is to include the local ID in UE CONTEXT SETUP REQUEST message (i.e., step 3 in Fig. 5). Meanwhile, the contribution indicates that the RRCReconfiguration message to relay UE should include the new local ID of remote UE, which has been agreed in RAN2.

- Admission control at gNB-DU side for remote UE ([10, Samsung])

In legacy, the inclusion of DU to CU RRC Container IE is to indicate the admission at the gNB-DU side. However, for sidelink relay case, such IE may not be needed since the remote UE does not need Uu interface configuration. Thus, the presence of the DU to CU RRC Container IE cannot be used to indicate the admission result of gNB-DU for remote UE. In other words, the gNB-DU should provide an explicit indication in INITIAL RRC MESSAGE TRANSFER message to indicate the admission result of remote UE at gNB-DU, i.e.,

Proposal 6-3: gNB-DU provides an explicit indication in INITIAL UL RRC MESSAGE TRANSFER to reflect the admission result of the remote UE.

- Mapping configuration at the relay UE ([10, Samsung])

According to the running RRC CR, the mapping configuration at Relay UE contains the LCID of the egress Uu RLC channel, which aims at configuring UL mapping from E2E RB to Uu RLC bearer. Thus, the gNB-CU should be aware of the logical channel of Uu RLC bearer. Based on the legacy F1 signaling design, the logical channel assignment for Uu RLC channel is performed at the gNB-DU side, and then provided to the gNB-CU via RRC container when configuring the Uu RLC channel for the relay UE. In other words, the gNB-CU does not know the logical channel for Uu RLC channel so that the above “sl-Egress-RLC-Channel-Uu-r17” cannot be generated by the gNB-CU. To resolve this issue, the following options can be considered:

- Option 1: the gNB-CU provides LCID of the Uu RLC channel

- Option 2: the gNB-DU provides LCID of the Uu RLC channel along with the lower layer configuration for sidelink

- Option 3: change the RAN2 CR by indicating the Uu RLC channel ID instead of LCID

Q8: Please provide your views on the following additional issues:

              - Inter-gNB-DU mobility: the UE CONTEXT SETUP REQUEST message is enhanced to include local ID of remote UE

              - Admission control at gNB-DU side for remote UE:  an explicit indication is added to INITIAL UL RRC MESSAGE TRANSFER to indicate the admission result of remote UE

              - Mapping configuration at the relay UE: select one option among Option 1/2/3

NOTE: if any other issues are missing, please raise it out here.

| Company | Comments |
| --- | --- |
| Samsung | - Agree - Agree - Option 2 |
| Qualcomm | a. Agree b. No. During remote UE initial access, DU to CU RRC Container still needs to be included in INTIAL UL RRC MESSAGE TRANSFER for sending the lower layer configuration (e.g., RLC, MAC, PHY) for PC5 RLC channel and for SRB1 PC5 RLC CH configuration for remote UE. Hence the admission can still be done based on the presence of this container and no need of an explicit IE. c. Wait for RAN2 conclusion on the following Proposal: Proposal 11 (low priority): Regarding how to allocate LCID for PC5 RLC channel of remote UE Uu RBs including SRB2 and DRBs, RAN2 to down select the following options. FFS on SRB1 a. Alt 1: allocated by UE same as in R16 SL b. Alt 2: up to gNB dedicated configuration same as in Uu If RAN2 selects Alt1, then the LCID corresponding to the PC5 MAC logical channel is assigned by the Relay UE/Remote UE (this means CU or DU don’t have the PC5 LCID), whereas the LCID corresponding to the Uu MAC logical channel is assigned by the DU (like legacy). Option 2 in Proposal 6-3 only solves the LCID problem for Uu RLC channel (i.e., the Uu mapping config) and not the PC5 mapping config. To be consistent for both, it might be better to rely on RLC Channel ID for both hops and not use LCID for bearer mapping. CU should manage the bearer mapping configuration and assign the RLC Channel ID for both Uu and PC5. |
| Lenovo, Motorola Mobility | a. Agree b. no, agree with Qualcomm c. Option 3 seems cleaner. We are fine to wait for RAN2 progress. |
| Nokia | a. agree b. no. agree with QC. c. wait for RAN2 decision. |
| Huawei | a. Agree b. Same view with QC c. gNB-CU can allocate a Uu RLC channel ID to indicate that a new RLC channel is configured or a legacy RLC channel is reconfigured. Then, gNB-DU can determine whether the Uu RLC channel ID is the same with LCID, or can be mapped to another LCID. In this way, the logical channel assignment for Uu RLC channel is still performed at the gNB-DU side, which align with legacy. Anyway, we can wait for RAN2 conclusion. |
| E/// | a. Yes b. No. Don’t see the motivation to have a new indicator, while legacy container works. c. Wait |
| China Telecom | a. Yes. b. No. Same view as QC. c. We can wait for RAN2 decision. |
| CATT | a. agree b. no. Agree with QC. c. The mapping should from E2E RB to Uu RLC channel ID hence option3 is more reasonable. We are ok to wait for RAN2 decision. |
| CMCC | a. Agree b. No c. Option 2, we are fine to wait for RAN2 agreements on it. |
| ZTE | a. Yes b. no. Agree with QC. c. Option 3 is more reasonable and aligned with IAB design. |

----------

- All companies agree to introduce the local ID of remote UE in UE CONTEXT SETUP REQUEST message to address inter-gNB-DU mobility

- Majority (9/10) disagree the enhancement, and 8 companies believe the existing DU to CU RRC Container IE is still needed to include the lower layer configurations for PC5 RLC channel and for SRB1 PC5 RLC CH configuration for remote UE.

- Majority (8/10) agrees to wait for RAN2 progress.

The moderator’s comment: for b, we may need consider how to deal with CellGroupConfig in DU to CU RRC Container since it is mandatory IE if DU to CU RRC Container IE is included in INITIAL UL RRC MESSAGE.

So, the moderator has the following proposal:

Proposal 8:RAN3 agrees the following:

  - the UE CONTEXT SETUP REQUEST message is enhanced to include local ID of remote UE for, e.g., inter-gNB-DU mobility

  - the INITIAL UL RRC MESSAGE can include the DU to CU RRC Container IE including the lower layer configurations of PC5 RLC channel and SRB1 PC5 RLC channel configuration for remote UE, FFS: on how to deal with the mandatory IE, i.e., CellGroupConfig, in DU to CU RRC Container ID in this case.

-----------

Conclusion, Recommendations [if needed]

If needed

References

[1] R3-220237 Discussion on the Open Issues of Control Plane Procedures (CATT)

[2] R3-220276 Control Plane procedures for U2N L2 relays (Qualcomm Incorporated)

[3] R3-220325 (TP for SLrelay BLCR for 38.473) Support of Uu adaptation layer for U2N relay (Huawei)

[4] R3-220350 Network signaling for NR SL Relay (Ericsson)

[5] R3-220375 Discussion on SL relay architecture and RRC connection management of remote UE (ZTE)

[6] R3-220376 Discussion on F1 signalling design for the PC5&Uu RLC channel and bearer mapping configuration (ZTE)

[7] R3-220377 (F1AP CR) Support of NR SL relay (ZTE)

[8] R3-220396 Discussion on control plane procedures for SL Relay (China Telecommunication)

[9] R3-220496 (TP for TS 38.401) Control plane issues on SL relay (Lenovo, Motorola Mobility)

[10] R3-220569 (TP to TS38.473 on sidelink relay) Discussion on open issues for sidelink relay (Samsung)

[11] R3-220570 CR to TS38.401 on Rel-17 Sidelink Relay (Samsung)

[12] R3-220867 Discussion on CP issue for SL relay (CMCC)