3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-221305

# Online, January 17th – 26th 2022

Agenda Item:	15.2.2

Source:	Ericsson

Title:	(TP for QoE BL CR for TS 38.423): Mobility Support

Document for:	Agreement

# Text Proposal for QoE BL CR for TS 38.423

-------------------------------------------Change 1-------------------------------------------

### 9.2.3.b UE Application Layer Measurement Information

The IE defines the information about the QoE Measurement Collection (QMC) configuration.

Editor’s note: The presence of configuration container in the IE is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for S-based Application Layer Measurement Configuration | O (WA) |  | Octet string (1..)[FFS] | Contains the s-based QoE measurement configuration, see Annex L in [xx]. | - | - |
| QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the trace activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. | - | - |
| Measurement Configuration Application Layer ID | M |  |  | This IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [10]. | - | - |
| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. | - | - |
| QoE Measurement Status (WA) | M |  | ENUMERATED (ongoing, configured...) | Indicates whether a measurement has been activated. | - | - |
| CHOICE MDT Alignment Information | O |  |  | Indicates the MDT measurements with which alignment is required. | - | - |
| >S-based MDT |  |  |  |  |  |  |
| >>NG-RAN Trace ID | M |  | 9.2.3.97 | Indicates the MDT measurements with which alignment is required. | - | - |
| >M-based MDT |  |  |  |  |  |  |
| >>Trace Reference | M |  | OCTET STRING (SIZE(6)) | Trace Reference defined in TS 32.422 [23]. | - | - |
| Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. | - | - |
| CHOICE Area Scope of QMC | C-ifQoEMeasTypeSignalingBased |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |  | - |
| >>>NR CGI | M |  | 9.2.2.7 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |  | - |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  | - | - |
| >>>TAI | M |  | 9.2.3.20 |  | - | - |
| >PLMN based |  |  |  |  |  | - |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |  | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | - | - |
| S-NSSAI List | O | 0..1 |  |  | - | - |
| >S-NSSAI Item |  | 1 .. <maxnoofSNSSAIforQMC> |  |  | - | - |
| >>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  | - | - |

| Condition | Explanation |
| --- | --- |
| ifQoEMeasTypeSignalingBased | This IE shall be present if the QoE Measurement Type IE is set to the value “s-based” |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell IDs comprising the QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA comprising the QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |
| maxnoofSNSSAIforQMC | Maximum no. of S-NSSAIs comprising the QMC scope. Value is 16. |

-------------------------------------------End of changes-------------------------------------------