3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221463

Online, January 17th – 26th 2022

Title:	LS on Support for Configuration and Reporting of RAN Visible QoE Measurements

Response to:	-

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN3

To:	CT1

Cc:	RAN2, SA4

Contact Person:

Name: 		Filip Barać

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description

In the legacy QoE management framework (defined for UTRA and E-UTRA), the QoE measurement configuration and the QoE measurement report are delivered from the network to the UE and from the UE to the network, respectively, inside containers (i.e., XML files) whose contents are not visible to the RAN. CT1 has, in the specification TS 27.007, defined the AT commands +CAPPLEVMC and +CAPPLEVMR for delivering the QoE configuration and report containers between the UE AS and the UE Application layer.

In the Rel-17 RAN2/3 Work Item on “NR QoE management and optimizations for diverse services”, RAN3 agreed to support the concept RAN visible QoE measurements, where:

- The RAN assembles a RAN visible QoE measurement configuration and delivers it to the UE AS, which then delivers it to the UE Application layer.

- The UE Application layer delivers the RAN visible QoE measurement report to the UE AS, which then delivers the report to the RAN. The RAN can use the corresponding reports for various types of optimizations.

To enable RAN visible QoE measurements, the RAN visible QoE measurement configuration and RAN visible QoE report can be delivered between the UE AS and the UE Application layer separately from the XML files containing the legacy QoE measurement configuration and legacy QoE measurement report. So, the RAN visible QoE configuration and report can be transferred without any corresponding legacy QoE configuration and report being included in the same AT command.

In addition to the RAN visible QoE measurements, the ID of the PDU session used by the corresponding measured service also needs to be included in the RAN visible QoE report.

The above requires to be supported in AT commands. RAN2 will provide the detailed content of the RAN visible QoE configuration and report, once they complete their work.

2. Actions:

To 3GPP CT1:

Given that it is necessary to deliver the RAN visible QoE configuration and RAN visible QoE report between the UE AS and the UE Application layer, RAN3 respectfully asks CT1 to specify the corresponding AT commands. RAN2 will provide the detailed content of the RAN visible QoE configuration and report, once they complete their work.

3. Date of next TSG RAN WG3 meeting:

RAN3#115-e                         February 21st - March 3rd 2022		Online