3GPP TSG-RAN WG3 Meeting #114bis-e                                                      	R3-220274

E-Meeting: Jan 17th – Jan 26th, 2022

Agenda item:	15.3

Source:	Qualcomm Inc.

Title:	RAN visible QoE

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further on RAN Visible QoE based on some agreements and open issues identified in previous meetings.

# Discussion

## RVQoE metrics and values

- Interaction latency or comparable quality viewport switching latency metric is NOT considered as a RAN visible QoE metric in Rel-17

- Buffer level is confirmed as a RAN visible QoE metric for DASH and VR service types

- Playout delay for media startup is confirmed as a RAN visible QoE metric for DASH and VR service types

- FFS whether to include stalling related events during an application session (e.g., number of stalling occurrences) as part of RAN visible QoE. FFS how this is indicated e.g., by counting how many times a stop reason is specified as "rebuffering" in Play List or via indicating Buffer Level as 0. FFS whether to consider this under RAN visible QoE metrics or values.

- FFS whether and how to support RAN visible QoE values. If supported, the following is to be clarified:

  - Granularity: Whether the RAN visible QoE value is calculated based on measurements of multiple metrics or per metric

  - Definition: Whether the RAN visible QoE value is an objective/qualitative representation of QoE metrics (e.g., on a score of 0-10, poor/medium/good) or derived information from QoE metrics (e.g., the number of stalling events, buffer level alarm)

- FFS which entity (UE or NG-RAN or MCE) should generate the RAN visible QoE values

- FFS whether the calculation of RAN visible QoE values is pre-defined in SA4 specifications in case it is agreed that UE has to report the RAN visible QoE values

- FFS whether NG-RAN can configure a threshold/target for RAN visible QoE values, only upon crossing which UE would report the RAN visible QoE values.

The following possibilities can be considered in the context of RAN visible QoE values:

| Granularity | Definition | Example |
| --- | --- | --- |
| Derived information from multiple QoE metrics | QoE score | For example, RVQoE value = a* Buffer Level + b* Playout Delay, where a and b are coefficients for each QoE metric |
| Derived information from single QoE metric | QoE score | If Buffer level < X RVQoE value = 1 or “Good” else if X <= Buffer level < Y RVQoE value = 2 or “Medium” Else if Buffer level >= Y RVQoE value = 3 or “Poor” |
|  | Alarm e.g., Buffer level alarm | If Buffer level > X (pre-defined or configured by NG-RAN) Buffer Level Alarm = “True” |
|  | Statistical information | Number of stalling events |

Observation 1: Defining RAN visible QoE values as an objective/qualitative representation of QoE metrics (e.g., on a score of 0–10, poor/medium/good) requires knowledge of the typical values of QoE metrics and the need to define appropriate formulas/thresholds, which is beyond RAN3’s understanding and also extensive work for SA4 if pursued.

Observation 2: Defining RAN visible QoE values based on measurements of multiple metrics is convoluted and it is best to define RVQoE values based on a single metric

Hence the following is proposed:

Proposal 1: There is no need to define any RAN visible QoE values as an objective/qualitative representation of QoE metrics (e.g., on a score of 0-10, poor/medium/good). RAN3 can instead focus on supporting RAN visible QoE values which are statistical information derived from legacy QoE metrics

Proposal 2: RAN visible QoE values are to be calculated based on QoE measurements of a single metric (i.e., not from multiple metrics)

Proposal 3: Stalling related information during an application session can be derived from legacy QoE metric “Play List” and supported as a RAN visible QoE value. Number of stalling occurrences in the last measurement interval can be calculated by counting how many times a stop reason is specified as "rebuffering" in Play List

Proposal 4: UE APP should generate the statistical information or alarms which are to be reported as RAN visible QoE values and indicated to UE AS via AT command. Send LS to SA4 to confirm.

Proposal 5: NG-RAN can optionally configure an “alarm threshold” for each RVQoE metric, upon crossing which UE reports an alarm for the corresponding RVQoE metric instead of reporting the absolute value

## RVQoE Configuration

- In split gNB architecture, gNB-CU should generate the RAN visible QoE configuration.

- RAN Visible QoE and legacy QoE can be configured together or separately. In case RAN visible QoE is configured separately, it can be configured only after configuring legacy QoE.

- NG-RAN can release a list of RAN visible QoE configurations while not releasing the corresponding legacy QoE configurations

- If the legacy QoE configuration is released, the corresponding RAN visible QoE configuration is released as well

- RAN visible QoE configuration can include at least the RAN visible QoE metrics to be reported, service type and a measurement ID for the RAN visible QoE. Whether existing IEs can be reused for service type and measurement ID and the signaling design is up to RAN2

- There is no need to consider Start Time, Duration and Sample Percentage in the RAN Visible QoE configuration in Rel-17

- RAN3 should discuss whether the existing identified RAN visible QoE metrics (or values if agreed) justifies the need of a separate reporting periodicity for RAN visible QoE

- RAN3’s decision on whether to have a different reporting periodicity for RAN visible QoE is independent of RAN2’s decision on which SRB to use for RAN visible QoE

- Send an LS to SA4 checking the feasibility of supporting a different reporting periodicity for RAN visible QoE metrics, from the application perspective.

- FFS whether to define event triggers in RAN visible QoE configuration.

- WA: If the legacy QoE configuration is paused/resumed, the corresponding RVQOE configuration is paused/resumed as well

In the last meeting, it was proposed to also consider non-QoE metric related event triggers e.g., during a handover. For the sake of simplicity, we propose to not consider any such event triggers in Rel-17.

Proposal 6: There is no need to define any event triggers (except alarm thresholds if agreed in Proposal 5) in RAN visible QoE configuration

Enabling separate reporting periodicities for RVQoE will add to the processing complexities and increase storage requirements at UE Application to handle RVQoE and legacy QoE independently. We therefore propose to not define a separate reporting periodicity for RAN visible QoE.

Proposal 7: There is no need to define a separate reporting periodicity for RAN visible QoE

Proposal 8: Convert the following WA into agreement:

WA: If the legacy QoE configuration is paused/resumed, the corresponding RVQOE configuration is paused/resumed as well

## RVQoE report

- FFS whether RAN visible QoE and legacy QoE can be reported separately or should be always reported together. Note: This depends on whether a separate reporting periodicity is agreed to be defined in RAN visible QoE configuration.

Proposal 9: RAN visible QoE should always be reported together with legacy QoE

## Per slice RVQoE

- RAN3 assumes that slice ID is included inside the transparent QoE reporting container, which is up to SA4’s decision. LS to SA4 on the support of including slice ID in the QoE reporting container in R3-216225

- There is no need to include slice ID as an explicit IE over Uu outside the QoE configuration and reporting container for legacy QoE. FFS whether and how to support per slice RVQoE configuration and reporting.

- WA: Include PDU or QoS related information in RVQoE report

Application layer is aware of the PDU sessions and slices, so when it needs to report QoE, it can include PDU session ID together with RAN visible QoE metrics. UE AS need not know the mapping between an application session to a DRB/PDU session, it just needs to transfer the PDU session ID and RVQoE metrics to gNB. gNB can get the mapping for PDU session to DRBs/QoS flows.

Proposal 10: Include PDU session ID in RVQoE report

## Handling RVQoE during mobility

- FFS whether the RAN visible QoE configuration can be propagated from the source to target node upon mobility and during context retrieval.

Proposal 11: RAN visible QoE configuration can be propagated from the source to target node upon mobility and during context retrieval

Proposal 12: RAN visible QoE report can be signalled from the target to the source node after a successful handover.

## Alignment between radio related measurements and RVQoE

- The details of alignment between radio-related measurements and RVQoE measurements can be discussed in RAN3#114-bis-e.

- The alignment of RVQoE and MDT measurements reuses the solution for the alignment of legacy QoE and MDT measurements. RAN node can reuse RRM measurements as well

Proposal 13: Similar to OAM based alignment, NG-RAN can configure RVQoE and MDT at the same time in case an alignment is required

# Conclusion

Observation 1: Defining RAN visible QoE values as an objective/qualitative representation of QoE metrics (e.g., on a score of 0–10, poor/medium/good) requires knowledge of the typical values of QoE metrics and the need to define appropriate formulas/thresholds, which is beyond RAN3’s understanding and also extensive work for SA4 if pursued.

Observation 2: Defining RAN visible QoE values based on measurements of multiple metrics is convoluted and it is best to define RVQoE values based on a single metric

Proposal 1: There is no need to define any RAN visible QoE values as an objective/qualitative representation of QoE metrics (e.g., on a score of 0-10, poor/medium/good). RAN3 can instead focus on supporting RAN visible QoE values which are statistical information derived from legacy QoE metrics

Proposal 2: RAN visible QoE values are to be calculated based on QoE measurements of a single metric (i.e., not from multiple metrics)

Proposal 3: Stalling related information during an application session can be derived from legacy QoE metric “Play List” and supported as a RAN visible QoE value. Number of stalling occurrences in the last measurement interval can be calculated by counting how many times a stop reason is specified as "rebuffering" in Play List

Proposal 4: UE APP should generate the statistical information or alarms which are to be reported as RAN visible QoE values and indicated to UE AS via AT command. Send LS to SA4 to confirm.

Proposal 5: NG-RAN can optionally configure an “alarm threshold” for each RVQoE metric, upon crossing which UE reports an alarm for the corresponding RVQoE metric instead of reporting the absolute value

Proposal 6: There is no need to define any event triggers (except alarm thresholds if agreed in Proposal 5) in RAN visible QoE configuration

Proposal 7: There is no need to define a separate reporting periodicity for RAN visible QoE

Proposal 8: Convert the following WA into agreement:

WA: If the legacy QoE configuration is paused/resumed, the corresponding RVQOE configuration is paused/resumed as well

Proposal 9: RAN visible QoE should always be reported together with legacy QoE

Proposal 10: Include PDU session ID in RVQoE report

Proposal 11: RAN visible QoE configuration can be propagated from the source to target node upon mobility and during context retrieval

Proposal 12: RAN visible QoE report can be signalled from the target to the source node after a successful handover.

Proposal 13: Similar to OAM based alignment, NG-RAN can configure RVQoE and MDT at the same time in case an alignment is required

# 4. References

# 5. Appendix

| RVQOE metric | Description | Potential benefits to RAN |
| --- | --- | --- |
| Buffer Level | Indicates the playout duration for which media data of all active media components is available starting from the current playout time. A list of buffer occupancy level measurements during playout at normal speed. Level of the buffer is included in milliseconds. Time of the measurement of the buffer level is also included. Annex D.4.5 in ISO/IEC 23009-1 defines the metrics for buffer level status events as follows: | Better mobility decisions at RAN based on buffer level (e.g., the selection of HO type between legacy HO, DAPS HO or CHO), leading to seamless connectivity and a lower risk of video stalling Assists NG-RAN to adjust the resource allocation for the UE. For example, the base station will consider scheduling more radio resources to those streams which reports higher buffer level so as to guarantee the stream will be processed properly. |
| Buffer level Alarm | Buffer level corresponds to the DL buffer at the UE application client, indicating the "playout duration for which media data of all active media components is available starting from the current playout time". In normal operation, when the buffer level falls below a given threshold, an end-to-end flow control in the client will request further data to be sent from the application server. If the application server, transport network or the RAN is congested, the client in the UE will not receive the needed information for continued streaming. In this situation we believe that availability of the absolute buffer level measurement in the RAN would not be easily exploitable. However it could be discussed whether there is a benefit that derived information, e.g. that the client determines that the buffer level falls below an "alarm" level (defined by a second threshold, lower than the one used for nominal E2E flow control) is available in the RAN. In that case the RAN could check the DL buffer in the PDCP layer for the corresponding DRB and prioritize it in case data is actually buffered in the RAN (which would mean that the bottle-neck is situated in the RAN). It is still an open question whether such additional mechanism will bring benefit on top of already existing mechanisms ensuring QoS for GBR bearers, or on top of mechanisms allocating resources while ensuring fairness between non-GBR bearers. | A UE going to be “starved” for DL streaming data could get higher scheduling priority, and that would help if DL data is already buffered in the CU-UP (PDCP buffer)). NG-RAN will have to compare the received information with own DL PDCP buffer status for the RB. The play-list may also serve a statistical purpose, redundant with buffer level information. |
| Playout Delay for Media Start-up | The playout delay for media start-up is measured as the time in milliseconds from the time instant of DASH player receives play-back-start trigger to the instant of media playout. This metric indicates the waiting time that the user experiences for media start-up. The metric is only logged at the time point when the media start-up happens. - If the MPD has been delivered earlier before the user clicks, it may include the process time of MPD, the fetch time of some media segments which are required for media presentation, the process time of segments, and the time for media decode and render to the user. - If no MPD has been fetched earlier, it also needs to add the fetch time of MPD. Key; Type. PlayoutDelayforMediaStartup; Integer. | RAN node can leverage this as a time budget to deliver the requested content without video stalling, while, at the same time, not over-allocating the precious radio resources to that service |
| Play List | A list of playback periods. A playback period is the time interval between a user action and whichever occurs soonest of the next user action, the end of playback or a failure that stops playback. Example for Playout - 5 seconds of initial stalling - 10 seconds playing with representation 1 - 15 seconds of stalling - 20 seconds playing with representation 2 - 30 seconds playing with representation 1 In last meeting, a simplified version was also proposed e.g., an indication from the application to the access stratum whenever the video representation changes or the video stalls | Presented video quality and video stalling statistics, can be valuable at RAN for radio resource partitioning and allocation among different users and services |
| VR Interaction latency | From TS 26.929, VR interaction latency is a key metric to evaluate the VR experience and it varies based on the VR content encoding, packing and delivery technologies, as well as VR device capabilities. The latency between user interaction and the corresponding response may involve interaction detection latency, DASH request latency, network delivery latency, receiver re-buffering latency and motion-to-photon latency. - VR Interaction detection latency - VR content access latency - VR decoding latency - VR rendering latency VR interaction latency has the same meaning as “Comparable quality viewport switching latency metric” in TS 26.118. The comparable quality viewport switching latency metric reports the latency and the quality-related factors when viewport movement causes quality degradations, such as when low-quality background content is briefly shown before the normal higher-quality is restored. Note that this metric is only relevant if the Advanced Video Media profile and region-wise packing is used. Also note that the metric currently does not report factors related to foveated rendering. Key; Type; Description. CompQualLatency; List; List of comparable quality viewport switching latencies. Entry; Object. firstViewport; ViewportItem; Specifies information about the first viewport. secondViewport; ViewportItem; Specifies information about the second viewport. worstViewport; ViewportItem; Specifies information about the worst viewport seen during the switch duration. time; Real-Time; Wall-clock time when the switch started. Mtime; Media-Time; Media presentation time when the switch started.. Latency; Integer; Specifies the switching delay in milliseconds.. Accuracy; Integer; Specifies the estimated accuracy of the latency metric in milliseconds. Cause; List; Specifies a list of possible causes for the latency.. Entry; Object. code; Enum; A possible cause for the latency. The value is equal to one of the following: - 0: Segment duration - 1: Buffer fullness - 2: Availability of comparable quality segment - 3: Timeout. |  |