3GPP TSG-RAN WG3 #114bis-e							                                             R3-220290

Online, January 17 - 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0193 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.401: Correction on IPv6 Flow Label handling for IAB using IPv6 tunnel mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2022-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When IPsec tunnel mode is used to protect DL F1 or X2 traffic to IAB nodes, the IPv6 Flow Label information needs to be available on the outer IP header, so that it can be used for the DL mapping at the IAB-donor-DU. This implies that for deployments with separate security gateway, the IPv6 Flow Label information is set on the inner IP header by the IAB-donor-CU for F1 traffic and by the MeNB for Xn traffic, and it must then be copied from the inner IP header to the outer IP header by the security gateway. The need of IPv6 Flow Label copying by the security gateway to support these IAB deployment scenarios is presently missing in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description to section “8.9.9.1 Traffic Mapping from IP-layer to Layer-2” on the copying of IPv6 Flow Label copying at the security gateway for IAB when using IPsec tunnel mode for F1 or X2 protection together with separate security gateway. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because the changes only impact the IAB functionality. The impact can be considered isolated because the change only affects IAB deployments with separate security gateway. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB deployments with separate security gateway cannot support fine granular QoS or traffic differentiation using IPv6 Flow Label. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START CHANGE

### 8.9.9	Traffic Mapping

#### 8.9.9.1	Traffic Mapping from IP-layer to Layer-2

When forwarding IP packets, the IAB-donor-DU performs the traffic mapping from IP-layer to layer-2 as defined in TS 38.340 [22]. The traffic mapping information is configured by the IAB-donor-CU, which contains the IP header information, and the BH information including the BAP routing ID and a list of egress link and BH RLC channel pairs.

Multiple traffic mappings can contain the same BAP routing ID and/or list of egress link and BH RLC channel pairs.

The traffic mappings can be configured as part of the UE Context Setup or UE Context Modification procedures. They may also be configured via the non-UE-associated BAP Mapping Configuration procedure.

NOTE:	Implementation must ensure the avoidance of potential race conditions, i.e. no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

The traffic mapping from IP-layer to layer-2 may include IPv6 Flow Label information. For DL F1 or X2 traffic, the IPv6 Flow Label information is set by the IAB-donor-CU or MeNB, respectively. When this traffic is protected via IPsec tunnel mode with separate security gateway, the IPv6 Flow Label is set on the inner IP header by the respective IAB-donor-CU or MeNB, and then copied from the inncer IP header to the outer IP header by the security gateway.

NOTE:	Implementation must ensure that IPv6 Flow Label collisions are avoided on the IP backhaul network between security gateway and IAB-donor-DU.

END OF CHANGE