3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-220295

E-meeting, January 17 – 26, 2022

Agenda Item:	13.2.3

Source: 	Qualcomm Incorporated

Title:	Inter-donor topology transport

Document for:	Discussion

# Introduction

In TSG RAN Meeting #114-e, the following agreements have been achieved [1]:

Agree to wait for RAN2's progress on BAP operation (e.g., header rewriting, routing, bearer mapping) E2E QoS requirement are divided into two parts: provided by its own topology fragment, provided by the non-F1-terminating CU’s topology fragment, which is up to implementation of CU1. For DL descendent node traffic: - CU1->CU2: - QoS info. - A list of DL IP addresses - FFS: L2 info (e.g. egress BAP routing ID, egress BH RLC CH) - CU2->CU1 - for each traffic: a list of {DSCP/IPv6 flow label, ingress BAP routing ID, ingress BH RLC CH ID} - FFS: prior-hop BAP address For UL descendent node traffic: - CU1->CU2: - QoS info. - FFS: ingress BAP routing ID, ingress BH RLC CH - CU2->CU1 - for each traffic,: egress BAP routing ID, egress BH RLC CH ID - FFS: next-hop BAP address for UL FFS: additional info, stage-3 details for signaling design. WA: If non-F1-terminating CU is not able to guarantee the per topology fragment QoS requirement, it should reject the request from F1-terminating CU. The granularity of the informed QoS requirement info is "per GTP-U tunnel“ or "per group of GTP-U tunnels" About non-F1-U traffic type, the information to be exchanged between the F1-termination donor and non-F1-termination donor include: {UE-associated F1AP, non-UE-associated F1AP, non-F1}, FFS for other info. WA: The following information is exchanged between F1-terminating CU (CU1) and non-F1-terminating CU (CU2) for boundary node traffic: - CU1->CU2 - QoS info per traffic type for non-UP traffic and per one or bundle of F1-U tunnels for UP traffic; content is FFS. - DL IP address info - CU2->CU1 - DL: IPv6 FL/DSCP value - UL: UL boundary node configuration, e.g., UL BH mapping, for each QoS info; pending RAN2.

This paper discusses Xn info exchange for inter-donor topology transport.

# Discussion

## QoS info content

For non-UP traffic: RAN3#114-e agreed that the Non-UP Traffic Type (UA F1AP, NUA F1AP, non-F1) is shared between CUs for non-UP boundary/descendant traffic. As per Rel-16, the Control Plane Traffic Type (Integer 1-3) for non-UP traffic is still missing.

For UP traffic: RAN3#113-e/114-e agreed that QoS info for UP traffic is shared between CUs with the granularity of one or a bundle of boundary/descendant F1-U GTP-U tunnels. Since the latter are transported via same BH RLC CH(s), the QoS info should include a BH RLC CH QoS as defined in Rel-16.

Proposal 1: QoS info includes Control Plane Traffic Type for non-UP boundary/descendant traffic and BH RLC CH QoS for UP boundary/descendant traffic.

## How to avoid 1:N mapping at the boundary node?

For UL descendant traffic: CU1 provides indices of ingress BAP routing IDs and indices of ingress BH RLC CHs along with QoS info of offloaded traffic. CU2 avoids mapping two offloaded traffic instances corresponding to one index of an ingress BAP routing ID/ingress BH RLC CH to distinct egress BAP routing IDs/egress BH RLC CHs.

For DL descendant traffic: CU2 provides indices of egress BAP routing IDs and indices of egress BH RLC CHs along with QoS info of offloaded traffic. CU2 avoids mapping two offloaded traffic instances corresponding to distinct indices of egress BAP routing IDs/egress BH RLC CHs to same ingress BAP routing ID/ingress BH RLC CH.

Proposal 2: To avoid 1:N mapping at the boundary node, CU1 provides indices of ingress BAP routing IDs/BH RLC CHs and indices of egress BAP routing IDs/BH RLC CHs to CU2 for respective UL and DL descendant traffic offloaded to CU2’s topology.

## BAP address of the boundary node’s parent node in CU2’s topology

RAN3#111-e agreed that CU2 provides CU1 with ingress BH RLC CH ID(s) for DL traffic and egress BH RLC CH ID(s) for UL traffic to support bearer mapping at the boundary node.

Bearer mapping has the following format:

(prior-hop BAP address, ingress BH RLC CH ID)  (next-hop BAP address, egress BH RLC CH ID)

Bearer mapping is configured by CU1 at the boundary node, whereas the prior-hop BAP address in DL or the next-hop BAP address in UL corresponds to the BAP address of the boundary node’s parent node in CU2’s topology. Consequently, CU2 should provide this BAP address to CU1.

Proposal 3: CU2 provides CU1 the prior-hop BAP address for DL traffic or equivalently the next-hop BAP address for UL traffic.

## Maintenance of the transport path in CU2’s topology

RAN3#114-e made a WA that CU2 should reject the request for traffic offload from CU1 if CU2 cannot guarantee the per-topology-fragment QoS requirement. It should also be the case that CU2 shall initiate a request to CU1 to release the transport path in CU2’s topology if CU2 can no longer guarantee the per-topology-fragment QoS requirement.

CU2 may further reconfigure BAP IDs in CU2’s topology upon which it provides the updated BAP IDs of the transport path to CU1.

Proposal 4: CU2 may indicate to CU1 the release of the transport path in CU2’s topology or provide updated BAP IDs of the transport path.

# Conclusion

This paper discussed Xn info exchange for inter-donor topology transport. The following proposals have been made:

Proposal 1: QoS info includes Control Plane Traffic Type for non-UP boundary/descendant traffic and BH RLC CH QoS for UP boundary/descendant traffic.

Proposal 2: To avoid 1:N mapping at the boundary node, CU1 provides indices of ingress BAP routing IDs/BH RLC CHs and indices of egress BAP routing IDs/BH RLC CHs to CU2 for respective UL and DL descendant traffic offloaded to CU2’s topology.

Proposal 3: CU2 provides CU1 the prior-hop BAP address for DL traffic or equivalently the next-hop BAP address for UL traffic.

Proposal 4: CU2 may indicate to CU1 the release of the transport path in CU2’s topology or provide updated BAP IDs of the transport path.

# References

[1]	Chairman report, 3GPP RAN TSG WG3 Meeting #114e, Electronic Meeting, November 1 – 11, 2021