3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220502

# Electronic meeting, 17th – 26th Jan 2022

Agenda item:		18.4.2

Source:	Intel Corporation

Title:	AI/ML based load balancing

Document for:		Discussion and Decision

# Introduction

During RAN3 #114e meeting, the following open issues were captured for load balancing use case:

Editor’s Note: FFS other input information required for AI/ML-based load balancing. Editor’s Note: FFS other output information expected from AI/ML-based load balancing. Editor’s Note: FFS other feedback expected from AI/ML-based load balancing

In this contribution, we mainly focus on the remaining open issues for AI-based load balancing.

# Discussion

## Output data of Load Balancing use case

During RAN3 #114e meeting, it is FFS whether following information are considered as output of AI/ML-based load balancing:

- The UE(s) selected to be handed-over to target NG-RAN node

- Predicted time stamp of handover

In this section, we will focus on the motivation of having such output data for AI/ML-based load balancing.

Selected UE(s) and handover target cell(s) for load balancing handover

As mentioned above, to improve service quality of user experience and improve system capacity, source NG-RAN node may select certain UE(s) to be handover to one/more target NG-RAN node(s). Hence, AI/ML model can directly decide and select suitable UE(s) which need to be handed-over to neighboring NG-RAN node(s) based on current/predicted resource status of current and neighboring NG-RAN nodes.

- The predicted selected UE(s) to be handed-over to target NG-RAN node is generated as output of AI/ML-based load balancing use case.

Predicted time stamp to handover

Although the source NG-RAN node receives predicted UE(s) and corresponding target cell(s) for handover, it does not mean the NG-RAN node should perform the predicted handover decision immediately. AI/ML model can provide the predicted output in a near future, which allows the network to prepare for certain decision and reduce service interruption.

- The handover decision for AI/ML-based load balancing can be performed in future after predicted output(s) are generated from Model Inference, which can further help to reduce service interruption during handover.

For load balancing handover, according to the predicted resource status of current and neighboring NG-RAN nodes, AI/ML model may select the best timing to perform load balancing handover according to the resource status. For example, assuming both source NG-RAN node and neighboring NG-RAN nodes are performing worse at Time N, if the resource status of neighboring cell is predicted becoming better at Time M and the neighboring cell could also handle the traffic from the selected UE(s) properly, AI/ML model should provide this predicted handover time, i.e. Time M, for the corresponding handover decision as the output of model inference. The predicted time stamp to handover is related to the corresponding time stamp of predicted resource status of current and neighboring NG-RAN node.

- The predicted time stamp to handover is generated as output of AI/ML-based load balancing use case.

Handover Validity Time

During RAN3 #114e meeting, it was FFS whether validity time is applied to all outputs produced by the Model Inference function.

Till RAN3 #114e meeting, following information were agreed as output of AI/ML-based load balancing:

•	Selection of target cell for mobility load balancing

•	Predicted own resource status information: this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

•	Predicted resource status information signalled from neighbor NG-RAN node(s): this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

•	Validity time for the Model Inference output predictions. FFS whether validity time is applied to all outputs produced by the Model Inference function.

The validity time represents how long the predicted results are valid. Meanwhile, after admission control, the target NG-RAN node may also temporarily hold resource pre-allocated for the selected UE(s) during this validity time window. If validity time window expires, the target NG-RAN node can release pre-allocated resource for the predicted handed-over UE(s). From the source NG-RAN node point of view, the corresponding target NG-RAN node and handover decision is not valid after validity time window expires.

When multiple outputs are considered for AI/ML-based load balancing, it can be considered as a multi-agent decision making problem, where all output decisions are generated at the same time based on the same set of input data (in either time domain or data set). Hence, the validity time of all output decisions is the same.

- When multiple outputs are considered for AI/ML-based load balancing, validity time of all output decisions is the same.

Based on above observation, the validity time should be corresponding to the set of AI/ML-based load balancing decisions.

- The validity time is corresponding to the set of AI/ML-based load balancing outputs and applied to all outputs produced by the Model Inference function.

A text proposal for the above solutions and standard impact is provided in Annex A.

- Agree the corresponding text proposal to TR 37.817 in Annex A.

# Conclusion

In this contribution, we mainly discussed the output of AI/ML-based load balancing.

We propose the following observations and proposals:

Proposal 1: The predicted selected UE(s) to be handed-over to target NG-RAN node is generated as output of AI/ML-based load balancing use case.

Observation 1: The handover decision for AI/ML-based load balancing can be performed in future after predicted output(s) are generated from Model Inference, which can further help to reduce service interruption during handover.

Proposal 2: The predicted time stamp to handover is generated as output of AI/ML-based load balancing use case.

Observation 2: When multiple outputs are considered for AI/ML-based load balancing, validity time of all output decisions is the same.

Proposal 3: The validity time is corresponding to the set of AI/ML-based load balancing outputs and applied to all outputs produced by the Model Inference function.

Proposal 4: Agree the corresponding text proposal to TR 37.817 in Annex A.

# References

- R3-216278, Draft TR37.817 Study on enhancement for Data Collection for NR and EN-DC

# Annex A – Text Proposal to TR 37.817

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# 5       Use Cases and Solutions for Artificial Intelligence in RAN

## 5.2	Load Balancing

### 5.2.1	Use case description

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 5.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

The following solutions can be considered for supporting AI/ML-based load balancing:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM.

Other possible locations of the AI/ML Model Inference are FFS.

To improve the load balancing decisions at a gNB (gNB-CU), a gNB can request load predictions from a neighbouring node. Details of the procedure are FFS.

If existing UE measurements are needed by a gNB for AI/ML-based load balancing, RAN3 shall reuse the existing framework (including MDT and RRM measurements). FFS on whether new UE measurements are needed.

To increase the awareness of the traffic dynamics and enable more improved traffic steering decisions it is possible to complement load measurements currently exposed over RAN interfaces with information related to predicted load from neighbouring RAN nodes as well as UE measurements and information.

- An NG-RAN node can also predict its own load. This can be achieved by considering the own load and load information received from neighbour RAN nodes. Load predictions can be signalled between RAN nodes.

- An NG-RAN node can also derive load prediction using UE measurements and information, for example MDT and RRM measurements, or UE location information (e.g. velocity, position). For the aspects concerning the configuration and the reporting of UE measurements and information the impacted protocol is RRC. RAN2 needs to be consulted for details during the normative phase.

Signalling of information used to derive Model Inference outputs may be achieved over the Xn interface by reusing existing or new procedures.  The details are to be discussed during normative work.

A high-level signalling flow for the AI/ML use case related to Load Balancing is shown in Figure 5.2-2-1 below.

Figure 5.2-2-1: AI/ML for Load Balancing use case

- Step 1: an AI/ML Model Training is located at NG-RAN node 1. NG-RAN node 2 is assumed to have capabilities in providing NG-RAN node 1 with useful input information, such as predicted resource status and/or mobility predictions.

- Steps 2-3: NG-RAN node 1 can request and obtain UE measurements and location information (e.g. RRM measurements, MDT measurements, velocity, position).

- Step 4-5: NG-RAN node 1 can request Resource Status information from the neighbouring NG-RAN node 2. Details and name of the procedure are FFS.

- Step 6: NG-RAN node 1 can perform Mobility Load Balancing predictions (e.g. for cells of NG-RAN node 1).

- Step 7: NG-RAN node 1 takes Mobility Load Balancing decision and UEs are moved from NG-RAN node 1 to NG-RAN node 2.

- Step 8: NG-RAN node 2 sends Feedback to NG-RAN node 1 (e.g. resource status updates after load balancing). It is FFS whether “Feedback” is signalled after receiving a Feedback Request.

#### 5.2.2.1	Input of AI/ML-based Load Balancing

To predict the optimized load balancing decisions, NG-RAN may need following information as input data for AI/ML-based load balancing:

From the local node:

- Own resource status information (e.g. per cell, per SSB Area): e.g., this can be calculated using predictions of some or all of the resource information specified in current XnAP

- Predicted own resource status information: e.g., this can be calculated using predictions of some or all of the resource information specified in current XnAP

From the UE:

- UE location information (e.g. from RLF reports, SCG Failure Information, Successful Handover Report)

- UE Radio Measurements, e.g., RSRP, RSRQ, SINR

- UE Mobility History Information

From neighbour NG-RAN Nodes:

- Neighbour resource status information (e.g. per cell, per SSB Area): it may include, e.g., some or all of the resource information in current Xn: Resource Status Update procedure

- Predicted neighbour resource status information: this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

Editor’s Note: FFS other input information required for AI/ML-based load balancing.

#### 5.2.2.2	Output of AI/ML-based Load Balancing

AI/ML-based load balancing model can generate following information as output:

- Selection of target cell for mobility load balancing

- Predicted own resource status information: this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

- Predicted resource status information signalled from neighbor NG-RAN node(s): this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

- Predicted selected UE(s) to be handed-over to target NG-RAN node

- Predicted time stamp to handover to target NG-RAN node

- Validity time for the Model Inference output predictions. Validity time is corresponding to the set of outputs and applied to all outputs produced by the Model Inference function.

Editor’s Note: FFS other output information expected from AI/ML-based load balancing.

#### 5.2.2.3	Feedback of AI/ML-based Load Balancing

To optimize the performance of AI/ML-based load balancing model, following feedback can be considered to be collected from NG-RAN nodes:

- UE performance information from target NG-RAN (for those UEs handed over from the source NG-RAN node)

- Resource status information updates from target NG-RAN

Editor’s Note: FFS other feedback expected from AI/ML-based load balancing