3GPP TSG-RAN WG3 #114bis-e		                                   R3-220545

17th – 26th Jan, 2022

Online

Source:	CATT

Title:	TP for 38.413 on session management for broadcast service

Agenda Item:	22.2.2

Document for:	Discussion and Decision

Introduction

In previous RAN3 meeting, some agreements are reached on session management for broadcast. In this contribution, we provide TP for 38.413 on session management for broadcast service.

Discussion

It is agreed that the following three procedures should be introduced for Broadcast service.

-	Broadcast Session Resource Setup;

-	Broadcast Session Resource Release;

For Broadcast session update, it was ever discussed whether one single update procedure could be used for both broadcast and multicast. Technically speaking, it is feasible to support the update of Broadcast and multicast service using one single procedure. However, considering the mechanism for broadcast and multicast is different, it seems clearer to use separate procedure for broadcast and multicast. So, we have a preference to define a broadcast session Resource Update procedure.

Proposal 1: It is proposed to have separate procedure for broadcast session update and multicast session update.

With this, 3 following procedures should be introduced for broadcast service:

-	Broadcast Session Resource Setup;

-	Broadcast Session Resource Release;

-	Broadcast Session Resource Update;

The TP for 38.413 is provided

Text Proposal to 38.413

- 8	NGAP Procedures

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Resource Setup | BROADCAST SESSION RESOURCE SETUP REQUEST | BROADCAST SESSION RESOURCE SETUP RESPONSE | BROADCAST SESSION RESOURCE SETUP FAILURE |
| Broadcast Session Resource Update | BROADCAST SESSION RESOURCE UPDATE REQUEST | BROADCAST SESSION RESOURCE UPDATE RESPONSE | BROADCAST SESSION RESOURCE UPDATE FAILURE |
| Broadcast Session Resource Release | BROADCAST SESSION RESOURCE RELEASE REQUEST | BROADCAST SESSION RESOURCE RELEASE RESPONSE |  |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |

8.x	Broadcast Session Management Procedure

8.x.1	Broadcast Session Resource Setup

- 8.x.1.1	General

The purpose of the Broadcast Session Resource Setup procedure is to request the gNB to notify UEs about an upcoming MBS Session of an given broadcast serivce.

The procedure uses non-UE associated signalling.

- 8.x.1.2	Successful Operation

Figure 8.x.1.2-1. Broadcast Session Resource Setup procedure. Successful operation.

The AMF initiates the procedure by sending an BROADCAST SESSION RESOURCE SETUP REQUEST message to the gNB. If the gNB accepts the Broadcast session resource setup request, the gNB responds with the BROADCAST SESSION RESOURCE SETUP RESPONSE message. If IP multicast is used,the gNB shall select and join the IP multicast (identified by the IP multicast address and the IP address of the multicast source as described by the TNL Information IE or, if present, the Alternative TNL Information IE) to enable the reception of broadcast data.Otherwise,the gNB should include the Transport Layer Information IE in the BROADCAST SESSION RESOURCE SETUP RESPONSE message.

The gNB shall report to the AMF in the BROADCAST SESSION RESOURCE SETUP RESPONSE message the result for the broadcast session resource requested to be setup.Upon reception of the BROADCAST SESSION RESOURCE SETUP RESPONSE message the AMF shall, transfer transparently the MBS Session Resource Setup Response Transfer IE or MBS Session Resource Setup Failure Transfer IE to the MB-SMF associated with the concerned broadcast session.

- 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1. Broadcast Session Resource Setup procedure. Unsuccessful operation.

If the gNB is not capable of correctly processing the request(e.g. the MBS resources could not be established at all in any cell), the gNB shall send BROADCAST SESSION RESOURCE SETUP FAILURE message.

- 8.x.1.4	Abnormal Conditions

Void.

8.x.2	Broadcast Session Resource Update

- 8.x.2.1	General

The purpose of the Broadcast Session Resource Update procedure is to request gNB to update the information related to the previously established MBS session.

The procedure uses non-UE associated signalling.

- 8.x.2.2	Successful Operation

Figure 8.x.2.2-1. Broadcast Session Resource Update procedure. Successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION RESOURCE UPDATE REQUEST message to the gNB.

If the MBS Service Area IE is included in the BROADCAST SESSION RESOURCE UPDATE REQUEST message, the gNB shall update the MBS service area and send the BROADCAST SESSION RESOURCE UPDATE RESPONSE message.

If the MBS Session Resource Update Request Transfer IE is included in the BROADCAST SESSION RESOURCE UPDATE REQUEST message, the gNB shall update the MBS context and resources and send the BROADCAST SESSION RESOURCE UPDATE RESPONSE message.

- 8.x.2.3	Unsuccessful Operation

Figure 8.x.2.3-1. Broadcast Session Resource Update procedure. Unsuccessful operation.

If the gNB is not capable of correctly processing the request, the gNB shall send BROADCAST SESSION RESOURCE UPDATE FAILURE message.

- 8.x.2.4	Abnormal Conditions

Void.

- 8.x.3		Broadcast Session Resource Release

- 8.x.3.1	General

The purpose of the Broadcast Session Resource Release procedure is to release the MBS context corresponding to the previous established broadcast session.

The procedure uses non-UE assocated signalling.

- 8.x.3.2	Successful Operation

Figure 8.x.3.2-1. Broadcast Session Resource Release procedure. Successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION RESOURCE RELEASE REQUEST message to the gNB.

Upon receiption of the BROADCAST SESSION RESOURCE RELEASE REQUEST message, the gNB shall respond with the BROADCAST SESSION RESOURCE RELEASE RESPONSE message. The gNB node shall disable the reception of MBS data and release the affected resources and remove the MBS bearer context.

- 8.x.3.3	Abnormal Conditions

Void.

============ Next change ==============

- 9.2.x		Broadcast Session Management Messages

- 9.2.x.1	BROADCAST SESSION RESOURCE SETUP REQUEST

This message is sent by the AMF to establish a MBS context .

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.z |  |  |  |
| Service Area | M |  | 9.3.4.y |  |  |  |
| MBS Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Setup Request Transfer IE specified in subclause 9.3.4.x |  |  |

- 9.2.x.2 BROADCAST SESSION RESOURCE SETUP RESPONSE

This message is sent by the gNB to report the successful outcome of the request from the BROADCAST SESSION RESOURCE REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.Z |  |  |  |
| MBS Session Resource Setup Response Transfer | O |  | OCTET STRING | Containing the MBS Session Resource Setup Response Transfer IE specified in subclause 9.3.4.xx |  |  |

- 9.2.x.3	BROADCAST SESSION RESOURCE SETUP FAILURE

This message is sent by the gNB to report the unsuccessful outcome of the request from the BROADCAST SESSION SETUP REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Setup Failure Transfer | O |  | OCTET STRING | Containing the MBS Session Resource Setup Failure Transfer IE specified in subclause 9.3.4.z |  |  |

- 9.2.x.4	BROADCAST SESSION RESOURCE UPDATE REQUEST

This message is sent by the AMF to modify a MBS context of broadcast service.

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.z |  |  |  |
| Service Area | O |  | 9.3.4.y |  |  |  |
| MBS Session Resource Update Request Transfer | O |  | OCTET STRING | Containing the MBS Session Resource Updata Request Transfer IE specified in subclause 9.3.4.xx |  |  |

- 9.2.x.5	BROADCAST SESSION RESOURCE UPDATE RESPONSE

This message is sent by the gNB to report the successful outcome of the request from the BROADCAST SESSION RESOURCE UPDATE REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.z |  |  |  |
| MBS Session Resource Update ResponseTransfer | O |  | OCTET STRING | Containing the MBS Session Resource Update Response Transfer IE specified in subclause 9.3.4.yy |  |  |

- 9.2.x.6	BROADCAST SESSION RESOURCE UPDATE FAILURE

This message is sent by the gNB to report the unsuccessful outcome of the request from the BROADCAST SESSION RESOURCE UPDATE REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Update Failure Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Modify Failure Transfer IE specified in subclause 9.3.4.zz |  |  |

- 9.2.x.7	BROADCAST SESSION RESOURCE RELEASE REQUEST

This message is sent by the AMF to release the corresponding broadcast session context and the MBS-service-associated logical NG connection.

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.z |  |  |  |
| MBS Session Resource Release Request Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Release Request Transfer IE specified in subclause 9.3.4.xxx |  |  |

- 9.2.x.8	BROADCAST SESSION RESOURCE RELEASE RESPONSE

This message is sent by the gNB to acknowledge the BROADCAST SESSION RESOURCE RELEASE REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.z |  |  |  |
| MBS Session Resource Release Response Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Release Response Transfer IE specified in subclause 9.3.4.YYY |  |  |

- 9.3.4.Y	Service area

The service area indicate the area within which data of one or multiple MBS session(s) are sent.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Choice Service area |  |  |  |  |
| >Cell |  |  |  |  |
| >> Cell list for service area | M |  |  |  |
| >>> Cell item |  | 1..<maxnoofCellsinservicearea> |  |  |
| >>>> NR CGI |  |  | 9.3.1.7 |  |
| >TAI |  |  |  |  |
| >>TAI list for service area | M |  |  |  |
| >>>TAI item |  | 1..<maxnoofTAIsinservicearea> |  |  |
| >>>>TAI |  |  | 9.3.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinservicearea | Maximum no. of cells in one service area. Value is FFS |
| maxnoofTAIsinservicearea | Maximum no. of TAIs in one service area. Value is FFS. |

- 9.3.4.Z	TMGI

The TMGI uniquely identifies the MBMS Bearer Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMGI |  |  |  |  |
| >PLMN identity | M |  | 9.3.3.5 |  |
| >Service ID | M |  | OCTET STRING (SIZE (3)) |  |

- 9.3.4.X	MBS Session Resource Setup Request Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TNL Information | O |  |  |  |  |  |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >GTP DL TEID | M |  | 9.3.2.5 |  |  |  |
| QoS Flows To Be Setup List |  | 1 |  |  |  |  |
| >QoS Flows To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |  |  |

- 9.3.4.Y	MBS Session Resource Setup Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP Transport Layer Information | O |  | 9.3.2.2 |  |  |  |

- 9.3.4.Z	MBS Session Resource Setup Failure Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |  |  |

- 9.3.4.XX	MBS Session Resource Update Request Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TNL Information | O |  |  |  |  |  |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >GTP DL TEID | M |  | 9.3.2.5 |  |  |  |
| QoS Flows To Be Setup List |  | 0…1 |  |  |  |  |
| >QoS Flows To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |  |  |

- 9.3.4.YY	MBS Session Resource Update Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP Transport Layer Information | O |  | 9.3.2.2 |  |  |  |

- 9.3.4.ZZ	MBS Session Resource Update Failure Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |  |  |

- 9.3.4.XXX	MBS Session Resource Release Command Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | O |  | 9.3.1.2 |  |  |  |

- 9.3.4.YYY	MBS Session Resource Release Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| FFS | O |  |  |  |  |  |