3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220841

Electronic Meeting, January 17th – 26th, 2022

Agenda item:	24.2

Source: 	Intel Corporation

Title: 	(TP for RA-SDT BL CR for TS 38.300)

Document for:	Other

# 1		Introduction

This contribution proposes text proposal for RA-SDT BL CR for TS 38.300 [1] based on enhancements proposed for RACH SDT as discussed in [2] and [3].

# 2		Reference

[1] R3-220076, “RA-SDT BLCR to TS 38.300", ZTE

[2] R3-220839, "(TP for RA-SDT BL CR for TS 38.423) Discussion on RACH based SDT", Intel Corporation

[3] R3-220840, "(TP for RA-SDT BL CR for TS 38.423) Assistance Information from New gNB for RACH based SDT", Intel Corporation

# 3		Text Proposal for RA-SDT BLCR to TS 38.300 [1]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## XX.1	Support of RACH based SDT

For RACH based SDT, the UE in RRC_INACTIVE initiates RACH and requests RRC resume together with UL SDT data. If the UE accesses a gNB other than the last serving gNB, the UL SDT data is buffered at the receiving gNB before the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. The receiving gNB may also indicate its relocation preference to the last serving gNB to be taken into account by the last serving gNB when deciding whether to relocate the UE context or not. Other SDT assistance information may also be provided by the receiving gNB to help the decision.

Editor’s Note: Details of assistance information is pending to future discussion.

Editor’s Note: UL SDT data may also need to be buffered in other cases, details are pending to the discussion.

If the last serving gNB decides not to relocate the UE context, at least information necessary for the receiving gNB to handle SDT is retrieved to the receiving gNB.

Editor’s Note: FFS on details of the retrieved information and how in case of no relocation.

If the last serving gNB decides not to relocate the UE context, in case SDT is used for signalling, SRB PDCP PDUs is transferred between the receiving gNB and the last serving gNB via the XnAP SRB TRANSFER message.

### XX.2	RA-based SDT with UE context relocation

The overall procedure for RACH based small data transmission with UE context relocation is illustrated in the figure XX.2-1.

Figure XX.2-1. RA-based SDT with UE context relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB locates the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE requested for SDT and may also provides SDT assistance information.

Editor’s Note: Details of assistance information is pending to future discussion.

3.  The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered to the UPF.

4-6. The receiving gNB decides to keep UE in RRC Inactive state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses. The receiving gNB also initiates NG-AP Path Switch Request procedure to establish a NG UE-associated signalling connection to the serving AMF. After path switch, the UL NAS PDU, if any, is delivered to the AMF.

Editor’s Note: FFS on how and when to establish forwarding tunnel for DL SDT data.

7. After SDT is completed, the receiving gNB generates and sends RRCRelease including the Suspend configuration to the UE to move the UE back to RRC_INACTIVE.

8. The receiving gNB indicates the last serving gNB to remove the UE context by sending XnAP UE CONTEXT RELEASE message.

Editor’s Note: Details of the whole procedure is FFS.

### XX.3	RA-based SDT without UE context relocation

The following figure describes the UE initiated RACH based SDT procedure when the UE is still within the configured RNA and the last serving gNB decides not to relocate the UE context.

Figure XX.3-1: RA-based SDT without UE context relocation

Editor's note: The detailed descriptions for each step are FFS.