# 3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221465

# Online, January 17th – 26th 2022

Title:	LS on Support for Configuration and Reporting of RAN Visible QoE Measurements

Response to:	-

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN3

To:	RAN2

Cc:	SA4

Contact Person:

Name: 		Filip Barać

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall description:

RAN3 agreed to support the RAN visible QoE. The consequent RRC impact, in addition to the impact indicated in previous LSs to RAN2, is captured in the following RAN3#114bis-e agreements:

- Include PDU session ID in RAN Visible QoE report, FFS on Slice information.

- RAN visible QoE reports and legacy QoE reports can use different periodicity, the reporting periodicity can be ms120, ms240, ms480, ms640, ms1024, FFS for (ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, min60).

- During handover preparation, source NG-RAN node sends to the target NG-RAN node:

  - (WA) in RRC container: RAN visible QoE metrics configured at the UE

- Target node may generate new RAN visible QoE configuration and send to UE during handover or RRC resume procedure.

The PDU session ID(s) corresponding to the service that is measured should be transmitted from UE to NG-RAN in a PDU session List, since one QMC measurement session may be associated with one or more PDU sessions. The format of PDU Session ID is INTEGER (0..255), and it is provided per measConfigAppLayerId.

The second agreement implies that a separate RAN visible QoE reporting periodicity in the RAN visible QoE configuration may need to be indicated from NG-RAN to the UE, when applicable.

In addition, RAN2 has requested RAN3 to provide more explanations about why to put higher priority on RAN visible QoE measurements and more clarifications about RAN visible QoE measurements usage in the reply LS (R3-220111/R2-211160).

RAN3’s understanding is that RAN visible QoE reports, which include the related RAN visible QoE metrics, could be utilized by the NG-RAN node for radio network optimization during an ongoing application/QMC session. However, there is no consensus in RAN3 with respect to whether the delivery of RAN visible QoE reports is with a higher priority than legacy QoE reporting, and the final decision with respect to which SRB should be used for RAN visible QoE reporting can be made by RAN2.

# 2. Actions:

RAN3 respectfully asks RAN2 to take the above RAN3 agreements into account and provide the necessary RRC signalling support.

# 3. Date of next TSG RAN WG3 meeting:

RAN3#115-e                         February 21st - March 3rd 2022		Online