3GPP TSG-RAN WG3 Meeting #114bis-e	 R3-221575

# Online,  17th – 26th January, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0729 | Rev | 2 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MultiSIM support over Xn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Samsung, ZTE, Huawei, vivo, Radisys, Reliance JIO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Support MUSIM in Rel 17 XnAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include Paging Cause for RAN Pagining procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for MUSIM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.2, 9.1.1.7, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR TS 38.473 CR TS 38.463 CR TS 36.413 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2: resubmission, including more co-signing companies. |  |  |  |  |  |  |  |  |

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards an ng-eNB (e.g. NG-RAN node2).

When available, the NG-RAN node1 shall include the Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

#### 8.2.5.3	Unsuccessful Operation

Not applicable.

#### 8.2.5.4	Abnormal Condition

Void.

*******************************

Skip to the next Change

*******************************

#### 9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34]. | YES | ignore |
| Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| Paging Cause | O |  | ENUMERATED(voice, …) | Coding is FFS |  |  |

*******************************

Skip to the next Change

*******************************

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

*******************************

Skip to the next Change

*******************************

id-NRV2XServicesAuthorized,

id-oldNG-RANnodeUEXnAPID,

id-OldtoNewNG-RANnodeResumeContainer,

id-PagingCause,

id-PagingDRX,

*******************************

Skip to the next Change

*******************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue				CRITICALITY reject		TYPE UEIdentityIndexValue							PRESENCE mandatory}|

{ ID id-UERANPagingIdentity					CRITICALITY ignore		TYPE UERANPagingIdentity							PRESENCE mandatory}|

{ ID id-PagingDRX							CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea						CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-PagingPriority						CRITICALITY ignore		TYPE PagingPriority									PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging			CRITICALITY ignore		TYPE AssistanceDataForRANPaging					PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging			CRITICALITY ignore		TYPE UERadioCapabilityForPaging					PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue		CRITICALITY ignore		TYPE ExtendedUEIdentityIndexValue				PRESENCE optional }|

{ ID id-PagingeDRXInformation				CRITICALITY ignore		TYPE PagingeDRXInformation						PRESENCE optional }|

{ ID id-UESpecificDRX						CRITICALITY ignore		TYPE UESpecificDRX									PRESENCE optional }|

{ ID id-PagingCause						    CRITICALITY ignore		TYPE PagingCause									PRESENCE optional },

...

}

*******************************

Skip to the next Change

*******************************

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

*******************************

Skip to the next Change

*******************************

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PagingCause ::= ENUMERATED {

voice,

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

*******************************

Skip to the next Change

*******************************

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

*******************************

Skip to the next Change

*******************************

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-PagingCause																						ProtocolIE-ID ::= 25x

END

-- ASN1STOP