3GPP TSG-RAN WG3 Meeting #115-e	R3-221685

# Online, February 21st – March 3rd 2022

Agenda Item:	13.3.2

Source:	Ericsson

Title:	(TP for IAB BL CR for TS 38.473): Inter-Donor-DU Tunnelling

Document for:	Agreement

# Introduction

At the RAN3#114bis-e meeting, the following agreements and TBCs were captured:

The static tunnel can be configured by implementation or by donor-CU. Discussions on CU-based configuration are stopped for Rel17.

The release of IP prefixes and/or a list of IP address(es) of UL rerouted packets is left to donor-DU2 implementation.

CU1 sends to CU2 a list of potential IP prefixes and/or IP address(es) present in the source field of the UL packets to be transmitted from CU2’s donor-DU to CU1’s donor-DU

Whether the tunnel type is up to implementation.

RAN3 further discusses how target donor-DU knows which tunnel a re-routed UL packet should be delivered to.

This paper proposes a way forward on the open issues related to inter-donor-DU UL rerouting.

# Discussion

Open issues related to inter-donor-DU UL rerouting are discussed below.

At the RAN3#114bis-e meeting, the following TBCs were captured:

Whether the tunnel type is up to implementation.

RAN3 further discusses how target donor-DU knows which tunnel a re-routed UL packet should be delivered to.

Given that only one meeting is left in this WI, and that there is not enough time for a thorough analysis of all the alternatives, we propose that the above issues are left to implementation.

Proposal 1: The inter-donor-DU tunnel type is up to implementation.

Proposal 2: It is up to implementation how an IAB-donor-DU knows which inter-donor-DU tunnel a packet should be delivered to.

As discussed in the paper R3-221682, submitted to the AI 13.2.2, the RAN3 framework for inter-donor-DU routing can be also used for avoidance of descendant IP address reconfiguration. As argued in R3-221682, CU1 needs to configure donor-DU1 with the list of destination IP addresses to be tunnelled towards donor-DU2. In that respect, RAN3 already agreed:

Target donor-DU determines the UL packet to be re-routed, by comparing IP prefixes and/or a list of IP address(es) configured by donor-CU, and the source address field of the UL IP packet.

In the context of UL inter-donor routing, the IP addresses exempt from IP filtering are source addresses. To enable inter-donor-DU routing of DL traffic as well, the addresses indicated to the donor-DU are destination addresses. So, an additional F1AP flag is needed to indicate whether the IP prefixes/addresses are source or destination addresses, which corresponds to UL and DL traffic, respectively.

The analysis in R3-221682 concludes the following:

Conclusion: The only additional specification impact for enabling the avoidance of descendant node IP address reconfiguration is an indication to the donor-DU of whether an IP address/prefix subject to tunnelling/exempt from IP address filtering is a source or a destination address/prefix.

So, in the F1AP and XnAP signalling for indicating to the donor-DU a list of IP prefixes/addresses subject to tunnelling, it is also signalled whether the IP address/prefix is a source or a destination address/prefix.

We note that the F1-Terminating Topology BH Information IE from the IAB BL CR for TS 38.423 already contains the DL TNL Address IE, meaning that the flag is needed only on F1AP.

Proposal 3: The F1AP signalling for informing the donor-DU about the IP addresses/prefixes subject to tunnelling/exempt from filtering contains an indication of whether the IP address/prefix is a source or a destination IP address/prefix.

A TP for IAB BL CR for TS 38.473 reflecting the proposal above is presented in the Annex.

Proposal 4: Agree the TP for IAB BL CR for TS 38.473, presented in the Annex.

# Conclusion

In this paper we discuss the remaining issues on inter-donor-DU rerouting. We conclude and propose the following:

Proposal 1: The inter-donor-DU tunnel type is up to implementation.

Proposal 2: It is up to implementation how an IAB-donor-DU knows which inter-donor-DU tunnel a packet should be delivered to.

Conclusion: The only additional specification impact for enabling the avoidance of descendant node IP address reconfiguration is an indication to the donor-DU of whether an IP address/prefix subject to tunnelling/exempt from IP address filtering is a source or a destination address/prefix.

Proposal 3: The F1AP signalling for informing the donor-DU about the IP addresses/prefixes subject to tunnelling/exempt from filtering contains an indication of whether the IP address/prefix is a source or a destination IP address/prefix.

Proposal 4: Agree the TP for IAB BL CR for TS 38.473, presented in the Annex.

# Annex: TP for IAB BL CR for TS 38.473

-------------------------------------------Start of changes-------------------------------------------

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| IAB TNL Address Exemption |  | IAB TNL ADDRESS EXEMPTION |  |

-------------------------------------------Next change-------------------------------------------

## 8.10	IAB Procedures

>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<

### 8.10.x	IAB TNL Address Exemption

#### 8.10.x.1		General

The purpose of the IAB TNL Address Exemption procedure is informing the gNB-DU about the TNL addresses that are exempt from TNL address filtering. This procedure is only applicable to IAB.

The procedure uses non-UE associated signalling.

#### 8.10.x.2		Successful Operation

Figure 8.10.x.2-1: IAB TNL Address Exemption procedure

The IAB TNL Address Exemption procedure is initiated by the gNB-CU, to inform the gNB-DU about the source and destination TNL addresses and/or prefixes that are exempt from TNL packet filtering at the gNB-DU.

#### 8.10.x.3		Abnormal Conditions

Void.

-------------------------------------------Next change-------------------------------------------

## 9.2.9	IAB messages

>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<

### 9.2.9.a	IAB TNL ADDRESS EXEMPTION

This message is sent by the gNB-CU to provide the list of TNL addresses to be exempt from TNL address filtering at the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| IAB TNL Address Exemption List |  | 1 |  |  | YES | reject |
| >IAB TNL Address Exemption Item |  | 1..<maxnoofTLAsIAB> |  |  | EACH | ignore |
| >>IAB TNL Address | M |  | 9.3.1.102 |  | - |  |
| >>Direction | O |  | ENUMERATED (UL, DL) | The value ”UL” corresponds to source TNL address. The value “DL” corresponds to destination TNL address. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum no. of IPv6 addresses or IPv6 address prefixes and/or individual IPv4 addresses that can be exempt from TNL address filtering. The value is 1024. |

-------------------------------------------End of changes-------------------------------------------