3GPP TSG-RAN WG3 Meeting #114-e	R3-221695

21 February 2022 – 3 March 2022

Online

Agenda Item:	18.4.1

Source:	InterDigital

Title:	Closing open issues in AI/ML Energy Saving use case

Document for:	Discussion and Agreement

# 1	Introduction

The study item Enhancement for Data Collection for NR and EN-DC has progressed 3 use cases individually over the past several meetings. Each use case has open issues and errors that need to be resolved before the end of the study which is after this meeting. This contribution tries to address those aspects to help complete the study.

# 2	Discussion

#### 2.1 FFS in Input from neighbouring NG-RAN nodes

There is an FFS on this topic:

-	FFS Accept/reject of offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons

Our understanding is that the offloading plan is a result of the model inference of both OAM and non-OAM cases. After the inference, there is a possible mobility strategy to be executed. For every handover type, the source node will request from the target node to handover a UE. The target node(s) is then going to perform admission control. This by itself is a validation of the offloading plan. Moreover, the purpose of using ML techniques in these use cases is to create some form of automation of processes, and the feedback loops serves the purpose of correcting them. The main value of this proposal is to allow for the NG-RAN node 2 to pre-approve the model’s handovers basically a pre-handover group call admission control concept. We feel this would perhaps be worth discussing. Therefore, we propose three options for this FFS:

- Completely remove the full sentence, i.e. reject the optimization of pre-sharing of the offloading plan or group CAC

- Remove “FFS”, agree on a reasonable name for a procedure, and add the standards impact for this procedure in section 5.1.2.7 accordingly.

- Since this input would only occur after the Energy Savings prediction, the acceptance/rejection of the offloading plan is actually part of feedback not inputs, and thus this text without the FFS should be moved to the feedback section, and also added to the standards impact section..

Proposal 1: To properly address the FFS on the offloading plan the following 3 options are possible:

- Completely remove the full sentence, i.e. reject the optimization of pre-sharing of the offloading plan or group CAC

- Remove “FFS”, agree on a reasonable name for a procedure, and add the standards impact for this procedure in section 5.1.2.7 accordingly.

- Since this input would only occur after the Energy Savings prediction, the acceptance/rejection of the offloading plan is actually part of feedback not inputs, and thus this text without the FFS should be moved to the feedback section, and also added to the standards impact section.

We support option c here since it fits better as feedback, but could support option b.

#### 2.2 FFS in Output of AI/ML-based Network Energy Saving

There is an FFS on this topic:

-	FFS Validity time of the predicted energy saving decisions

The validity time of the prediction when made in NG-RAN node 1 and then acted upon in NG-RAN node 1 is less critical then if NG-RAN node 1 predicted and then is dependent on NG-RAN node 2 to act within a validity window. We should also consider that the deployed models may be called upon for inference purposes at unknown rates, depending on the target optimization taking place. For example, the NG-RAN node may infer an output (to be executed upon within a validity time), but then receive a measurement report where it realizes the UE situation has completely changed. Therefore, there are other ways to consider the validity of the predictions, not necessarily by standardizing a parameter for it. So, we feel this could or could not be included as an output. We feel it is worth pointing out that there is a similar quantity addressed in the other use cases. We propose that either way, the decision to keep this should be synchronized with the Load Balancing use case.

Proposal 2: Align the existence and description (if any) of Validity time as an output in the Energy Savings use case with the load balancing use case. The options are:

- Remove “Validity time” as an output.

- Remove “FFS” and keep it as an output.

- Replace “FFS” with “Validity time (internal node use only)”.

- Replace “FFS” with “Validity time use outside the internal node will be discussed during the work item phase”.

We propose option c, to recognize it as a parameter but used internally for this use case. However, we can accept any of the other options if aligned with the load balancing use case.

#### 2.3 Clarifications

#### 2.3.1	UE measurements in message flows

The message flow diagram in this section shows a box for step 2 named “Measurement”. Since the UE can be potentially asked to perform multiple measurements for input to the model, we feel plural would be more suitable. This is located in two places both in section 5.1.2.2 and 5.1.2.3.

Proposal 3: In both section 5.1.2.2 and 5.1.2.3, Rename the box in the message flow chart in step 2 to “Measurement(s)”, and modify the description of step 2 as The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells. Likewise in step 3 and step 7/8 the reports can be plural, thus the sentence should be modified as The UE sends the measurement report message(s) to NG-RAN node 1.

#### 2.3.2	Model Training in message flows

Step 5 in the descriptions of training in sections 5.1.2.2 reads as:

Step 5: Model Training at OAM. Required measurements are leveraged to train AI/ML models for network energy saving.

A better description would be (to match what is written in step 4) with the inclusion of input data from other NG-RAN nodes is:

Step 5: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for network energy saving.

Proposal 4: In section 5.1.2.2, Modify the description of step 5 as:

Step 5: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for network energy saving.

# 3	Conclusion

Our proposals:

Proposal 1: To properly address the FFS on the offloading plan the following 3 options are possible:

- Completely remove the full sentence, i.e. reject the optimization of pre-sharing of the offloading plan or group CAC

- Remove “FFS”, agree on a reasonable name for a procedure, and add the standards impact for this procedure in section 5.1.2.7 accordingly.

- Since this input would only occur after the Energy Savings prediction, the acceptance/rejection of the offloading plan is actually part of feedback not inputs, and thus this text without the FFS should be moved to the feedback section, and also added to the standards impact section.

We support option c here since it fits better as feedback, but could support option b.

Proposal 2: Align the existence and description (if any) of Validity time as an output in the Energy Savings use case with the load balancing use case. The options are:

- Remove “Validity time” as an output.

- Remove “FFS” and keep it as an output.

- Replace “FFS” with “Validity time (internal node use only)”.

- Replace “FFS” with “Validity time use outside the internal node will be discussed during the work item phase”.

We propose option c, to recognize it as a parameter but used internally for this use case. However, we can accept any of the other options if aligned with the load balancing use case.

Proposal 3: In both section 5.1.2.2 and 5.1.2.3, Rename the box in the message flow chart in step 2 to “Measurement(s)”, and modify the description of step 2 as The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells. Likewise in step 3 and step 7/8 the reports can be plural, thus the sentence should be modified as The UE sends the measurement report message(s) to NG-RAN node 1.

Proposal 4: In section 5.1.2.2, Modify the description of step 5 as:

Step 5: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for network energy saving.

# 4. Annex – TP for 37.817

## 5.1	Network Energy Saving

### 5.1.1	Use case description

To meet the 5G network requirements of key performance and the demands of the unprecedented growth of the mobile subscribers, millions of base stations (BSs) are being deployed. Such rapid growth brings the issues of high energy consumption, CO2 emissions and operation expenditures (OPEX). Therefore, energy saving is an important use case which may involve different layers of the network, with mechanisms operating at different time scales.

Cell activation/deactivation is an energy saving scheme in the spatial domain that exploits traffic offloading in a layered structure to reduce the energy consumption of the whole radio access network (RAN). When the expected traffic volume is lower than a fixed threshold, the cells may be switched off, and the served Ues may be offloaded to a new target cell.

Efficient energy consumption can also be achieved by other means such as reduction of load, coverage modification, or other RAN configuration adjustments. The optimal EE decision depends on many factors including the load situation at different nodes, RAN nodes capabilities, KPI/QoS requirements, number of active Ues and UE mobility, cell utilization, etc.

However, the identification of actions aimed at energy efficiency improvements is not a trivial task. Wrong switch-off of the cells may seriously deteriorate the network performance since the remaining active cells need to serve the additional traffic. Wrong traffic offload actions may lead to a deterioration of Energy Efficiency instead of an improvement. The current energy-saving schemes are vulnerable to potential issues listed as follows:

- Inaccurate cell load prediction. Currently, energy-saving decisions rely on current traffic load without considering future traffic load.

- Conflicting targets between system performance and energy efficiency. Maximizing the system’s key performance indicator (KPI) is usually done at the expense of energy efficiency. Similarly, the most energy efficient solution may impact system performance. Thus, there is a need to balance and manage the trade-off between the two.

- Conventional energy-saving related parameters adjustment. Energy-saving related parameters configuration is set by traditional operation, e.g., based on different thresholds of cell load for cell switch on/off which is somewhat a rigid mechanism since it is difficult to set a reasonable threshold.

- Actions that may produce a local (e.g. limited to a single RAN node) improvement of Energy Efficiency, while producing an overall (e.g. involving multiple RAN nodes) deterioration of Energy Efficiency.

To deal with issues listed above, ML techniques could be utilized to leverage on the data collected in the RAN network. ML algorithms may predict the energy efficiency and load state of the next period, which can be used to make better decisions on cell activation/deactivation for ES. Based on the predicted load, the system may dynamically configure the energy-saving strategy (e.g. the switch-off timing and granularity, offloading actions) to keep a balance between system performance and energy efficiency and to reduce the energy consumption.

### 5.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 5.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network energy saving:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 5.1.2.2	Model Training at OAM and Model Inference at NG-RAN

In this solution, NG-RAN makes energy decisions using AI/ML model trained from OAM.

Figure 5.1.2.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM.

Step 5: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for network energy saving.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-17 scope.

Step 7: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 8: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 9: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output(s) (e.g. energy saving strategy, handover strategy, etc).

Step 10: NG-RAN node 1 sends Model Performance Feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 11: NG-RAN node 1 may select the most appropriate target cell for each UE before it performs handover, if the output is handover strategy.

Step 12: NG-RAN node 2 provides feedback to OAM.

Step 13: NG-RAN node 1 provides feedback to OAM.

#### 5.1.2.3	Model Training and Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and generates energy saving decisions.

Figure 5.1.2.2-1. Model Training and Model Inference at NG-RAN

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to NG-RAN node 1 including the required measurement result.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based network energy saving.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based energy saving based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based energy saving optionally, which can also generate predicted results/actions.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g. energy saving strategy, handover strategy, etc).

Step 9: NG-RAN node 1 may select the most appropriate target cell for each UE before it performs handover, if the output is handover strategy.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

#### 5.1.2.4	Input of AI/ML-based Network Energy Saving

To predict the optimized network energy saving decisions, NG-RAN may need following information as input data for AI/ML-based network energy saving:

Input Information from Local node:

- UE mobility/trajectory prediction

- Current/Predicted Energy efficiency

- Current/Predicted resource status

Input Information from UE:

-      UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

Input from neighbouring NG-RAN nodes:

- Current/Predicted energy efficiency

- Current/Predicted resource status

- Current energy state (e.g., active, high, low, inactive)

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements).

#### 5.1.2.5	Output of AI/ML-based Network Energy Saving

AI/ML-based network energy saving model can generate following information as output:

- Energy saving strategy, such as recommended cell activation/deactivation.

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy efficiency

- Predicted energy state (e.g., active, high, low, inactive)

- Validity time of the predicted energy saving decisions (internal node use only)

#### 5.1.2.6	Feedback of AI/ML-based Network Energy Saving

To optimize the performance of AI/ML-based network energy saving model, following feedback can be considered to be collected from NG-RAN nodes:

- Resource status of neighbouring NG-RAN nodes

- Energy efficiency

- UE performance affected by the energy saving action (e.g. handed-over UEs), including bitrate, packet loss, latency.

- System KPIs (e.g. throughput, delay, RLF of current and neighboring NG-RAN node)

- Response from offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons

#### 5.1.2.7	Standard Impact

Potential Xn interface impact:

- New signalling procedure or enhanced existing procedure to collect the input data information

  - Predicted energy efficiency between neighboring NG-RAN nodes and source NG-RAN node

  - Predicted resource status between neighboring NG-RAN nodes and source NG-RAN node

New signalling procedure or enhanced existing procedure to retrieve feedback information including at least:

- Offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons