3GPP TSG-RAN WG3 Meeting #114-e	R3-221697

21 February 2022 – 3 March 2022

Online

Agenda Item:	18.4.3

Source:	InterDigital

Title:	Closing open issues in AI/ML Mobility Optimization use case

Document for:	Discussion and Agreement

# 1	Introduction

The study item Enhancement for Data Collection for NR and EN-DC has progressed 3 use cases individually over the past several meetings. Each use case has open issues and errors that need to be resolved before the end of the study which is after this meeting. This contribution tries to address those aspects to help complete the study.

# 2	Discussion

#### 2.1 Treatment of existing FFSs in the TR

#### 2.1.1 FFS on the need for new UE measurements

The input data section has the following sentence at the very end:

If existing UE measurements are needed by a gNB for AI/ML-based mobility optimization, RAN3 shall reuse the existing framework (including MDT and RRM measurements). FFS on whether new UE measurements are needed.

It is clear that the scope of the study has not, so far, included new measurements. However, new measurements or new parameters in mobility procedures have not been ruled out yet, but more importantly, they haven't been studied either. This makes it clear that there isn’t yet an understanding of whether the current measurement framework is able to provide enough data for proper model training. Model training in supervised learning is renowed for requiring, in many instances, large amounts of data. It is also well known that some data features in training phase are more highly correlated to the target prediction, than others. It is therefore fundamental that this study is at the very least kept open, under penalty that some target optimizations cannot be executed properly, based on the lack of completeness and low quantity of the collected data. Excluding a further assessment related to UE generated data, when in all use cases, it is the “RRC Measurement Report” message that is being used to transport this UE generated data, is an unnecessary limitation with plenty of potential to create difficulties when creating proper datasets that network nodes can use to train models or even to perform inference.

We consider this a fundamental requirement for all the AI/ML use cases – to at least study whether new UE measurements or parameters are needed to be considered, especially in the context of the content of the "RRC Measurement Report” message.

We therefore propose two options for this FFS to be removed

Proposal 1:

- At the end of the input data section remove the FFS in “FFS on whether new UE measurements are needed”, by changing it to: “Whether new measurements are needed for input or new parameters for the existing mobility procedures are needed due to AI/ML model impacts or feedback is to be discussed during the work item phase.” This should be aligned with the decision made for other use cases.

- Alternatively, and because this is true for the other use cases, the whole FFS should be deleted here and a general statement like the proposed could be put in a general section the best one being the conclusion section.

We favour option b but can accept option a

#### 2.1.2 Standard impact – FFS on details of feedback from a neighbouring node

In the feedback and standard impact section, there are two FFSs related to the details for a gNB requesting feedback from a neighbour node. We feel feedback information is a fundamental part of any optimization process and should therefore be included. We propose to remove therefore the acronym “for further study” in both instances and keep the study of the feedback procedure for the working item phase.

Proposal 2: In the feedback section, replace the sentence “Performance information from target NG-RAN, FFS on performance information details” with “Performance information from target NG-RAN, performance information details to be agreed during the work item phase”

Proposal 3: In the standard impact section, replace the sentence “To improve the mobility decisions at a gNB (gNB-CU), a gNB can request mobility feedback from a neighbouring node. Details of the procedure are FFS.” with “To improve the mobility decisions at a gNB (gNB-CU), a gNB can request mobility feedback from a neighbouring node, details to be agreed during the work item phase”

#### 2.2 FFS treatment of “to be continued” inputs/outputs

#### 2.2.1 Validity time

The validity time of the prediction when made in NG-RAN node 1 and then acted upon in NG-RAN node 1 is less critical then if NG-RAN node 1 predicted and then is dependent on NG-RAN node 2 to act within a validity window. We should also consider that the deployed models may be called upon for inference purposes at unknown rates, depending on the target optimization taking place. For example, the NG-RAN node may infer an output (to be executed upon within a validity time), but then receive a measurement report where it realizes the UE situation has completely changed. Therefore, there are other ways to consider the validity of the predictions, not necessarily by standardizing a parameter for it. So, we feel this could or could not be included as an output. We feel it is worth pointing out that there is a similar quantity addressed in the Energy savings and Load Balancing use cases. We propose that either way, the decision to keep this should be synchronized with these use cases. However, since the mobility optimization is not dealing with moving groups of UEs, it is likely even less needed than the other two use cases

Proposal 4: Align the existence and description (if any) of Validity time as an output in the Mobility optimization use case with the other use cases. The options are:

- Don’t add “Validity time” as an output.

- Add “Validity time” as an output.

- Add “Validity time (internal node use only)”.

- Add “Validity time use outside the internal node will be discussed during the work item phase”.

We propose option c, to recognize it as a parameter but used internally for this use case. However, we can accept any of the other options if aligned with the other use cases.

#### 2.2.2 UE trajectory prediction

The output data section has the following FFS:

-	Note: FFS whether the UE trajectory prediction is an internal output to the node hosting the Model Inference function

The UE trajectory predicitons are very likely to be made easier by the UE itself. NG-RAN nodes would need some extra information from the UE to be able to properly predict its trajectory. If the UE can make these predictions, then it would be more efficient that the UE is simply able to provide them, as opposed to sourcing some data to NG-RAN nodes, so that NG-RAN nodes can then predict the trajectory. It should also be pointed out that a basic assumption of working with data to train models, is that that data provides mobility patterns of UEs. These patterns will only show up in the context of data analytics or trained AI/ML models if there is some form of repetition on the UEs’ behavior. Repetition here aims exactly at the UEs mobility patterns. Contiguous NG-RAN nodes have valuable radio planning information in order to establish/understand what these patterns are. Moreover, the UE trajectory can be extremely beneficial in terms of resource preparation in the context of any HO type. Therefore, for all these reasons, we do not think there is any benefit (quite the opposite) in introducing a limitation on which node is able to produce these predictions, as well as having nodes exchanging this information. The scope of this current work doesn’t consider the UE using AI/ML to assist in determining its trajectory, but since the source NG-RAN node can and does consider it, passing it along to the target NG-RAN node makes sense.

Proposal 5a: In the output data section, remove “Note: FFS whether the UE trajectory prediction is an internal output to the node hosting the Model Inference function”.

Proposal 5b: In the standard impact section, add “UE Mobility/Trajectory from target NG-RAN” as Potential Xn interface impact

Proposal 5c: In the feedback section include the updated UE trajectory received at NG-RAN node 1, received from NG-RAN node 2, so that NG-RAN node 1 can improve its predictions

#### 2.3.4 Unsuccessful Handover input

In the input data section, the first input from neighbouring NG-RAN nodes mentions UE successful handover information, but for completeness it should also include unsuccessful handover since an RLF report could also be used to help the AI/ML model. Both reports are important as they can contain relevant information for the assessment of the model’s performance, but an unsuccessful handover report (RLF Report) would even be more relevant, in the sense that it would help the NG-RAN node 1 to understand faster, via feedback (where the delta for the feedback is very likely higher), that the attained mobility decision was not optimal. In the successful case, the feedback would, for the same reasons, also be important. However, because the end result is positive in a way (the handover was completed without problems), then the adjustment would be naturally smaller.

Proposal 6: In the Input Information from the neighbouring RAN nodes section modify -	“UE’s successful handover information in the past and received from neighboring RAN nodes” to – “UE’s successful and unsuccessful handover information in the past and received from neighboring RAN nodes.”

#### 2.3.5 Traffic prediction output

This use case is about mobility optimization in general. Based on current discussions and state of TR 37.817, it is clear that companies agree to include current and predicted UE traffic as part of the relevant inputs for optimizing the mobility process. The traffic needs of each UE in mobility dictate the need to activate features such as traffic steering and data forwarding. When the current optimal mobility decision is made at NG-RAN node 1, it is likely that the UE’s current and predicted traffic dictate the need to activate one of these features. Both of these features require resource reservation at the target NG-RAN node(s) which is accepted in 3GPP as a problem worth addressing, being addressed in release 15 and continuing to today.

Therefore, these features should be considered when devising the list of inputs/outpuits for model training and inference. In order to consider the traffic steering and data forwarding features in mobility and make these an integral part, as they are, of the mobility process, the following output should also be added:

Proposal 7a: in the output data section to handle the traffic steering part of the use case and help with data forwarding decisions, add an output as follows: “Traffic predictions for resource allocation purposes in mobility (e.g., for CA/DC activation/deactivation and Data Forwarding related decision optimization)”

Proposal 7b: in the standard impact section to handle the traffic steering part of the use case and help with data forwarding decisions, add in potential Xn impacts “Traffic predictions for resource allocation purposes between source and target NG-RAN nodes”

#### 2.5 Clarification of Predicted handover target node

One final clarification should be made, the mobility use case should cover all handover types. Since CHO has a group of candidate cells and DAPS and regular handover have one target cell, the output dealing with predicted handover target should have some minor changes, so that all handover types are covered clearly. Moreover, the confidence of the prediction is an important aspect that should be considered and that could influence the decision-making process. We propose therefore a rewording of the predicted handover output data bullet.

Proposal 8: In the output data section clarify “Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication” by modifying to “Predicted handover target node for regular and DAPS handover, candidate cells in for CHO, may together with the confidence of the prediction.

# 3	Conclusion

Our proposals are:

Proposal 1:

- At the end of the input data section remove the FFS in “FFS on whether new UE measurements are needed”, by changing it to: “Whether new measurements are needed for input or new parameters for the existing mobility procedures are needed due to AI/ML model impacts or feedback is to be discussed during the work item phase.” This should be aligned with the decision made for other use cases.

- Alternatively, and because this is true for the other use cases, the whole FFS should be deleted here and a general statement like the proposed could be put in a general section the best one being the conclusion section.

We favour option b but can accept option a

Proposal 2: In the feedback section, replace the sentence “Performance information from target NG-RAN, FFS on performance information details” with “Performance information from target NG-RAN, performance information details to be agreed during the work item phase”

Proposal 3: In the standard impact section, replace the sentence “To improve the mobility decisions at a gNB (gNB-CU), a gNB can request mobility feedback from a neighbouring node. Details of the procedure are FFS.” with “To improve the mobility decisions at a gNB (gNB-CU), a gNB can request mobility feedback from a neighbouring node, details to be agreed during the work item phase”

Proposal 4: Align the existence and description (if any) of Validity time as an output in the Mobility optimization use case with the other use cases. The options are:

- Don’t add “Validity time” as an output.

- Add “Validity time” as an output.

- Add “Validity time (internal node use only)”.

- Add “Validity time use outside the internal node will be discussed during the work item phase”.

We propose option c, to recognize it as a parameter but used internally for this use case. However, we can accept any of the other options if aligned with the other use cases.

Proposal 5a: In the output data section, remove “Note: FFS whether the UE trajectory prediction is an internal output to the node hosting the Model Inference function”.

Proposal 5b: In the standard impact section, add “UE Mobility/Trajectory from target NG-RAN” as Potential Xn interface impact

Proposal 5c: In the feedback section include the updated UE trajectory received at NG-RAN node 1, received from NG-RAN node 2, so that NG-RAN node 1 can improve its predictions

Proposal 6: In the Input Information from the neighbouring RAN nodes section modify -	“UE’s successful handover information in the past and received from neighboring RAN nodes” to –“UE’s successful and unsuccessful handover information in the past and received from neighboring RAN nodes.”

Proposal 7a: in the output data section to handle the traffic steering part of the use case and help with data forwarding decisions, add an output as follows: “Traffic predictions for resource allocation purposes in mobility (e.g., for CA/DC activation/deactivation and Data Forwarding related decision optimization)”

Proposal 7b: in the standard impact section to handle the traffic steering part of the use case and help with data forwarding decisions, add in potential Xn impacts “Traffic predictions for resource allocation purposes between source and target NG-RAN nodes”

Proposal 8: In the output data section clarify “Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication” by modifying to “Predicted handover target node for regular and DAPS handover, candidate cells in for CHO, may together with the confidence of the prediction.

# 4. Annex – TP for 37.817

## 5.3	Mobility Optimization

### 5.3.1	Use case description

Mobility management is the scheme to guarantee the service-continuity during the mobility by minimizing the call drops, RLFs, unnecessary handovers, and ping-pong. For the future high-frequency network, as the coverage of a single node decreases, the frequency for UE to handover between nodes becomes high, especially for high-mobility UE. In addition, for the applications characterized with the stringent QoS requirements such as reliability, latency etc., the QoE is sensitive to the handover performance, so that mobility management should avoid unsuccessful handover and reduce the latency during handover procedure. However, for the conventional method, it is challengeable for trial-and-error-based scheme to achieve nearly zero-failure handover. The unsuccessful handover cases are the main reason for packet dropping or extra delay during the mobility period, which is unexpected for the packet-drop-intolerant and low-latency applications. In addition, the effectiveness of adjustment based on feedback may be weak due to randomness and inconstancy of transmission environment. Besides the baseline case of mobility, areas of optimization for mobility include dual connectivity, CHO, and DAPS, which each have additional aspects to handle in the optimization of mobiltity.

Mobility aspects of SON that can be enhanced by the use of AI/ML include

- Reduction of the probability of unintended events

- UE Location/Mobility/Performance prediction

- Traffic Steering

Reduction of the probability of unintended events associated with mobility.

Examples of such unintended events are:

- Intra-system Too Late Handover: A radio link failure (RLF) occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

- Intra-system Too Early Handover: An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

- Intra-system Handover to Wrong Cell: An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

- Successful Handover: During a successful handover, there is underlying issue.

RAN Intelligence could observe multiple HO events with associated parameters, use this information to train its ML model and try to identify sets of parameters that lead to successful Hos and sets of parameters that lead to unintended events.

UE Location/Mobility/Performance Prediction

Predicting UE’s location is a key part for mobility optimisation, as many RRM actions related to mobility (e.g. selecting handover target cells) can benefit from the predicted UE location/trajectory. UE mobility prediction is also one key factor in the optimization of early data forwarding particularly for CHO. UE Performance prediction when the UE is served by certain cells is a key factor in determining which is the best mobility target for maximisation of efficiency and performance.

Traffic Steering

Efficient resource handling can be achieved adjusting handover trigger points and selecting optimal combination of Pcell/PSCell/Scells to serve a user.

Existing traffic steering can also be improved by providing a RAN node with information related to mobility or dual connectivity.

For example, before initiating a handover, the source gNB, could use feedbacks on UE performance collected for successful handovers occurred in the past and received from neighboring gNBs.

Similarly, for the case of dual connectivity, before triggering the addition of a secondary gNB or triggering SN change, an eNB could use information (feedbacks) received in the past from the gNB for successfully completed SN Addition or SN Change procedures.

In the two reported examples, the source RAN node of a mobility event, or the RAN node acting as Master Node (a eNB for EN-DC, a gNB for NR-DC) can use feedbacks received from the other RAN node, as input to an AI/ML function supporting traffic related decisions (e.g. selection of target cell in case of mobility, selection of a PSCell / Scell(s) in the other case), so that future decisions can be optimized.

### 5.3.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 5.3.2.1			Locations for AI/ML Model Training and AI/ML Model Inference

Considering the locations of AI/ML Model Training and AI/ML Model Inference for mobility solution, the following two options are considered:

- The AI/ML Model Training function is deployed in OAM, while the Model Inference function resides within the RAN node

- Both the AI/ML Model Training function and the AI/ML Model Inference function reside within the RAN node

Furthermore, for CU-DU split scenario, following option is possible:

- AI/ML Model Training is located in CU-CP or OAM, and AI/ML Model Inference function is located in CU-CP

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM.

#### 5.3.2.2			AI/ML Model Training in OAM and AI/ML Model Inference in NG-RAN node

Figure 5.3-1  AI/ML Model Training in OAM and AI/ML Model Inference in NG-RAN node

Step 0. NG-RAN node 2 is assumed to optionally have an AI/ML model, which can generate required input such as resource status and utilization prediction/estimation etc.

Step 1. The NG-RAN node configures the measurement information on the UE side and sends configuration message to UE including configuration information.

Step 2. The UE collects the indicated measurement, e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3. The UE sends measurement report message to NG-RAN node 1 including the required measurement.

Step 4. The NG-RAN node 1 sends the input data for training to OAM, where the input data for training includes the required input information from the NG-RAN node 1 and the measurement from UE.

Step 5. The NG-RAN node 2 sends the input data for training to OAM, where the input data for training includes the required input information from the NG-RAN node 2. If the NG-RAN node 2 executes the AI/ML model, the input data for training can include the corresponding inference result from the NG-RAN node 2.

Step 6. Model Training. Required measurements are leveraged to training AI/ML model for UE mobility optimization.

Step 7. OAM sends AI/ML Model Deployment Message to deploy the trained/updated AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-17 scope.

Step 8. The NG-RAN node 1 obtains the measurement report as inference data for UE mobility optimization.

Step 9. The NG-RAN node 1 obtains the input data for inference from the NG-RAN node 2 for UE mobility optimization, where the input data for inference includes the required input information from the NG-RAN node 2. If the NG-RAN node 2 executes the AI/ML model, the input data for inference can include the corresponding inference result from the NG-RAN node 2.

Step 10. Model Inference. Required measurements are leveraged into Model Inference to output the prediction, e.g.  UE trajectory prediction, target cell prediction, target NG-RAN node prediction, etc.

Step 11. The NG-RAN 1 sends the model performance feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 12. According to the prediction, recommended actions or configuration are executed for Mobility Optimization.

Step 13. The NG-RAN node 1, the target NG-RAN node (represented by NG-RAN node 2 of this step in the flowchart), and UE do the handover procedure to hand over UE from NG-RAN node 1 to the target NG-RAN node.

Step 14. The NG-RAN node 1 sends the feedback information to OAM.

#### 5.3.2.3			AI/ML Model Training and AI/ML Model Inference in a NG-RAN node

Figure 5.3-2: Model Training and Model Inference both located in RAN node

Step 0. NG-RAN node 2 is assumed to optionally have an AI/ML model, which can generate required input such as resource status and utilization prediction/estimation etc.

Step 1. NG-RAN node1 configures the measurement information on the UE side and sends configuration message to UE including configuration information.

Step 2. UE collects the indicated measurement, e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3. UE sends measurement report message to NG-RAN node1 including the required measurement.

Step 4. The NG-RAN node 1 obtains the input data for training from the NG-RAN node2, where the input data for training includes the required input information from the NG-RAN node 2. If the NG-RAN node 2 executes the AI/ML model, the input data for training can include the corresponding inference result from the NG-RAN node 2.

Step 5. Model training. Required measurements are leveraged to training AI/ML model for mobility optimization.

Step 6. NG-RAN node1 obtains the measurement report as inference data for real-time UE mobility optimization.

Step 7. The NG-RAN node 1 obtains the input data for inference from the NG-RAN node 2 for UE mobility optimization, where the input data for inference includes the required input information from the NG-RAN node 2. If the NG-RAN node 2 executes the AI/ML model, the input data for inference can include the corresponding inference result from the NG-RAN node 2.

Step 8. Model Inference. Required measurements are leveraged into Model Inference to output the prediction, including e.g., UE trajectory prediction, target cell prediction, target NG-RAN node prediction, etc.

Step 9. According to the prediction, recommended actions are executed for Mobility Optimization.

Step 10. The NG-RAN node 1, the target NG-RAN node (represented by NG-RAN node 2 of this step in the flowchart), and UE do the handover procedure to hand over UE from NG-RAN node 1 to the target NG-RAN node.

Step 11. The NG-RAN node 2 sends feedback information after mobility optimization action to the NG-RAN node 1.

Note: UE mobility information for training purposes is only sent to gNBs that request such information or when triggered.

#### 5.3.2.4			Input data

The following data is required as input data for mobility optimization.

Input Information from the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by Gnb implementation when available.

- Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

- UE historical serving cells and their locations

- Moving velocity

Input Information from the neighbouring RAN nodes:

- UE’s successful and unsuccessful handover information in the past and received from neighboring RAN nodes

- UE’s history information from neighbor

- Position, resource status, QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc.)

- Resource status and utilization prediction/estimation

- SON Reports of handovers that are successful, too-early, too-late, or handover to wrong (sub-optimal) cell

- Information about the performance of handed over Ues

- Resource status prediction

Input Information from the local node:

- UE trajectory prediction output (will be used by the RAN node internally)

- Local resource status prediction

- Current/predicted UE traffic

If existing UE measurements are needed by a Gnb for AI/ML-based mobility optimization, RAN3 shall reuse the existing framework (including MDT and RRM measurements). Whether new measurements are needed for input or new parameters for the existing mobility procedures are needed due to AI/ML model impacts or feedback is to be discussed during the work item phase

#### 5.3.2.5			Output data

- UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time)

- Estimated arrival probability in CHO and relevant confidence interval

- Predicted handover target node for regular and DAPS handover, candidate cells for CHO, may together with the confidence of the prediction.

- Priority, handover execution timing, predicted resource reservation time window for CHO

- Traffic predictions for resource allocation purposes in mobility (e.g. for CA/DC activation/deactivation and Data Forwarding related decision optimization)

- Validity time (internal node use only)

          - 5.3.2.6			Feedback

The following data is required as feedback data for mobility optimization.

- QoS parameters such as throughput, packet delay of the handed-over UE, etc

- Resource status information updates from target NG-RAN

- Performance information from target NG-RAN, performance information details to be agreed during the work item phase.

- UE trajectory received at NG-RAN node 1, received from NG-RAN node 2

#### 5.3.2.7			Standard impact

To improve the mobility decisions at a Gnb (Gnb-CU), a Gnb can request mobility feedback from a neighbouring node, details to be agreed during the work item phase

Potential Xn interface impact:

  - Predicted resource status info and performance info from candidate target NG-RAN node to source NG-RAN node

  - Predicted UE trajectory info from source NG-RAN node to target NG-RAN node

  - Predicted UE traffic info from source NG-RAN node to target NG-RAN node

  - Traffic predictions for resource allocation purposes in mobility (e.g. for CA/DC activation/deactivation and Data Forwarding related decision optimization)

  - Current/predicted UE traffic

  - UE Mobility/Trajectory from target NG-RAN”

  - Traffic predictions for resource allocation purposes between source and target NG-RAN nodes.

  - New signaling procedure or existing procedure to retrieve input information via Xn interface.

  - New signaling procedure or existing procedure to retrieve feedback information via Xn interface.