3GPP TSG-RAN WG3 #115-e																	      R3-221792

21th Feb – 3rd Mar 2022

Online

Agenda Item:	30.2

Source:	ZTE

Title:	(TP for BLCR Introduction of MultiSIM support over S1) Support of Multi-SIM

Document for:	Approval

- Introduction

In the last RAN3 #114bis e-meeting, we have the following progress.

Name the Baseline CRs as “Introduction of MultiSIM support over NG or Xn or etc.”. Paging Cause is included in NGAP/XnAP/F1AP/E1AP/S1AP. The Coding is to be discussed further. Paging Cause support indication is included in NGAP. Both the name and the coding is to be discussed further. Paging Cause support indication over XnAP needed? No. It is agreed that the procedural text state that the “Paging Cause support indication “IE indicates “UE support”. No change need in the BL CR. It is agreed that the “Reject Paging” cause value in S1AP is not needed. BL CR should remove it. It is agreed that the “Paging Cause support indication” IE over E1AP is not introduced. It can be further discussed based on contribution. BL CR should remove it. It is agreed that the “MUSIM Gap” over F1AP is not introduced. It can be further discussed based on RAN2 progress. BL CR should remove it. The IEs related to the topics in the “Remaining issues to be discussed at RAN3#115-e” are marked as FFS in the BL CRs. It is agreed that the BL draft CR on 38.300 is needed (on Radisys). “Paging Cause support indication” IE: naming: Using “Paging Cause Indication for Voice Service Supported”, which is aligned with SA2; and Coding: One code point with extension mark is agreeable. Further: Using “supported, …” which is aligned with SA2. Paging Cause IE Coding. Choose from the below two Options: Option 1: “voice, …” Option 2: A new IE structure as in below: This IE indicates the paging cause for paging a UE. New Cause value over NGAP “Release due to MUSIM”, discuss if needed / what is the usage. To be continued...

This paper further discusses the remaining issues.

- Discussion

  - Paging Cause

In the LSin R2-2111330, RAN2 has agreed to introduce paging cause for voice service.

- Paging cause for service indication will be introduced in NR and LTE - RAN2 agreed to provide the paging cause value “voice” to UEs by extending the Paging message. The presence of the extension, which can be empty list if none of the UEs have paging cause “voice”, indicates that network supports transmission of paging cause for service indication.

According to above LSin, from UE’s point of view, when UE receiving Paging message.

- Case 1：If “Paing cause” not included, then UE judges the serving eNB/gNB is legacy eNB/gNB (i.e., not support Multi-SIM feature).

- Case 2：If “Paging cause” included and with value “voice”, then UE judges the serving eNB/gNB supports Multi-SIM feature and the service is IMS-voice.

- Case 3：If If “Paging cause” included but without value “voice” (i.e., value as empty), then UE judges the serving eNB/gNB supports Multi-SIM feature and the service is non IMS-voice.

How can the serving eNB/gNB decide to include “Paging cause” in the S1AP/NGAP Paging message and indicate the service is non IMS-voice (Case 3)?

Actually, the “Paging Cause” is generated by Core network (i.e., MME/AMF). For case 3, when the serving eNB/gNB receives S1AP/NGAP: Paging message from Core network, including “Paging Cause” IE with value“non IMS-voice”, it can page UE accordingly.

Proposal 1: Core network sends S1AP/NGAP: Paging message to eNB/gNB including “Paging Cause” IE. Furthermore, if the service is IMS-voice, then its sub-IE “Paging Cause” is set to “voice”, otherwise, its sub-IE “Paging Cause” is empty.

#### 9.2.1.xx Paging Cause This information element indicates the paging cause for paging a UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Paging Cause; O; ENUMERATED (voice, …); Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present but PagingRecord is presen, it implies that the corresponding paging entry is for a service other than IMS voice..

  - Paging Cause support indication

For the Paging Cause support indication, RAN3 has achieved the following progress in the last meeting.

Paging Cause support indication is included in NGAP. Both the name and the coding is to be discussed further. Paging Cause support indication over XnAP needed? No. It is agreed that the procedural text state that the “Paging Cause support indication “IE indicates “UE support”. No change need in the BL CR. It is agreed that the “Paging Cause support indication” IE over E1AP is not introduced. It can be further discussed based on contribution. BL CR should remove it. “Paging Cause support indication” IE: naming: Using “Paging Cause Indication for Voice Service Supported”, which is aligned with SA2; and Coding: One code point with extension mark is agreeable. Further: Using “supported, …” which is aligned with SA2.

The following CRs are attached in the SA2 in LS S2-2109303.

TS 23.501 (S2-2109301) The "RRC Inactive Assistance Information" includes: - UE specific DRX values; - UE specific extended idle mode DRX values (cycle length and Paging Time Window length); - The Registration Area provided to the UE; - Periodic Registration Update timer; - If the AMF has enabled MICO mode for the UE, an indication that the UE is in MICO mode; - Information from the UE identifier, as defined in TS 38.304 [50] for NR and TS 36.304 [52] for E-UTRA connected to 5GC, that allows the RAN to calculate the UE's RAN paging occasions; - An indication that Paging Cause Indication for Voice Service is supported. The RRC Inactive Assistance Information mentioned above is provided by the AMF during N2 activation with the (new) serving NG-RAN node (i.e. during Registration, Service Request, Handover) to assist the NG RAN's decision whether the UE can be sent to RRC Inactive state. If the AMF allocates a new Registration Area to the UE, the AMF should update the NG-RAN with the new Registration Area by sending the RRC Inactive Assistance Information accordingly. The Paging Cause Indication for Voice Service is used to assist NG RAN to perform RAN based paging.

Since SA2 has agreed to introduce a new IE “Paging Cause Indication for Voice Service” to assist NG RAN to perform RAN based paging, RAN3 can reuse the same IE name defined in SA2.

Proposal 2: Add a new IE “Paging Cause Indication for Voice Service” included in the “Core Network Assistance Information for RRC INACTIVE” IE.

#### 9.3.1.xy Paging Cause Indication for Voice Service This IE is used to notify whether the UE supports the feature of indication of paging cause for voice services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Paging Cause Indication for Voice Service; M; ENUMERATED (supported, …).

- Conclusion

Based on the above, RAN3 is requested to discuss and agree on the following proposals:

Proposal 1: Core network sends S1AP/NGAP: Paging message to eNB/gNB including “Paging Cause” IE. Furthermore, if the service is IMS-voice, then its sub-IE “Paging Cause” is set to “voice”, otherwise, its sub-IE “Paging Cause” is empty.

Proposal 2: Add a new IE “Paging Cause Indication for Voice Service” included in the “Core Network Assistance Information for RRC INACTIVE” IE.

TP for 36.413 is as below. TP for 38.413/38.423/38.473 is R3-221793

- Text Proposal for TS36.413 V16.8.0

---------------------------------------------Start Change--------------------------------------------------------------------------

## 8.5	Paging

### 8.5.1	General

The purpose of the Paging procedure is to enable the MME to page a UE in the specific eNB.

### 8.5.2	Successful Operation

Figure 8.5.2-1: Paging procedure

The MME initiates the paging procedure by sending the PAGING message to the eNB.

At the reception of the PAGING message, the eNB shall perform paging of the UE in cells which belong to tracking areas as indicated in the List of TAIs IE.

The CN Domain IE shall be transferred transparently to the UE.

The Paging DRX IE may be included in the PAGING message, and if present the eNB shall use it according to TS 36.304 [20].

A list of CSG IDs may be included in the PAGING message.

If included, the E-UTRAN may use the list of CSG IDs to avoid paging the UE at CSG cells whose CSG ID does not appear in the list.

For each cell that belongs to any of the TAs indicated in the List of TAIs IE, the eNB shall generate one page on the radio interface.

The Paging Priority IE may be included in the PAGING message, and if present the eNB may use it according to TS 23.401 [11] and TS 23.272 [17].

If the UE Radio Capability for Paging IE is included in the PAGING message, the eNB may use it to apply specific paging schemes. If the Enhanced Coverage Restricted IE is included in the PAGING message, the eNB shall, if supported, use it as defined in TS 23.401 [11].

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present according to TS 36.300 [14].

If the Assistance Data for CE capable UEs IE is included in the Assistance Data for Paging IE, it may be used for paging the indicated CE capable UE, together with the Paging Attempt Information IE according to TS 36.300 [14].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 36.300 [14].

If the Paging eDRX Information IE is included in the PAGING message, the eNB shall, if supported, use it according to TS 36.304 [20]. If the Paging Time Window IE is included in the Paging eDRX Information IE, the eNB shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [20]. The eNB should take into account the reception time of the PAGING message on the S1-MME interface to determine when to page the UE.

If the Extended UE Identity Index Value IE is included in the PAGING message, the eNB shall, if supported, use it to identify the paging resources to be used according to TS 36.304 [20]. The MME shall, if supported, include the Extended UE Identity Index Value IE in the PAGING message.

If the NB-IoT Paging eDRX Information IE is included in the PAGING message, the eNB shall, if supported, use it according to TS 36.304 [20]. If the NB-IoT Paging Time Window IE is included in the NB-IoT Paging eDRX Information IE, the eNB shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [20]. The eNB should take into account the reception time of the PAGING message on the S1-MME interface to determine when to page the UE.

If the NB-IoT UE Identity Index Value IE is included in the PAGING message, the eNB shall, if supported, use it to identify the paging resources to be used according to TS 36.304 [20].

If the CE-Mode-B Restricted IE is included in the PAGING message and the Enhanced Coverage Restricted IE is not set to restricted, the eNB shall use it as defined in TS 23.401 [11].

If the Data Size IE is included in the PAGING message, the eNB shall, if supported, use it to decide whether to initiate Mobile Terminated EDT procedures towards the UE as described in TS 36.300 [14].

If the WUS Assistance Information IE is included in the PAGING message, the eNB shall, if supported, use it to determine the WUS group for the UE, as specified in TS 36.304 [20].

If the NB-IoT Paging DRX IE is included in the PAGING message, the eNB shall use it according to TS 36.304 [20].

If the Paging Cause IE is included in the PAGING message, the NG-RAN node shall, if supported, transfer it to the UE according to TS 36.331 [21].

### 8.5.3	Unsuccessful Operation

Not applicable.

### 8.5.4	Abnormal Conditions

Not applicable.

9.2.1.xx	Paging Cause

This information element indicates the paging cause for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present, it implies that the corresponding paging entry is for a service other than IMS voice. |