3GPP TSG-RAN WG3#115-e                                             	                     R3-221809

E-Meeting, 21 February – 03 March 2022

Agenda item:	11.2

Source:	Nokia, Nokia Shanghai Bell

Title: 	(TP for TS 38.413) Coordination of Redcap Capability across gNBs with no Xn

Document for:	Approval

# Introduction

The RAN3 part for the work item of “support of redcap UEs” in [2] has started at RAN3#113. The following agreements were reached at RAN3#114bis:

- OAM is not precluded (may be sufficient in some deployments)

- Support the exchange of RedCap access configuration via Xn Setup/Configuration Update (solution 2)

- Support sending RedCap access configuration over F1AP

- For mobility between nodes without Xn, assume OAM, but allow to revisit next meeting also taking into account RACS discussion if applicable)

This paper continues the discussion on the mobility without Xn and potential solution beyond O&M.

# Discussion

As was explained at RAN3#114, there are two main scenarios to consider:

- The neighbour cell belongs to a gNB which does not support Redcap (1)

- The neighbour cell has turned on/off redcap barring e.g. due to overload. (2)

For (1) it was acknowledged that usually in a network the feature should be deployed on all gNBs or none. If deployed on all gNBs no solution is needed. If this is not the case, the support/non support will be rather static information and O&M setting would suffice. Therefore for (1) O&M is good enough.

For (2) it was concluded at RAN3#114bis that the barring of the cell could be more frequent in some networks and that O&M might not be good enough. The solution 2 using the Xn Setup/Configuration Update was agreed for the case where there is an Xn interface. Nevertheless, for the case where the Xn interface is not available, the solution 2 using the Xn Setup/Configuration Update cannot work and we need a solution which work with NG handover.

One solution could simply be to exchange the support of Redcap cell within the Target to Source container and Target to Source Failure Container of any NG handover messages.

#### 9.3.1.30	Target NG-RAN Node to Source NG-RAN Node Transparent Container

This IE is produced by the target NG-RAN node and is transmitted to the source NG-RAN node. For inter-system handovers to 5G, the IE is transmitted from the target NG-RAN node to the external relocation source.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Non Redcap Cells List |  | 0..1 |  |  | YES | ignore |
| >Non Redcap Cells Item |  | 1..<maxnoofCells> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.x |  | - |  |

In this solution, there could be two possible scenarios, considering a handover from cells of gNB1 to a cell 2 of a neighbour gNB2:

Scenario 1: cell 2 decides to bar redcap UEs, gNB1 not yet aware

Here, at the first handover from gNB1 to gNB2, gNB2 will update gNB1 about the redcap support of its cells.

There are two subcases:

- Subcase 1: This first handover happening from any cell of gNB1 to any cell of gNB2 is for a non-Redcap UE: the handover will succeed and gNB1 is informed of new cell2 status through the successful handover: no impact.

- Subcase 2: by lack of chance, the first handover happening from any cell of gNB1 to any cell of gNB2 concerns a Redcap UE handed over precisely to the cell 2 which has just turned on the barring: the handover is rejected and gNB1 is updated with the Redcap support of gNB2 cells including cell 2 status.

As can be seen from above, gNB1 is updated with this mechanism in all cases. There is however only one failure in the rare case where the ever first UE being handed from any cell of gNB1 to any cell of gNB2 happens to be a redcap UE moving to cell 2! We estimate this failure case as statistically rare and not a problem.

Scenario 2: cell 2 decides to “unbar” redcap UEs, gNB1 not yet aware

Here again, at the first handover from gNB1 to gNB2, gNB2 will update gNB1 about the redcap support of its cells

- Subcase 1: time T0 between cell 2 unbarring and any first one non-Redcap UE being handed over from any cell of gNB1 to any cell of gNB2: during this time T0, gNB1 refrains from handing over redcap UEs to cell 2 whereas cell 2 is unbarred. This is peharps not perfect but this is not worse than during all the time during which cell 2 was barred for redcap UEs. It is just as if redcap barring of cell 2 would last a little bit longer for UE under gNB1. Assuming that this new semi-dynamic update of barring was to fight congestion, we don’t think this drawback is big also considering that this time T0 should be short (see subcase 2).

- Subcase 2: first any non-Redcap UE of any cell of gNB1 is being handed over to any cell of gNB2 (i.e. not only cell 2) then the handover is successful and gNB1 gets in the successful return container the update of redcap support for gNB2 cells including cell 2 is unbarred now.

The above scenario shows that the piggyback solution in Target to Source container works with the following drawbacks:

- Rare failure cases for barring scenario 1,

- Small delay for informing the unbarring in scenario 2.

We evaluate these drawbacks as not critical and seldom, not jeopardizing the purpose and efficiency of the function.

Proposal 1: update the Redcap cell support via NGAP Target to Source NGAP Container when no Xn is available.

# Conclusion and Proposals

This paper has made some analysis on the remaining case without Xn interface available and concluded:

The above scenarios show that the piggyback solution in Target to Source container works with the following drawbacks:

- Rare failure cases for barring scenario 1.

- Small delay for informing the unbarring in scenario 2.

We evaluate these drawbacks as not critical and seldom, not jeopardizing the purpose and efficiency of the function.

Proposal 1: update the Redcap cell support via NGAP Target to Source NGAP Container when no Xn is available.

The TP is available below with limited impact on NGAP Target to Source NGAP Container IE:

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- RP-211574, Support for reduced capability NR devices

- R3-221142, Summary of discussions on Redcap, workplan, LS in, RAN3#114bis

# TP for 38.413 for Solution 2bis

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source NG-RAN node initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving AMF. When the source NG-RAN node sends the HANDOVER REQUIRED message, it shall start the timer TNGRELOCprep. The source NG-RAN node shall indicate the appropriate cause value for the handover in the Cause IE.

# Not modified

If the DAPS Request Information IE is included for a DRB in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE within the HANDOVER REQUIRED message, it indicates that the request concerns a DAPS Handover for that DRB, as described in TS 38.300 [8].

If the Non Redcap Cells List IE is included in the Target to Source Transparent Container IE within the HANDOVER COMMAND message, it indicates the list of cells to which handover of Redcap UEs should be prevented.

Interactions with other NGAP procedures:

# Not modified

#### 8.4.1.3	Unsuccessful Operation

Figure 8.4.1.3-1: Handover preparation: unsuccessful operation

If the 5GC or the target side is not able to accept any of the PDU session resources or a failure occurs during the Handover Preparation, the AMF sends the HANDOVER PREPARATION FAILURE message with an appropriate cause value to the source NG-RAN node.

If the Target to Source Failure Transparent Container IE has been received by the AMF from the handover target then the transparent container shall be included in the HANDOVER PREPARATION FAILURE message.

If the Target to Source Failure Transparent Container IE is received in the HANDOVER PREPARATION FAILURE message including the Cell CAG Information IE, the source NG-RAN node shall, if supported, store and replace the PNI-NPN information associated with the indicated cell.

If the Non Redcap Cells List IE is included in the Target to Source Failure Transparent Container IE within the HANDOVER PREPARATION FAILURE message, it indicates the list of cells to which handover of Redcap UEs should be prevented.

Interaction with Handover Cancel procedure:

# Not modified

#### 9.3.1.30	Target NG-RAN Node to Source NG-RAN Node Transparent Container

This IE is produced by the target NG-RAN node and is transmitted to the source NG-RAN node. For inter-system handovers to 5G, the IE is transmitted from the target NG-RAN node to the external relocation source.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC HandoverCommand message as defined in TS 38.331 [18] if the target is a gNB. Includes the RRC HandoverCommand message as defined in TS 36.331 [21] if the target is an ng-eNB. | - |  |
| DAPS Response Information List |  | 0..1 |  |  | YES | ignore |
| >DAPS Response Information Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>DRB ID | M |  | 9.3.1.53 |  | - |  |
| >>DAPS Response Information | M |  | 9.3.1.189 | Indicates the response to a requested DAPS Handover | - |  |
| Redcap Support | O |  |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

#### 9.3.1.187	Target NG-RAN Node to Source NG-RAN Node Failure Transparent Container

This IE is produced by the target NG-RAN node and is transmitted to the source NG-RAN node in case of preparation failure.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell CAG Information | O |  | 9.3.1.185 |  |  |  |
| Redcap Support | O |  |  |  | YES | ignore |

#### 9.3.1.x	Redcap Support

This IE is used to provide information on Redcap support of the gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Support | M |  |  |  |
| >all Cells |  |  | NULL | Indicates that Redcap support is available in all cells of the gNB. |
| >not All Cells |  |  |  |  |
| >>Non Redcap Cells List | M |  | 9.3.1.y |  |

#### 9.3.1.y	Non Redcap Cells List

The purpose of the Non Redcap Cells List IE is to provide the list of cells to which handover of Redcap UEs should be prevented.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Non Redcap Cells List |  | 1..<maxnoofCellsingNB> |  |  |
| >NR CGI | M |  | NR CGI 9.3.1.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsingNB | Maximum no. of cells in a gNB. Value is 16384. |

# Not modified

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

# Not modified

NonDynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDL	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUL	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

}

NonRedcapCellsList ::= SEQUENCE (SIZE(1.. maxnoofCellsingNB)) OF NR-CGI

NotAllowedTACs ::= SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC

NotificationCause ::= ENUMERATED {

fulfilled,

not-fulfilled,

...

}

# Not modified

RecommendedRANNodeItem ::= SEQUENCE {

aMFPagingTarget		AMFPagingTarget,

iE-Extensions		ProtocolExtensionContainer { {RecommendedRANNodeItem-ExtIEs} }	OPTIONAL,

...

}

RecommendedRANNodeItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RedcapSupport ::= CHOICE {

allCells		NULL,

notAllCells		NonRedcapCellsList,

choice-Extensions		ProtocolIE-SingleContainer { {RedcapSupport-ExtIEs} }

}

RedcapSupport-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

RedirectionVoiceFallback ::= ENUMERATED {

possible,

not-possible,

...

}

# Not modified

TargetID ::= CHOICE {

targetRANNodeID			TargetRANNodeID,

targeteNB-ID			TargeteNB-ID,

choice-Extensions		ProtocolIE-SingleContainer { {TargetID-ExtIEs} }

}

TargetID-ExtIEs NGAP-PROTOCOL-IES ::= {

{ID id-TargetRNC-ID		CRITICALITY reject	TYPE TargetRNC-ID PRESENCE mandatory },

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer ::= SEQUENCE {

rRCContainer		RRCContainer,

iE-Extensions		ProtocolExtensionContainer { {TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSResponseInfoList						CRITICALITY ignore	EXTENSION DAPSResponseInfoList	PRESENCE optional }|

{ ID id-RedcapSupport				CRITICALITY ignore	EXTENSION RedcapSupport	PRESENCE optional },

...

}

TargetNGRANNode-ToSourceNGRANNode-FailureTransparentContainer ::= SEQUENCE {

cell-CAGInformation		Cell-CAGInformation			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TargetNGRANNode-ToSourceNGRANNode-FailureTransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargetNGRANNode-ToSourceNGRANNode-FailureTransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RedcapSupport				CRITICALITY ignore	EXTENSION RedcapSupport	PRESENCE optional },	...

}

TargetRANNodeID ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {TargetRANNodeID-ExtIEs} } OPTIONAL,

...

}

TargetRANNodeID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

# Not modified

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

# Not modified

id-QosFlowFeedbackList									ProtocolIE-ID ::= 278

id-BurstArrivalTimeDownlink								ProtocolIE-ID ::= 279

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-PduSessionExpectedUEActivityBehaviour					ProtocolIE-ID ::= 281

id-MicoAllPLMN											ProtocolIE-ID ::= 282

id-QosFlowFailedToSetupList								ProtocolIE-ID ::= 283

id-RedcapSupport										ProtocolIE-ID ::= xxx

END

-- ASN1STOP