3GPP TSG-RAN WG3 Meeting #115-e	R3-221833

E-meeting, 21 Feb – 3 Mar 2022

Title: 	(TP for SON BLCR for 38.473, 38.401,38.300, 38.470, 38.423) Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	10.2.2

Document Type:	other

# 1. Introduction

There were some progress in RAN3#114bis:

Agree that the OAM configures the NG-RAN with Alternative Coverage Configurations, namely with the set of parameters defining the Cell/Beam configuration corresponding to each Cell/Beam Coverage State. Boundary ranges define the range of configuration changes for each alternative coverage configuration.

gNB-CU does not provide CCO coverage modification suggestions to the gNB-DU. Such agreement may be revisited when a decision on alternative/suitable coverage configurations from OAM is taken.

And the following open issues were identified

It is proposed to continue discussions on the following:

1)	Do companies see a need to signal neighbour Cell/Beam Coverage State values form the gNB-CU to the gNB-DU?

2)	If the answer to 1) is “no”, how should the gNB-DU deduce the Cell/Beam Coverage State of neighbour cells/beams and therefore adapt its cell/beam coverage to it?

In this paper we propose how to further progress with CCO and provide come new proposals and corrections to previously agreed text.

# 2. Background

During discussions in LTE, there where proposals trying to describe the coverage configuration in terms of coverage change, i.e. to indicate how much bigger the cell becomes. Some different options are possible here, e.g. describing how the parameters change (tilt, Tx power) or trying to describe the geographical shape of the cell. This would however require a lot from the access node – he would have to have access to radio planning tools/models to understand what a modified parameter would result in.

This is the main reason why a more abstract configuration indicator is selected in LTE. This gives an indication that a change has occurred and also gives a chance for the receiving node to learn what this configuration in a neighbour node means. It should also be noted that there is a relationship between the coverage configuration from OAM and what is signalled over X2. Therefore, by defining the different OAM configurations, the operator also has a control over what is signalled over X2.

Observation 1: In LTE, there was a relationship between the OAM configurations and the indicator used over X2.

Another important part is that the usage of this abstract indicator was specified only for MRO. When a new indicator is received, the node can store the current MRO state (all corrections and also all statistics that are used or not yet used to influence the mobility parameters) and retrieve the state matching the indicator that is received. This means that MRO does not have to start from scratch every time the coverage state in a neighbour node is updated. Other usage is of course not precluded, but there is no support for this in the LTE specification. In NR we are opening the door for adding new usage for the CCO indicator.

Observation 2. In LTE, usage of CCO exchange was specified only for MRO, but in NR we are discussing to add additional usage for the CCO indicator.

There are also some principles to keep in mind. The first principle is that coverage modifications are a sensitive subject for an operator. Changing the coverage pose the risk of creating coverage holes which have an immediate and negative impact on the end customer. Especially if the coverage problem is reoccurring and happening more frequently in certain locations.

Observation 3: Changing the coverage pose the risk of creating coverage holes which have an immediate and negative impact on the end customer.

Another important factor is that the requirements on coverage may be very strict. This could include regulator requirements. There could for example be a requirement to guarantee the coverage in one area but also a requirement not to interfere in a certain area.

Observation 4: There may be strict requirements associated with coverage.

Another thing that is worth discussing is the cell replacement functionality which was included in LTE and is also agreed for NR. The principle is that one node can indicate to a neighbour node that a cell is being replaced with another cell. In LTE the scenario was for example to switch between omni and 3.sector cells to reduce the power consumption. To reduce the impact of mobility, a functionality was added to warn a neighbour node of an upcoming change. This is captured as follows in stage2:

If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving eNB may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending eNB adds cells in inactive state, the receiving eNB may use this information to avoid connection or re-establishment failures.

The receiving node may also use the notification to reduce the impact on mobility. For example, the receiving eNB should avoid triggering handovers towards cell(s) that are indicated to be inactive.

The detailed behaviour is captured in stage3 as follows:

If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the eNB2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell. If the Cell Deployment Status Indicator IE is present and the Cell Replacing Info IE contains non-empty cell list, the eNB2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets. If the Cell Deployment Status Indicator IE is not present, the eNB2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE.

Observation 5: The cell replacement functionality from LTE is agreed for NR and provides a way to inform neighbours about replacing one or multiple cells with another cell to reduce impact on mobility.

# 3. Discussion

## 3.1 Issue 1- OAM configurations

We have already agreed to have alternative coverage configurations to define the freedom for a single node to modify the coverage configurations. In this section we will discuss the benefits of also including suitable coverage configurations, i.e. a list of which of these alternative coverage configurations work well together, or more specifically which combinations provide the required/intended coverage.

This can apply to intra-node operations but becomes more important in inter node scenarios. It would for example be desirable to make coverage modifications in one node and compensate this with changes in a neighbour node. This would allow for more freedom for the network to adjust to e.g. user distribution (to avoid having users on the cell edge) without being limited to compensating within the same node which is directly related to the selection of sites.

Cell1, Conf1

Cell2, Conf1

Cell1, Conf2

Cell2, Conf2

One first step is to simply use alternative coverage configurations with no further guidance which allows the extended/reduced coverage of cells handled by these nodes (see figure above).

But this poses a similar risk that was discussed in in issue1: by changing the coverage we may create coverage holes. This since only the serving node knows the resource situation and can know whether a compensation is possible. So in the example above, if cell 1 reduces the coverage and cell 2 is not able to meet this proposed change, there will be a coverage hole.

In a system with peer network nodes there is no way to control how the neighbour node compensates for a modified coverage in the neighbour cell. During discussions in LTE, there where proposals trying to describe the coverage configuration in terms of coverage change, i.e. to indicate how much bigger the cell becomes. This was however ruled out since it was seen impossible to characterize how modifications in one node would impact the coverage in the border area to a neighbour node. Hence, the coverage configuration that is signalled between nodes does not really indicate the nature of the change, but just provides and index to the configuration that is used.

This means that without any additional configuration from OAM, it would be a quite big task iterating all possible coverage configurations to identify the one that is most suitable for a specific configuration in the neighbour node. If there is no guiding information the only way to learn is to try and monitor the performance. But the risk is that this would create coverage holes during the learning process. We do not exclude that this would be feasible in some scenarios and especially employing AI/ML , but we think that we should also not exclude the possibility that OAM provides additional information to reduce any such impact.

If we take the example above into account, instead of just configuring each node with the alternative coverage configurations, we propose to also indicate a set of suitable coverage combinations.

For the example above, the configuration may look something like this:

Cell 1

AltCovConf

>Conf1

>Conf2

SuitCovComb

>Cell1/Conf1 – Cell2/Conf2

>Cell/Conf2 – Cell2/conf2

Cell 2

AltCovConf

>Conf1

>Conf2

SuitCovComb

>Cell1/Conf1 – Cell2/Conf2

>Cell/Conf2 – Cell2/conf2

Therefore we would propose to agree that whether a node can compensate for another node or not depends on the details of the alternative coverage configurations. We also propose that OAM can give an indication on suitable coverage combinations with neighbour cells. This indication could be binding (the node shall blindly follow this) or used as an indication to help the nodes quicker achieving the most optimal solution. This would allow for a more centrally controlled coverage modification where OAM and radio planning tools are involved to identify suitable configurations as well an enabling a more distributed scheme where we give more freedom to the NG-RAN node and advanced implementations to optimize the coverage.

Similar to the examples in the previous section, we can also have the case where these suitable coverage combinations are not defined. In this case, it would be up to each node to decide whether and how to compensate for coverage modifications in a neighbour cell.

OAM may provide a binding or informative list of suitable coverage combinations to be used with coverage configurations of cells in neighbour nodes.

Note that if we do not specify these suitable coverage combinations, we will lose the possibility to put requirements on nodes for jointly guaranteeing an aggregated coverage. This means the inter node coverage compensation cannot be used in cases where there are strict rules on coverage apply.

## 3.2 Issue 3 – CU/DU functionality split

In the previous discussions, we have mainly discussed the aggregated case, where the communication is on peer-to-peer basis. This approach could also be used for gNB-DUs. So the principles of configuration discussed above could apply, i.e. that the gNB-DU would be configured by OAM as outlined above with the alternative coverage configurations and the suitable coverage combinations.

But when we consider CU/DU split, the situation is slightly different. The gNB-CU is managing a set of gNB-DU. In deployments with virtualised nodes, the CU may handle a large number of DUs. The gNB-CU has a lot of knowledge about the problems occurring since he is managing the RRC (e.g. receives UE measurements) and will receive all failure event reports. The gNB-CU is also aware about the load situation in each cell. Therefore, we think the gNB-CU could play a different role especially when it comes to selecting suitable coverage combinations. As the central entity, the gNB-CU would have a more global view and could suggest to reconfigure multiple gNB-DU at once.

Hence the general idea would be that gNB-DU is configured with the alternative coverage configurations and the gNB-CU is configured with the suitable coverage combinations as illustrated below:

gNB-DU1

Cell 1

AltCovConf

>Conf1

>Conf2

gNB-DU2

Cell 2

AltCovConf

>Conf1

>Conf2

gNB-CU

SuitCovComb

>Cell1/Conf1 – Cell2/Conf2

>Cell/Conf2 – Cell2/conf2

AltCovConf

>Conf1

>Conf2

SuitCovComb

>Cell1/Conf1 – Cell2/Conf2

>Cell/Conf2 – Cell2/conf2

Similar to what was discussed above, this suitable coverage combinations could be binding or informative, i.e. the gNB-CU is either bound to use only these combinations or is free to use any but may use these as a starting point.

Based on the above discussion, we propose that for the CU/DU split, the gNB-CU is configured with the suitable coverage combinations and the gNB-DU is configured with the alternative coverage configurations and that we introduce signalling to support gNB-CU sending proposed coverage configurations to the gNB-DU.

gNB-CU may be configured with the suitable coverage combinations and may in that case send a set of proposed coverage configurations to the gNB-DU

Note that the configuration of suitable coverage combinations is optional and if absent, the decision would be fully distributed to the gNB-DU.

## 3.3 Stability of the system

We have agreed that it is always up to the gNB-DU to decide whether a configuration is possible to apply. This since it is only the gNB-DU that knows the resources available. The same problem would apply for coverage compensation case between aggregated gNB, where only the node handling the cell would know the resource situation.

One problem with allowing this kind of distributed final decisions for coverage compensation and at the same time controlling the suitable coverage configurations between nodes to avoid coverage holes is that we may end up in scenarios where one node wants to perform a change but the required compensation is not possible in the neighbour node.

The previously outlined suitable coverage combinations could provide a baseline for this negotiation. By exchanging the information between nodes, each node is aware about what the actions in one node means for other nodes. On the other hand, if this information is not available, there is no way for a node to judge whether his actions are reasonable or whether they will create a coverage hole.

If we first look at how the suitable coverage combinations could be used, a typical solutions in RAN3 for this kind of situation would be to introduce some form of negotiation prior to the change, or by exchanging a model of resource availability between nodes to increase the knowledge in the initiating node. This would however introduce more complexity and may lead to discussion regarding "which node would decide" or "which node needs this change the most".

Instead, we would propose to keep the solution as simple s as possible:

- One node (gNB or gNB-CU) initiate the change.

- Each involved node (gNB/gNB-DU) checks whether he can meet this proposed change, and acknowledges his corresponding change

- If one involved node is not able to perform the change,

  - this is indicated to the initiating node

  - The initiating node reverts back to previous configuration and informs neighbours which then all follows

If we on the other hand try to achieve this without the framework of suitable coverage combinations we would have to rely on each node learning the impact of coverage configurations in neighbour nodes. In order to achieve this without creating excessive number of dropped UEs, we would probably have to slowly modify the coverage area and see whether the neighbour node will follow. This could possibly be done if we want could express small steps, and possibly the relationships between them. But as explained before, there indicated coverage configurations does not map to a specific coverage characteristics, i.e. the size of the cell. Hence this would be very difficult. Further, the actual algorithm to optimize this would be proprietary and it would be difficult to also express intention behind adjustment, e.g. "I am now trying small step".

Based on the above, we prefer that we instead enable a simple mechanism based on the suitable coverage combinations:

If coverage compensation is allowed and is to be strictly controlled by an OAM configuration of suitable coverage configurations, the initiating node propose a change and reverts back in case all involved nodes are not able to meet the proposed change according to the suitable coverage combinations.

## 3.4 Cell replacement functionality

The question was raised whether cell replacement functionality is important in NR. It is true that the NR as such is much more flexible compared to LTE to adjust the signalling overhead to the traffic situation, and that the scenario of switching between omni and sectorized cells may not bring the same benefit as in LTE.

On the other hand, in NR we have the new scenario of centralised gNB-CUs handling a set of gNB-DUs. One advantageous adaption to traffic would be to switch off some gNB-DUs and replace this coverage with neighbour gNB-DUs. But since the functionality of switching on and off cells in gNB-DU is managed by the gNB-CU, this means that the gNB-CU must be involved.

Over F1, it is the gNB-CU who decides whether a cell is activated or not. The gNB-CU may also modify the NR-CGI and PCI of the cell in the gNB-DU in the GNB-CU CONFIGURATION UPDATE message. But if the gNB-CU shall be allowed to be involved, he must have the information available to understand the possible options and he has to have the means to control the gNB-DU and whether there is any gNB-DU that is able to replace a cell that is about to be switched off. This is essentially boils down to a similar scenario as described in 3.1 and could be solved with the same means as in 3.2.

Another small impact is probably that the gNB-DU shall inform which cell is to be inactivated and the gNB-CU may then inactivate the cell using the normal method for deactivation. Otherwise we will diverge from current principle which may cause IoT issues

Hence, we propose:

Cell replacement functionality shall be supported in NG-RAN. This can be supported by proposal 1 and 2 above.

## 3.5 Beam replacement functionality

There is also a discussion whether there is a need to exchange beam replacement information. Here, we think it would be important to discuss the added benefit of this. Is there any benefit for a neighbour node to know which beam is replacing another beam. How will this information be used in the receiver? If we move to this more granular information, how accurate is this information, i.e. how well can a node predict that one SSB is replaced by another one from the point of view of the receiving node.

Discuss whether there is any benefit of beam replacement functionality

The next related question is whether the early indication of a beam replacement is needed. For the cell it is used to stop mobility towards a cell that will be replaced. Is this really needed also for the beam? Or is the cell information enough. At this point we do not see the big benefit of including this for the beam information but we are open to hear the proponents view on the benefit.

Do not include SSB Deployment Status Indicator.

## 3.6 Neighbour information exchange

If the gNB-CU is informed about neighbour changes from the gNB-DU, the question is how the gNB-DU shall use this information. The gNB-DU has limited knowledge about neighbouring cells. He may receive information about failed mobility events.

One possible solution is to use the suitable coverage configurations discussed in the previous sections. Instead of configuring the gNB-CU with this information, the gNB-DU can be configured. With this information, the gNB-DU may take decision based on neighbour information. But the question is whether there is any benefit of this solution compared to letting the gNB-CU providing the recommendation. The gNB-DU does not have so much additional information available and it is likely that the gNB-CU is able to make a better recommendation.

In addition to what was previously was discussed, we believe that it is important to include the CCO issue in the neighbour information. The reason is that if we expect nodes to be able to compensate for another node, the receiving node need to know the type of correction.

- If the problem is coverage related, i.e. the sending node feels it is important to modify to bridge a coverage hole, there is no need for immediate compensation of the receiving node, while

- If the problem is cell edge interference related, the need to compensate is much bigger.

Therefore we propose:

Discuss how the gNB-DU can use the neighbour information

Agree to include the CCO issue in the neighbour information

## 3.7 Other corrections to the BLCR

The following further miscellaneous corrections are included in the F1AP TP (non-exhaustive list):

- CCO Assistance Information List is changed to a single item. We expect that we will signal one issue at a time to each gNB-DU. In case a list is needed the Range have to be different (e.g. maxNbrIssues)

- range bounds added in tabular

- tabular is aligned with ASN.1

- IEs renamed to start with CCO (to align the naming)

- Adjusted ASN.1 according to the above

The following further miscellaneous corrections are included in the XnAP TP (non-exhaustive list):

- added a separate list/item structure in the tabular

- corrected the range of cell overage state

- Adjusted ASN.1 to include extension containers

# 4. Conclusion

In our opinion, the discussion on how to control CCO

Based on the discussion in this paper, we propose the following:

Proposal 1:	OAM may provide a binding or informative list of suitable coverage combinations to be used with coverage configurations of cells in neighbour nodes.

Proposal 2:	gNB-CU may be configured with the suitable coverage combinations and may in that case send a set of proposed coverage configurations to the gNB-DU

Proposal 3:	If coverage compensation is allowed and is to be strictly controlled by an OAM configuration of suitable coverage configurations, the initiating node propose a change and reverts back in case all involved nodes are not able to meet the proposed change according to the suitable coverage combinations.

Proposal 4:	Cell replacement functionality shall be supported in NG-RAN. This can be supported by proposal 1 and 2 above.

Proposal 5:	Discuss whether there is any benefit of beam replacement functionality

Proposal 6:	Do not include SSB Deployment Status Indicator.

Proposal 7:	Agree to include similar neighbour information as Xn on F1

Proposal 8:	Agree to include the CCO issue in the neighbour information

# Annex 1 – TP for TS 38.473

Start of the change

## 8.2	Interface Management procedures

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the gNB-CU TNL Association To Add List IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use it to establish the TNL association(s) with the gNB-CU. The gNB-DU shall report to the gNB-CU, in the gNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU as follows:

-	A list of TNL address(es) with which the gNB-DU successfully established the TNL association shall be included in the gNB-CU TNL Association Setup List IE;

-	A list of TNL address(es) with which the gNB-DU failed to establish the TNL association shall be included in the gNB-CU TNL Association Failed To Setup List IE.

If the GNB-CU CONFIGURATION UPDATE message includes gNB-CU TNL Association To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by both received TNL endpoints towards the gNB-CU. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by the received endpoint IP address(es).

If the gNB-CU TNL Association To Update List IE is contained in the gNB-CU CONFIGURATION UPDATE message the gNB-DU shall, if supported, overwrite the previously stored information for the related TNL Association(s).

If in the gNB-CU CONFIGURATION UPDATE message the TNL Association usage IE is included in the gNB-CU TNL Association To Add List IE or the gNB-CU TNL Association To Update List IE, the gNB-DU node shall, if supported, use it as described in TS 38.472 [22].

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Protected E-UTRA Resources List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall protect the corresponding resource of the cells indicated by E-UTRA Cells List IE for spectrum sharing between E-UTRA and NR.

If the GNB-CU CONFIGURATION UPDATE message contains the Protected E-UTRA Resource Indication IE, the receiving gNB-DU should forward it to lower layers and use it for cell-level resource coordination. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE when expressing its desired resource allocation during gNB-DU Resource Coordination procedure. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same gNB-DU.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE is absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows IAB-node access or not.

If the BAP Address IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the CCO Assistance Information IE or the CCO Neighbour Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may use it to determine a new cell and/or beam configuration. If the CCO recommendation IE is included within the the CCO Assistance Information IE, the gNB-DU may use this recommendation to further it to determine a new cell and/or beam configuration.

If the RACH Cell List IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may use, store or update this information to perform RACH optimization. If the Neighbour NR Cells for SON List IE is present in the Cells for SON List IE, the gNB-DU may store this information and use it to resolve PRACH conflicts.

Editor’s note:	The paragraph above may needs refinement.

Editor’s note: It is FFS whether gNB-DU may or shall store the information.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU Configuration Update: Unsuccessful Operation

If the gNB-DU cannot accept the update, it shall respond with a GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-CU CONFIGURATION UPDATE message towards the same gNB-DU.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

## 9.2	Message Functional Definition and Content

### 9.2.1	Interface Management messages

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >> E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>> E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | M |  | 9.3.1.x1 |  | YES | Ignore |
| RACH Cell List [Name FFS] | O |  | 9.3.1.y1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-DU to a gNB-CU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instance, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| >Cells Failed to be Activated Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| gNB-CU TNL Association Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| gNB-CU TNL Association Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Failed To Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE List |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - | - |
| >>NR CGI | M |  | 9.3.1.12 |  | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| CCO Modification Notification | O |  | 9.3.1.x3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.x1	CCO Assistance Information

This IE indicates the Capacity and Coverage (CCO) actions for specific CCO issues detected.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO issue detection | M |  | ENUMERATED (coverage, cell edge capacity ...) | Indicates the type of CCO issue detected |
| Affected Cells and Beams | O |  | 9.3.1.x2 |  |
| CCO Recommendation | O |  | 9.3.1.x5 |  |

#### 9.3.1.x2	Affected Cells and Beams

This IE includes a list of cells and/or SS/PBCH block indexes affected by the detected CCO issue.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Affected Cell List |  | 1 .. < maxCellingNBDU> |  |  |
| >NG-RAN CGI | M |  | 9.2.3.25 |  |
| >Affected SSB List |  | 0..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

Editor´s Note: the content of this IE is FFS

#### 9.3.1.x3	CCO Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO Modification NotificationList |  | 1 |  |  |
| > CCO Modification Indication Item | M | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is to be inactivated. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. |
| >>SSB Coverage Modification List |  | 0..<maxnoofSSBAreas> |  |  |
| >>>SSB Index | M |  | INTEGER (0..63) |  |
| >>> SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.3.1.x5	CCO Recommendation

This IE includes a recommendation of a coverage configuration modification

| CCO Recommendation List |  | 1 |  |  |
| --- | --- | --- | --- | --- |
| > CCO Recommendation Item | M | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is to be inactivated. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

### 9.4.4	PDU Definitions

<<next change>>

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

SuccessfulHOReportInformationList,

CCO-Modification-Notification,

CCO-Assistance-Information,

RACHCell-List

<<next change>>

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-SuccessfulHOReportInformationList,

id-CCO-Modification-Notification,

id-CCO-Assistance-Information,

id-RACHCell-List,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

<<next change>>

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	 Cells-to-be-Activated-List						PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	 Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	 Cells-to-be-Barred-List							PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	 Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	 Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	 Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	 UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 		BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information			CRITICALITY ignore	TYPE 	CCO-Assistance-Information					PRESENCE optional	}|

{ ID id-RACHCell-List							CRITICALITY ignore	TYPE	 RACHCell-List											PRESENCE optional },

...

}

<<next change>>

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBCUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBCUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-Failed-to-be-Activated-List			CRITICALITY reject	TYPE Cells-Failed-to-be-Activated-List				PRESENCE optional}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-Setup-List			CRITICALITY ignore	TYPE GNB-CU-TNL-Association-Setup-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-Failed-To-Setup-List	CRITICALITY ignore	TYPE GNB-CU-TNL-Association-Failed-To-Setup-List	PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List				CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info					CRITICALITY ignore	TYPE Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-CCO-Modification-Notification				CRITICALITY ignore	TYPE CCO-Modification-Notification					PRESENCE optional	},

...

}

### 9.4.5	Information Element Definitions

<<next change>>

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofSuccessfulHOReports,

maxnoofNR-UChannels,

maxServedCellforSON,

maxNeighbourCellforSON,

maxnoofNeighbourCCOCells

<<next change>>

CCO-Modification-Notification ::= SEQUENCE (

cCO-ModificationNotificationList				CCO-ModificationNotificationList,

iE-Extensions								ProtocolExtensionContainer { { CCO-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

CCO-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

CCO-ModificationNotificationList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CCO-ModificationNotificationItem

CCO-ModificationNotificationItem ::= SEQUENCE {

nRCGI					NRCGI,

cellCoverageState		CellCoverageState,

sSB-Coverage-Modification-List		SSBCoverageModification-List OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { CCO-ModificationNotificationItem-ExtIEs } }		OPTIONAL,

...

}

CCO-ModificationNotificationItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellCoverageState ::= INTEGER (0..63, ...)

CCO-Assistance-Information ::= SEQUENCE {

cCO-issue-detection				ENUMERATED {coverage, cell-edge-capacity, ...},

affectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

cCORecommendation				CCORecommendation 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

<<next change>>

CCORecommendation ::= SEQUENCE {

cCORecommendationList			CCORecommendationList,

iE-Extensions			ProtocolExtensionContainer { { CCORecommendation-ExtIEs } }		OPTIONAL,

...

}

CCORecommendation-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CCORecommendationList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CCORecommendationItem

CCORecommendationItem ::= SEQUENCE {

nRCGI							NRCGI,

cellCoverageState				CellCoverageState,

iE-Extensions			ProtocolExtensionContainer { { CCORecommendationItem-ExtIEs } }		OPTIONAL,

...

}

CCORecommendationItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<next change>>

SSBCoverageModification-List ::= SEQUENCE (SIZE (0..maxnoofSSBAreas)) OF SSBCoverageModification-Item

SSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

sSBCoverageState				SSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { SSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

SSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBCoverageState ::= INTEGER (0..15, ...)

<<next change>>

### 9.4.7	Constant Definitions

<<next change>>

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofSuccessfulHOReports				INTEGER ::= 32 -- FFS

maxnoofNR-UChannels						INTEGER ::= 32 -- FFS

maxServedCellforSON						INTEGER ::= 256

maxNeighbourCellforSON					INTEGER ::= 32

maxnoofNeighbourCCOCells				INTEGER ::= 512

<<next change>>

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-SliceRadioResourceStatus							ProtocolIE-ID ::= 900 -- Assigned by MCC

id-CompositeAvailableCapacity-SUL					ProtocolIE-ID ::= 901 -- Assigned by MCC

id-SuccessfulHOReportInformationList				ProtocolIE-ID ::= 902 -- Assigned by MCC

id-NR-U-Channel-List								ProtocolIE-ID ::= 903 -- Assigned by MCC

id-Coverage-Modification-Notification				ProtocolIE-ID ::= 904 -- Assigned by MCC

id-CCO-Assistance-Information					ProtocolIE-ID ::= 905 -- Assigned by MCC

id-Neighbor-node-CCO-Assistance-Information-List	ProtocolIE-ID ::= 906 -- Assigned by MCC

id-RACHCell-List									ProtocolIE-ID ::= 907 -- Assigned by MCC

End of the change

# Annex 2 – TP for 38.401

# 7	NG-RAN functions description

Start of the change

## 7.X	Coverage and Capacity Optimisation Function

The Coverage and Capacity Optimisation Function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, Coverage and Capacity Optimisation Function detection function is located at the gNB-CU. To perform Coverage and Capacity optimisation at gNB-DU, gNB-CU may send information about coverage modifications in neighbours cells as well as CCO related issues and optionally a set of recommended configurations to gNB-DU via F1AP signalling. The gNB-DU selects a suitable configuration and signals the selected configuration to gNB-CU.

End of the change

# Annex 3 – TP for 38.300

Start of the change

### 15.5.X	Support for CCO

#### 15.5.X.1	General

The objective of NR Capacity and Coverage Optimization (CCO) function is to detect and mitigate coverage and cell edge interference issues.

#### 15.5.X.2	OAM requirements

Each NG-RAN node may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contains relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted. OAM may also provide suitable coverage combinations indicating which of these alternative coverage combinations should be used concurrently in cooperating nodes.

#### 15.5.X.3	Dynamic coverage configuration changes

An NG-RAN node may autonomously select and switch between these configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes about the coverage reconfiguration using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell and its coverage state indicator and optionally the SSB index of each modified SSB and its coverage state indicator.

The indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the indicator to retrieve a previously stored Mobility Robustness Optimisation state. If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving NG-RAN node may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending NG-RAN node adds cells in inactive state, the receiving NG-RAN node may use this information to avoid connection or re-establishment failures. The receiving NG-RAN node may also use the notification to reduce the impact on mobility. For example, the receiving NG-RAN node should avoid triggering handovers towards cell(s) that are indicated to be inactive.

The indicator may also be used at the receiving NG-RAN node to trigger adjustments of its own coverage modification.

# Annex 4 – TP for 38.470

## 5.2	F1-C functions

### 5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of  intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. With the gNB-CU Configuration Update function, energy saving with cell activation/deactivation can be supported as defined in TS 38.300 [8]. For coverage and capacity optimisation, the gNB-CU may send information about identified issues and optionally a set of recommended configurations to the gNB-DU and the gNB-DU responds with the selected configuration. The gNB-CU may provide information about ncoverage modifications in neighbour cells to the gNB-DU. The gNB-DU may provide information on coverage modifications executed in the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to inform the S-NSSAI(s), CAG ID(s) and NID(s) supported by the gNB-DU.

The F1 setup and gNB-CU Configuration Update functions allow to inform the NID(s) available at the gNB-CU.

The F1 resource coordination function is used to transfer information about frequency resource sharing between gNB-CU and gNB-DU. In case of split gNB architecture, the gNB-CU may consolidate the outgoing messages from multiple gNB-DUs and distribute the incoming messages to the involved gNB-DUs, to perform resource coordination.

The gNB-DU status indication function allows the gNB-DU to indicate overload status to gNB-CU.

The network access rate reduction function is used to indicate to the gNB-DU that the rate at which UEs are accessing the network need to be reduced.

The F1 removal function is used to remove the interface instance and all related resources between the gNB-DU and the gNB-CU in a controlled manner.

# Annex 4 – TP for 38.423

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name |  | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List |  | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType |  | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |  |
| >>Served Cells To Update NR |  | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA |  | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | M |  |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | O |  |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  |  | 0 ..1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  |  | 0..<maxnoofCellsinNG-RAN node> |  |  |  |  |
| >>CCO issue detection | M |  |  | ENUMERATED (coverage, cell edge capacity ...) | Indicates the type of CCO issue detected | _ |  |
| >Global NG-RAN Cell Identity | M |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell to be modified. | – |  |
| >Cell Coverage State | M |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >Cell Deployment Status Indicator | O |  |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  |  | – |  |
| >>Replacing Cells |  |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>Global NG-RAN Cell Identity |  |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >SSB Coverage Modification List |  |  | 0..<maxnoofSSBAreas> |  | List of SSB beams with modified coverage. | – |  |
| >>SSB Index | M |  |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>SSB Coverage State | M |  |  | INTEGER (0..15, …) | Value '0' indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |
| ifSSBDeploymentStatusIndicatorPresent(FFS) | This IE shall be present if the SSB Deployment Status Indicator IE is present. |

## 9.3	Message and Information Element Abstract Syntax (with ASN.1)

### 9.3.5	Information Element definitions

<<next change>>

Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofCellsinNG-RANnode)) OF Coverage-Modification-Item

Coverage-Modification-Item ::= SEQUENCE {

cCOIssueDetection				ENUMERATED { coverage, cell-edge-capacity, ...},

globalNG-RANCell-ID				GlobalCell-ID,

cellCoverageState				INTEGER (0..63, ...),

cellDeploymentStatusIndicator	CellDeploymentStatusIndicator			OPTIONAL,

cellReplacingInfo				CellReplacingInfo						OPTIONAL,

-- Included in case the Cell Deployment Status Indicator IE is present

sSB-Coverage-Modification-List  SSB-Coverage-Modification-List,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofSSBAreas)) OF SSB-Coverage-Modification-Item

SSB-Coverage-Modification-Item ::= SEQUENCE {

sSBIndex				         INTEGER(0..63),

sSBCoverageState				INTEGER (0..15, ...),

iE-Extension			ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

iE-Extension			ProtocolExtensionContainer { { SSB-Coverage-Modification-Item-ExtIEs} } OPTIONAL,

...

}

SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

### 9.3.7	Constant definitions