3GPP TSG-RAN WG3 Meeting #115-e	R3-221860

# Online,  Feb 21st 2022 - March 3rd 2022

Agenda Item:	31.2.1

Source:	Ericsson

Title:	 On the use of Local NG-RAN node Identifier

Document for:	Discussion, Decision

# 1	Introduction

This paper discusses some Stage 2 and Stage 3 details of the solution concerning the definition and the use of Local NG-RAN node Identifier.

# 2	Discussion

## Local NG-RAN node Identifier of neighbour’s neighbour nodes

The exchange of a Local NG-RAN node Identifier (or similar names, such as Local node ID, etc), has been agreed as a mean to achieve interoperability in Resume scenario, where the Local NG-RAN node Identifier is used by a NG-RAN to disambiguate the identity (Global NG-RAN Node ID) of the NG-RAN node that allocated the UE context for the UE attempting to resume.

As part of the general discussion, one of the requirements considered has been impact on signaling and the need for keep that to a reasonable level.

We note that the most recent CR endorsed for XnAP in R3-216187 describes the possibility for a NG-RAN node 1 to send to a peer NG-RAN node 2 a list of Local NG-RAN Node Identifiers for a very high number of Neighbor NG-RAN Node (i.e. a neighbor’s neighbor node), using the Neighbour NG-RAN Node List IE.

Our current understand is that this proposal aims at providing a tool for a gNB to build a table (or equivalent) for “monitoring” the allocation of “Local NG-RAN Node Identifier” in one geographical area, and support an implementation in resolving potential conflicts in Local NG-RAN Node Identifiers that may occur in that area.

However, we don’t think this is really needed, for the following reasons:

- Different implementations may decide to handle conflicts in different ways, without the need for maintaining information of all Local NG-RAN Node Identifiers of neighbour’s neighbours.

- The impact in the signaling can be quite heavy.  For example, a node may have to send quite many NG-RAN node configuration update messages to inform a neighbor of changes occurred in one (or more) of its neighbors and since each node can decide to change its Local NG-RAN node Identifier independently from other nodes, the number of required messages can increase substantially.

Therefore, we propose to not signal Neighbour NG-RAN Node List IE neither in Xn Setup procedure nor in the NG-RAN node Configuration Update procedure.

Proposal 1: Do not include the Neighbour NG-RAN Node List IE neither in Xn Setup procedure nor in the NG-RAN node Configuration Update procedure.

## Interaction with other procedures

In the current version of CR endorsed for XnAP in R3-216187 the text below is found in section 8.4.1.2 related to Xn Setup procedure:

Interactions with other procedures:

If the NG-RAN node1 receives a XN SETUP RESPONSE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding XN SETUP REQUEST, the NG-RAN node1 should initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

If the NG-RAN node1 receives a XN SETUP RESPONSE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding XN SETUP REQUEST, the NG-RAN node1 should initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

We think this text is not needed since it should be up to implementation to decide how to resolve a conflict situation. In the proposed text (that has to be written from the receiver point of view), the NG-RAN node 1 is the node that should initiate an update, but different implementation may decide to address the conflict from NG-RAN node 2 instead. For example, the NG-RAN node 1 can be a very big node serving very many users, while NG-RAN node 2 is a pico node. It does not seem a good approach that the big node changes its Local NG-RAN node Identifier instead of the pico node. It is sufficient that the protocol provides the means for the nodes to understand that there is a conflict and then the respective node initiates handling if they think it is needed

For the reason explained above, there is no need for the text related to Neighbour NG-RAN Node List IE.

Proposal 2: Remove the text for Interactions with other procedures in section 8.4.1.2.

A similar text is also present in the section 8.4.2.2 related to NG-RAN node Configuration Update procedure:

Interactions with other procedures:

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE, the NG-RAN node1 should initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE, the NG-RAN node1 should initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

In addition to the reasons already explained, we note that there is a fundamental difference between the messages used in Xn Setup procedure as compared to the NG-RAN node Configuration Update which needs to be considered. The Xn Setup procedure is specified with a “Request-Response” pattern, meaning that in the Response message NG-RAN node 2 can put information about itself.

The NG-RAN node Configuration Update procedure follows a “Request-Acknowledge” pattern, meaning that in the return (Ack) message the NG-RAN node 2 does not need to put information about itself. If the NG-RAN node 2 has information about any configuration change it would initiate a NG-RAN node Configuration Update procedure itself.

Hence, we propose to remove the text for Interaction with other procedures in section 8.4.2.2.

Proposal 3: Remove the text for Interactions with other procedures in section 8.4.2.2.

## Xn Setup Response

As mentioned, in the Xn Setup Response message NG-RAN node 2 does not need to put information about itself. On this, we should also consider what is stipulated in XnAP section 4.1:

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included. For requirements on including Criticality Diagnostics IE, see section 10.

The Local NG-RAN node Identifier is an optional IE, so we need to add some procedural text to explain when the NG-RAN node 2 needs to put such optional IE in the XN SETUP RESPONSE message. We also note that a XN SETUP RESPONSE message can contain a Local NG-RAN node Identifier only if such identifer has been taken into use by the node. It is better, therefore, to explicitly mention this condition in the procedural text.

Proposal 4: Add a sentence in section 8.4.1.2 to clarify the inclusion of the optional Local NG-RAN node Identifier in the XN SETUP RESPONSE message.

## NG-RAN node Configuration Update

As said, the NG-RAN node Configuration Update procedure follows a “Request-Acknowledge” pattern, meaning that in the return (Ack) message the NG-RAN node 2 does not need to put information about itself.

Hence, we propose to remove the text indicating the presence of Local NG-RAN Node Identifier in NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, in section 8.4.2.2.

Proposal 5: Remove the text for describing the presence of Local NG-RAN Node ID in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE in section 8.4.2.2.

## On the use of more Local NG-RAN node Identifiers

We think that the use of more than one Local NG-RAN node Identifiers can help to tackle a couple of scenarios:

- In case of conflict, a node decides to revoke the use of an “Old” Local NG-RAN node Identifier and take a “New” Local NG-RAN node Identifier into use. However, there can still be a number of users to which the “Old” identifier has been assigned. It is reasonable to assume that the “Old” and the “New” identifier can coexist for some time.

- In case of reaching the maximum capacity in terms of number of users that can be addressed with a specific I-RNTI profile, a node can decide to take into use a “Second” Local NG-RAN node Identifier, in addition to a “First” Local NG-RAN node Identifier already in use (i.e. without revoking the “First” one).

For these reasons, we think it can be beneficial to enable the use of a List of Local NG-RAN node Identifier. At the same time, it’s good to keep the List short, to limit the impact in signaling. We think that a list of 6 elements is reasonable, but we are open to discuss other options.

In case a NG-RAN node takes an additional Local NG-RAN Node identifier into use or removes a Local NG-RAN Node identifier currently in use it informs its neighbor NG-RAN nodes about this change.

Proposal 6: An NG-RAN node can use a list of Local NG-RAN node Identifier. The number of elements in the list is proposed to be 6.

## I-RNTI profile

To support an NG-RAN node in resolving the Global NG-RAN node ID of the NG-RAN node that allocated the I-RNTI, we propose to define an I-RNTI structure containing three parts, as follows:

- the first part (starting from the MSB of the I-RNTI) identifies the I-RNTI profile. It is 2 bits long for a Full I-RNTI or 1 bit long for a short I-RNTI, and it indicates the length of a Local NG-RAN Node Identifier of the NG-RAN Node that allocated the I-RNTI.

- the second part, immediately following the first part, is a Local NG-RAN Node Identifier.

- the third part, immediately following the second part, identifies the UE context stored in the NG-RAN node that allocated the I-RNTI.

The I-RNTI profiles for Full I-RNTI can be defined as in the table below:

I-RNTI profiles for Full I-RNTI

| I-RNTI Profile Identity | I-RNTI profile value (binary encoding) |
| --- | --- |
| 0 | 0b00 |
| 1 | 0b01 |
| 2 | 0b10 |
| 3 | 0b11 |

The I-RNTI profiles for Short I-RNTI can be defined as in the table below:

I-RNTI profiles for Short I-RNTI

| I-RNTI Profile Identity | I-RNTI profile value (binary encoding) |
| --- | --- |
| 0 | 0b0 |
| 1 | 0b1 |

Proposal 7: Agree on the definitions for I-RNTI profiles to use in Full I-RNTI and Short I-RNTI.

The proposals above are captured in the proposed CRs for XnAP and draftCR for 38.300 in Appendix.

Proposal 8: Agree on the proposed CRs for XnAP and 38.300.

# Conclusion

In this paper the below proposals have been made:

Proposal 1: Do not include the Neighbour NG-RAN Node List IE neither in Xn Setup procedure nor in the NG-RAN node Configuration Update procedure.

Proposal 2: Remove the text for Interactions with other procedures in section 8.4.1.2.

Proposal 3: Remove the text for Interactions with other procedures in section 8.4.2.2.

Proposal 4: Add a sentence in section 8.4.1.2 to clarify the inclusion of the optional Local NG-RAN node Identifier in the XN SETUP RESPONSE message.

Proposal 5: Remove the text for describing the presence of Local NG-RAN Node ID in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE in section 8.4.2.2.

Proposal 6: An NG-RAN node can use a list of Local NG-RAN node Identifier. The number of elements in the list is proposed to be 6.

Proposal 7: Agree on the definitions for I-RNTI profiles to use in Full I-RNTI and Short I-RNTI.

Proposal 8: Agree on the proposed CRs for XnAP and 38.300.

# Reference

[1] R3-221861 Support flexible I-RNTI partitioning [RRCInactive]

# Appendix – Draft CR for 38.300

3GPP TSG-RAN WG3 Meeting #115-e	R3-xxxxxx

# Online,  Feb 21st 2022 - March 3rd 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Local NG-RAN Node IDs for RRC_INACTIVE [RRCInactive] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A standardized solution is needed to enable inter-vendor interoperable way for an NG RAN node to deduce the identity of another NG RAN node from the received I-RNTI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new section to describe the use and structure of I-RNTI to disambiguate the identity of the NG RAN node hosting the UE Context. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Collision would happen often due to conflicts of NG-RAN Node IDs in the inter-vendor scenario, which introduce extra latency and signaling exchange. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.2, Annex xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------  Start of Changes ---------------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[9]	3GPP TS 37.324: " E-UTRA and NR; Service Data Protocol (SDAP) specification".

[10]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[15]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[16]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[17]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[20]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer"

[21]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[22]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[23]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[24]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[25]	Void.

[26]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[27]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[28]	3GPP TS 24.501: "NR; Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[30]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[31]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[32]	3GPP TS 38.470: "NG-RAN; F1 application protocol (F1AP) ".

[33]	3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[34]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[35]	3GPP TS 38.101-2: "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[36]	3GPP TS 38.101-3: "User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[37]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[38]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[39]	3GPP TS 22.104 "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[41]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[42]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[43]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[44]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[xx]	3GPP TS 38.423: “Xn application protocol (XnAP)”

-----------------------------------------------  Next Change ---------------------------------------------------------

## 8.2	Network Identities

The following identities are used in NG-RAN for identifying a specific network entity:

-	AMF Name: used to identify an AMF.

-	NR Cell Global Identifier (NCGI): used to identify NR cells globally. The NCGI is constructed from the PLMN identity the cell belongs to and the NR Cell Identity (NCI) of the cell. The PLMN ID included in the NCGI should be the first PLMN ID within the set of PLMN IDs associated to the NR Cell Identity in SIB1, following the order of broadcast.

NOTE 1:	How to manage the scenario where a different PLMN ID has been allocated by the operator for an NCGI is left to OAM and/or implementation.

-	gNB Identifier (gNB ID): used to identify gNBs within a PLMN. The gNB ID is contained within the NCI of its cells.

-	Global gNB ID: used to identify gNBs globally. The Global gNB ID is constructed from the PLMN identity the gNB belongs to and the gNB ID. The MCC and MNC are the same as included in the NCGI.

NOTE 2:	It is not precluded that a cell served by a gNB does not broadcast the PLMN ID included in the Global gNB ID.

-	Tracking Area identity (TAI): used to identify tracking areas. The TAI is constructed from the PLMN identity the tracking area belongs to and the TAC (Tracking Area Code) of the Tracking Area.

-	Single Network Slice Selection Assistance information (S-NSSAI): identifies a network slice.

-	Network Identifier (NID): identifies an SNPN in combination with a PLMN ID.

-	Closed Access Group Identifier: identifies a CAG within a PLMN.

-	Local NG-RAN Node Identifier: used as reference to the NG-RAN node in the I-RNTI.

-----------------------------------------------  Next Change ---------------------------------------------------------

Annex xx (normative):

Use and structure of the I-RNTI

The I-RNTI provides an NG-RAN node with a reference to the UE context and a reference to the NG-RAN node that allocated the UE context.

To support an NG-RAN node in resolving the Global NG-RAN node ID of the NG-RAN node that allocated the I-RNTI, the I-RNTI structure is as follows:

- the first part (starting from the MSB of the I-RNTI) identifies the I-RNTI profile. It is 2 bits long for a Full I-RNTI or 1 bit long for a short I-RNTI, and it indicates the length of a Local NG-RAN Node Identifier of the NG-RAN Node that allocated the I-RNTI.

- the second part, immediately following the first part, is a Local NG-RAN Node Identifier.

- the third part, immediately following the second part, identifies the UE context stored in the NG-RAN node that allocated the I-RNTI.

In case a NG-RAN node takes an additional Local NG-RAN Node identifier into use or removes a Local NG-RAN Node identifier currently in use it informs its neighbor NG-RAN nodes about this change.

The I-RNTI profiles for Full I-RNTI are described in table XX-1.

Table XX-1: I-RNTI profiles for Full I-RNTI

| I-RNTI Profile Identity | I-RNTI profile value (binary encoding) |
| --- | --- |
| 0 | 0b00 |
| 1 | 0b01 |
| 2 | 0b10 |
| 3 | 0b11 |

The I-RNTI profiles for Short I-RNTI are described in table XX-2.

Table XX-2: I-RNTI profiles for Short I-RNTI

| I-RNTI Profile Identity | I-RNTI profile value (binary encoding) |
| --- | --- |
| 0 | 0b0 |
| 1 | 0b1 |