3GPP TSG-RAN WG3 Meeting #115-e	R3-221865

E-meeting, 21 February – 3 March, 2022

Agenda item:	15.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining open points on QMC/MDT alignment

Document for:	Discussion and Decision

# 1	Introduction

In this paper we provide a brief overview of open points for QMC/MDT alignment

# 2	Discussion

- Whether any enhancements for MDT/QMC alignment in split architecture scenarios should be considered. (RP-213473)

Our view: One of the aspects of MDT/QMC alignment agreed at RAN3#114bis-e is to delay start of radio measurements in the UE requested as part of immediate MDT, until the QoE session starts in the UE. It seems reasonable to apply the same principle for measurements collected from the gNB-DU and the gNB-CU-UP in order to avoid unnecessary measurements to be sent to the TCE.

Proposal 1: Aligned MDT measurements from the gNB-DU and gNB-CU-UP are activated when the network has been notified that the QoE session has started in the UE.

- FFS whether to support the alignment between s-based QoE and m-based MDT.

Our view: The first problem is that alignment with m-based MDT comes with the drawback that m-based MDT is not propagated in case of inter-node handover. The second problem is that the legacy mechanism for selection of UEs for m-based MDT could not be used, i.e. the network will have to select the UE that uses s-based QoE also for m-based MDT. This means that such alignment mechanism will have impact on legacy MDT specification which should be avoided.

Proposal 2: Alignment between s-based QoE and m-based MDT is not supported.

- FFS whether to support the scenario where QoE measurement session span across multiple gNBs configured with m-based MDT with different Trace Reference.

Our view: Support of such scenario would require the OAM system to provide a list containing a TR for each gNB the UE could potentially visit. The entity constructing this list would therefore need extensive information about gNB adjacencies, and the list could also become very long. We therefore believe this is not the good direction to take. A reasonable compromise can be that QMC/MDT alignment for m-based MDT is supported for intra-node operation.

Proposal 3: QMC/MDT alignment for m-based MDT is supported for intra-node operation.

# 3	Conclusion

Proposal 1: Aligned MDT measurements from the gNB-DU and gNB-CU-UP are activated when the network has been notified that the QoE session has started in the UE.

Proposal 2: Alignment between s-based QoE and m-based MDT is not supported.

Proposal 3: QMC/MDT alignment for m-based MDT is supported for intra-node operation.