3GPP TSG-RAN WG3 Meeting #115-e	R3-221923

E-meeting, 21 Feb-3 Mar 2022

Title: 	Discussion about location report and mapping in NTN-IoT

Source: 	Huawei

Agenda item:	30.1

Document Type:	discussion

# 1. Introduction

This paper discusses one particular issue of user location report and mapping in NTN-IoT, which can be quite different to the scope of NR NTN.

# 2. Discussion

In NR NTN, one significant achievement in RAN3 was rely on UE location report for NNSF and the associated mapping of CGI and TAC on fixed geographical area.

The most recent agreement in NR NTN is that UE does not report to the NG-RAN its coarse GNSS coordinates during initial access (before AS security is activated) [1]:

Due to possible privacy issues indicated by SA3, RAN2 is likely to decide that UE does not report to the NG-RAN its coarse GNSS coordinates during initial access (before AS security is activated), for example, for service request and registration area update procedures. RAN2 assumes UE location information can be reported after AS security is activated and network has NTN specific user consent. RAN2 has asked SA3 to work on the NTN specific user consent in Rel-17. RAN2 also understands that, if needed, NG-RAN can reselect an AMF serving a PLMN corresponding to the available UE's current location. This location can be determined by the AMF by invoking UE location procedure (LCS) in connected mode(once AS security is activated) and provided to the NG-RAN.

Thus, with RAN2’s agreement, UE will not report any location, i.e., even the coarse one, before AS security. When AS security is activated, however, the UE is able to obtain accurate location information thanks to GNSS capability. Correspondingly, the NG-RAN is able to map the location into mapped cells which is at a comparable level with terrestrial cell size, and report the mapped CGI as well as the accurate TAI to CN via NG interface.

Observation 1:  Fine location information can only be provided with AS security activation.

We note that for eMTC, AS security and measurement procedures are supported thus NR agreements regarding location report and cell mapping can just be followed.

Proposal 1:  For eMTC, NR agreements regarding location report and cell mapping can be followed

Nevertheless, it is not the same case for NB-IoT. For NB-IoT, AS security is supported with the user plane solution and is not supported with the control plane solution. Measurement procedures are not supported with any of the solution. The non-support of AS Security for NB-IoT greatly challenges the reuse of NR NTN agreements regarding location report. In align with SA3’s requirement, if no AS security is activated, unprotected location can’t be reported. This has been verified by the latest RAN2 LS as follows [2]:

Following liaisons with SA2, SA3 and RAN3 (see R2-2109216, R2-2200145, R2-2200149, R2-2202542) on the reporting of UE location information in NR NTN, RAN2 has started to discuss location reporting aspects for IoT NTN. For NB-IoT UEs AS security cannot be assumed to be supported. This has lead RAN2 to assume that sending a detailed location over AS cannot be supported and RAN2 has thus discussed whether to send coarse location. However, in latest received LS by SA3 in R2-2200149/S3-213360 regarding NR NTN location reporting, it is stated that even sending coarse location unprotected should be avoided, so RAN2 has assumed that not even coarse location may be reported over AS for NB-IoT. RAN2 also discussed the possibility UE providing location to the MME via NAS procedures as an alternative to deliver the location information via AS. RAN2 have the following questions to RAN3/SA2 and CT1: - To RAN3/SA2, we ask whether there are any issues from a RAN3 and SA2 point of view that a NB-IoT UE does not provide any location information over AS to eNB. - RAN2 assumes that not providing location over AS may lead to impact on E-UTRAN selecting correct core network, NNSF and constructing the mapped cell ID and any potential ULI. - To RAN3/SA2/CT1, if location is not provided over AS and the location is needed for network operation is it feasible from a RAN3 and SA2 point of view that UE instead provides location to the MME via NAS, which is assumed to be protected. - If providing location over NAS is a viable solution that is acceptable for network operation, then is it feasible that this is introduced in Rel-17.

With RAN2’s agreement, there will be no location report from NB-IoT UEs using CP CIoT EPS Optimization only to eNB, and RAN3 need to question the location report mechanism as well as the mapping mechanism. As also pointed out in the LS, first, this means eNB may lack of knowledge to select the correct country (core network), reporting the accurate TAI where the UE is located. The re-selection is also under discussion, because even if NB-IoT is not subject to mobility there are already deployment considering move of NB-IoT device, e.g. rent bike in China. As positioning also require the activation of AS security, this basically means eNB has no way to obtain any fine location information, at any time, which also challenge all CGI mapping concept introduced in NR NTN. The introduction of mapped CGI came from SA2 requirement to avoid massive changes in the side of CN, the mapping between cell IDs and geographical areas was configured in RAN and CN and is ideally used to identify a small area (hopefully comparable to terrestrial cell size) where the UE is located in. In the absence of AS security, hence the absence of any fine location, the eNB has no knowledge of UE location and is not capable to define a mapped E-UTRAN CGI and report it to the CN. Therefore, for the first question in the LS, we tend to think we can confirm that not providing any location over AS may lead to failure of selecting correct core network, NNSF and constructing the mapped cell ID and any potential ULI.

In the LS, RAN2 also asked is it feasible that UE instead provides location to the MME via NAS, which is assumed to be protected. For this question, we think this is mainly up to SA2 and CT1. RAN3 can hardly judge whether a NAS solution is feasible.

Proposal 2: In the absence of AS security in NB-IoT, RAN3 can confirm not providing any location over AS may lead to failure of selecting correct core network, NNSF and constructing the mapped cell ID and any potential ULI.

Proposal 3：RAN3 can leave the question about feasibility of NAS based solution to SA2 and CT1.

A corresponding LS out is shown in [3]

# 3. Conclusion

Based on the discussion in this paper, we propose to discuss the following proposal:

Observation 1:  Fine location information can only be provided with AS security activation.

Proposal 1:  For eMTC, NR agreements regarding location report and cell mapping can be followed

Proposal 2: In the absence of AS security in NB-IoT, RAN3 can confirm not providing any location over AS may lead to failure of selecting correct core network, NNSF and constructing the mapped cell ID and any potential ULI.

Proposal 3：RAN3 can leave the question about feasibility of NAS based solution to SA2 and CT1.

# 4. Reference

[1] R3-221675 LS on UE location during initial access in NTN, RAN2

[2] R2-2201957 LS on UE providing Location Information for NB-IoT, RAN2

[3] R3-221925 [draft] Reply LS on UE providing location information for NB-IoT, Huawei