3GPP TSG-RAN WG3 Meeting #115-e	R3-221941

# Electronic meeting, 21st Feb – 3rd Mar 2022

Agenda item:		18.1

Source:	Intel Corporation

Title:	Consideration on Alignment for AI/ML in NG-RAN Rel-17 SI

Document for:		Discussion and Decision

# Introduction

During Rel-17, RAN3 has studied how to enable AI/ML in NG-RAN to support network energy saving, load balancing and mobility optimization use cases. Potential solutions and the corresponding standard impacts are identified in [1]. During study item phase, solutions, potential standard impacts and required input/output/feedback information were discussed separately under each use case. However, there are many similarities among use cases, as well as same input/output/feedback information.

In this contribution, we first identify the similarities among use cases, then focus on how to align among use cases to avoid duplicate effort during work item phase.

# Discussion

## Similarity among Use Cases

### Location for AI/ML Model Training and AI/ML Model Inference

As described in section 5.1.2.1, section 5.2.2.1 and section 5.3.2.1 in [1], below table summarizes the location for AI/ML Model Training and AI/ML Model Inference agreed for three use cases:

|  | CU/DU non-split architecture |  | CU/DU split architecture |  |
| --- | --- | --- | --- | --- |
| Location for AI/ML Model Training | OAM | gNB | OAM | gNB-CU (CU-CP) |
| Location for AI/ML Model Inference | gNB | gNB | gNB-CU (CU-CP) | gNB-CU (CU-CP) |

Additionally, for all three use cases, when AI/ML Model Training is located in OAM and AI/ML Model Inference is located in gNB, gNB is allowed to continue perform model training based on AI/ML model trained in OAM.

Observation 1: All three use cases (i.e. AI/ML-based network energy saving, load balancing and mobility optimization) have common deployment scenarios for AI/ML Model Training and AI/ML Model Inference.

As captured in [1], it was agreed that AI/ML model can be deployed/updated from Model Training to Model Inference, and Model Inference can also provide model performance feedback to Model Training when applicable.

When Model Training and Model Inference are both located at gNB or gNB-CU (CU-CP), model deployment/update and model performance feedback are performed internally within gNB or gNB-CU, which can be realized by implementation without impacting standards.

However, when Model Training is located in OAM, while Model Inference is located in gNB or gNB-CU, it requires interaction between OAM and gNB to support the transmission of model deployment/update and model performance feedback. As replied in [5], SA5 plans to make it a first priority to address the AI/ML model management capabilities to support AI/ML in NG-RAN for this deployment scenario in their Rel-18 new SI “Study on AI/ML management”.

Observation 2: SA5 will study and address model training in OAM and model inference in NG-RAN in their Rel-18 SI “Study on AI/ML management”.

Based on above observation, for all three use cases, since RAN3 has agreed model deployment/update and model performance feedback (when applicable) need to be supported between OAM and NG-RAN for the corresponding deployment scenario, it is proposed to capture potential impact on SA5 specifications as below:

- Model deployment/update, which is used to initially deploy a trained, validated, and tested AI/ML model to Model Inference function or to deliver an updated model to the Model Inference function.

- Model performance feedback, which may be used for monitoring the performance of the AI/ML model

Considering this is a common standard impact for all three use cases, it is proposed to have a new chapter to capture potential impacts on SA5 for this deployment scenario.

Proposal 1: Capture the following SA5 impact in TR 37.817: “When Model Training is located in OAM and Model Inference is located in gNB or gNB-CU, SA5 considers the following standard impacts: 1) model deployment/update; 2) model performance feedback.”

### Input/Output/Feedback

Input

Among three use cases, following input information are commonly collected from local node, neighbouring RAN nodes and UE.

From local node:

- UE trajectory prediction

| AI/ML-based Network energy saving | AI/ML-based Load Balancing | AI/ML-based Mobility Optimization |
| --- | --- | --- |
| UE mobility/trajectory prediction | UE trajectory prediction | UE trajectory prediction output (will be used by the RAN node internally) |

- Current/predicted resource status

| AI/ML-based Network energy saving | AI/ML-based Load Balancing | AI/ML-based Mobility Optimization |
| --- | --- | --- |
| Current/predicted resource status | Predicted own resource status information: e.g., this can be calculated using predictions of some or all of the resource information specified in current XnAP | Local resource status prediction |

From neighbouring RAN nodes:

- Current/predicted resource status

| AI/ML-based Network energy saving | AI/ML-based Load Balancing | AI/ML-based Mobility Optimization |
| --- | --- | --- |
| UE mobility/trajectory prediction | UE trajectory prediction | UE trajectory prediction output (will be used by the RAN node internally) |

From the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

| AI/ML-based Network energy saving | AI/ML-based Load Balancing | AI/ML-based Mobility Optimization |
| --- | --- | --- |
| UE measurement report | UE radio measurements, e.g. RSRP, RSRQ, SINR | Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g. RSRP, RSRQ, SINR |

Observation 3: Same input information collected from UE/local node/neighbouring NG-RAN node can be used for all three use cases, including UE trajectory prediction from local node, current/predicted resource status from local and neighbouring NG-RAN nodes, UE location information and UE measurement report from the UE.

Output

As each use case has its unique optimization target using AI/ML, outputs generated by each use cases are different.

Observation 4: There’s no common output information between three use cases.

Feedback

Similar as input information, resource status of neighbouring NG-RAN node as feedback information is also commonly used for all three use cases. Besides, to continuously tune AI/ML model performance for model inference, all three use cases agreed that UE performance and system KPI are used as performance information, which are transmitted from neighbouring NG-RAN node to source NG-RAN node.

- Resource status of neighbouring NG-RAN nodes (duplicated with input information)

- Performance information

  - UE performance

  - System KPIs

Observation 5: Same feedback information can be used for all three use cases, including resource status of neighbouring NG-RAN nodes, UE performance, and system KPIs.

Standard Impact

Based on above observations, common information used for three use cases will lead to the same standard impacts and specification works. As a result, for work item phase, we don't have to work on specifications in each use case separately for the same information. It would be more straightforward to work on the common standard impacts among use cases and avoids duplicate efforts.

Proposal 2: During WI phase, it is recommended to work on information (input/output/feedback) common for all three uses cases together rather than individually for each use case, which would anyway lead to common standard impacts and specification works.

## Standard Impact on other WGs

Impact on SA5

Another key objective of the study item is to identify potential impact to other WGs in Rel-18. As discussed in Section 2.1.1, SA5 will be impacted to support the deployment scenario “Model Training in OAM, Model Inference in NG-RAN”, meanwhile, supporting model deployment/update and model performance feedback.

Additionally, as captured in solution figures for three use cases [1], when Model Training is deployed at OAM, NG-RAN node should also provide input information for Model training and feedback information to OAM, which may include UE performance information as well as system KPIs. Therefore, SA5 will also be impacted to support receiving feedback information for AI/ML-based use cases.

Proposal 3: SA5 will be impacted to collect input and feedback information for three use cases.

Impact on RAN2

Other than SA5, during RAN #94e meeting, it was also discussed whether RAN2 will be impacted during Rel-18 WI. Based on current progress and discussion in each use case, whether there will be some impacts on RAN2 or not mainly depends on data collection from UE for model training and model inference over Uu interface.

It was summarized in Section 2.1.2 that, for all three use cases, the following information is collected from the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

The UE Mobility History Information is collected for the load balancing use case. The UE historical serving cells and their locations, moving velocity are collected for the mobility optimization use case.

So far, the above information identified to be collected from the UE is all supported in the current NR RRC specification [6], which has no new RAN2 impact.

| Information collected from the UE | Supported IE in TS38.331 |
| --- | --- |
| UE location information | LocationInfo |
| UE measurement report | MeasResults |
| UE mobility history information | VisitedCellInfoList |
| UE historical serving cells and locations |  |
| Moving velocity | CommonLocationInfo -> velocityEstimate |

Proposal 4: No RAN2 impact is foreseen for AI/ML enabled NG-RAN in Rel-18 WI. The information identified to be collected from the UE is already supported in TS 38.331.

Different from data collection for SON/MDT, to generate and train a well-formulated AI/ML model, it normally requires huge amount of data collection from UEs. However, this doesn’t mean that a UE needs to report massive data (required for model training) in a burst fashion through successive RRC messages. The UE can still report the required data according to SON/MDT procedures, while it is gNB or OAM’s responsibility to store and maintain enough data for model training. No additional RAN2 impact is foreseen.

Observation 6: For AI/ML model training, the UE does not need to report massive data required for model training in a burst fashion to gNB. It is the responsibility of the Data Collection function to store the UE’s RRC reports continuously and maintain enough data for model training.

Proposal 5: Data collection for SON/MDT can be used as baseline for data collection for AI/ML enabled NG-RAN without any RAN2 impact.

## Alignment among Use Cases

As observed in Section 2.1.2, there are many input/output/feedback information common for all three use cases. To better facilitate readers to understand what exact information is used for certain use case, it is proposed to use the same terminology for the same information across all three use cases.

Proposal 6: Align terminology for the same information across all three use cases, including UE trajectory prediction, current/predicted resource status, UE measurement report, mobility history information, etc.

To have a better overview, it is suggested to consider the alignment among use cases in General agenda item, including terminology, input/output/feedback, solution flow chart, standard impact, etc. The corresponding text proposals are proposed in the companion contributions [2][3][4].

Proposal 7: The "General" agenda item should cater alignments across use cases, including terminology, input/output/feedback, solution flow chart, standard impact, etc.

Proposal 8: Agree the corresponding text proposal to TR 37.817 in Annex A.

# Conclusion

In this contribution, we analyzed the similarities among three use cases and summarized common input/feedback information. We further analyzed potential standard impacts on other WGs, including SA5 and RAN2 with following observations and proposals:

Observation 1: All three use cases (i.e. AI/ML-based network energy saving, load balancing and mobility optimization) have common deployment scenarios for AI/ML Model Training and AI/ML Model Inference.

Observation 2: SA5 will study and address model training in OAM and model inference in NG-RAN in their Rel-18 SI “Study on AI/ML management”.

Observation 3: Same input information collected from UE/local node/neighbouring NG-RAN node can be used for all three use cases, including UE trajectory prediction from local node, current/predicted resource status from local and neighbouring NG-RAN nodes, UE location information and UE measurement report from the UE.

Observation 4: There’s no common output information between three use cases.

Observation 5: Same feedback information can be used for all three use cases, including resource status of neighbouring NG-RAN nodes, UE performance, and system KPIs.

Observation 6: For AI/ML model training, the UE does not need to report massive data required for model training in a burst fashion to gNB. It is the responsibility of the Data Collection function to store the UE’s RRC reports continuously and maintain enough data for model training.

Proposal 1: Capture the following SA5 impact in TR 37.817: “When Model Training is located in OAM and Model Inference is located in gNB or gNB-CU, SA5 considers the following standard impacts: 1) model deployment/update; 2) model performance feedback.”

Proposal 2: During WI phase, it is recommended to work on information (input/output/feedback) common for all three uses cases together rather than individually for each use case, which would anyway lead to common standard impacts and specification works.

Proposal 3: SA5 will be impacted to collect input and feedback information for three use cases.

Proposal 4: No RAN2 impact is foreseen for AI/ML enabled NG-RAN in Rel-18 WI. The information identified to be collected from the UE is already supported in TS 38.331.

Proposal 5: Data collection for SON/MDT can be used as baseline for data collection for AI/ML enabled NG-RAN without any RAN2 impact.

Proposal 6: Align terminology for the same information across all three use cases, including UE trajectory prediction, current/predicted resource status, UE measurement report, mobility history information, etc.

Proposal 7: The "General" agenda item should cater alignments across use cases, including terminology, input/output/feedback, solution flow chart, standard impact, etc.

Proposal 8: Agree the corresponding text proposal to TR 37.817 in Annex A.

# References

- R3-221610, TR37.817 Study on enhancement for Data Collection for NR and EN-DC v1.2.0

- R3-221942, AI/ML-based network energy saving, Intel Corporation

- R3-221943, AI/ML-based load balancing, Intel Corporation

- R3-221944, AI/ML-based mobility optimization, Intel Corporation

- R3-220133, LS reply on model deployment and update from OAM to NG-RAN, SA5

- TS38.331, Radio Resource Control (RRC) protocol specification (Release 16)

# Annex A – Text Proposal to TR 37.817

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# 6      Conclusion

Potential impact on SA5 specifications:

- Support Model deployment/update, which is used to initially deploy a trained, validated, and tested AI/ML model to Model Inference function or to deliver an updated model to the Model Inference function.

- Support Model performance feedback, which may be used for monitoring the performance of the AI/ML model

- Support data collection of Input and Feedback information used for AI/ML-based network energy saving, AI/ML-based load balancing and AI/ML-based mobility optimization, as defined in Section 5.1.2.4, 5.1.2.6, 5.2.2.4, 5.2.2.6, 5.3.2.4 and 5.3.2.6.

RAN3 should work on standard impacts of each use case, which are identified in Section 5.1.2.7, 5.2.2.7 and 5.3.2.7, during work item phase in Rel-18.

Common information (input/output/feedback) for all three uses cases should be worked together. Potential common Xn interface impact among three use cases:

- New signalling procedure or enhanced existing procedure to collect the input data information

  - Predicted resource status between neighboring NG-RAN nodes and source NG-RAN node

- New signalling procedure or enhanced existing procedure to retrieve feedback information

  - UE performance information between neighboring NG-RAN nodes and source NG-RAN node

  - System KPIs between neighboring NG-RAN nodes and source NG-RAN node

# Other standard impact of input/output/feedback can be found in Section 5.1.2.7, 5.2.2.7 and 5.3.2.7.