3GPP TSG-RAN WG3 Meeting #115-e	R3-221943

# Electronic meeting, 21st Feb – 3rd Mar 2022

Agenda item:		18.4.2

Source:	Intel Corporation

Title:	AI/ML based load balancing

Document for:		Discussion and Decision

# Introduction

During RAN3 #114bis-e meeting, the following open issues were captured for load balancing use case:

FFS whether validity time is applied to all outputs produced by the Model Inference function.

In this contribution, we mainly focus on the remaining open issues for AI-based load balancing.

# Discussion

## Output data of Load Balancing use case

Handover Validity Time

During RAN3 #114bis-e meeting, it was FFS whether validity time is applied to all outputs produced by the Model Inference function.

Following information were agreed as output of AI/ML-based load balancing:

•	Selection of target cell for mobility load balancing

•	Predicted own resource status information: this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

•	Predicted resource status information signalled from neighbor NG-RAN node(s): this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

•	Validity time for the Model Inference output predictions. FFS whether validity time is applied to all outputs produced by the Model Inference function.

•	The predicted UE(s) selected to be handed over to target NG-RAN node (will be used by RAN node internally)

The validity time represents how long the predicted results are valid. Meanwhile, after admission control, the target NG-RAN node may also temporarily hold resource pre-allocated for the selected UE(s) during this validity time window. If validity time window expires, the target NG-RAN node can release pre-allocated resource for the predicted handed-over UE(s). From the source NG-RAN node point of view, the corresponding target NG-RAN node and handover decision is not valid after validity time window expires.

When multiple outputs are considered for AI/ML-based load balancing, it can be considered as a multi-agent decision making problem, where all output decisions are generated at the same time based on the same set of input data (in either time domain or data set). Hence, the validity time of all output decisions is the same.

Observation 1: When multiple outputs are considered for AI/ML-based load balancing, validity time of all output decisions is the same.

Based on above observation, the validity time should be corresponding to the set of AI/ML-based load balancing decisions.

Proposal 1: The validity time is corresponding to the set of AI/ML-based load balancing outputs and applied to all outputs produced by the Model Inference function.

Predicted time stamp to handover

As discussed in [2], there were 6/12 companies supporting to introduce predicted time stamp to handover as output for AI/ML-based load balancing use case. It was agreed that the source NG-RAN node provides predicted UE(s) and corresponding target cell(s) for handover. This does not mean the target NG-RAN node should perform the predicted handover decision immediately. AI/ML model can provide the predicted output in the future, which allows the network to prepare for certain decision and reduce service interruption.

Additionally, by using AI/ML to support load balancing use case, the decision of mobility load balancing does not need to be fast reacting similar to legacy load balancing use case.

Observation 2: The handover decision for AI/ML-based load balancing can be performed in future and does not need to be a fast-reacting decision. This allows the target NG-RAN node to prepare resources in advance and further help to reduce service interruption during handover.

For load balancing handover, according to the predicted resource status of current and neighboring NG-RAN nodes, AI/ML model may select the best timing to perform load balancing handover according to the resource status. For example, assuming both source NG-RAN node and neighboring NG-RAN nodes are performing worse at Time N, if the resource status of neighboring cell is predicted becoming better at Time M and the neighboring cell could also handle the traffic from the selected UE(s) properly, AI/ML model should provide this predicted handover time, i.e. Time M, for the corresponding handover decision as the output of model inference. The predicted time stamp to handover is related to the corresponding time stamp of predicted resource status of current and neighboring NG-RAN node.

Proposal 2: The predicted time stamp to handover is generated as output of AI/ML-based load balancing use case.

## Alignment with other use cases

As discussed in the companion contribution [3], different use cases should be aligned if there’s any common information/solution is used. Based on such motivation, we proposal to consider following alignment for network energy saving use case.

In Figure 5.2.2-1:

- Change step 1 into “measurement configuration”

- Change step 2 into “UE measurement report”

- add box “measurement” at UE side after step 1.

- Step 3 and step 4, input data for AI/ML model training -> input data for Model Training

- Step 5: AI/ML Model Deployment -> model deployment/update

- Step 8: Mobility load balancing predictions (Model Inference) -> Model Inference

- Add handover as a separate step after step 10

- Split step 11 into separate steps

In Figure 5.2.2-2:

- Change step 1 into “measurement configuration”

- Change step 2 into “UE measurement report”

- Add box “measurement” at UE side after step 1

- Remove request signaling from NG-RAN node 1 to UE

- Remove request signaling from NG-RAN node 1 to NG-RAN node 2

- Step 8: Mobility load balancing predictions (Model Inference) -> Model Inference

- Add handover as a separate step after step 10

Input:

- Bullet 1 & 2 of input from local node, merge into current/predicted resource status

- change “UE trajectory prediction” into “UE mobility/trajectory prediction”

- change “UE Radio Measurements, e.g. RSRP, RSRQ, SINR” into “UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements”

- bullet 1&2 of input from neighbour NG-RAN node, merge into current/predicted resource status

A text proposal for the above solutions and standard impact is provided in Annex A.

Proposal 3: Agree the corresponding text proposal to TR 37.817 in Annex A.

# Conclusion

In this contribution, we mainly discussed the remaining open issues and alignment of AI/ML-based load balancing.

We propose the following observations and proposals:

Observation 1: When multiple outputs are considered for AI/ML-based load balancing, validity time of all output decisions is the same.

Observation 2: The handover decision for AI/ML-based load balancing can be performed in future and does not need to be a fast-reacting decision. This allows the target NG-RAN node to prepare resources in advance and further help to reduce service interruption during handover.

Proposal 1: The validity time is corresponding to the set of AI/ML-based load balancing outputs and applied to all outputs produced by the Model Inference function.

Proposal 2: The predicted time stamp to handover is generated as output of AI/ML-based load balancing use case.

Proposal 3: Agree the corresponding text proposal to TR 37.817 in Annex A.

# References

- R3-216278, Draft TR37.817 Study on enhancement for Data Collection for NR and EN-DC

- R3-221435, Summary of CB: # AIRAN4_LB, ZTE

- R3-221941, Consideration on alignment for AI/ML in NG-RAN Rel-17 SI, Intel Corporation

# Annex A – Text Proposal to TR 37.817

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 5.2	Load Balancing

### 5.2.1	Use case description

The rapid traffic growth and multiple frequency bands utilized in a commercial network make it challenging to steer the traffic in a balanced distribution. To address the problem, load balancing had been proposed. The objective of load balancing is to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cells or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to improve network performance. This can be done by means of optimization of handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

However, the optimization of the load balancing is not an easy task as follows:

- Currently the load balancing decisions relying on the current/past-state cell load status are insufficient. The traffic load and resource status of the network changes rapidly, especially in the scenarios with high-mobility and large number of connections, which may lead to ping-pong handover between different cells, cell overload and degradation of user service quality.

- It is difficult to guarantee the overall network and service performance when performing load balancing. For the load balancing, the UEs in the congested cell may be offloaded to the target cell, by means of handover procedure or adapting handover configuration. For example, if the UEs with time-varying traffic load are offloaded to the target cell, the target cell may be overloaded with new-arrival heavy traffic. It is difficult to determine whether the service performance after the offloading action meets the desired targets.

To deal with the above issues, solutions based on AI/ML model could be introduced to improve the load balancing performance. Based on collection of various measurements and feedbacks from UEs and network nodes, historical data, etc. AI/ML model-based solutions and predicted load could improve load balancing performance, in order to provide higher quality user experience and to improve the system capacity.

### 5.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

          - 5.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based load balancing:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM.

Other possible locations of the AI/ML Model Inference are FFS.

          - 5.2.2.2 AI/ML Model Training in OAM and AI/ML Model Inference in a NG-RAN node

A high-level signalling flow for the AI/ML use case related to Load Balancing with Model Training in OAM and Model Inference in NG-RAN is shown in Figure 5.2.2-1 below.

Figure 5.2.2-1 Model Training at OAM, Model Inference at NG-RAN

Step 0:  NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with useful input information, such as predicted resource status, etc.

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE reports to NG-RAN node 1 requested measurements and/or location information (e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells, velocity, position).

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM.

Step 5: NG-RAN node 2 also sends input data for Model Training to OAM.

Step 6: AI/ML Model Training is located at OAM. The required measurements are leveraged to train the AI/ML models for load balancing.

Step 7: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node is allowed to continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-17 scope.

Step 8: The UE collects and reports to NG-RAN node 1 requested measurements or location information.

Step 9: The NG-RAN node 1 receives from the neighbouring NG-RAN node 2 the input information for load balancing model inference.

Step 10: NG-RAN node 1 performs Mobility Load Balancing predictions (e.g. for cells of NG-RAN node 1).

Step 11. The NG-RAN 1 sends the model performance feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 12: NG-RAN nod 1 executes Mobility Load Balancing actions and UEs are moved from NG-RAN node 1 to NG-RAN node 2.

Step 13: NG-RAN node 1 selects the most appropriate target cell for each UE and performs handover.

Step 14: NG-RAN node 1 sends feedback information to OAM.

Step 15: NG-RAN node 2 sends feedback information to OAM.

          - 5.2.2.3 AI/ML Model Training and AI/ML Model Inference in a NG-RAN node

A high-level signalling flow for the AI/ML use case related to Load Balancing with Model Training and Model Inference in a NG-RAN node is shown in Figure 5.2.2-2 below.

Figure 5.2.2-2: Model Training and Model Inference in a NG-RAN node

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with useful input information, such as predicted resource status, etc.

Step 1: The NG-RAN node 1 configures UE to provide measurements and/or location information(e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE reports to NG-RAN node 1 the requested measurements and/or location information (e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells, velocity, position).

Step 4: The NG-RAN node 1 receives from the neighbouring NG-RAN node 2 the input information for load balancing model training.

Step 5: An AI/ML Model Training is located at NG-RAN node 1. The required measurements are leveraged to train the AI/ML model.

Step 6: NG-RAN node 1 receives UE measurements and/or location information.

Step 7: NG-RAN node 1 can receive from the neighbouring NG-RAN node 2 the input information for load balancing model inference.

Step 8: NG-RAN node 1 performs Mobility Load Balancing predictions (e.g., for cells of NG-RAN node 1).

Step 9: NG-RAN node 1 takes Mobility Load Balancing decision and UEs are moved from NG-RAN node 1 to NG-RAN node 2.

Step 10: NG-RAN node 1 selects the most appropriate target cell for each UE and performs handover.

Step 11: NG-RAN node 2 sends feedback information to NG-RAN node 1 (e.g. resource status updates after load balancing, etc).

#### 5.2.2.4	Input of AI/ML-based Load Balancing

To predict the optimized load balancing decisions, NG-RAN may need following information as input data for AI/ML-based load balancing:

From the local node:

- Current/Predicted resource status

- UE mobility/trajectory prediction

From the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

- UE Mobility History Information

From neighbour NG-RAN Nodes:

- Current/Predicted resource status

- UE performance measurement at traffic offloaded neighbour cell

#### 5.2.2.5	Output of AI/ML-based Load Balancing

AI/ML-based load balancing model can generate following information as output:

- Selection of target cell for mobility load balancing

- Predicted own resource status information: this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

- Predicted resource status information signalled from neighbor NG-RAN node(s): this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

- Validity time for the Model Inference output predictions. FFS whether validity time is applied to all outputs produced by the Model Inference function.

- The predicted UE(s) selected to be handed over to target NG-RAN node (will be used by RAN node internally)

#### 5.2.2.6	Feedback of AI/ML-based Load Balancing

To optimize the performance of AI/ML-based load balancing model, following feedback can be considered to be collected from NG-RAN nodes:

- UE performance information from target NG-RAN (for those UEs handed over from the source NG-RAN node)

- Resource status information updates from target NG-RAN

- System KPIs (e.g., throughput, delay, RLF of current and neighbours)

#### 5.2.2.7	Standard impacts

Potential Xn interface impact:

- New signalling procedure or enhanced existing procedure to collect the input data information

  - Predicted energy efficiency between neighboring NG-RAN nodes and source NG-RAN node

  - Predicted resource status between neighboring NG-RAN nodes and source NG-RAN node

- New signalling procedure or enhanced existing procedure to provide the output data information

  - Predicted resource status between neighboring NG-RAN nodes and source NG-RAN node

  - Validity time for the Model Inference output predictions.

- New signalling procedure or enhanced existing procedure to retrieve feedback information

  - UE performance information between neighboring NG-RAN nodes and source NG-RAN node

  - System KPIs between neighboring NG-RAN nodes and source NG-RAN node