3GPP TSG-RAN WG3 Meeting #115-e	R3-221957

# E-meeting, 21 Feb – 3 Mar 2022													  Revision of R3-215136

Title: 	Assistance information for UL duplication

Source: 	Huawei, China Unicom, China Telecom, CATT

Agenda item:	31.2.2

Document Type:	discussion

1. Introduction

The UL PDCP duplication for MR-DC was discussed at previous meetings with the following minutes in the Chairman notes.

RAN3-114-e meeting Propose to agree the following and capture into the Chairman minutes: For R15/R16 status quo, both MN and SN (without node coordination) can send the activation/deactivation MAC CE to the UE. And the UE just follows the received MAC CE. Consider if possible to have a joint solution (including both CP based, UP based approach after taking respective potential issues into account). Consider to close this topic if still no consensus (i.e. up to contribution driven later on). To be continued…. RAN3-113-e meeting: Solutions on the on the table: - Sol1: indicate MAC entity control - Sol2: Radio Quality information exchange - Sol3: all RLC status exchange - Sol4: partial RLC status exchange Find the compromised solution? To be continued... RAN3-112-e meeting: The topic is to be discussed in TEI17. No RAN2 impact is expected. it is understood that we do not challenge the status quo for Rel-16.

This document further discusses the assistance information for UL duplication coordination.

2. Discussion

Last RAN3#114-e meeting confirmed that that for R15/R16 status quo, both MN and SN (without node coordination) can send the activation/deactivation MAC CE to the UE. And the UE just follows the received MAC CE. This is already supported that at RAN2#110 meeting, as follows for R16 IIoT:

- The UE just follows the received MAC CE, even if the RLCi field belongs to the other node. No specification change is required.

2.2 Potential solutions

2.2.1 Solution 1: indicating MAC entity control

This was proposed in [3], where the PDCP entity indicates when the assisting node can use the Rel-16 MAC CE. Typically, for SN terminated PDCP, the indicator is used to indicate whether the M-NG-RAN to perform MAC CE control; For MN terminated PDCP, the indicator is used to indicate whether the S-NG-RAN to perfrom MAC CE control.

We may want to re-iterate our understanding below: for URLLC services, the high reliability/low latency has the highest priority, even at the cost of the radio resource inefficiency. And diverse resource repetition/redundant mechanisms have been introduced across RAN groups. This means that any new solutions considered should not deteriorate and violate this utmost objective.

Also we see many disadvantages and issues as follows.

-	It cannot ensure the high-reliability. In case one connection is poor (assuming the connection with PDCP entity node), the other node can not send the MAC CE to activate the PDCP duplication timely.

-	Lack of flexibility. Each node should have its own decision to activate/deactivate to ensure URLLC services performance based on its channel/load status. But this solution limits to a single node (the PDCP entity node) to control the MAC-CE.

-	Still blind activation/deactivation command without the knowledge of another link. Then it comes with resource waste, or even worse long latency.

Hence we think that this solution breaks the R15/R16 status quo. And this solution may perform worse compared with R16 under some scenarios.

- Solution 1 (only single node controlling MAC CE duplication command) is not considered for IIOT R17.

2.2.2 Solution 2: UL duplication coordination assistance information

As discussed above, several alternatives are proposed for the UL duplication coordination. One feasible solution is to reuse the radio quality assistance information defined for Rel-15 DL duplication for Rel-16 UL duplication. Currently the Assistance Information Type already defines several information as follows, wherein the UL radio quality index, and the power headroom report can be used for UL in addition to other useful information. Hence this assistance information type can also be used for uplink. In order to differentiate that DL and UL duplication,  an additional ‘Assistance Info. Ind per RLC for UL’ is needed.

#### 5.5.3.38 Assistance Information Type Value range: {0=UNKNOWN, 1=Average CQI, 2=Average HARQ Failure, 3=Average HARQ Retransmissions, 4= DL Radio Quality Index, 5= UL Radio Quality Index, 6= Power Headroom Report, 7-228=reserved for future value extensions, 229-255=reserved for test purposes}.

As the following figure shows, in case of CU/DU architecture,  there may need to transfer multiple radio quality assistance information per RLC entity in a single GTP-U tunnel. For example, the radio quality assistance information of two secondary RLC entities need to be transferred from the DU1 to the DU2. Hence the logical ID should be added.

- For solution 2, reuse the radio quality assistance information for UL duplication coordination with new indicator and the LCH ID in the ASSISTANCE INFORMATION DATA (PDU Type 2).

In addition, this information for the UL assistance information per RLC entity needs to be included in the PDU Type 0 from the DU2 to the DU1.

Figure 3 PDCP duplication for NR DC scenario and CU/DU split

- For solution 2, add the Radio Quality Assistance Information and the LCH ID in the DL USER DATA (PDU Type 0).

2.2.3 Potential way forward

The solution2/3/4 are relying on the User Plane-based indication. For example, solution2 replies on the radio quality information exchange, while solution 3/4 exchanges the full/partial exact RLC status exchange.

For solution 3 and 4, in case each node updates its duplication state of all/partial RLC entities, it should notify another node. This dynamic exchange RLC state seems already discussed in R16. The additional latency and possibly outdated transfer should be considered further. One possible way is to define the UL duplication suggestion information per RLC entity similar to the DL duplication activation suggestion command in R15.

Whether the combination of solution 2/3/4 is necessary or not, RAN3 can further discuss in detail if solution 2 is adopted.

- RAN3 can further discuss the combination of solutions 2/3/4 if solution2 is adopted.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- Solution 1 (only single node controlling MAC CE duplication command) is not considered for IIOT R17.

- For solution 2, reuse the radio quality assistance information for UL duplication coordination with new indicator and the LCH ID in the ASSISTANCE INFORMATION DATA (PDU Type 2).

- For solution 2, add the Radio Quality Assistance Information and the LCH ID in the DL USER DATA (PDU Type 0).

- RAN3 can further discuss the combination of solutions 2/3/4 if solution2 is adopted.

The CR is provided in [4].

5. Reference

- R3-215179 Handling PDCP Duplication, Ericsson, Intel Corporation, Nokia, Nokia Shanghai Bell.

- R3-204168 Response LS on Network Coordination for UL PDCP Duplication, RAN2

- R3-213401 Handling PDCP Duplication (Ericsson, Intel Corporation, Nokia, Nokia Shanghai Bell)

- R3-221958	Assistance information for UL duplication [UL_duplication]	Huawei, China Unicom, China Telecom, CATT