3GPP TSG-RAN WG3 Meeting #115-e	R3-221964

E-meeting, 21 Feb - 3 Mar 2022

Title: 	(TP for NR_Slice BLCR for TS 38.300) Supporting Service Continuity

Source: 	Huawei

Agenda item:	17.2

Document Type:	Other

1. Introduction

During the last RAN3#114bis-e meeting [1] , issues related to service continuity were discussed and following agreements were achieved:

Repartitioning solution

The slice RRM policies/restrictions are configured from (SA5) NRM O&M for configuration based and re-partitioning solutions

Current SA5 definition and model (TS 28.541) related to RRM dedicated policy is kept unchanged from RAN3 perspective

Current resource types for RRM policy utilization measurement as defined in TS 28.541 are sufficient.

Pre-emption

WA: Pre-emption in the shared pool can rely on existing QoS flow ARP based mechanism. Nothing additional is needed.

WA: For prioritized pool, RAN assumes that SA5 statement on prioritized resource of the Slice member list prevails over QoS ARP of non Slice member list. Nothing additional is needed.

Propose the following to be continued:

SCG resources outside RA (to be continued when receiving SA2 reply LS)

Whether gNB should receive the RA of the UE from AMF in order to help operator which think that SCG resources shall not added outside the RA.

Resource shortage in Mobility scenarios (to be continued online if time allows or at next meeting)

Whether to signal time critical handover to help target gNB determine if MCRS using DC can be used at target

•	(to be debated: handover time critical is how quick the connection with source gNB will disappear is not related to delay criticality of an application)

Whether a slice resource change indication should be included in the handover request acknowledge message to help source gNB in future handover decisions

•	(to be debated: per slice RRC connected UEs and per slice DRBs are currently not exchanged over Xn Resource Status procedure)

In this contribution, the discussion mainly focuses on supporting service continuity.

2. Discussion

Last meeting agreed the BLCR [2] with addition of descriptions of the slice resources. Two editor’s notes of FFS are remained to be checked as follows.

- Editor’s Note 1: whether the “new cell” can be outside the RA of the UE is FFS.

After the RAN3#114-e meeting, a LS [3] was sent to SA2 for asking the above FFS and no reply has received yet. Before receiving their reply LS, we understand that:

- There is no need to in addition signal the UE’s RA to the NG-RAN node for the Multi-Carrier Resource Sharing to ensure the slice continuity. Note that the Core Network Assistance Information for RRC INACTIVE already includes the TAI list for RRC inactive. But this is for the RNA configuration purpose, nothing else.

- As specified in clause 9.4 in TS 38.300 [4] below, it is clear that the roaming and access restrictions, or the mobility restrictions list already provides information to allow/prohibit the UE to access. The NG-RAN can use it to select the proper SCG accordingly, no matter the new cell is within the RA or outside of the RA.

9.4	Roaming and Access Restrictions

The roaming and access restriction information for a UE includes information on restrictions to be applied for subsequent mobility action during CM-CONNECTED state. It may be provided by the AMF and also may be updated by the AMF later.

It includes the forbidden RAT, the forbidden area and the service area restrictions as specified in TS 23.501 [3]. It also includes serving PLMN/SNPN and may include a list of equivalent PLMNs. It may also include PNI-NPN mobility restrictions (i.e. list of CAGs allowed for the UE and whether the UE can also access non-CAG cells).

Upon receiving the roaming and access restriction information for a UE, if applicable, the gNB should use it to determine whether to apply restriction handling for subsequent mobility action, e.g., handover, redirection.

If the roaming and access restriction information is not available for a UE at the gNB, the gNB shall consider that there is no restriction for subsequent mobility actions.

- Over Xn setup/RAN configuration update messages, neighbour RAN nodes can exchange their supported slices per TA, so that the SCG can be setup based on these slices.  Hence this is no new things either.

Hence before receiving the reply LS, we propose to remove the editor note, and add reference of clause 9.4 to the stage2 BL CR.

- Remove the editor’s note on the “new cell”, and add reference of clause 9.4 to the stage2 BL CR.

- Editor’s Note 2: the above FFS sentence is pending confirmation from SA5.

In [5], the attributes of RRMPolicyRatio are shown in the following table:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rRMPolicyMaxRatio | M | T | T | F | T |
| rRMPolicyMinRatio | M | T | T | F | T |
| rRMPolicyDedicatedRatio | O | T | T | F | T |

From our understanding, the value of each attribute could be used to deduce how O&M may maintain the corresponding parameters. Taken this table as an example, the value of “ isWritable ” is “ T ” means RRMPolicyRatio is configurable and the value of “ isInvariant ” is “ F ”  means it shall be variant. With the above information it is clear that O&M should be able to update the RRM policies/restrictions configuration.

Hence before receiving the reply LS from SA5, we propose to remove the editor’s note.

- Remove the editor’s note on “pending confirmation from SA5” from the stage2 BL CR.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- Remove the editor’s note on the “new cell”, and add reference of clause 9.4 to the stage2 BL CR.

- Remove the editor’s note on “pending confirmation from SA5” from the stage2 BL CR.

For proposals above, the corresponding TP for TS 38.300 can be found in the section 5.

4. Reference

- R3-221042, Summary of Offline Discussion – Slicing Service Continuity (NOKIA).

- R3-221526, Support of Enhanced RAN Slicing	Nokia, Nokia Shanghai Bell.

- R3-216237, LS on Clarification of SCG outside RA for slicing.

- 3GPP TS 38.300 V16.8.0, NR; NR and NG-RAN Overall Description; Stage 2 (Release 16).

- 3GPP TS 28.541 V17.5.0, 5G Network Resource Model (NRM); Stage 2 and stage 3 (Release 17 ).

5. TP for BLCR for TS 38.300 on top of R3-221526

Change Begins

### 16.3.3	Resource Isolation and Management

Resource isolation enables specialized customization and avoids one slice affecting another slice.

Hardware/software resource isolation is up to implementation. Each slice may be assigned with either shared, prioritized or dedicated radio resource up to RRM implementation and SLA as in TS 28.541 [xx].

To enable differentiated handling of traffic for network slices with different SLA:

-	NG-RAN is configured with a set of different configurations for different network slices by OAM;

-	To select the appropriate configuration for the traffic for each network slice, NG-RAN receives relevant information indicating which of the configurations applies for this specific network slice.

16.3.3.1	Handling of Slice Resources

The NG-RAN node may use Multi-Carrier Resource Sharing or Resource Repartitioning to allocate resources to a slice during the procedures described in clause 16.3.4 to support the slice service continuity in case of slice resources shortage.

In Multi-Carrier Resource Sharing the RAN node can setup the dual connectivity or carrier aggregation with different frequency and overlapping coverage where the same slice is available and when it is allowed based on the Roaming and Access Restrictions as specified in clause 9.4.

The Resource Repartitioning allows a slice to use resources from the shared pool or/and prioritized pool when its own dedicated or prioritized resources are not available as specified in TS 28.451 [xx].

Slice RRM policies/restrictions associated with Resource Repartitioning are configured from O&M.

Measurements of RRM policy utilization according to resource types defined in TS 28.451 are reported from RAN nodes to O&M and may lead O&M to update the Slice RRM policies/restrictions configuration.

<Unchanged Text Omitted>

Change Ends