3GPP TSG-RAN WG3 Meeting #115	R3-221971

E-meeting, 21 Feb – 3 Mar 2022

Title: 	(TP for Powersaving BLCR for TS 38.423) Supporting UE Power Saving Enhancements

Source: 	Huawei

Agenda item:	30.3

Document Type:	other

- Introduction

Last RAN3#114bis-e meeting discussed the paging subgrouping for UE power saving, with the following agreements and open issues.

The CN provides subgroup ID included in subgroup information. Include UE paging subgroup information into the NG Paging message for CN paging. Introduce UE subgroup information into NGAP Core Network Assistance Information. Introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE. Adopt the terminology "PEIPS Assistance Information" as the IE name to align with SA2 terminology, containing a CN subgroup ID. The CN assigned subgroup ID is encoded as 0…7. (note: further checking maybe needed) Add some clarification on stage 2 behaviour related PEIPS assistance information for RRC INACTIVE for TS38.300. the following WA achieved: WA: NG-RAN node can know the total number of subgroups supported by CN via OAM. RAN3 needs to wait RAN'2 further progress for Xn impact. It is FFS for F1 signaling impact of UEID-based subgrouping capability..

With the latest progress in RAN2, in this document, we provide the further impact analysis.

2. Discussion

The running TS 38.300 CR captures the basic flowchart of the CN-controlled subgrouping and the UE-ID based subgrouping, as specified in [1].

Fig.1 Procedure for CN controlled subgrouping

Figure 2: Procedure for UE ID based subgrouping

2.1 Impact analysis on XnAP

On the PEI monitoring, the latest RAN2#116bis-e meeting has made the following agreements.

UE is configured to monitor PEI, either only in the last used cell or any other cells (after cell reselection). FFS how the configuration is provided in [SI, RRCRelease, or NAS message].

Though the final solution on how the configuration is provided is not clear, it can be observed that the PEI monitoring is “configurable”. That is, for RRC-idle/inactive UEs,

  - the PEI can only be monitored in the last used cell, or,

  - the PEI can be monitored at any other cells.

Then over Xn interface, the PEIPS assistance information can be included in the RAN paging message, when the PEI monitoring is not limited to the last used cell (which is determined by the last serving cell).

- Remove the editor notes and FFSs in the XnAP BLCR, so that the PEIPS assistance information may be included in the RAN paging message, when the PEI monitoring is not limited to the last used cell.

2.2 Impact analysis on the F1AP

- Determination of Nsg-UEID (number of UEID-based subgroups) and subgroupsNumPerPO (total number of subgroups in a PO):

As observed below, RAN2 agreed to introduce a new parameter Nsg-UEID to indicate its support of UE-ID based subgrouping and the subgroupsNumPerPO as the total number of subgroups in a PO, in SIB1. Then it can be determined by the DU itself, since this is a cell level parameter.

RAN configuration (of subgrouping) includes the two parameters Nsg-UEID (number of UEID-based subgroups) and subgroupsNumPerPO (total number of subgroups in a PO): - - If only CN-assigned subgrouping is used, subgroupsNumPerPO is present (the value then equals to the number of CN-assigned subgroups), and Nsg-UEID is absent. - - If only UEID-based subgrouping is used, subgroupsNumPerPO and Nsg-UEID are present, and Nsg-UEID has the same value as subgroupsNumPerPO. - - If both subgrouping methods are used, both subgroupsNumPerPO and Nsg-UEID are present, and 0 < Nsg-UEID < subgroupsNumPerPO.

- The gNB-DU determines the Nsg-UEID, and the subgroupsNumPerPO and no F1 impact is foreseen.

- UE’s capability of supporting the UE ID-based subgrouping

Beside, RAN2 agrees that UE’s capability of supporting the UE ID-based subgrouping is reported to RAN by AS UE capability signalling as follows.

UE’s capability of supporting the UE ID-based subgrouping is reported to RAN by AS UE capability signalling while R2 assumes that UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling.

However, currently in F1AP Paging message, the UE radio capability for paging is not present, thus the gNB-DU can not derive whether the UE supports the UE-ID based subgrouping or not.

Hence the simple solution is to introduce an indicator indicating that the UE is capable of the UE ID-based subgrouping. Below provides the exemplary example of this simple indicator. Note that it should be noted that in case that when the PEI Assistance Information is included in the F1AP paging message, the UE-ID Based Subgrouping Indication is not needed.

- The gNB-DU needs to know the UE capability for UE-ID based subgrouping in case that the CN controlled paging subgroup ID is not received. A new indicator is introduced in the F1AP Paging message.

9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Paging Origin | O |  | 9.3.1.79 |  | YES | Ignore |  |
| UE-ID Based Subgrouping Indication | O |  | ENUMERATED (true, ...) |  | YES | Ignore |  |

- Support of TRS/CSI-RS for RRC idle/inactive

The UE power saving in idle/inactive may be enabled by using the connected TRS/CSI-RS to sleep longer before waking up for its PO. These TRS/CSI-RS configuration is provided in SIBx. Below provides the tentative structure in the running CR in R2-2201814:

-- ASN1START -- TAG-SIBx-START SIBx-r17 ::= SEQUENCE { trs-ResouceSetConfig-r17 SEQUENCE (SIZE (1..maxNrofTRS-ResourceSets-r17)) OF TRS-ResourceSet-r17 OPTIONAL, -- Need R validityDuration-r17 ENUMERATED {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]} OPTIONAL, -- Need S lateNonCriticalExtension OCTET STRING OPTIONAL, ... } TRS-ResourceSet-r17 ::= SEQUENCE { powerControlOffsetSS-r17 ENUMERATED{db-3, db0, db3, db6}, scramblingID-r17 ScramblingId, firstOFDMSymbolInTimeDomain-r17 INTEGER (0..9), startingRB-r17 INTEGER (0..maxNrofPhysicalResourceBlocks-1), nrofRBs-r17 INTEGER (24..maxNrofPhysicalResourceBlocksPlus1), ssb-Index-r17 SSB-Index, periodicityAndOffset-r17 CSI-ResourcePeriodicityAndOffset, frequencyDomainAllocation-r17 BIT STRING (SIZE (4)), indBitID-r17 INTEGER (0..5), ... } -- TAG-SIBx-STOP -- ASN1STOP

Over F1 interface, the gNB-DU can generate the SIBx since the PHY related parameters are included. Then the gNB-DU system information need to be extended to include SIBx message as well.

- The gNB-DU System Information need to be extended to include SIBx, and sent to the gNB-CU.

2.3 Impact analysis on the NGAP

There is working assumption on the total number of subgroups supported by the CN, determined by the OAM. And RAN2#116bis meeting discussed the RAN sharing case, and made the following assumption.

-  RAN2 assumes that there is no particular impact to Uu signalling to support RAN sharing. It is further assumed that Core Networks must have consistent policy if subgrouping is used by multiple Core Networks.

Also in TS 23.501, it is captured that:

- If paging subgroups are being allocated by the AMF, then all AMFs connected to one gNB (including the AMFs in different PLMNs using 5G MOCN network sharing) shall use a consistent policy in allocating UEs to the paging subgroups. - NOTE: Because the UE uses the AMF allocated paging subgroup across all cells in its TAI-list, and, different, overlapping TAI lists can be allocated to different UEs, then to avoid UE power consumption increasing, it is likely to be necessary that all AMFs with an overlapping coverage area use a consistent policy in allocating UEs to the paging subgroups.

Figure 3 depicts this RAN sharing scenario where the CNs belong to different operators.

Fig.3 RAN sharing scenario

For the RAN sharing scenario, a gNB can also be connected to multiple AMFs. Thus for the RAN sharing scenario, the question is how the shared cell can support the CN controlled subgrouping for multiple CNs of different operators.

Considering that different CNs may have different subgrouping strategies, if the CN assigned subgroups from different CNs are mixed in cell/RAN, UEs connected to different CNs would impact each other, which will degrade the overall CN subgrouping performance in the system. For instance, CN1 assigns UEs with low paging probability into subgroup 1 while CN2 assigns UEs with high paging probability into subgroup 1. Then in the shared cell, the subgroup 1 would contain both high-probability and low-probability UEs if we do not introduce any scheme for separation.

Considering that the RAN sharing scenario is popular in the current network deployment, we think the design of CN controlled subgrouping mechanism should take this scenario into account.

- RAN3 further discuss that the CN controlled subgroup number assignment for the RAN sharing case.

  - Other impact

The last RAN2#116bis meeting also agreed that:

- PEI subgroup indices are allocated to CN-assigned subgroups first. The 1st bit in the PEI bitmap corresponds to the CN-assigned subgroup #1, 2nd bit in the bitmap corresponds to the CN-assigned subgroup #2, and so on. After CN-assigned subgroups, the subsequent bits, if any, correspond to the UEID-based subgroup #1, #2, and so on. - Both subgroupNumPerPO and Nsg-UEID range from 1 to 8.

Also in the running TS 38.331 CR in R2-2201814, the parameter subgroupsNumForUEID is starting from 1.

SubgroupConfig-r17 ::=         SEQUENCE {

subgroupsNumPerPO-r17                       INTEGER (1.. maxNrofPagingSubgroups-r17),

subgroupsNumForUEID-r17                     INTEGER (1.. maxNrofPagingSubgroups-r17)                    OPTIONAL,              -- Need R

...

}

Hence the PEIPS Assistance Information can be INTEGER (1..8,…). Also the reference to TS 38.304 can be updated to TS 38.331.

- PEIPS Assistance Information can be INTEGER (1..8, …). Also the reference to TS 38.304 is updated to TS 38.331.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- Remove the editor notes and FFSs in the XnAP BLCR, so that the PEIPS assistance information may be included in the RAN paging message, when the PEI monitoring is not limited to the last used cell.

- The gNB-DU determines the Nsg-UEID, and the subgroupsNumPerPO and no F1 impact is foreseen.

- The gNB-DU needs to know the UE capability for UE-ID based subgrouping in case that the CN controlled paging subgroup ID is not received. A new indicator is introduced in the F1AP Paging message.

- The gNB-DU System Information need to be extended to include SIBx, and sent to the gNB-CU.

- RAN3 further discuss that the CN controlled subgroup number assignment for the RAN sharing case.

- PEIPS Assistance Information can be INTEGER (1..8, …). Also the reference to TS 38.304 is updated to TS 38.331.

The corresponding TP for BLCR for TS 38.423 is provided in the following section. And the TP for BLCR for NG and F1 is in [3].

4. Reference

- R2-2201157 38.300 running CR for introduction of UE power saving enhancements	Huawei, HiSilicon

- R2-2201814	38.331 running CR for introduction of UE power saving enhancements	CATT

- R3-221972	(TP for Powersaving BLCR for TS 38.473 and TS 38.413) Supporting UE Power Saving Enhancements	Huawei

5. TP for BLCR for TS 38.423 on top of R3-221593

<Unchanged Text Omitted>

8.2.5	RAN Paging

8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards an ng-eNB (e.g. NG-RAN node2).

When available, the NG-RAN node1 shall include the Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [90].

<Unchanged Text Omitted>

9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34]. | YES | ignore |
| Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | Ignore |
| PEIPS Assistance Information | O |  | 9.2.3.x |  | YES | ignore |

<Unchanged Text Omitted>

#### 9.2.3.x	PEIPS Assistance Information

This IE provides the information related to CN paging subgrouping for a particular UE, as specified in TS 38.331 [10].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Subgroup ID | M |  | INTEGER (1..8, …) |  |

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

SNTriggered,

SCGIndicator,

UESpecificDRX,

PEIPSassistanceInformation

<Unchanged Text Omitted>

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-PEIPSassistanceInformation,

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue				CRITICALITY reject		TYPE UEIdentityIndexValue							PRESENCE mandatory}|

{ ID id-UERANPagingIdentity					CRITICALITY ignore		TYPE UERANPagingIdentity							PRESENCE mandatory}|

{ ID id-PagingDRX							CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea						CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-PagingPriority						CRITICALITY ignore		TYPE PagingPriority									PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging			CRITICALITY ignore		TYPE AssistanceDataForRANPaging					PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging			CRITICALITY ignore		TYPE UERadioCapabilityForPaging					PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue		CRITICALITY ignore		TYPE ExtendedUEIdentityIndexValue				PRESENCE optional }|

{ ID id-PagingeDRXInformation				CRITICALITY ignore		TYPE PagingeDRXInformation						PRESENCE optional }|

{ ID id-UESpecificDRX						CRITICALITY ignore		TYPE UESpecificDRX									PRESENCE optional }|

{ ID id-PEIPSassistanceInformation				CRITICALITY ignore		TYPE PEIPSassistanceInformation							PRESENCE optional },

...

}

<Unchanged Text Omitted>

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

CHOinformation-Ack-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHO-Probability ::= INTEGER (1..100)

CNsubgroupID ::= INTEGER (1..8, ...)

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

<Unchanged Text Omitted>

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PEIPSassistanceInformation ::= SEQUENCE {

cNsubgroupID			CNsubgroupID,

iE-Extensions		ProtocolExtensionContainer { {PEIPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

PEIPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PacketLossRate ::= INTEGER (0..1000, ...)

PagingDRX	::= ENUMERATED {

v32,

v64,

v128,

v256,

... ,

v512,

v1024

}

<Unchanged Text Omitted>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-PEIPSassistanceInformation																				ProtocolIE-ID ::= aaa

<Unchanged Text Omitted>