3GPP TSG-RAN WG3 Meeting #115e	R3-221992

E-meeting, 21st Feb – 3rd March 2022

Agenda item:	  22.2.4

Source:	  Lenovo, Motorola Mobility

Title:		(TP to TS 38.425 BL CR) Remaining issues on Flow control and F1-U tunnel

Document for:		Discussion and Decision

1	Introduction

In RAN3#114e, the following agreements were made [1]:

-	For Broadcast and Multicast, optional use of DL flow control in the shared F1-U tunnel.

-	For split MRB with common PDCP, shared F1-U tunnel is used, existing NR user plane protocol functions need to be reviewed for their applicability for MBS.

-	For Multicast, reuse the existing UE-associated F1AP procedures to provide per UE the joined MBS Session IDs, further FFS UE specific MBS information and MBS context information (FFS).

-	Introduce a set of E1 procedures to control NR MBS resources in gNB-CU-UP including NG-U and F1-U terminations. FFS whether the E1 procedures are defined to apply for both, BC and MC. FFS whether the E1 procedures are defined on MRB context or MBS Session level.

-	If F1-U flow control is applied for NR MBS

-	Reuse existing PDU Type 0 and Type 1 to support flow control for multicast and broadcast

-	No additional protocol elements required to be specified in TS 38.425 for support of BC MBS NR

-	Discussions on additional procedure text are necessary for the DDDS procedure to clarify how to specify how the receiving node shall interpret the contained information in case DDDS is applied on an MRB.

This contribution continues to discuss the remaining issues on F1 interface for MBS.

2	Discussion

F1-U tunnel for PTM only and PTP only MRB

In last meeting, it was agreed that a shared F1-U tunnel is established between gNB-CU and gNB-DU for PTP/PTM transmission associated with split MRB with common PDCP. Besides split MRB, there are other two bearer types: PTM only MRB and PTP only MRB according to RAN2’s agreements [2]:

In RRC signalling, one MRB can be configured with PTM only or PTP only or both PTM and PTP.  Whether PTM, PTM+PTP or PTP-only can be changed from one to other via RRC signaling.

According to the agreement, RRC based MRB bearer type change is also supported. In the email discussion [3], all companies agrees that a common PDCP entity is used for MRB bearer type change. With this assumption, the shared F1-U tunnel can be used for PTM only MRB and PTP only MRB. Using a dedicated F1-U tunnel for PTM/PTP only MRB is not necessary and will waste F1-U resource.

Besides split MRB, the shared F1-U tunnel is also used for PTM only MRB and PTP only MRB.

Decision of RRC based bearer type change

For split MRB with a common PDCP, the working assumption is the decision of using PTP (RLC leg) or PTM (RLC leg) is made by the gNB-DU. For RRC based MRB bearer type change, it is straight forwards that the gNB-CU makes the decision. When gNB-CU decides to change the MRB bearer type, the gNB-CU needs to indicate the requested MRB bearer type to the gNB-DU so that the gNB-DU can provide corresponding L2/L1 configuration parameters.

gNB-CU makes the decision on RRC based MRB bearer type change and gNB-CU indicates the requested MRB bearer type to gNB-DU.

F1-U tunnel for forwarding and retransmission data

When a UE is handed over to target gNB, data forwarding for a MRB is beneficial to minimize the data loss. According to RAN2 discussion [3], PDCP status report may be triggered and reported by the UE during RRC based MRB bearer type change. The gNB may retransmit the missing PDCP SDU according to the PDCP status report. Both forwarding data and retransmission data are per UE basis. To transmit the forwarding data and retransmission data, there are two solutions:

-	Solution 1: a dedicated F1-U tunnel is established for the for transmitting the forwarding data or retransmission data. For example, when gNB-CU receives the PDCP status report from the UE, the gNB-CU identifies the receiving status and decides to retransmit the missing data to the UE. Then gNB-CU sends the missing packet over the retransmission GTP-U tunnel to the gNB-DU. When data arrives at the gNB-DU, the gNB-DU identifies the retransmission data according to the TNL information, e.g. the TEID in the GTP-U header. And the gNB-DU sends the retransmission date as a higher priority.

-	Solution 2: the shared F1-U tunnel is used for the for transmitting the forwarding data or retransmission data. For example, when the gNB-CU receives the PDCP status report from the UE, the gNB-CU identifies the receiving status and decides to retransmit the missing data to the UE. Then the gNB-CU adds MBS data retransmission information (e.g. PDCP SNs or SN range of retransmission packets) with UE index in the user plane protocol e.g. in the frame DL USER DATA. When the data arrives the gNB-DU, the gNB-DU identifies the retransmission data according to the MBS data retransmission information in the user plane protocol e.g. in the frame DL USER DATA. And then the gNB-DU sends the retransmission date as a higher priority.

For transmitting the forwarding data or retransmission data, RAN3 selects one of the following solutions:

- Solution 1: a dedicated F1-U tunnel is established for the for transmitting the forwarding data or retransmission data.

- Solution 2: the shared F1-U tunnel is used for the for transmitting the forwarding data or retransmission data.

Flow control for multicast MRB

In RAN3#114e meeting, it was agreed that:

If F1-U flow control is applied for NR MBS

-	Reuse existing PDU Type 0 and Type 1 to support flow control for multicast and broadcast

-	No additional protocol elements required to be specified in TS 38.425 for support of BC MBS NR

-	Discussions on additional procedure text are necessary for the DDDS procedure to clarify how to specify how the receiving node shall interpret the contained information in case DDDS is applied on an MRB.

For broadcast session, the existing flow control mechanism can be reused. For multicast session, there are multiple bearer type, PTP only MRB, PTM only MRB and spit MRB. If the flow control information is carried by the shared F1-U, the gNB-DU needs to aggregate. the status of PTM transmission and PTP transmissions for all the involved UEs. It is very complicated and difficult to specify all details on how to calculate the highest PDCP SN and desired buffer size. In order to minimize the specification impacts and standard effort, it is suggested to leave the flow control for multicast MRB to implementation.

The expected change on TS 38.425 is given as following:

------------------------------------------------------------------- Change Start -----------------------------------------------------------

5.2	NR user plane protocol layer services

The following functions are provided by the NR user plane protocol:

-	Provision of NR user plane specific sequence number information for user data transferred from the node hosting NR PDCP to the corresponding node for a specific data radio bearer.

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were not delivered to the UE or not transmitted to the lower layers.

-	Information of NR PDCP PDUs transmitted to the lower layers for user data associated with a specific data radio bearer.

-	Information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer;

-	Information of the currently desired buffer size at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer.

-	Information of the currently desired data rate in bytes at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for retransmission user data associate with a specific data radio bearer;

-	Information of NR PDCP PDUs transmitted to the lower layers for retransmission user data associated with a specific data radio bearer.

-	Information of the specific events at the corresponding node.

-	Information on Radio Link Quality from the corresponding node for user data associated with a specific data radio bearer.

-	Information for QoS monitoring from the corresponding node for user data associated with a specific data radio bearer.

-	In this version of the present document, the above information is also applied to MRB. For multicast MRB with shared F1-U GTP-U tunnel, how to formulate the information is up to implementation.

------------------------------------------------------------------- Change Stop -----------------------------------------------------------

For multicast MRB with shared F1-U GTP-U tunnel, how to formulate the flow control information is up to implementation.

3	Conclusion

This contribution discusses the remaining issues on F1 interface for MBS. And we propose:

- Besides split MRB, the shared F1-U tunnel is also used for PTM only MRB and PTP only MRB.

- gNB-CU makes the decision on RRC based MRB bearer type change and gNB-CU indicates the requested MRB bearer type to gNB-DU.

For transmitting the forwarding data or retransmission data, RAN3 selects one of the following solutions:

- Solution 1: a dedicated F1-U tunnel is established for the for transmitting the forwarding data or retransmission data.

- Solution 2: the shared F1-U tunnel is used for the for transmitting the forwarding data or retransmission data.

- For multicast MRB with shared F1-U GTP-U tunnel, how to formulate the flow control information is up to implementation.

Reference

[1] RAN3#114e Chairman Notes.

Annex Text Proposal

------------------------------------------------------------------- Change Start -----------------------------------------------------------

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MRB	MBS Radio Bearer

MR-DC	Multi-RAT Dual Connectivity

------------------------------------------------------------------- Next Change -----------------------------------------------------------

5.2	NR user plane protocol layer services

The following functions are provided by the NR user plane protocol:

-	Provision of NR user plane specific sequence number information for user data transferred from the node hosting NR PDCP to the corresponding node for a specific data radio bearer.

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were not delivered to the UE or not transmitted to the lower layers.

-	Information of NR PDCP PDUs transmitted to the lower layers for user data associated with a specific data radio bearer.

-	Information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer;

-	Information of the currently desired buffer size at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer.

-	Information of the currently desired data rate in bytes at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for retransmission user data associate with a specific data radio bearer;

-	Information of NR PDCP PDUs transmitted to the lower layers for retransmission user data associated with a specific data radio bearer.

-	Information of the specific events at the corresponding node.

-	Information on Radio Link Quality from the corresponding node for user data associated with a specific data radio bearer.

-	Information for QoS monitoring from the corresponding node for user data associated with a specific data radio bearer.

-	In this version of the present document, the above information is also applied to MRB. For multicast MRB with shared F1-U GTP-U tunnel, how to formulate the information is up to implementation.

------------------------------------------------------------------- Change End -----------------------------------------------------------