3GPP TSG-RAN WG3 Meeting #115-e	R3-222122

E-meeting, 21 February – 3 March 2022

Agenda item:	18.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TR 37.817) Final Discussions in AI/ML Load Balancing

Document for:	Text Proposal

# 1	Introduction

In this contribution, we discuss some remaining open points in AI/ML Load Balancing and try to provide some editorial corrections removing repetitions in the text of the TR.

# 2	Addressing Remaining Open Points in TR 37.817

In the last meeting we agreed the following:

- Model Inference in the DU is not allowed for CU/DU split architectures.

This agreement implies that there won’t be any other locations for AI/ML Model inference considered in this study.

Proposal 1: Remove from the text of the TR “Other possible locations of the AI/ML Model Inference are FFS.”

Another aspect that has been discussed in several meetings is that of validity time for predictions. Even though we have discussed validity time over several meetings, we haven’t captured its definition in the TR. Validity time seems to be a model specific parameter related to how long a prediction is valid. How it can be defined exactly is unclear:

- Is validity time a time instant in the horizon that a prediction is expected to be valid?

- Is validity time a time period/window inside which a prediction is valid?

Different definitions of this time will affect the result of the prediction since the statistics of the predicted result depend on it.

Observation 1: There doesn’t seem to be a unique definition of what validity time means.

The problem is essentially to identify Model output validity. Model output validity though may not necessarily depend on time, but may depend on other information as well, e.g., related to the area where the prediction is made.

Observation 2: Model output validity may depend on information other than time.

Furthermore, defining a time of validity of a prediction may not be a simple task. It is perhaps more natural to assume that once a prediction becomes invalid, Model Inference sends a new prediction.

Observation 3: A prediction can be assumed to be valid until a new prediction is sent.

It is therefore a bit ambiguous still how validity time can be used to indicate validity of Output information. For consistency with our view on the other use cases, here also we support the following:

Proposal 2: Do not send a validity time in Model Inference Output. Model output validity can be discussed in normative phase per inference output.

The FFS related to the details of the procedure of requesting load predictions from a neighbouring node is redundant. This FFS is addressed already in the potential interface impacts and should therefore be deleted.

Proposal 3: Remove from the text “Details of the procedure are FFS.” where this procedure relates to requesting load predictions from a neighbouring node to improve load balancing decisions at a gNB (gNB-CU).

Another open point is related to whether new UE measurements are needed for AI/ML based load balancing. In our view, the captured UE measurements are sufficient and no other UE measurements are needed to be considered in the input for this study.

Proposal 4: No new UE measurements are needed. Remove the related FFS from TR 37.817.

A final open point is related to whether MDT/RRM needs to be enhanced in order to collect consecutive UE information. Collecting consecutive UE information could help to obtain information from UEs that switch between RRC Connected and RRC Idle states. However, it is unclear whether network should rely extensively on obtaining input data for Training purposes from the UE. It is not possible always to rely that a UE is willing to provide the needed information and therefore it seems that utilizing data available at the network side related to e.g., a UE’s location is a more reliable method.

Observation 4: The use case creating a requirement for collecting consecutive UE information is of unclear necessity.

Proposal 5: Do not consider MDT/RRM enhancement to collect consecutive UE information.

Finally, we propose to align the figure related to Model Training and Model Inference in the RAN to resemble the generality of the other figures captured for AI/ML Energy Saving and Mobility Optimization use cases. The current request for input data for load balancing training at step 3 may be mistakenly assumed that it requests input data both for training at step 4 and for inference at step 7. The exact details of these procedures will be detailed in work item phase.

Proposal 6: Remove the request for input data for load balancing model training from Figure 5.2.2-2 illustrating Model Training and Model Inference at an NG-RAN node.

# 3	Way Forward for Rel. 18 Work Item

During the Rel. 17 study, slicing hasn’t been discussed as part of the prioritized use cases. Legacy SON/MDT mechanisms inherently support slicing information.

Observation 5: Even though slicing is part of existing procedures, slicing hasn’t been explicitly discussed in AI/ML Rel.17 study.

During this study we have introduced new information that can be communicated to enable AI/ML in the RAN. A big part of this information is based on predictions, calculated at a gNB, e.g., resource status and utilization predictions. In our view, when applicable we should use legacy SON/MDT as a baseline in normative work. As an example, load predictions can be based on existing metrics used for load reporting such as CAC, PRB, etc. Furthermore, the prediction granularity for the load as defined in SON/MDT can be considered e.g., per SSB area, per cell, etc.. We believe that the already defined granularity is also supported when it comes to predictions of information.

Proposal 7: Use existing SON/MDT procedures as a baseline when possible in normative work, i.e., use predictions of existing metrics (e.g., CAC, PRB) with their defined granularities (e.g., per SSB area, per cell).

Load related measurements can be reported per slice in Resource Status Reporting. Slice Available Capacity indicates the network resources on a slice in the Uplink and Downlink directions  Predicted load information could include slicing information so that reporting is on a per slice basis. Slicing enhancements have also been part of Rel-17. There are several possible ways forward on how to handle slicing information in this Rel.17 study and subsequent Rel.18 work:

- Include in the scope of the study predictions of existing metrics that may be made on a per slice granularity and in this way introduce slicing aspects into the load balancing use case.

- Exclude slicing aspects completely from the study and revisit those under a new use case covering slicing enhancements for a possible Rel.18 SI.

Proposal 8:  RAN3 is invited to discuss and make a decision on the preferred way forward between the following 2 options with respect to slicing information in the AI/ML Rel.17 study/Rel.18 work:

- Include in the scope of the study predictions of existing metrics that may be made on a per slice granularity and in this way introduce slicing aspects into the load balancing use case.

- Exclude slicing aspects completely from the study and revisit those under a new use case covering slicing enhancements for a possible Rel.18 SI.

We have a preference to the first approach

# 4	Conclusion

We make the following observations and proposals:

Proposal 1: Remove from the text of the TR “Other possible locations of the AI/ML Model Inference are FFS.”

Observation 1: There doesn’t seem to be a unique definition of what validity time means.

Observation 2: Model output validity may depend on information other than time.

Observation 3: A prediction can be assumed to be valid until a new prediction is sent.

Proposal 2: Do not send a validity time in Model Inference Output. Model output validity can be discussed in normative phase per inference output.

Proposal 3: Remove from the text “Details of the procedure are FFS.” where this procedure relates to requesting load predictions from a neighbouring node to improve load balancing decisions at a gNB (gNB-CU).

Proposal 4: No new UE measurements are needed. Remove the related FFS from TR 37.817.

Observation 4: The use case creating a requirement for collecting consecutive UE information is of unclear necessity.

Proposal 5: Do not consider MDT/RRM enhancement to collect consecutive UE information.

Proposal 6: Remove the request for input data for load balancing model training from Figure 5.2.2-2 illustrating Model Training and Model Inference at an NG-RAN node.

Observation 5: Even though slicing is part of existing procedures, slicing hasn’t been explicitly discussed in AI/ML Rel.17 study.

Proposal 7: Use existing SON/MDT procedures as a baseline when possible in normative work, i.e., use predictions of existing metrics (e.g., CAC, PRB) with their defined granularities (e.g., per SSB area, per cell).

Proposal 8:  RAN3 is invited to discuss and make a decision on the preferred way forward between the following 2 options with respect to slicing information in the AI/ML Rel.17 study/Rel.18 work:

- Include in the scope of the study predictions of existing metrics that may be made on a per slice granularity and in this way introduce slicing aspects into the load balancing use case.

- Exclude slicing aspects completely from the study and revisit those under a new use case covering slicing enhancements for a possible Rel.18 SI.

# Annex - TP for TR 37.817

### 5.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 5.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based load balancing:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM.

#### 5.2.2.2 AI/ML Model Training in OAM and AI/ML Model Inference in a NG-RAN node

A high-level signalling flow for the AI/ML use case related to Load Balancing with Model Training in OAM and Model Inference in NG-RAN is shown in Figure 5.2.2-1 below.

Figure 5.2.2-1 Model Training at OAM, Model Inference at NG-RAN

Step 0:  NG-RAN node 2 may have an AI/ML model, which can provide NG-RAN node 1 with useful input information, such as predicted resource status, etc.

Step 1: The NG-RAN node 1 requests the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects and reports to NG-RAN node 1 requested measurements and/or location information (e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells, velocity, position).

Step 3: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 also sends input data for Model Training to OAM.

Step 4: AI/ML Model Training is located at OAM. The required measurements are leveraged to train the AI/ML models for load balancing.

Step 5: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node is allowed to continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-17 scope.

Step 6: The UE collects and reports to NG-RAN node 1 requested measurements or location information.

Step 7: The NG-RAN node 1 receives from the neighbouring NG-RAN node 2 the input information for load balancing model inference.

Step 8: NG-RAN node 1 performs Mobility Load Balancing predictions (e.g. for cells of NG-RAN node 1).

Step 9. The NG-RAN 1 sends the model performance feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 10: NG-RAN nod 1 executes Mobility Load Balancing actions and UEs are moved from NG-RAN node 1 to NG-RAN node 2.

Step 11: NG-RAN node 1 and NG-RAN node 2 send feedback information to OAM.

#### 5.2.2.3 AI/ML Model Training and AI/ML Model Inference in a NG-RAN node

A high-level signalling flow for the AI/ML use case related to Load Balancing with Model Training and Model Inference in a NG-RAN node is shown in Figure 5.2.2-2 below.

Figure 5.2.2-2: Model Training and Model Inference in a NG-RAN node

Step 0: NG-RAN node 2 may have an AI/ML model optionally, which can provide NG-RAN node 1 with useful input information, such as predicted resource status, etc.

Step 1: The NG-RAN node 1 requests UE to provide measurements and/or location information(e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects and reports to NG-RAN node 1 the requested measurements and/or location information (e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells, velocity, position).

Step 3: The NG-RAN node 1 receives from the neighbouring NG-RAN node 2 the input information for load balancing model training.

Step 4: An AI/ML Model Training is located at NG-RAN node 1. The required measurements are leveraged to train the AI/ML model.

Step 5: NG-RAN node 1 receives UE measurements and/or location information.

Step 6: NG-RAN node 1 can receive from the neighbouring NG-RAN node 2 the input information for load balancing model inference.

Step 7: NG-RAN node 1 performs Mobility Load Balancing Model Inference (e.g., for cells of NG-RAN node 1).

Step 8: NG-RAN node 1 takes Mobility Load Balancing action and UEs are moved from NG-RAN node 1 to NG-RAN node 2.

Step 9: NG-RAN node 2 sends feedback information to NG-RAN node 1

#### 5.2.2.4	Input of AI/ML-based Load Balancing

To predict the optimized load balancing decisions, NG-RAN may need following information as input data for AI/ML-based load balancing:

From the local node:

- Own resource status information (e.g. per cell, per SSB Area): e.g., this can be calculated using predictions of some or all of the resource information specified in current XnAP

- Predicted own resource status information: e.g., this can be calculated using predictions of some or all of the resource information specified in current XnAP

- UE trajectory prediction

From the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE Radio Measurements, e.g., RSRP, RSRQ, SINR

- UE Mobility History Information

From neighbour NG-RAN Nodes:

- Neighbour resource status information (e.g. per cell, per SSB Area): it may include, e.g., some or all of the resource information in current Xn: Resource Status Update procedure

- Predicted neighbour resource status information: this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

- UE performance measurement at traffic offloaded neighbour cell

#### 5.2.2.5	Output of AI/ML-based Load Balancing

AI/ML-based load balancing model can generate following information as output:

- Selection of target cell for mobility load balancing

- Predicted own resource status information: this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

- Predicted resource status information signalled from neighbor NG-RAN node(s): this can be calculated using, e.g., predictions of some or all of the resource information specified in current XnAP

- The predicted UE(s) selected to be handed over to target NG-RAN node (will be used by RAN node internally)

Model output validity can be discussed in normative phase per inference output.

#### 5.2.2.6	Feedback of AI/ML-based Load Balancing

To optimize the performance of AI/ML-based load balancing model, following feedback can be considered to be collected from NG-RAN nodes:

- UE performance information from target NG-RAN (for those UEs handed over from the source NG-RAN node)

- Resource status information updates from target NG-RAN

- System KPIs (e.g., throughput, delay, RLF of current and neighbours)

#### 5.2.2.7	Standard impacts

To improve the load balancing decisions at a gNB (gNB-CU), a gNB can request load predictions from a neighbouring node.

If existing UE measurements are needed by a gNB for AI/ML-based load balancing, RAN3 shall reuse the existing framework (including MDT and RRM measurements).

To increase the awareness of the traffic dynamics and enable more improved traffic steering decisions it is possible to complement load measurements currently exposed over RAN interfaces with information related to predicted load from neighbouring RAN nodes as well as UE measurements and information.

- An NG-RAN node can also predict its own load. This can be achieved by considering the own load and load information received from neighbour RAN nodes. Load predictions can be signalled between RAN nodes.

- An NG-RAN node can also derive load prediction using UE measurements and information, for example MDT and RRM measurements, or UE location information (e.g. velocity, position). For the aspects concerning the configuration and the reporting of UE measurements and information the impacted protocol is RRC. RAN2 needs to be consulted for details during the normative phase.

Signalling of information used to derive Model Inference outputs may be achieved over the Xn interface by reusing existing or new procedures.  The details are to be discussed during normative work.

# Potential interface impacts:

- New or enhanced existing signaling procedure to request/retrieve predicted resource status information from neighbouring nodes via Xn interface.

- New or enhanced existing signaling procedure to request/retrieve predicted load balancing strategy information from neighbouring nodes via Xn interface.

- New or enhanced existing procedure to request/retrieve feedback information via Xn interface.