3GPP TSG-RAN WG3 Meeting #115-e	R3-222156

E-meeting, 21 Feb – 3 Mar 2022

Title: 	(TPs to TS 36.413, 38.413) CE based Carrier Selection for NB-IoT

Source: 	Huawei

Agenda item:	12.2

Document Type:	other

# 1. Introduction

In RAN3#114bis-e meeting, the following agreements/processes were made:

Summary of offline disc R3-221047 noted Draft LS to RAN2: LS on Coverage-Based Carrier Selection in R3-221150 final in R3-221162 Agreed The paged gNB/eNB should receive an indication whether the coverage -based paging carrier selection is supported and will be used or not for the UE. Send an LS to RAN2 to ask whether the paged eNB is able to know such information based on the inter-node RRC containers (i.e. UERadioPagingInformation-NB and UEPagingCoverageInformation-NB) received in S1AP/NGAP: PAGING message or whether they want RAN3 to include it in RAN3 IE. Based on RAN2 reply assess the RAN3 specification impact. To be continued...

In this contribution, we further discuss this topic based on RAN2 progress.

# 2. RAN2 related progress [1]

| Open Issue 5 | During the meeting discussion, company has mentioned that the assigned information to UE in dedicated signaling may also need to be delivered to core network. RAN3 has had some related discussion but pend to make agreement. It’s better RAN2 can provide some suggestion on how to provide such information? | Companies are invited to consider the following example questions and further provide comments in Pre117-e-offline discussion: - Whether the assigned information to UE in dedicated signaling also need to be delivered to core network and sent back to eNB in next paging? - Which way is preferred to deliver this information: - Alt1: in UEPagingCoverageInformation RRC container. This means RAN2 change. - Alt2: in S1/NG signaling, e.g., in Cell Identifier and Coverage Enhancement Level IE in TS36.413 and in Paging Assistance Data for CE Capable UE IE in TS38.413. This means RAN3 change. Based on the output of Pre117-e-offline discussion, RAN2 can further decide whether to send LS to RAN3. |
| --- | --- | --- |

The following Open issue is given and for Pre117-e-offline:

- [Open Issue 5]: Whether the assigned information to UE in dedicated signaling also need to be delivered to core network and sent back to eNB in next paging? If yes, whether there is RAN2 specification impact or it can be left to RAN3?

# 3. Discussion

As listed in section2, RAN2 will have further discussion on option issue 5 during Pre117-e-offline discussion, based on that RAN3 can make final decision on whether new NGAP/S1AP IE should be introduced or not.

Based on the previous discussion provided in [2], to support RAN2 Option 1c with Alt2 (fallback when cell change), the following conclusions were listed:

Conclusion 1: to support RAN2 option 1c, the “Carrier Selection Information” (i.e. coverage information (CEL/Rmax) for the carrier selection) needs to be included in the following messages:

- S1AP/NGAP: UE CONTEXT RELEASE COMPLETE message

- S1AP/NGAP: UE CONTEXT SUSPEND REQUEST message

- NGAP: UE CONTEXT RESUME REQUEST message

- S1AP/NGAP: PAGING message

Conclusion 2: it is assumed that RAN2 will extend the existing RRC container(s) (e.g. UERadioPagingInformation-NB and/or UEPagingCoverageInformation-NB) to support CE based carrier selection, there is no need to introduce new NGAP/S1AP IE.

Conclusion 3: to support fallback mechanism in RAN2 option 1c-Alt 2, extend the existing UEPagingCoverageInformation-NB message to carry the “Carrier Selection Information”, as it is exchanged together with last cell id over NGAP/S1AP messages.

With that, unless RAN2 feedback on the need of new S1AP/NGAP IE, there will be no need for RAN3 to introduce related new IEs. On the other hand, it is needed to clarify the new handling for the eNB by updating the procedural text of NGAP/S1AP Paging procedure, i.e. clarify that for the UE which supports CE based paging carrier selection, the eNB should use the received carrier selection information to choose the paging carrier as defined in TS36.304.

- Proposal 1: unless RAN2 feedback on the need of new S1AP/NGAP IE, there will be no need for RAN3 to introduce related new IEs

- Proposal 2: RAN3 only add clarification in the procedural text in NGAP and S1AP specs.

# 4. Conclusion

- In this contribution, we analyses the CE based carrier selection, get the following proposals:

- Proposal 1: unless RAN2 feedback on the need of new S1AP/NGAP IE, there will be no need for RAN3 to introduce related new IEs

- Proposal 2: RAN3 only add clarification in the procedural text in NGAP and S1AP specs.

- The S1AP TP to TS 36.413 is provided in section 6, and the NGAP TP to TS 38.413 is provided in section 7.

# 5. Reference

- R2-2201795 Report of [Post116bis-e][310][NBIOT/eMTC R17] Carrier Selection open issues (ZTE)

- R3-220423 (TPs to TS 36.413, 38.413) CE based Carrier Selection for NB-IoT (Huawei)

# 6. Text Proposal to TS 36.413

## 8.5	Paging

### 8.5.1	General

The purpose of the Paging procedure is to enable the MME to page a UE in the specific eNB.

### 8.5.2	Successful Operation

Figure 8.5.2-1: Paging procedure

The MME initiates the paging procedure by sending the PAGING message to the eNB.

//skip unchanged part

The Paging Priority IE may be included in the PAGING message, and if present the eNB may use it according to TS 23.401 [11] and TS 23.272 [17].

If the UE Radio Capability for Paging IE is included in the PAGING message, the eNB may use it to apply specific paging schemes. If the Enhanced Coverage Restricted IE is included in the PAGING message, the eNB shall, if supported, use it as defined in TS 23.401 [11].

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present according to TS 36.300 [14].

If the Assistance Data for CE capable UEs IE is included in the Assistance Data for Paging IE, it may be used for paging the indicated CE capable UE, together with the Paging Attempt Information IE according to TS 36.300 [14].

If the Assistance Data for CE capable UEs IE is included in the PAGING message, in case the cell supports CE based paging carrier selection, the eNB shall, if applicable, use the received information to determine the paging carrier as defined in TS 36.304 [20].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 36.300 [14].

If the Paging eDRX Information IE is included in the PAGING message, the eNB shall, if supported, use it according to TS 36.304 [20]. If the Paging Time Window IE is included in the Paging eDRX Information IE, the eNB shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [20]. The eNB should take into account the reception time of the PAGING message on the S1-MME interface to determine when to page the UE.

# 7. Text Proposal to TS 38.413

### 8.5.1	Paging

#### 8.5.1.1	General

The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

The AMF initiates the Paging procedure by sending the PAGING message to the NG-RAN node.

At the reception of the PAGING message, the NG-RAN node shall perform paging of the UE in cells which belong to tracking areas as indicated in the TAI List for Paging IE.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12] and TS 36.304 [29].

For each cell that belongs to any of the tracking areas indicated in the TAI List for Paging IE, the NG-RAN node shall generate one page on the radio interface.

If the Paging Priority IE is included in the PAGING message, the NG-RAN node may use it according to TS 23.501 [9].

If the UE Radio Capability for Paging IE is included in the PAGING message, the NG-RAN node may use it to apply specific paging schemes.

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present, according to TS 38.300 [8].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 38.300 [8].

If the Paging Origin IE is included in the PAGING message, the NG-RAN node shall transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the NB-IoT Paging eDRX Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it according to TS 36.304 [29]. If the NB-IoT Paging Time Window IE is included in the NB-IoT Paging eDRX Information IE, the NG-RAN node shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [29]. The NG-RAN node should take into account the reception time of the PAGING message on the NG interface to determine when to page the UE.

If the NB-IoT Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 36.304 [29].

If the Enhanced Coverage Restriction IE is included in the PAGING message, the NG-RAN node shall, if supported, use it as defined in TS 23.501 [9].

If the Paging Assistance Data for CE Capable UE IE is included in the Assistance Data for Paging IE in the PAGING message, it may be used for paging the indicated CE capable UE, according to TS 23.502 [10].

If the Paging Assistance Data for CE Capable UE IE is included in the PAGING message, in case the cell supports CE based paging carrier selection, the NG-RAN node shall, if applicable, use the received information to determine the paging carrier as defined in TS36.304 [29].

If the WUS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it to determine the WUS group for the UE, as specified in TS 36.304 [29].

If the Paging eDRX Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it according to TS 36.304 [29]. If the Paging Time Window IE is included in the Paging eDRX Information IE, the NG-RAN node shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [29]. The NG-RAN node should take into account the reception time of the PAGING message on the NGAP interface to determine when to page the UE.

If the CE-mode-B Restricted IE is included in the PAGING message and the Enhanced Coverage Restriction IE is not set to "restricted", the NG-RAN node shall, if supported, use it as defined in TS 23.501 [9].

If the NPN Paging Assistance Information IE is included in the Assistance Data for Paging IE, the NG-RAN node may take it into account when determining the cells where paging will be performed.

#### 8.5.1.3	Abnormal Conditions

Void.