3GPP TSG-RAN WG3 Meeting #115-e	R3-222160

E-meeting, 21 Feb – 3 Mar 2022

Title: 	Rapporteur clean-up for TS 38.401 BL CR

Source: 	Huawei

Agenda item:	22.1

Document Type:	other

# 1. Introduction

In the MBS BL CR [1] to TS 38.401, there are many Editor’s notes/FFSs, this contribution is provided to address these Editor’s notes/FFSs.

# 2. Discussion

In section 6.1.x Overall Architecture of NR MBS, there are several Editor’s Notes：

- Editor’s Note: Applicability of specified cardinalities may need to be revisited. - Editor’s Note: It is FFS whether the F1-U tunnel for the PTM transmission is established per DU or per cell. The definition and usage of the term “PTM” is FFS. Also, the definition of the term “MBS radio bearer” is FFS. - Editor’s Note: for the split MBS radio bearer with common PDCP, the statement may be re-visited after further progress in RAN2 and RAN3 on the data re-transmission and forwarding. - Editor’s Note: existing NR user plane protocol functions need to be reviewed for their applicability for MBS.

The 1) Editor’s Note was added at the beginning of the WI discussion, and based on the discussion until now, we should be able to confirm the applicability of specified cardinalities, and therefore it should be removed.

For the 2) Editor’s Note in section 6.1.x, currently the common understanding is that the shared F1-U tunnel could be established either per DU or per cell or per several cells. And PTM, MRB are well defined in TS 38.331 running CR, therefore this Editor’s node could also be removed, and we can use abbreviation MRB (MBS Radio Bearer) to replace the “MBS radio bearer” used in this BL CR.

For the 3) Editor’s Note in section 6.1.x, the sentence above this Editors’ note is fine for split MRB with common PDCP. The Editor’s Note should be removed as well.

For the 4) Editor’s Note in section 6.1.x, for flow control, the following progresses were made:

For Broadcast and Multicast, optional use of DL flow control in the shared F1-U tunnel.

If F1-U flow control is applied for NR MBS

-	Reuse existing PDU Type 0 and Type 1 to support flow control for multicast and broadcast

-	No additional protocol elements required to be specified in TS 38.425 for support of BC MBS NR

-	Discussions on additional procedure text are necessary for the DDDS procedure to clarify how to specify how the receiving node shall interpret the contained information in case DDDS is applied on an MRB.

Regardless of whether it is needed to add additional procedure text or not, this 4) Editor’s Note should be removed.

Proposal 1: remove the Editor’s Notes in section 6.1.x, and use abbreviation MRB (MBS Radio Bearer) to replace the “MBS radio bearer”.

In section 6.4	“UE associations in NG-RAN Node”, there is an Editor’s Note as follows:

For multicast, for each MRB, the MBR specific Uu configuration is incorporated into each UE’s individual CellGroupConfig, and the gNB-DU provides such information to the gNB-CU to configure the UE. Editor’s Note: The statement above concerning the incorporation of the MBR specific Uu configuration into the RRC CellGroupConfig IE needs to be checked against respective RAN2 decisions.

Based on current 38.331 running CR, this statement is fine for multicast, therefore it is needed to remove this Editor’s Note as well.

Proposal 2: remove the Editor’s Notes in section 6.4.

In Section 7.X.1 “Support of dynamic PTP and PTM switching”, there is an Editor’s Note as follows:

### 7.X.1 Support of dynamic PTP and PTM switching NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2]. In case of split gNB architecture, for a split MRB bearer with common PDCP, upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg). Editor’s note: The above paragraph would be re-visited after there is conclusion on flow control mechanism for shared F1-U tunnel.

Based on latest progress, this Editor’s Note can be removed as well.

Proposal 3: remove the Editor’s Notes in section 7.X.1.

# 3. Conclusion

- In this contribution, we discussed the Editor’s Notes in the current 38.401 BL CR, and get the proposals to remove all the Editors’ Notes, and use abbreviation MRB (MBS Radio Bearer) to replace the “MBS radio bearer”.

- The corresponding TP to do these clean-up is provided in section 5.

# 4. Reference

- R3-221521 Introduction of NR MBS, Huawei, CMCC

# 5. TP to TS 38.401 BL CR

--------------------------------Start of the First Change-----------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[5]	3GPP TS 38.414: "NG-RAN; NG data transport".

[6]	3GPP TS 38.424: "NG-RAN; Xn data transport".

[7]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[8]	ITU-T Recommendation G.823 (2000-03): "The control of jitter and wander within digital networks which are based on the 2048 kbit/s hierarchy".

[9]	ITU-T Recommendation G.824 (2000-03): "The control of jitter and wander within digital networks which are based on the 1544 kbit/s hierarchy".

[10]	ITU-T Recommendation G.825 (2001-08): "The control of jitter and wander within digital networks which are based on the synchronous digital hierarchy (SDH)".

[11]	ITU-T Recommendation G.8261/Y.1361 (2008-04): "Timing and Synchronization aspects in Packet networks".

[12]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[13]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[14]	3GPP TS 38.410: "NG-RAN; NG general aspect and principles".

[15]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles"

[16]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[17]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[18]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP Network Layer Security".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP TS 32.422: "Trace control and configuration management".

[21]	3GPP TS 37.470: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and NG-RAN; W1 general aspects and principles; Stage-2".

[22]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[23]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[24]	3GPP TS 38.425: "NG-RAN; NR user plane Protocol".

[25]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[26]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[xx]	3GPP TS 23.247: " Architectural enhancements for 5G multicast-broadcast services; Stage 2".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Associated QoS Flow: as defined in TS 23.247 [xx].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

Conditional Handover: as defined in TS 38.300 [2].

Conditional PSCell Change: as defined in TS 37.340 [12].

DAPS Handover: as defined in TS 38.300 [2].

en-gNB: as defined in TS 37.340 [12].

Early Data Forwarding: as defined in TS 38.300 [2].

gNB: as defined in TS 38.300 [2].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB that controls the operation of one or more gNB-DUs. The gNB-CU terminates the F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU.

gNB-CU-Control Plane (gNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the gNB-CU for an en-gNB or a gNB. The gNB-CU-CP terminates the E1 interface connected with the gNB-CU-UP and the F1-C interface connected with the gNB-DU.

gNB-CU-User Plane (gNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol of the gNB-CU for an en-gNB, and the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU for a gNB. The gNB-CU-UP terminates the E1 interface connected with the gNB-CU-CP and the F1-U interface connected with the gNB-DU.

IAB-node: as defined in TS 38.300 [2].

IAB-donor: as defined in TS 38.300 [2].

IAB-donor-CU: the gNB-CU of an IAB-donor, terminating the F1 interface towards IAB-nodes and IAB-donor-DU.

IAB-donor-DU: the gNB-DU of an IAB-donor, hosting the IAB BAP sublayer (as defined in TS 38.340 [22]), providing wireless backhaul to IAB-nodes.

IAB-DU: as defined in TS 38.300 [2].

IAB-MT: as defined in TS 38.300 [2].

Mapped QoS flows: Unicast QoS flows requested to be established, i.e. included in the legacy QoS flow lists in a way, that non-support RAN nodes would attempt to establish unicast QoS flows and supporting RAN nodes can identify them as mapped QoS flows based on the associated QoS information.

ng-eNB: as defined in TS 38.300 [2].

ng-eNB Central Unit (ng-eNB-CU): as defined in TS 37.470 [21].

ng-eNB Distributed Unit (ng-eNB-DU): as defined in TS 37.470 [21].

NG-RAN node: as defined in TS 38.300 [2].

PDU Session Resource: This term is used for specification of NG, Xn, and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support a PDU Session.

Public Network Integrated NPN: as defined in TS 23.501 [3].

Stand-alone Non-Public Network: as defined in TS 23.501 [3].

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BH	Backhaul

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

DAPS	Dual Active Protocol Stack

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

MBS	Multicast Broadcast Service

MRB	MBS Radio Bearer

NAS	Non-Access Stratum

NID	Network identifier

NPN	Non-Public Network

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM	Point to Multipoint

O&M	Operation and Maintenance

PWS	Public Warning System

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SAP	Service Access Point

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SM	Session Management

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

--------------------------------Start of the Next Change-----------------------------

## 6.1	Overview

//skip unchanged part

### 6.1.x	Overall Architecture of NR MBS

The overall architecture specified in section 6.1.1 and 6.1.2 applies for NR MBS.

Upon establishment of a MBS Session resource by the 5GC, the gNB-CU triggers the establishment of MRBs, involving the gNB-DU. If E1 is deployed, the gNB-CU-CP triggers establishment of respective MBS UP resources in the gNB-CU-UP.

The gNB-DU assigns the G-RNTI.

A shared F1-U tunnel is used between the gNB-CU and the gNB-DU for PTM transmission of a MRB, and for the data transmission of a split MRB. The gNB-DU assigns the DL GTP-U TEID and provides it to the gNB-CU. If E1 is deployed the gNB-CU-CP forwards it to the gNB-CU-UP.

For both broadcast and multicast, DL flow control maybe used for the shared F1-U tunnel established for the MRB, as specified in TS 38.425 [24].

//skip unchanged part

## 6.4	UE associations in NG-RAN Node

There are several types of UE associations needed in the NG-RAN node: the "NG-RAN node UE context" used to store all information needed for a UE and the associations between the UE and the logical NG and Xn connections used for NG/XnAP UE associated messages. An "NG-RAN node UE context" exists for a UE in CM_CONNECTED.

Definitions:

NG-RAN node UE context:

An NG-RAN node UE context is a block of information in an NG-RAN node associated to one UE. The block of information contains the necessary information required to maintain the NG-RAN services towards the active UE. An NG-RAN node UE context is established when the transition to RRC CONNECTED for a UE is completed or in the target NG-RAN node after completion of handover resource allocation during handover preparation, in which case at least UE state information, security information, UE capability information and the identities of the UE-associated logical NG-connection shall be included in the NG-RAN node UE context.

For Dual Connectivity an NG-RAN node UE context is also established in the S-NG-RAN node after completion of S-NG-RAN node Addition Preparation procedure.

If radio bearers are requested to be setup during a UE Context setup or modification procedure, the UE capabilities are signalled to the receiving node as part of the UE context setup or modification procedures.

Bearer context:

A bearer context is a block of information in a gNB-CU-UP node associated to one UE that is used for the sake of communication over the E1 interface. It may include the information about data radio bearers, PDU sessions and QoS-flows associated to the UE. The block of information contains the necessary information required to maintain user-plane services toward the UE.

MBS Session context in a gNB-DU:

The definition of an MBS Session context in a gNB-DU applicable for broadcast and multicast.

An MBS Session context in a gNB-DU

-	is a block of information associated to an MBS Session, which may consist of one or several MRB Contexts;

-	corresponds to either one or several F1-U tunnels.

MRB Context in a gNB-DU:

An MRB Context is a block of information in a gNB-CU-UP node associated to one or several MRBs (MRB “instances”). The gNB-DU sets up resources for each MRB

-	based on information provided within MBS Session Context related information as received by the gNB-DU (e.g. MRB QoS, MBS service area information, etc.), and,

-	for multicast, based on the UE Contexts established for RRC_CONNECTED UEs within the gNB-DU containing joining information of the UE for the respective multicast session.

For multicast, for each MRB, the MBR specific Uu configuration is incorporated into each UE’s individual CellGroupConfig, and the gNB-DU provides such information to the gNB-CU to configure the UE.

UE-associated logical NG/Xn/F1/E1 -connection:

NGAP, XnAP, F1AP and E1AP provide means to exchange control plane messages associated with the UE over the respectively NG-C, Xn-C, F1-C or E1 interface.

A UE-associated logical connection is established during the first NGAP/XnAP/F1AP message exchange between the NG/Xn/F1 peer nodes.

The connection is maintained as long as UE associated NG/XnAP/F1AP messages need to be exchanged over the NG/Xn/F1 interface.

The UE-associated logical NG-connection uses the identities AMF UE NGAP ID and RAN UE NGAP ID.

The UE-associated logical Xn-connection uses the identities Old NG-RAN node UE XnAP ID and New NG-RAN node UE XnAP ID, or M-NG-RAN node UE XnAP ID and S-NG-RAN node UE XnAP ID.

The UE-associated logical F1-connection uses the identities gNB-CU UE F1AP ID and gNB-DU UE F1AP ID.

The UE-associated logical E1-connection uses the identities gNB-CU-CP UE E1AP ID and gNB-CU-UP UE E1AP ID.

When a node (AMF or gNB) receives a UE associated NGAP/XnAP/F1AP/E1AP message the node retrieves the associated UE based on the NGAP/XnAP/F1AP/E1AP ID.

UE-associated signalling:

UE-associated signalling is an exchange of NGAP/XnAP/F1AP/E1AP messages associated with one UE over the UE-associated logical NG/Xn/F1/E1-connection.

NOTE1:	The UE-associated logical NG-connection may exist before the NG-RAN node UE context is setup in the NG-RAN node.

NOTE2:	The UE-associated logical F1-connection may exist before the UE context is setup in the gNB-DU.

NOTE3:	The general principle described in this section also applies to ng-eNB and W1 interface, if not explicitly specified otherwise.

## 6.x	MBS Session associations in NG-RAN Node

The following MBS Session associations are defined in the NG-RAN node to support NR MBS:

NG RAN MBS Session Resource Context: Encompasses CP and UP, transport and radio resources to support an MBS Session. For multicast it also encompasses the MBS Session state (active, de-activated) information about joined UEs. If an MBS Session Resource within a gNB serves multiple MBS service areas, as specified in TS 23.247 [x] the same NG MBS Session Resource context may be associated with multiple NG-U resources. During an ongoing multicast session, NG-U resources are setup or released by the gNB upon UE mobility or UEs leaving or joining the MBS multicast session.

//skip unchanged part

# 7	NG-RAN functions description

//skip unchanged part

## 7.x	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

### 7.X.1	Support of dynamic PTP and PTM switching

NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for a split MRB bearer with common PDCP, upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg).

--------------------------------Start of the Next Change-----------------------------

# 8	Overall procedures in gNB-CU/gNB-DU Architecture

//skip unchanged part

## 8.2	Intra-gNB-CU Mobility

### 8.2.1	Intra-NR Mobility

//skip unchanged part

## 8.xx	Overall procedures for NR MBS

The following clauses describe the overall procedures for NR MBS involving E1 and F1.

--------------------------------End of the Changes-----------------------------