3GPP TSG-RAN WG3 Meeting #115-e	R3-222228

E-meeting, 21 Feb – 3 Mar 2022

Title: 	Further discussions on energy saving

Source: 	Huawei

Agenda item:	18.4.1

Document Type:	Other

1. Introduction

The AI based energy saving was discussed during RAN3#114bis-e. In R3-221439 [1], the summary of the offline discussion was presented. This paper tried to have further discussions on the remaining open issues.

2. Background

FFS Accept/reject of offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons

FFS Validity time of the predicted energy saving decisions

3. Discussion

For the validity time of energy strategy, as discussed in our paper [2], there is no need to include the validity time together with the output information every time. Firstly, the assumption made by RAN3 so far is that the received information are valid till a new version of such information is received. Secondly, the output information can be reported to the subscribed node with a given periodicity, which is just like the existing mechanism where on entity could raise request of load status to another entity with a given periodicity, this mechanism could be reused, which also imply that the output information is valid during the requested time period (validity time).

Last but not least, we think it is not proper for the inference to give such a validity time, since the inference block keeps functioning and outputting the predictions, what if a new prediction comes within the validity time, and why a new prediction could not be available within the validity time. Thus we think there is no need to introduce the validity time which will not help improve the performance but just bring uncertainties.

No need to introduce the validity time of the predicted energy saving decisions.

For the acceptation/rejection of offloading plan for a certain number of UEs from neighbouring NG-RAN node, it was argued that the benefit is to avoid energy efficiency deterioration at the target. However, until now, the energy saving builds upon the possibility for the NG-RAN node owning a capacity booster cell to autonomously decide to switch off such cell to lower energy consumption, where the decision is typically based on cell load information. In addition, in current TR, the NG-RAN node is aware of the current/predicted energy efficiency and resource status of the neighbouring NG-RAN nodes. This means the NG-RAN node takes the decision after evaluating the whole situation and believes that the decision to be made is the best for energy efficiency; on the other hand, it is a normal operation or behaviour that a node is allowed to reject an incoming request, i.e. an offloading request (HO request, for instance). However, this is just up to receiving node’s implementation whether to do or not; under normal circumstance, the receiving node should not reject an offloading request since, as analysed above, this request is for the good of the energy efficiency; but the receiving node is allowed to reject since there might be exceptional case that the decision from AI/ML model is not optimal.

An NG-RAN node is allowed to accept or reject an offloading plan from neighbouring NG-RAN node, which is up to implementation.

- With this proposal above, we think there is no need to explicitly introduce such an agreement of introducing acceptation/rejection of offloading plan from neighbour NG-RAN node, since it is just a normal stage 3 procedural behaviour.

- Here we assume that, an NG-RAN node doesn’t need the suggestion/recommendation of handover/offloading strategy for its served UEs from its neighbour node, handover/offloading strategy should be generated from the inference model, which is part of the “Output of AI/ML-based Network Energy Saving”

4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	No need to introduce the validity time of the predicted energy saving decisions.

Proposal 2:	An NG-RAN node is allowed to accept or reject an offloading plan from neighbouring NG-RAN node, which is up to implementation.

5. Reference

- R3-221439, Summary of CB: # AIRAN3_ES, Intel.

- R3-215664, Further discussions on remaining open issues about general principles and framework, Huawei.

Annex – TP

START OF CHANGE

5.1.2.2	Model Training at OAM and Model Inference at NG-RAN

In this solution, NG-RAN makes energy decisions using AI/ML model trained from OAM.

Figure 5.1.2.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends measurement report message to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM.

Step 5: Model Training at OAM. Required measurements are leveraged to train AI/ML models for network energy saving.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 scope.

Step 7: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 8: UE sends UE measurement report to NG-RAN node 1.

Step 9: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output(s) (e.g. energy saving strategy, handover strategy, etc).

Step 10: NG-RAN node 1 sends Model Performance Feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 11: NG-RAN node 1 may select the most appropriate target cell for each UE before it performs handover, if the output is handover strategy.

Step 12: NG-RAN node 2 provides feedback to OAM.

Step 13: NG-RAN node 1 provides feedback to OAM.

5.1.2.3	Model Training and Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and generates energy saving decisions.

Figure 5.1.2.2-1. Model Training and Model Inference at NG-RAN

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends measurement report to NG-RAN node 1 including the required measurement result.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based network energy saving.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based energy saving based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based energy saving optionally, which can also generate predicted results/actions.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 7: UE sends UE measurement report to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g. energy saving strategy, handover strategy, etc).

Step 9: NG-RAN node 1 may select the most appropriate target cell for each UE before it performs handover, if the output is handover strategy.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

5.1.2.4	Input of AI/ML-based Network Energy Saving

To predict the optimized network energy saving decisions, NG-RAN may need following information as input data for AI/ML-based network energy saving:

Input Information from Local node:

- UE mobility/trajectory prediction

- Current/Predicted Energy efficiency

- Current/Predicted resource status

Input Information from UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

Input from neighbouring NG-RAN nodes:

- Current/Predicted energy efficiency

- Current/Predicted resource status

- Current energy state (e.g., active, high, low, inactive)

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements).

5.1.2.5	Output of AI/ML-based Network Energy Saving

AI/ML-based network energy saving model can generate following information as output:

- Energy saving strategy, such as recommended cell activation/deactivation.

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy efficiency

- Predicted energy state (e.g., active, high, low, inactive)

5.1.2.6	Feedback of AI/ML-based Network Energy Saving

To optimize the performance of AI/ML-based network energy saving model, following feedback can be considered to be collected from NG-RAN nodes:

- Resource status of neighbouring NG-RAN nodes

- Energy efficiency

- UE performance affected by the energy saving action (e.g. handed-over UEs), including bitrate, packet loss, latency.

- System KPIs (e.g. throughput, delay, RLF of current and neighboring NG-RAN node)

5.1.2.7	Standard Impact

Potential Xn interface impact:

- New signalling procedure or enhanced existing procedure to collect the input data information

  - Predicted energy efficiency between neighboring NG-RAN nodes and source NG-RAN node

  - Predicted resource status between neighboring NG-RAN nodes and source NG-RAN node

- New signalling procedure or enhanced existing procedure to retrieve feedback information