3GPP TSG-RAN WG3 #115-e                                                                              R3-222255

Online, Feb 21 – Mar 03, 2022 e-Meeting

Title: 	(TP for TS 38.300) Discussion on Mobility between MBS Supporting and non-MBS Supporting Nodes

Source: 	CMCC

Agenda item:   	22.3.2

Document for: 		Other

Introduction

Mobility between MBS-supporting and non-MBS-supporting nodes was discussed in RAN3#114bis-e and the related agreements are as follows:

RAN3 decided to minimize data loss and agreed on the solution to eliminate duplicates via using the same Core Network Sequence Numbers over both the unicast N3 tunnel and shared N3 tunnel for the multicast related handover from non-MBS supporting gNB to MBS supporting gNB.

In this contribution, we continue to discuss the potential issues on the scenario for mobility from MBS supporting node to non-MBS supporting node.

Discussion

Discussion on Mobility from MBS supporting node to non-MBS supporting node

In this section, mobility from MBS supporting node to non-MBS supporting node is investigated. We have basically reached the agreement that source gNB is aware of the MBS support of the target gNB before the handover. It is still waiting to be discussed that using which means for source gNB to learn in advance. From our view, there are two potential options for source gNB perceiving MBS capability of target gNB.

Option 1: Based on OAM setting;

Option 2: Exchange capability during Xn setup or configuration update procedure;

Assumed that the capability of a gNB supporting the MBS is a static configuration, we think that propagating via OAM configuration is enough and it is not necessary to take any enhancement on XnAP. Additionally, considered that there is only one meeting left in Rel-17, it is not realistic to discuss the stage2 and stage3 details of capability exchange over Xn. Thus, OAM configuration is an appropriate way to inform source gNB in advance.

Proposal 1: Do not exchange whether MBS supporting or not in Xn Setup/Configuration Update.

Furthermore, the issue of how to stop data forwarding still waits to be resolved. Source gNB needs to stop data forwarding after UE gets access to target gNB. Based on the conclusion in previous meeting, we continue to working on user plane solutions. Two potential solutions are summarized in last meeting:

Solution 1: The core network provides UE individual end marker in the shared NG-U tunnel to the source gNB. And then the source gNB forwards the end marker to the target gNB.

Solution 2: The core network provides UE individual end marker in the UE associated NG-U tunnels to the source gNB. And then the source gNB forwards the end marker to the target gNB. UPF sends this UE-individual End Marker packet with the mapped QFI i.e. the QFI of the unicast QoS flow associated with the MBS flow. The gNB can identify from the received mapped QFI what is the corresponding MBS flow for which data forwarding should stop.

If applying solution1, a UE individual end marker carrying specific UE ID is provided from MB-UPF to source gNB in common GTP-U tunnel. MB-UPF is not aware of which gNB is the serving gNB and will transmit the packets to all gNBs in provision of MBS service. gNBs does not serve that UE will discard the packet and only the serving gNB understand the UE ID will receive the packet. Assumed that adopting solution1, we need to find out a UE ID for identification. Actually, it is not simple to find an appropriate ID.

Observation 1: If solution1 applied, effect on the non-serving gNBs and extra efforts are needed to consider when specifying UE identification.

If applying solution2, MBS data may be transmitted via 5GC Individual MBS traffic delivery, which requires a UE PDU session associated to a Multicast MBS session and the associated QoS flow belongs to the associated PDU session. The end marker is transmitted through a unicast tunnel with the mapped QFI of unicast flow and gNB will identify the MBS flow for which data forwarding should stop, for the specific UE. Legacy HO can be performed and no extra specification workload to support solution2.

Observation 2: If solution2 applied, there is no extra specification workload.

Compared with solution1, solution2 is easy to implement and the legacy handover is performed after the DRB is activated before the handover. Hence, aiming to perform an easily operated data forwarding, we prefer introducing UE individual End Marker packet with the mapped QFI.

Proposal 2: For mobility from MBS supporting node to non-MBS supporting node, introducing UE individual end Marker packet with the mapped QFI transmitted via UE associated NG-U tunnels is preferred.

Conclusions

In this paper, we continue the discussion on scenario of mobility from MBS supporting nodes to non-MBS supporting node. The proposals are listed below:

Observation 1: If solution1 applied, effect on the non-serving gNBs and extra efforts are needed to consider when specifying UE identification.

Observation 2: If solution2 applied, there is no extra specification workload.

Proposal 1: Do not exchange whether MBS supporting or not in Xn Setup/Configuration Update.

Proposal 2: For mobility from MBS supporting node to non-MBS supporting node, introducing UE individual end Marker packet with the mapped QFI transmitted via UE associated NG-U tunnels is preferred.

References

- TS 23.247, Architectural enhancements for 5G multicast-broadcast services

- R3-221079, Summary of Offline Discussion – MBS Mobility Non Supporting, Nokia

TP for TS 38.300

#### 16.x.5.3.2	Handover between Multicast Supporting cell and Multicast non-supporting cells

During an active multicast MBS session, at mobility from an MBS-supporting NG-RAN node to a non-MBS supporting NG-RAN node, the target NG-RAN node sets up PDU Session resources associated to the multicast MBS Session. The SMF infers from the absence of an ‘MBS-support“ indication in the Path Switch Request message (Xn handover) or Handover Request Acknowledge message (NG handover) that the 5GC has to switch to 5GC individual MBS traffic delivery for that UE as specified in TS 23.247 [x].

If data forwarding is applied the source NG-RAN node infers from the handover preparation response message that the target NG-RAN node does not support MBS and changes the QFI(s) in the forwarded packets to the associated unicast QFI(s) if respective mapping information is available. Once the source NG-RAN node receives the UE-individual end marker packet with the QFI of the unicast QoS flow associated with the MBS flow via the NG-U tunnel. The source NG-RAN node can identify from the received mapped QFI what is the corresponding MBS flow of the MBS session and stop forwarding the packets of the MBS session to the target NG-RAN node.

The source NG-RAN may also learn before the handover preparation response message that the target NG-RAN node is non-MBS supporting via OAM configuration.

For mobility from non-MBS supporting NG-RAN node to MBS-supporting NG-RAN node, the existing Xn/NG handover procedures apply. The 5GC infers from the presence of the ‘MBS-supporting“ indicator in the Path Switch Request message (Xn handover) or Handover Request Acknowledge message (NG handover) that PDU sessions resources associated with active multicast MBS session(s) can be switched from 5GC MBS individual traffic delivery to 5GC shared traffic delivery. The target NG-RAN node may buffer the packets during the MRB reconfiguration from DRB to MRB and use sequence number received from both unicast N3 tunnel and shared N3 tunnel to eliminate duplicates.