3GPP TSG-RAN WG3 #115-e                                                                              R3-222269

Online, 21th Feb – 3rd Mar 2022

Title: 	(TP to TS 38.423) UE history information in MR-DC

Source: 	CMCC, CATT

Agenda item:	10.2.1.4

Document Type:	other

# 1. Introduction

This contribution is the TP for TS 38.423 BLCR with addition of UE history information for SN.

# 3. Annex – TP to TS 38.423 BLCR

Start of the first change

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

-------- skip unchanged part ----------

For each QoS flow which has been successfully established in the S-NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, use it for RAN part delay reporting. In case such a QoS flow is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring. If the QoS Monitoring Reporting Frequency IE is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it for RAN part delay reporting.

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

Upon reception of the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, start collecting the SCG information and continue for as long as the UE stays in one of its cells.

If the UE History Information from the UE IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the PSCell Change Indication IE set to "true" is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall send the SCG UE History Information in the S-NODE MODIFICATION REQUIRED message to the M-NG-RAN node upon the change of PScell.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

============ Next Change ==============

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell change Indication | O |  | ENUMERATED (true, ...) | Indicates that M-NG-RAN node subscribes the PSCell change | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

-----------------------------next change-------------------------

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

CellAssistanceInfo-NR,

CHOinformation-Req,

CHOinformation-Ack,

CHO-MRDC-EarlyDataForwarding,

CHO-MRDC-Indicator,

CPTransportLayerInformation,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

CriticalityDiagnostics,

XnUAddressInfoperPDUSession-List,

DAPSResponseInfo-List,

DataTrafficResourceIndication,

DeliveryStatus,

DesiredActNotificationLevel,

DRB-ID,

DRB-List,

DRB-Number,

DRBsSubjectToDLDiscarding-List,

DRBsSubjectToEarlyStatusTransfer-List,

DRBsSubjectToStatusTransfer-List,

DRBToQoSFlowMapping-List,

E-UTRA-CGI,

ExpectedUEActivityBehaviour,

ExpectedUEBehaviour,

ExtendedUEIdentityIndexValue,

FiveGCMobilityRestrictionListContainer,

GlobalCell-ID,

GlobalNG-RANNode-ID,

GlobalNG-RANCell-ID,

GUAMI,

InterfaceInstanceIndication,

I-RNTI,

LocationInformationSNReporting,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

LTEUESidelinkAggregateMaximumBitRate,

LTEV2XServicesAuthorized,

MR-DC-ResourceCoordinationInfo,

ServedCells-E-UTRA,

ServedCells-NR,

ServedCellsToUpdate-E-UTRA,

ServedCellsToUpdate-NR,

MAC-I,

MaskedIMEISV,

MDT-Configuration,

MDTPLMNList,

MobilityRestrictionList,

NG-RAN-Cell-Identity,

NG-RANnodeUEXnAPID,

NR-CGI,

NE-DC-TDM-Pattern,

NRUESidelinkAggregateMaximumBitRate,

NRV2XServicesAuthorized,

PagingDRX,

PagingeDRXInformation,

PagingPriority,

PartialListIndicator,

PLMN-Identity,

PDCPChangeIndication,

PDUSessionAggregateMaximumBitRate,

PDUSession-ID,

PDUSession-List,

PDUSession-List-withCause,

PDUSession-List-withDataForwardingFromTarget,

PDUSession-List-withDataForwardingRequest,

PDUSessionResourcesAdmitted-List,

PDUSessionResourcesNotAdmitted-List,

PDUSessionResourcesToBeSetup-List,

PDUSessionResourceChangeRequiredInfo-SNterminated,

PDUSessionResourceChangeRequiredInfo-MNterminated,

PDUSessionResourceChangeConfirmInfo-SNterminated,

PDUSessionResourceChangeConfirmInfo-MNterminated,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupInfo-SNterminated,

PDUSessionResourceSetupInfo-MNterminated,

PDUSessionResourceSetupResponseInfo-SNterminated,

PDUSessionResourceSetupResponseInfo-MNterminated,

PDUSessionResourceModificationInfo-SNterminated,

PDUSessionResourceModificationInfo-MNterminated,

PDUSessionResourceModificationResponseInfo-SNterminated,

PDUSessionResourceModificationResponseInfo-MNterminated,

PDUSessionResourceModConfirmInfo-SNterminated,

PDUSessionResourceModConfirmInfo-MNterminated,

PDUSessionResourceModRqdInfo-SNterminated,

PDUSessionResourceModRqdInfo-MNterminated,

PDUSessionType,

PC5QoSParameters,

QoSFlowIdentifier,

QoSFlowNotificationControlIndicationInfo,

QoSFlows-List,

RANPagingArea,

ResetRequestTypeInfo,

ResetResponseTypeInfo,

RFSP-Index,

RRCConfigIndication,

RRCResumeCause,

SCGConfigurationQuery,

SecurityIndication,

S-NG-RANnode-SecurityKey,

SpectrumSharingGroupID,

SplitSRBsTypes,

S-NG-RANnode-Addition-Trigger-Ind,

S-NSSAI,

TargetCellList,

TAISupport-List,

Target-CGI,

TimeToWait,

TraceActivation,

UEAggregateMaximumBitRate,

UEContextID,

UEContextInfoRetrUECtxtResp,

UEContextKeptIndicator,

UEHistoryInformation,

UEIdentityIndexValue,

UERadioCapabilityForPaging,

UERadioCapabilityID,

UERANPagingIdentity,

UESecurityCapabilities,

UPTransportLayerInformation,

UserPlaneTrafficActivityReport,

XnBenefitValue,

RANPagingFailure,

TNLConfigurationInfo,

MaximumCellListSize,

MessageOversizeNotification,

NG-RANTraceID,

MobilityInformation,

InitiatingCondition-FailureIndication,

HandoverReportType,

TargetCellinEUTRAN,

C-RNTI,

UERLFReportContainer,

Measurement-ID,

RegistrationRequest,

ReportCharacteristics,

CellToReport,

ReportingPeriodicity,

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

SuccessfulHOReportInformation,

PSCellHistoryInformationRetrieve,

Coverage-Modification-List,

SCGFailureReportContainer

PSCellchangeIndication

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

id-AMF-Region-Information-To-Delete,

id-AssistanceDataForRANPaging,

id-AvailableDRBIDs,

id-Cause,

id-cellAssistanceInfo-EUTRA,

id-cellAssistanceInfo-NR,

id-CellAndCapacityAssistanceInfo-EUTRA,

id-CellAndCapacityAssistanceInfo-NR,

id-ConfigurationUpdateInitiatingNodeChoice,

id-UEContextID,

id-CriticalityDiagnostics,

id-XnUAddressInfoperPDUSession-List,

id-DesiredActNotificationLevel,

id-DRBsSubjectToStatusTransfer-List,

id-ExpectedUEBehaviour,

id-ExtendedUEIdentityIndexValue,

id-FiveGCMobilityRestrictionListContainer,

id-GlobalNG-RAN-node-ID,

id-GUAMI,

id-indexToRatFrequSelectionPriority,

id-List-of-served-cells-E-UTRA,

id-List-of-served-cells-NR,

id-LocationInformationSN,

id-LocationInformationSNReporting,

id-LocationReportingInformation,

id-LTEUESidelinkAggregateMaximumBitRate,

id-LTEV2XServicesAuthorized,

id-MAC-I,

id-MaskedIMEISV,

id-MDT-Configuration,

id-MDTPLMNList,

id-MN-to-SN-Container,

id-MobilityRestrictionList,

id-M-NG-RANnodeUEXnAPID,

id-new-NG-RAN-Cell-Identity,

id-newNG-RANnodeUEXnAPID,

id-NRUESidelinkAggregateMaximumBitRate,

id-NRV2XServicesAuthorized,

id-oldNG-RANnodeUEXnAPID,

id-OldtoNewNG-RANnodeResumeContainer,

id-PagingDRX,

id-PagingeDRXInformation,

id-PagingPriority,

id-PartialListIndicator-EUTRA,

id-PartialListIndicator-NR,

id-PCellID,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourcesActivityNotifyList,

id-PDUSessionResourcesAdmitted-List,

id-PDUSessionResourcesNotAdmitted-List,

id-PDUSessionResourcesNotifyList,

id-PDUSessionToBeAddedAddReq,

id-PDUSessionToBeReleased-RelReqAck,

id-procedureStage,

id-RANPagingArea,

id-requestedSplitSRB,

id-RequiredNumberOfDRBIDs,

id-ResetRequestTypeInfo,

id-ResetResponseTypeInfo,

id-RespondingNodeTypeConfigUpdateAck,

id-RRCResumeCause,

id-selectedPLMN,

id-ServedCellsToActivate,

id-servedCellsToUpdate-E-UTRA,

id-ServedCellsToUpdateInitiatingNodeChoice,

id-servedCellsToUpdate-NR,

id-sourceNG-RANnodeUEXnAPID,

id-SpareDRBIDs,

id-S-NG-RANnodeMaxIPDataRate-UL,

id-S-NG-RANnodeMaxIPDataRate-DL,

id-S-NG-RANnodeUEXnAPID,

id-TAISupport-list,

id-Target2SourceNG-RANnodeTranspContainer,

id-targetCellGlobalID,

id-targetNG-RANnodeUEXnAPID,

id-TimeToWait,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-TraceActivation,

id-UEContextInfoHORequest,

id-UEContextInfoRetrUECtxtResp,

id-UEContextKeptIndicator,

id-UEContextRefAtSN-HORequest,

id-UEHistoryInformation,

id-UEIdentityIndexValue,

id-UERANPagingIdentity,

id-UESecurityCapabilities,

id-UserPlaneTrafficActivityReport,

id-XnRemovalThreshold,

id-PDUSessionAdmittedAddedAddReqAck,

id-PDUSessionNotAdmittedAddReqAck,

id-SN-to-MN-Container,

id-RRCConfigIndication,

id-SplitSRB-RRCTransfer,

id-UEReportRRCTransfer,

id-PDUSessionReleasedList-RelConf,

id-BearersSubjectToCounterCheck,

id-PDUSessionToBeReleasedList-RelRqd,

id-ResponseInfo-ReconfCompl,

id-initiatingNodeType-ResourceCoordRequest,

id-respondingNodeType-ResourceCoordResponse,

id-PDUSessionToBeReleased-RelReq,

id-PDUSession-SNChangeRequired-List,

id-PDUSession-SNChangeConfirm-List,

id-PDCPChangeIndication,

id-PC5QoSParameters,

id-SCGConfigurationQuery,

id-UEContextInfo-SNModRequest,

id-requestedSplitSRBrelease,

id-PDUSessionAdmitted-SNModResponse,

id-PDUSessionNotAdmitted-SNModResponse,

id-admittedSplitSRB,

id-admittedSplitSRBrelease,

id-PDUSessionAdmittedModSNModConfirm,

id-PDUSessionReleasedSNModConfirm,

id-s-ng-RANnode-SecurityKey,

id-PDUSessionToBeModifiedSNModRequired,

id-S-NG-RANnodeUE-AMBR,

id-PDUSessionToBeReleasedSNModRequired,

id-target-S-NG-RANnodeID,

id-S-NSSAI,

id-MR-DC-ResourceCoordinationInfo,

id-RANPagingFailure,

id-UERadioCapabilityForPaging,

id-PDUSessionDataForwarding-SNModResponse,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-NE-DC-TDM-Pattern,

id-InterfaceInstanceIndication,

id-S-NG-RANnode-Addition-Trigger-Ind,

id-SNTriggered,

id-DRBs-transferred-to-MN,

id-TNLConfigurationInfo,

id-MessageOversizeNotification,

id-NG-RANTraceID,

id-FastMCGRecoveryRRCTransfer-SN-to-MN,

id-FastMCGRecoveryRRCTransfer-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-CHOinformation-Req,

id-CHOinformation-Ack,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-DAPSResponseInfo-List,

id-CHO-MRDC-EarlyDataForwarding,

id-CHO-MRDC-Indicator,

id-MobilityInformation,

id-InitiatingCondition-FailureIndication,

id-UEHistoryInformationFromTheUE,

id-HandoverReportType,

id-HandoverCause,

id-SourceCellCGI,

id-TargetCellCGI,

id-ReEstablishmentCellCGI,

id-TargetCellinEUTRAN,

id-SourceCellCRNTI,

id-UERLFReportContainer,

id-NGRAN-Node1-Measurement-ID,

id-NGRAN-Node2-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReport,

id-ReportingPeriodicity,

id-CellMeasurementResult,

id-NG-RANnode1CellID,

id-NG-RANnode2CellID,

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-SuccessfulHOReportInformation,

id-PSCellHistoryInformationRetrieve,

id-NG-RANnode1SSBOffsets,

id-NG-RANnode2SSBProposedOffsets,

id-NG-RANnode2SSBOffsetsModificationRange,

id-Coverage-Modification-List,

id-PSCellCGI,

id-FailedPSCellCGI,

id-SCGFailureReportContainer

id-PSCellchangeIndication

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional}|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional}|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional}|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional}|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional}|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional}|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation						PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE	CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellchangeIndication	CRITICALITY ignore	TYPE PSCellchangeIndication				PRESENCE optional },

...

}

--------------------------Next change-----------------------------

# PLMN-Identity ::= OCTET STRING (SIZE(3))

# PCIListForMDT ::= SEQUENCE (SIZE(1.. maxnoofNeighPCIforMDT)) OF NRPCI

# PNI-NPN-Restricted-Information ::= ENUMERATED { restriced, not-restricted, ...}

# PortNumber ::= BIT STRING (SIZE (16))

# PriorityLevelQoS ::= INTEGER (1..127, ...)

# ProtectedE-UTRAResourceIndication ::= SEQUENCE {

activationSFN					ActivationSFN,

protectedResourceList			ProtectedE-UTRAResourceList,

mbsfnControlRegionLength		MBSFNControlRegionLength					OPTIONAL,

pDCCHRegionLength				INTEGER (1..3),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResourceIndication-ExtIEs} } 	OPTIONAL,

...

}

# ProtectedE-UTRAResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

# ProtectedE-UTRAResourceList ::= SEQUENCE (SIZE (1.. maxnoofProtectedResourcePatterns)) OF ProtectedE-UTRAResource-Item

# ProtectedE-UTRAResource-Item ::= SEQUENCE {

resourceType							ENUMERATED {downlinknonCRS, cRS, uplink, ...},

intra-PRBProtectedResourceFootprint		BIT STRING (SIZE(84, ...)),

protectedFootprintFrequencyPattern		BIT STRING (SIZE(6..110, ...)),

protectedFootprintTimePattern			ProtectedE-UTRAFootprintTimePattern,

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResource-Item-ExtIEs} } 	OPTIONAL,

...

}

# ProtectedE-UTRAResource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

# ProtectedE-UTRAFootprintTimePattern ::= SEQUENCE {

protectedFootprintTimeperiodicity			INTEGER (1..320, ...),

protectedFootrpintStartTime					INTEGER (1..20, ...),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAFootprintTimePattern-ExtIEs} } 	OPTIONAL,

...

}

# ProtectedE-UTRAFootprintTimePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PSCellchangeIndication::= ENUMERATED { true, ...}

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-scgFailureInformationReport									ProcedureCode ::= xx

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs						INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN    	INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64 FFS

maxnoofPSCellsPerSN 						INTEGER ::= xx FFS

maxnoofNR-UChannels			                 INTEGER ::= FFS

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered  																					ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-PagingeDRXInformation																			ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-SuccessfulHOReportInformation																	ProtocolIE-ID ::= xxx

id-SliceRadioResourceStatus-List																	ProtocolIE-ID ::= yyy

id-CompositeAvailableCapacitySupplementaryUplink													ProtocolIE-ID ::= zzz

id-SCGUEHistoryInformation																	ProtocolIE-ID ::= www

id-NG-RANnode1SSBOffsets																			ProtocolIE-ID ::= x01

id-NG-RANnode2SSBProposedOffsets																	ProtocolIE-ID ::= x02

id-NG-RANnode2SSBOffsetModificationRange															ProtocolIE-ID ::= x03

id-Coverage-Modification-List																		ProtocolIE-ID ::= xxx

id-NR-U-Channel-List					                                                          ProtocolIE-ID ::= xx1

id-PSCellCGI						                                                              ProtocolIE-ID ::= xxx

id-FailedPSCellCGI				                                                                  ProtocolIE-ID ::= yyy

id-SCGFailureReportContainer		                                                              ProtocolIE-ID ::= zzz

id-PSCellchangeIndication

ProtocolIE-ID ::= xxx

END

End of the last change