3GPP TSG-RAN3 Meeting #115-e	R3-222278

Electronic meeting, 21 Feb – 3 Mar 2022

Agenda item:	15.2.1.1

Source: 	Samsung

Title: 	(TP for BL CR to TS 38.300) Support of NR QoE

Document for:	Other & Agreement

Introduction

This TP is to capture the agreed solutions in the previous meetings.

For RVQoE, it was agreed to use RVQoE for scheduling in previous meetings, we propose to update the stage 2 description in TS 38.300 accordingly.

For the alignment of MDT and QoE, it was agreed to use UE assisted solution, we propose the update the stage 2 to capture the agreements below

UE assisted solution can be used for MDT-QoE alignment. UE can indicate to gNB via a flag whether QoE Measurement Collection (QMC) started/ended in the UE. If the NG-RAN knows there is an MDT configuration associated with a QoE configuration (e.g., upon receiving NG-RAN Trace ID in the QoE configuration from OAM),

· NG-RAN can configure the UE with that associated MDT configuration upon receiving the QMC start indication from the UE

· NG-RAN can deactivate the associated MDT configuration upon receiving the QMC end indication from the UE

TP for BL CR TS 38.300

<Start of change>

- 10.1	Basic Scheduler Operation

In order to utilise radio resources efficiently, MAC in gNB includes dynamic resource schedulers that allocate physical layer resources for the downlink and the uplink. In this clause, an overview of the scheduler is given in terms of scheduler operation, signalling of scheduler decisions, and measurements.

Scheduler Operation:

-	Taking into account the UE buffer status and the QoS requirements of each UE and associated radio bearers, schedulers assign resources between UEs;

-	Schedulers may assign resources taking account the radio conditions at the UE identified through measurements made at the gNB and/or reported by the UE;

-	Resource assignment consists of radio resources (resource blocks).

Signalling of Scheduler Decisions:

-	UEs identify the resources by receiving a scheduling (resource assignment) channel.

Measurements to Support Scheduler Operation:

-	Uplink buffer status reports (measuring the data that is buffered in the logical channel queues in the UE) are used to provide support for QoS-aware packet scheduling;

-	Power headroom reports (measuring the difference between the nominal UE maximum transmit power and the estimated power for uplink transmission) are used to provide support for power aware packet scheduling.

-  RAN visible QoE reports (measuring the QoE metrics in UE Application layer such as Buffer Level and Playout Delay etc.) are used to provide support for QoE-aware packet scheduling.

<Next change>

  - X.5	 Alignment of MDT and QoE Measurements

Radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The immediate MDT can be configured before the QoE measurement is configured or the two could be configured simultaneously. The TCE/MCE is responsible for the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. The NG-RAN node includes time stamp information to the QoE reports to enable the correlation of corresponding measurement results of MDT and QoE at the MCE/TCE. In addition, the NG-RAN node includes the MDT configuration identifiers (either Trace Reference for the management-based MDT, or Trace ID for the signalling-based MDT) in the QoE report.

UE assisted solution can be used for MDT-QoE alignment. UE configured with QoE measurement can send to the QMC start indication and the QoE measurement end indication to notify the NG-RAN node the start and end of the QoE measurement session in UE application. Based on the indications, the NG-RAN node can activate and deactivate the associated MDT. In case of split architecture, the gNB-CU-CP can activate and deactivate the associated MDT in the gNB-CU-UP and gNB-DU upon the reception of QMC start/end indication from the UE.

<End of change>