3GPP TSG-RAN3 Meeting #115-e	R3-222280

Electronic meeting, 21 Feb – 3 Mar 2022

Agenda item:	15.3

Source: 	Samsung

Title: 	(TP for BL CR to TS 38.473) RAN visible QoE

Document for:	Discussion & Agreement

Introduction

In this contribution, we’d like to further discuss QoE-aware scheduling, RVQoE reporting at overload and mobility based on the previous discussions.

Discussion

2.1 QoE-aware scheduling

RAN3 agreed to introduce a new message from CU to DU over F1 to transfer the RVQoE information in the privious meeting, and the purpose is to provide support for QoE-aware packet scheduling. According to TS 38.300, the resources assignment in scheduler is per logical channel, which is mapped to one DRB, if we take below PDU session of IMS voice for example, one PDU session will have multiple QoS flows to serve different services, e.g. voice only, or voice and video or SIP signalling, for each service, the QoS flow will be mapped to different DRBs as they have different QoS requirement. If only PDU session ID is reported in RVQoE report, the gNB will consider the RVQoE metrics for all the DRBs in the reported PDU session ID, even the RVQoE report only reflect the experience of a specific service on one DRB, i.e. only DRB3 serves the QoS flow 3 for video service in the example below, but QoE-aware scheduling will impact other DRBs like DRB1 and DRB2 for other services, we don’t think PDU session ID is a good reference for scheduling.

On the other hand, application layer is also aware of QoS flow according to TS 27.007, the reference is shown in R3-222281 [1], so it is feasible to include QoS flow identifier in the RVQoE, which is way much better than PDU session ID for scheduling.

Observation 1, one PDU session may have different QoS flows to serve different services.

Observation 2, only reporting PDU session ID along with RVQoE report is ambiguous for scheduling.

Observation 3, UE Application is aware of QoS flow identifier just the same as PDU session ID.

Proposal 1, the QoS flow ID should be included in RVQoE report over Uu to realize QoE-aware scheduling.

In split architecture, the RVQoE information in RVQoE report should be transferred from gNB-CU to gNB-DU, the DRB ID or QoS flow ID should be included in the RVQoE information so that the scheduler in MAC layer knows the RVQoE information is for which DRB and then make a proper scheduling decision.

Proposal 2, either DRB ID or QoS flow ID should be included in QoE information transfer message over F1AP for accurate scheduling.

2.2 RAN visible QoE reporting at overload

In last RAN3 meeting, RAN3 had one open issue about “FFS whether RAN visible QoE reporting should not be paused at overload.” As RVQoE report is for radio network optimization, we think the reporting in-time is important, and the message size of RVQoE is much less than legacy QoE, so RAN3 can down select below options for handling RVQoE reporting at RAN overload.

- Option 1, RVQoE reporting will not be paused even the legacy QoE reporting is paused. We think the spec impact will be adding corresponding descriptions in RAN2 spec.

- Option 2, RVQoE reporting will be paused together with legacy QoE reporting upon receiving the pause indication from the network. However, as one of the purposes of RVQoE is for short term radio network optimization, e.g. the gNB will adjust the scheduling priority according to the RVQoE report, the original reporting time (i.e. the time when the RVQoE is generated in UE Application) should be reported to gNB as well, so that gNB can make a proper decision according to the RVQoE and its original reporting time.

In our view, we think both options are fine as long as the information in RVQoE can help RAN to make proper decision on optimization and scheduling.

Proposal 3, RAN3 to down select below options for handling RVQoE reporting at overload.

- Option 1, RVQoE reporting will not be paused even the legacy QoE reporting is paused.

- Option 2, RVQoE reporting will be paused together with legacy QoE reporting upon receiving the pause indication from the network, when reporting resumed, the original reporting time should be included in the resumed RVQoE report for better decision in RAN side.

Proposal 4, send LS to RAN2 to notify the agreement.

2.2 RAN visible QoE reporting at mobility

In last RAN3 meeting, below reporting periodicities had been agreed for RVQoE reporting.

RAN visible QoE reports and legacy QoE reports can use different periodicity, the reporting periodicity can be ms120, ms240, ms480, ms640, ms1024

On the other hand, the handover related parameters, such as TimeToTrigger and t304 have below range.

TimeToTrigger ::=                   ENUMERATED {

ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256,

ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560,

ms5120}

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

from above time duration, we can find that the RVQoE reporting period is close to the event trigger time and handover execution time, which means the RVQoE measurement period may cover the handover phase, below are two examples of RVQoE reporting at handover.

From the figure above, it’s highly possible that the RVQoE report will be generated during HO exe (exampl2 2) or after HO exe (example 1). For example 1, the latter RVQoE report reflects the UE experience during handover, it will be a very good reference for the upcoming handover UEs, if many handover UEs from gNB1 to gNB2 had bad experience according to the RVQoE report from different UEs, the source gNB (i.e. Gnb1) may adjust the handover parameters e.g. A3-TimeToTrigger, threshold, or the source gNB will not choose the gNB2 as Target node for the upcoming handover UEs or offloading UEs, the operation is mostly the same as MRO, just with more information for reference.

For legacy QoE report during handover, RAN2 had below agreements in the last meeting. So we think the RVQoE report can follow the same principle.

Except for restarts transmission of QoE reports after handover

After handover, the target gNB should transfer the RVQoE report to the source if any RVQoE report is received.

The corresponding TP for BL CR to TS 38.423 to introduce the new RVQoE Report Transfer message in R3[]

Observation 4, the RVQoE reporting period is close to the handover triggering time and execution time

Observation 5, it is highly possible that the RVQoE report can reflect the UE experience during handover.

Observation 6, the legacy QoE report should be re-transmitted after handover.

Proposal 5, RAN visible QoE report should be transmitted on Xn.

The corresponding TP for BL CR to TS 38.473 is in the annex.

Proposal 6, RAN3 agrees the TP for BL CR to TS 38.473 in the annex.

Conclusion

In this paper, we discussed RVQoE, the following are observations and proposals:

Observation 1, one PDU session may have different QoS flows to serve different services.

Observation 2, only reporting PDU session ID along with RVQoE report is ambiguous for scheduling.

Observation 3, UE Application is aware of QoS flow identifier just the same as PDU session ID.

Proposal 1, the QoS flow ID should be included in RVQoE report over Uu to realize QoE-aware scheduling.

Proposal 2, either DRB ID or QoS flow ID should be included in QoE information transfer message over F1AP for accurate scheduling.

Proposal 3, RAN3 to down select below options for handling RVQoE reporting at overload.

- Option 1, RVQoE reporting will not be paused even the legacy QoE reporting is paused.

- Option 2, RVQoE reporting will be paused together with legacy QoE reporting upon receiving the pause indication from the network, when reporting resumed, the original reporting time should be included in the resumed RVQoE report for better decision in RAN side.

Proposal 4, send LS to RAN2 to notify the agreement.

Observation 4, the RVQoE reporting period is close to the handover triggering time and execution time

Observation 5, it is highly possible that the RVQoE report can reflect the UE experience during handover.

Observation 6, the legacy QoE report should be re-transmitted after handover.

Proposal 5, RAN visible QoE report should be transmitted on Xn.

Proposal 6, RAN3 agrees the TP for BL CR to TS 38.473 in the annex.

References

- R3-222281 (TP for BL CR to TS 38.473 and TS 38.463) Alignment of MDT and QoE

Annex 1 (TP for BL CR to TS 38.473)

<Start of change>

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Response message |  | Response message |  |
| Reset | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |  |
| --- | --- | --- |
| Error Indication | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging | PAGING |  |
| Notify | NOTIFY |  |
| PWS Restart Indication | PWS RESTART INDICATION |  |
| PWS Failure Indication | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |
| Deactivate Trace |  | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |
| Access Success |  | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |  |
| QoE Information Transfer | QOE INFORMATION TRANSFER |  |

<Next change>

## 8.X	QoE Information Transfer procedures

### 8.X.1	QoE Information Transfer

#### 8.X.1.1	General

The purpose of the QoE Information Transfer procedure is to transfer RAN visible QoE information from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.X.1.2	Successful operation

Figure 8.X.1.2-1: QoE Information Transfer procedures.

The gNB-CU initiates the procedure by sending the QOE INFORMATION TRANSFER message to the gNB-DU.

The gNB-DU considers QoE information for scheduling.

#### 8.X.1.3	Abnormal Conditions

Not applicable.

<Next change>

### 9.2.X	QoE Information Transfer messages

#### 9.2.X.1	QOE INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to indicate RAN visible QoE information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| QoE Information List |  |  |  |  |  |  |
| >QoE Information Item |  |  | 1..<maxnoofQoEInformation> |  |  |  |
| >>QoE Metric List | O |  | FFS |  | - | - |
| >>>Buffer level | O |  | FFS |  |  |  |
| >>>Playout delay | O |  | FFS |  |  |  |
| >DRBID List |  |  | 9.3.1.x |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoEInformation | Maximum no. of QoE information for one UE, the maximum value is FFS. |

<Next change>

#### 9.3.1.x	DRB ID List

This IE provides DRB information for the alignment of MDT and QoE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRB ID List |  |  | 1 .. <maxnoofDRBs> |  | – |  |
| >DRB ID | M |  | 9.3.1.8 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |