3GPP TSG-RAN3 Meeting #115-e	R3-222284

Electronic meeting, 21 Feb - 3 Mar 2022

Agenda item:	17.3

Source: 	Samsung

Title: 	Support of Target NSSAI

Document for:	Other & Agreement

Introduction

In RAN3 114-bie-e meeting, RAN3 had discussed whether transferring Target NSSAI during mobility is needed or not, it seems that companies have different understandings, so we’d like to further discuss this issue in the contribution.

Discussion

How the NG-RAN node use Target NSSAI is specified in TS 23.501 as below

“The NG-RAN shall attempt to find cells of TAs that can support all the S-NSSAIs in the Target S-NSSAIs, and if no such cell of a TA is available the RAN can attempt to select cells of TAs that best match the Target S-NSSAI. The NG-RAN shall attempt to ensure continuity of the PDU Sessions with activated User Plane associated with the S-NSSAIs in the Allowed NSSAI which are in the Target NSSAI. Also, the NG-RAN should attempt to ensure continuity of service for the S-NSSAIs of the Allowed NSSAI also available in the Target NSSAI, before prioritizing cells that are not supporting one or more of the S-NSSAI of the Allowed NSSAI also available in the Target NSSAI.”

From the above, we get two points,

- Point 1, it is clear that the NG-RAN attempt to find the cell that can support all the slices in Target NSSAI or best math the Target NSSAI, so it is possible that the current serving gNB cannot find the suitable cell for the UE to redirect

- Point 2, NG-RAN cannot redirect UE if there’s on-going active PDU sessions even the most suitable cell is found according to the Target NSSAI, as NG-RAN should ensure the service continuity at first.

The two points above proves that the current serving gNB cannot redirect the UE to the suitable cell.

Observation 1, it is possible that the current serving gNB cannot find the suitable cell for the UE to redirect

Observation 2, it is possible that the current serving gNB cannot redirect the UE even the suitable cell is found if there’s on-going PDU sessions.

Observation 3, it is highly possible that the current gNB cannot redirect the UE to the suitable cell even received Target NSSAI from AMF.

Before the current serving gNB find the suitable cell and can redirect the UE to the suitable cell that support the network slices in the Target NSSI, the UE may hand over to a new cell. If the new serving cell (i.e. Target gNB) have is not aware of the Target NSSAI, the target gNB cannot redirect the UE to the cell that supported all the slices in Target NSSAI even it exists.

Observation 4, the target gNB cannot redirect the UE to the cell that supported all the slices in Target NSSAI even it exists as the target gNB is not aware of the Target NSSAI.

So based on above observations, we think the Target NSSAI should be transferred to the Target gNB.

Proposal 1, RAN3 agrees that the Target NSSAI should be transferred to the target gNB during handover.

The enhancements could be as follows

- For Xn based handover, the Target NSSAI can either be included in XnAP handover request message or NGAP path switch request message.

- For NG based handover, the Target NSSAI can be included in NGAP handover request message.

Proposal 2, RAN3 agrees to include Target NSSAI in XnAP handover request message or NGAP path switch request message.

Proposal 3, RAN3 agrees to include Target NSSAI NGAP handover request message.

Conclusion

In this paper, we discuss the further issues about Target NSSAI and the following proposals are made:

Observation 1, it is possible that the current serving gNB cannot find the suitable cell for the UE to redirect

Observation 2, it is possible that the current serving gNB cannot redirect the UE even the suitable cell is found if there’s on-going PDU sessions.

Observation 3, it is highly possible that the current gNB cannot redirect the UE to the suitable cell even received Target NSSAI from AMF.

Proposal 1, RAN3 agrees that the Target NSSAI should be transferred to the target gNB during handover.

Proposal 2, RAN3 agrees to include Target NSSAI in XnAP handover request message or NGAP path switch request message.

Proposal 3, RAN3 agrees to include Target NSSAI NGAP handover request message.

References