3GPP TSG-RAN WG3 #115-e																		R3-222338

21 Feb - 3 Mar 2022

Online

Title: 	TP for SCG BL CR to TS 38.401

Source: 	ZTE

Agenda item:	14.2

Document for:	Approval

- Introduction

In the RAN3#114-bis-e-meeting we made the following progress.

The SCG Activation Request IE with two codepoints “activate SCG, deactivate SCG” shall be used in the SN Modification Required message. Add a new IE, e.g., “SCG Activation Status” with two codepoints in the Bearer Context Setup/Modification Request message. In case the UP can not follow the decision made in the CP, then the BEARER CONTEXT SETUP/MODIFICATION FAILURE message will be sent by the UP. Add a specific cause value for data arrival or data on-going in Xn/X2/F1AP. No need to add a specific cause value for UE related issues, e.g. UE overheating and UE power saving. No need to add a specific cause value for NW power saving. Define a general cause value, e.g., Failure due to SCG (de)activation, to indicate that the request is rejected due to the rejection of SCG (de)activation. For SCG activation during SN addition, only “SCG activated” will be used in the response message when accepting SN addition. FFS whether/how to enhance the E1 and F1 inactivity notification procedure. FFS when to perform the SCG status notification from CU-CP to CU-UP in TS38.401 BL CR.

This paper further discusses SCG activation and deactivation based on these agreements and open issues, and provides the corresponding TP for TS 38.401.

- Discussion

Whether/how to enhance the E1 and F1 inactivity notification procedure

The discussion on whether/how to enhance the F1 inactivity notification procedure can be seen in R3-222340. We think that only enhancing F1 AP is enough to help CU make the good SCG (de)activation decision. But we are fine to accept E1 enhancement for the sake of progress.

When to perform the SCG status notification from CU-CP to CU-UP in TS 38.401 BL CR

In the last meeting, we have agreed to add the SCG Activation Status IE in the Bearer Context Setup/Modification Request messages which indicates that CU-CP only needs to inform CU-UP about the decision of SCG (de)activation. In the current TS 38.401 BL CR, take the procedure of SN addition with SCG deactivation as an example, SN-CU-CP informs SN-CU-UP that the SCG is deactivated (step 2) before sending the SCG deactivation request to SN-DU (step 4). Note that we have agreed to support partial rejection for this case, which indicates that SN-DU may reject the SCG deactivation request via the UE context setup response message. If SN-DU rejects the request, SN-CU-UP may receive a wrong information on SCG status, e.g. it is informed that the SCG is deactivated but in fact the SCG is activated according to the response of SN-DU. Thus, it would be more accurate to inform SN-CU-CP about the actual SCG status after sending the SN Addition Request Acknowledge (step 6).

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

The modified procedure is shown in the following figure. The other procedures shall follow the principle of this procedure to avoid the above-mentioned issue, e.g. SN-CU-CP shall inform SN-CU-UP about the actual SCG status after sending the SN Addition Request Acknowledge or receiving the SN Modification Confirm message.

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

Observation 1: In the current TS 38.401 BL CR, take the procedure of SN addition with SCG deactivation as an example, SN-CU-CP informs SN-CU-UP that the SCG is deactivated (step 2) before sending the SCG deactivation request to SN-DU (step 4).

Observation 2: If SN-DU rejects the request, SN-CU-UP may receive a wrong information on SCG status, e.g. it is informed that the SCG is deactivated but in fact the SCG is activated according to the response of SN-DU.

Proposal 1: SN-CU-CP shall inform SN-CU-UP about the actual SCG status after sending the SN Addition Request Acknowledge or receiving the SN Modification Confirm message.

Stage 3 alignment:

It is noted that the following contents are added in the TS 38.423 BL CR, while the corresponding contents are not added in the TS 36.423 BL CR. Since there is general description as highlighted below, we can not foresee the necessity to add the specific description. Thus, it is proposed to remove the added contents in 8.3.4.3 in TS 38.423 BL CR to keep align with TS 36.423 BL CR.

8.3.4.3 Unsuccessful Operation Figure 8.3.4.3-1: S-NG-RAN node initiated S-NG-RAN node Modification, unsuccessful operation. In case the requested modification cannot be performed successfully the M-NG-RAN node shall respond with the S-NODE MODIFICATION REFUSE message to the S-NG-RAN node with an appropriate cause value in the Cause IE. In case that the Required Number of DRB IDs IE was included in the S-NODE MODIFICATION REQUIRED message and if the M-NG-RAN node is not able to provide additional DRB IDs, the M-NG-RAN node shall respond with the S-NODE MODIFICATION REFUSE with an appropriate cause value in the Cause IE. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE. If the SCG Activation Request IE is included in the S-NODE MODIFICATION REQUIRED message, and the M-NG-RAN node is not able to accept the Request, the M-NG-RAN node shall respond with the S-NODE MODIFICATION REFUSE with an appropriate cause value in the Cause IE.

Proposal 2: Remove the added contents in 8.3.4.3 in TS 38.423 BL CR to keep align with TS 36.423 BL CR.

Furthermore, there are some other misalignments between the TS 38.423 and 36.423 BL CRs. Therefore, it is also proposed to align these two BL CRs as presented in R3-222339.

Proposal 3: Align the other misalignments between the TS 38.423 and 36.423 BL CRs.

- Conclusion

Based on above analysis, we provide the following observations and proposal.

Observation 1: In the current TS 38.401 BL CR, take the procedure of SN addition with SCG deactivation as an example, SN-CU-CP informs SN-CU-UP that the SCG is deactivated (step 2) before sending the SCG deactivation request to SN-DU (step 4).

Observation 2: If SN-DU rejects the request, SN-CU-UP may receive a wrong information on SCG status, e.g. it is informed that the SCG is deactivated but in fact the SCG is activated according to the response of SN-DU.

Proposal 1: SN-CU-CP shall inform SN-CU-UP about the actual SCG status after sending the SN Addition Request Acknowledge or receiving the SN Modification Confirm message.

Proposal 2: Remove the added contents in 8.3.4.3 in TS 38.423 BL CR to keep align with TS 36.423 BL CR.

Proposal 3: Align the other misalignments between the TS 38.423 and 36.423 BL CRs.

- Text Proposal for TS38.401

### 8.4.x	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s).

#### 8.4.x.1	SN Addition with SCG Deactivation

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG deactivation.

2. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG deactivation.

3. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

4. The SN sends the SN addition Request Acknowledge towards the MN, indicates that the SCG is deactivated.

5. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the deactivated status of SCG.

6. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

#### 8.4.x.2	SN Addition with SCG Activation

Figure 8.4.x.2-1: SCG Activation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG activation.

2. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG activation.

3. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates that the SCG is activated.

4. The SN sends the SN addition Request Acknowledge towards the MN, indicates that the SCG is activated.

5. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the activated status of SCG.

6. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

#### 8.4.x.3	MN initiated SN Modification with SCG Deactivation

Figure 8.4.x.3-1: SCG Deactivation in MN initiated SN Modification procedure

1. SCG is activated.

2. The MN sends the SN Modification Request towards the SN, indicates the request of SCG deactivation.

3. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

4. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

5. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is deactivated.

6. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the deactivated status of SCG.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

#### 8.4.x.4	MN initiated SN Modification with SCG Activation

Figure 8.4.x.4-1: SCG Activation in MN initiated SN Modification procedure

1. SCG is deactivated.

2. MN sends the SN Modification Request towards the SN, indicates the request of SCG activation.

3. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

4. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is activated.

5. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is activated.

6. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the activated status of SCG.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP

#### 8.4.x.5	SN initiated SN Modification with SCG Deactivation

Figure 8.4.x.5-1: SCG Deactivation in SN initiated SN Modification procedure

1. SCG is activated.

2. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

3. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

4. The SN sends the SN Modification Require towards the MN, indicates the request of SCG deactivation.

5. The MN sends the SN Modification Confirm towards the SN.

6. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the deactivated status of SCG.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

#### 8.4.x.6	SN initiated SN Modification with SCG Activation

Figure 8.4.x.6-1: SCG Activation in SN initiated SN Modification procedure

1. SCG is deactivated.

2. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

3. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is activated.

4. The SN sends the SN Modification Required towards the MN, indicates the request of SCG activation.

5. The MN sends the SN Modification Confirm towards the SN.

6. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the activated status of SCG.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.