3GPP TSG-RAN WG3 Meeting #115-e	R3-222346

E-meeting, 21 Feb – 3 Mar 2022

Title: 	(TP to CPAC BL CR for TS 38.423) Preparation of Multiple T-SNs in SN initiated inter-SN CPC

Source: 	Huawei

Agenda item:	14.3

Document Type:	other

# 1. Introduction

As discussed in [1], the following proposals are made:

- Proposal 1: turn the WA of using MN initiated modification procedure as agreement.

- Proposal 2: introduce a T-SN List to be prepared IE in the SN Change Required message, to be used in case of multiple T-SN preparation, and the existing Target SN ID IE is ignored if this new IE is present.

- Proposal 3: introduce a Prepared T-SN List IE in the SN Change Confirm message, to be used in case of multiple T-SN is prepared.

- Proposal 4: introduce an Additional List of PDU Session Resource Change Confirm Info – SN terminated IE in the SN Change Confirm message, to carry different data forwarding address for different T-SNs, the existing PDU Session Resource Change Confirm Info – SN terminated IE is ignored if this new IE is present.

- Proposal 5: in case the CPC is executed by the UE, the MN sends SN Release Request message to cancel CPC in the target SNs.

In this document we provide the TP to support multiple T-SN preparation in SN initiated inter-SN CPC.

# 2. Reference

[1] R3-222158 (TP to CPAC BL CR for TS 37.340) SN initiated inter-SN CPC, Huawei

# 3. Text Proposal to CPAC BL CR of TS 38.423

----------Start of the First Change---------------

### 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

#### 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE CHANGE REQUIRED message may contain

-	the S-NG-RAN node to S-NG-RAN node Container IE.

If the M-NG-RAN node is able to perform the change requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE CHANGE CONFIRM message to the S-NG-RAN node. For DRBs configured with the PDCP entity in the S-NG-RAN node, the M-NG-RAN node may include data forwarding related information in the Data Forwarding Info from target NG-RAN node IE.

If the S-NODE CHANGE CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

The S-NG-RAN node may start data forwarding and stop providing user data to the UE and shall stop the timer TXnDCoverall upon reception of the S-NODE CHANGE CONFIRM message.

If the Conditional PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall consider that the requirement concerns CPAC, as described in TS 37.340 [8]. Accordingly, the M-NG-RAN node shall, if supported, include the Conditional PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Change Information Required IE included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall forward this information to the candidate target S-NG-RAN node, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the Target SN List to be prepared IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the requirement concerns preparation of multiple candidate target NG-RAN nodes, and act as specified in TS 37.340 [8]. In case multiple candidate target NG-RAN nodes are prepared, the M-NG-RAN node shall, if supported, include the Prepared Target SN List IE in the S-NODE CHANGE CONFIRM message

Interaction with M-NG-RAN node initiated S-NG-RAN node Release:

If the M-NG-RAN node receives the S-NODE CHANGE REQUIRED message indicating releasing target S-NG-RAN node(s) and cancelling all prepared PSCells in the target S-NG-RAN node(s), the M-NG-RAN shall trigger the M-NG-RAN node initiated S-NG-RAN node release procedure to the target S-NG-RAN node(s) and cancel all the prepared PSCells at the target S-NG-RAN node(s).

----------Start of the Next Change---------------

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Conditional PSCell Change Information Required | O |  |  |  | YES | reject |
| >CPAC Indicator | M |  | ENUMERATED (CPAC-initiation, CPC-modification, CPC-cancellation, ...) |  |  |  |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..FFS, …) | Indicates the maximum number of PSCells that the target SN may prepare. |  |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. |  |  |
| Target SN List to be prepared |  | 0.. <maxnoofTargetSNs> |  | This IE would be present only if the preparation of multiple candidate target SNs is required in case of SN initiated inter-SN CPC. | YES | ignore |
| >Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of candidate target SNs in case of SN initiated inter-SN CPC, Value is 4. |

------------------------------------------------Next change--------------------------------------------------

#### 9.1.2.12	S-NODE CHANGE CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node change was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session SN Change Confirm List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Confirm Item |  | 1 .. <maxnoof PDUsessions> |  | NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present in a PDU Session SN Change Confirm Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Confirm Info – SN terminated | O |  | 9.2.1.19 |  | – |  |
| >>Additional List of PDU Session Resource Change Confirm Info – SN terminated |  | 0..1 |  | This IE would be present only if multiple candidate target SNs are prepared in case of SN initiated inter-SN CPC. | YES | ignore |
| >>> Additional Item of PDU Session Resource Change Confirm Info – SN terminated |  | 1 .. <maxnoofTargetSNs> |  |  | - |  |
| >>>>PDU Session Resource Change Confirm Info – SN terminated | M |  | 9.2.1.19 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Conditional PSCell Change Information Confirm | O |  |  |  | YES | ignore |
| >Candidate PSCell ID List |  | 1 |  |  |  |  |
| >>Candidate PSCell ID Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |  |  |
| Prepared Target SN List |  | 0.. <maxnoofTargetSNs> |  | This IE would be present only if multiple candidate target SNs are prepared in case of SN initiated inter-SN CPC. | YES | ignore |
| >Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is FFS |
| maxnoofTargetSNs | Maximum no. of candidate target SNs in case of SN initiated inter-SN CPC, Value is 4. |

----------Start of the Next Change---------------

Asn.1 to be added during the meeting.

----------Start of the Next Change---------------

- Take XnAP spec as an example, the S-NODE CHANGE CONFIRM message could be updated as follows:

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.3.1 |  | YES |  | reject |  |
| M-NG-RAN node UE XnAP ID | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES |  | reject |  |
| S-NG-RAN node UE XnAP ID | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES |  | reject |  |
| Target S-NG-RAN node ID | M |  |  | Global NG-RAN Node ID 9.2.2.3 | This IE is ignored if the Target SN List to be prepared IE is present. | YES |  | reject |  |
| Cause | M |  |  | 9.2.3.2 |  | YES |  | ignore |  |
| PDU Session SN Change Required List |  |  | 0..1 |  |  | YES |  | ignore |  |
| >PDU Session SN Change Required Item |  |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |  |  |
| >>PDU Session ID | M |  |  | 9.2.3.18 |  | – |  |  |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  |  | 9.2.1.18 |  | – |  |  |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES |  | reject |  |
| Target SN List to be prepared |  | 0.. <maxnoofTargetSNs> |  |  | This IE would be present only if multiple Target SN is prepared in case of SN initiated inter-SN CPC. |  | YES |  | Reject |
| >Target S-NG-RAN node ID | M |  |  | Global NG-RAN Node ID 9.2.2.3 |  |  | - |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of Target SNs in case of SN initiated inter-SN CPC, Value is 4. |

- Take XnAP spec as an example, the S-NODE CHANGE CONFIRM message could be updated as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session SN Change Confirm List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Confirm Item |  | 1 .. <maxnoof PDUsessions> |  | NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present in a PDU Session SN Change Confirm Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Confirm Info – SN terminated | O |  | 9.2.1.19 | This IE is ignored if the Additional List of PDU Session Resource Change Confirm Info – SN terminated IE is present. | – |  |
| >>Additional List of PDU Session Resource Change Confirm Info – SN terminated |  | 0..1 |  | This IE would be present only if multiple Target SN is prepared in case of SN initiated inter-SN CPC. | YES | ignore |
| >>> Additional Item of PDU Session Resource Change Confirm Info – SN terminated |  | 1 .. <maxnoofTargetSNs> |  |  | - |  |
| >>>>PDU Session Resource Change Confirm Info – SN terminated | M |  | 9.2.1.19 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Prepared Target SN List |  | 0.. <maxnoofTargetSNs> |  | This IE would be present only if multiple Target SN is prepared in case of SN initiated inter-SN CPC. | YES | Reject |
| >Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of Target SNs in case of SN initiated inter-SN CPC, Value is 4. |