3GPP TSG-RAN WG3 Meeting #115-e	R3-222347

E-meeting, 21 Feb – 3 Mar 2022

Title: 	EPS User Plane Integrity Protection with non-supporting nodes

Source: 	Vodafone

Agenda item:	30.4

Document Type:	Agreement

_______________________________________________________________________________

1	Background

This meeting’s agenda records the agreements made on LTE UPIP in the last meeting. The BaseLine CRs that were endorsed in the last meeting have been re-submitted as:

| R3-221606 | R3-221434 | Support for User Plane Integrity Protection support for EPC connected architectures with EN-DC capable UE_E1AP | ZTE, China Telecom, Ericsson, Vodafone, Qualcomm, Nokia, Nokia Shanghai Bell, Huawei | 38.463 |
| --- | --- | --- | --- | --- |
| R3-221607 | R3-221448 | Introduction of User Plane Integrity Protection in EPS | Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Huawei, ZTE, Vodafone, Ericsson | 36.413 |
| R3-221608 | R3-221453 | Supporting EPS User Plane Integrity Protection | Huawei, Orange, CATT, ZTE, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Vodafone, Ericsson | 36.423 |

The open issues listed in the previous meeting minutes are:

How source eNB knows target eNB capability to support UPIP at S1 and X2 handover;

How MeNB knows SgNB capability to support UPIP

How CU-CP knows CU-UP capability to support UPIP

How (e.g. at intra MME handover to non supporting eNB, or PDN connection establishment using NAS signalling via non -supporting eNB) MME knows whether eNB supports UPIP.

Whether to add the “UPIP policy in use” to the source to target transparent container in the S1 interface HANDOVER REQUIRED and HANDOVER REQUEST messages

(Dependent upon SA3 and SA2 response) whether and how to do the Notification of “UPIP in use” in RAN 3’s interfaces

Whether any updates are needed to the parts of TS 36.300 or TS 37.340 that are regarded as being under “RAN 3 responsibility”?

Another issue that was mentioned in the meeting discussion is:

How do we ensure that at (X2 and S1) handover from an eNB using UPIP with UPIP policy = “preferred” to a non-supporting eNB, the target eNB will construct an RRCConnectionReconfigurationCommand that ensures that the UE does NOT use UPIP on the target-non-supporting cell.

This document discusses a couple of these issues.

2	LS Responses from SA2 and SA3

At the RAN 3 tdoc submission deadline, neither the SA2 nor the SA3 meetings had started.

However, the documents submitted to these meetings are available. From those documents it seems likely (but NOT certain) that:

- SA2 will wait for SA3 to decide on the notification procedure

- SA3 will indicate that the notification procedure is not needed (in EPS)

- SA3 will prefer that all packets are integrity protected.

The author understands that SA3 will attempt to treat the LS in their first week and so an answer at the beginning of the RAN 3 meeting can be anticipated.

3	Inter-MME handover to non-supporting MME and non-supporting eNB

Assuming that SA3 requires that all packets are UPIP protected if the policy is “UPIP=Required”, then we need a solution for the case that neither the target MME nor the target eNB support UPIP.

As the criticality IEs only operate on a single interface, we cannot rely on help from a non-supporting MME. Hence it seems necessary to get a supporting eNB to provide a positive acknowledgement (e.g. by inserting the UPIP status that it is implementing) into a new IE in the target to source transparent container in the S1-AP Handover Request Acknowledge and Handover Command messages. The absence of the positive acknowledgement is then a trigger for the source eNB to cancel the handover and re-initiate it only for the bearers that do not require UPIP.

Proposal 1: For S1 handover, a new IE (that indicates whether the bearer is UPIP protected or not) is added (per EPS bearer) into the target to source transparent container in the S1-AP Handover Request Acknowledge and Handover Command messages, and the source eNB is mandated to cancel handovers for when a “UPIP=required” policy cannot be maintained.

4	How does a UPIP-non-supporting target eNB turn off UPIP?

This is for the situation with UPIP in use on a source eNB and a policy of “UPIP=preferred”.

Some specification extracts are in the annex.

Examination of the X2 Handover Request and S1 Handover Request/Required indicates that the target eNB relies on the information in the TS 36.331 HandoverPreparationInformation to determine the current RRC Configuration and hence what information to put in the RRCConnectionReconfigurationCommand. The AS-Config is the IE that carries the information on the configuration of the PDCP from each bearer. For the bearers that don’t use NR-PDCP, the PDCP-Config is sent in the TS 36.331 RadioResourceConfigDedicated IE while for the bearers that use NR-PDCP, the as-ConfigNR-r15 (specified in TS 38.331) carries the PDCP configuration.

The as-ConfigNR-r15 is after this ellipsis in the TS 36.331 ASN.1 and hence would be ignored by a pre-Rel 15 eNB (or any eNB that has not been upgraded to support EN-DC?).

However, the as-ConfigNR-r15 maps to the TS 38.331 RadioBearerConfig and this contains the EPS bearer-ID along with the PDCP-Config. Then any legacy target eNB that ignores the as-ConfigNR would be missing information about one of the EPS bearer IDs that is listed in the X2-AP/S1-AP Handover Request message and this ought to trigger the target eNB to provide a Full RRC Reconfiguration to the UE which would then not configure UPIP.

Proposal 2: RAN 3 should check this analysis to verify that legacy eNBs will disable UPIP at handover of a UE from an eNB using UPIP with UPIP policy=preferred.

5	Proposals

Proposal 1: For S1 handover a new IE (that indicates whether the bearer is UPIP protected or not) is added (per EPS bearer) into the target to source transparent container in the S1-AP Handover Request Acknowledge and Handover Command messages, and the source eNB is mandated to cancel handovers for when a UPIP=required policy cannot be maintained.

Proposal 2: RAN 3 should check this analysis to verify that legacy eNBs will disable UPIP at handover of a UE from an eNB using UPIP with UPIP policy=preferred.

________________________________________________________________________

## Annex: TS 36.331 and TS 38.331 specification extracts

********* extract from TS 36.331 ***

#### HandoverPreparationInformation

This message is used to transfer the E-UTRA RRC information used by the target eNB or target ng-eNB during handover preparation or UE context retrieval, e.g. in case of resume or re-establishment, including UE capability information.

Direction: source eNB/ source RAN to target eNB or target ng-eNB

HandoverPreparationInformation message

-- ASN1START

HandoverPreparationInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverPreparationInformation-r8	HandoverPreparationInformation-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverPreparationInformation-r8-IEs ::= SEQUENCE {

ue-RadioAccessCapabilityInfo		UE-CapabilityRAT-ContainerList,

as-Config							AS-Config					OPTIONAL,		-- Cond HO

rrm-Config							RRM-Config					OPTIONAL,

as-Context							AS-Context				OPTIONAL,		-- Cond HO

nonCriticalExtension				HandoverPreparationInformation-v920-IEs		OPTIONAL

}

HandoverPreparationInformation-v920-IEs	::= SEQUENCE {

ue-ConfigRelease-r9					ENUMERATED {

rel9, rel10, rel11, rel12, v10j0, v11e0,

v1280, rel13, ..., rel14, rel15, rel16}		OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v9d0-IEs		OPTIONAL

}

/**** text deleted ******/

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Config The radio resource configuration. Applicable in case of intra-E-UTRA handover, resume or re-establishment. If the target receives an incomplete MeasConfig and/or RadioResourceConfigDedicated in the as-Config, the target eNB may decide to apply the full configuration option based on the ue-ConfigRelease.

as-Context Local E-UTRAN context required by the target eNB.

/*** text omoitted ***/

## 10.3	Inter-node RRC information element definitions

#### –	AS-Config

The AS-Config IE contains information about RRC configuration information in the source eNB which can be utilized by target eNB to determine the need to change the RRC configuration during the handover preparation phase. The information can also be used after the handover is successfully performed or during the RRC connection re-establishment or resume.

AS-Config information element

-- ASN1START

AS-Config ::=				SEQUENCE {

sourceMeasConfig					MeasConfig,

sourceRadioResourceConfig			RadioResourceConfigDedicated,

sourceSecurityAlgorithmConfig		SecurityAlgorithmConfig,

sourceUE-Identity					C-RNTI,

sourceMasterInformationBlock		MasterInformationBlock,

sourceSystemInformationBlockType1	SystemInformationBlockType1(WITH COMPONENTS

{..., nonCriticalExtension ABSENT}),

sourceSystemInformationBlockType2	SystemInformationBlockType2,

antennaInfoCommon					AntennaInfoCommon,

sourceDl-CarrierFreq				ARFCN-ValueEUTRA,

...,

[[	sourceSystemInformationBlockType1Ext	OCTET STRING (CONTAINING

SystemInformationBlockType1-v890-IEs)	OPTIONAL,

sourceOtherConfig-r9				OtherConfig-r9

-- sourceOtherConfig-r9 should have been optional. A target eNB compliant with this transfer

-- syntax should support receiving an AS-Config not including this extension addition group

-- e.g. from a legacy source eNB

]],

[[	sourceSCellConfigList-r10			SCellToAddModList-r10			OPTIONAL

]],

[[	sourceConfigSCG-r12					SCG-Config-r12		OPTIONAL

]],

[[	as-ConfigNR-r15						AS-ConfigNR-r15					OPTIONAL

]],

[[	as-Config-v1550						AS-Config-v1550					OPTIONAL

]],

[[	as-ConfigNR-v1570					AS-ConfigNR-v1570				OPTIONAL

]],

[[	as-ConfigNR-v1620					AS-ConfigNR-v1620				OPTIONAL

]]

}

/**** text deleted ***/

AS-ConfigNR-r15 ::=				SEQUENCE {

sourceRB-ConfigNR-r15				OCTET STRING			OPTIONAL,

sourceRB-ConfigSN-NR-r15				OCTET STRING			OPTIONAL,

sourceOtherConfigSN-NR-r15			OCTET STRING			OPTIONAL

}

/**** text deleted ***/

AS-Config field descriptions

antennaInfoCommon This field provides information about the number of antenna ports in the source PCell.

p-MaxEUTRA Indicates the p-MaxEUTRA in the source PCell.

sourceOtherConfigSN-NR Other NR config set by SN (cell group, measurements) in case of (NG)EN-DC i.e. as defined by the RRCReconfiguration message in TS 38.331 [82].

sourceRB-ConfigNR NR radio bearer config, as defined by RadioBearerConfig IE in TS 38.331 [82]. The field may e.g. be set by MN in case of (NG)EN-DC, by source eNB connected to 5GCN.

sourceRB-ConfigSN-NR NR radio bearer config set by SN in case of (NG)EN-DC or of SN terminated RB without SCG, as defined by RadioBearerConfig IE in TS 38.331 [82].

/**** text deleted ***/

RadioResourceConfigDedicated information element

-- ASN1START

RadioResourceConfigDedicated ::=		SEQUENCE {

srb-ToAddModList					SRB-ToAddModList			OPTIONAL,		-- Cond HO-Conn

drb-ToAddModList					DRB-ToAddModList			OPTIONAL,		-- Cond HO-toEUTRA

drb-ToReleaseList					DRB-ToReleaseList			OPTIONAL,		-- Need ON

mac-MainConfig						CHOICE {

explicitValue					MAC-MainConfig,

defaultValue					NULL

/*** ASN.1 omitted ***/

DRB-ToAddModList ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddModList-r15 ::=			SEQUENCE (SIZE (1..maxDRB-r15)) OF DRB-ToAddMod

DRB-ToAddModListSCG-r12 ::=		SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddModSCG-r12

DRB-ToAddModListSCG-r15 ::=		SEQUENCE (SIZE (1..maxDRB-r15)) OF DRB-ToAddModSCG-r12

DRB-ToAddMod ::=	SEQUENCE {

eps-BearerIdentity					INTEGER (0..15)			OPTIONAL,		-- Cond DRB-Setup

drb-Identity						DRB-Identity,

pdcp-Config							PDCP-Config				OPTIONAL,		-- Cond PDCP

rlc-Config							RLC-Config				OPTIONAL,		-- Cond SetupM

logicalChannelIdentity				INTEGER (3..10)			OPTIONAL,		-- Cond DRB-SetupM

logicalChannelConfig				LogicalChannelConfig	OPTIONAL,		-- Cond SetupM

...,

*****extract from TS 38.331 ***/

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIOBEARERCONFIG-START

RadioBearerConfig ::=                   SEQUENCE {

srb-ToAddModList                        SRB-ToAddModList                                        OPTIONAL,   -- Cond HO-Conn

srb3-ToRelease                          ENUMERATED{true}                                        OPTIONAL,   -- Need N

drb-ToAddModList                        DRB-ToAddModList                                        OPTIONAL,   -- Cond HO-toNR

drb-ToReleaseList                       DRB-ToReleaseList                                       OPTIONAL,   -- Need N

securityConfig                          SecurityConfig                                          OPTIONAL,   -- Need M

...

}

/*** text omitted ***/

DRB-ToAddModList ::=                    SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),

sdap-Config                             SDAP-Config

}                                                                                               OPTIONAL,   -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

[[

daps-Config-r16                         ENUMERATED{true}                                        OPTIONAL    -- Cond DAPS

]]

******************* end of TS 38.331 extract *********

The AS-Config is structured in a manner that would seem to lead to a Release 15 EN-DC aware target eNB being able to see that some bearers are integretigty protected

**********

Need X2 to work across multi-vendor X2 – eg at network sharing boundaries

****

Criticality Learning process ought to be documented… but seems to just involve one round trip extra per cell… seems OK on Service Request… and on PDN connection request for “required” just end up failing the default  bearer setup… check 23.401 5.10?

****

*********

For inter MME S1 handover, (if S3 need all packets to be UPIP protected) then need a solution that works in the case that neither target MME nor target eNB support UPIP  having a positive response from the target eNB that it has understood the UPIP command WITHIN the target to source transparent container allows the source eNB to do Handover Cancel if UPIP is not supported in the target eNB (and UPIP is required).

HW solution of sending selected policy from target eNB to target MME does not work well if target MME is also not supporting.

Overall, we may need to assume that (if using NR-PDCP in the source eNB) that the target also understands R15 otherwise it will just ignore the as-ConfigNR-r15 in the as-config sent in the HandoverPreparationInformation in the X2-Handover Request, and, will assume that the RadioResourceConfigDedicated in the AS-Config is ‘complete’ and hence that there is no need to send a ‘full configuration’ to the UE or to turn off integrity protection…. Or probably more generally the bearers using NR-PDCP get accidentally released.

## 10.3	Inter-node RRC information element definitions

#### –	AS-Config

The AS-Config IE contains information about RRC configuration information in the source eNB which can be utilized by target eNB to determine the need to change the RRC configuration during the handover preparation phase. The information can also be used after the handover is successfully performed or during the RRC connection re-establishment or resume.

AS-Config information element

-- ASN1START

AS-Config ::=				SEQUENCE {

sourceMeasConfig					MeasConfig,

sourceRadioResourceConfig			RadioResourceConfigDedicated,

sourceSecurityAlgorithmConfig		SecurityAlgorithmConfig,

sourceUE-Identity					C-RNTI,

sourceMasterInformationBlock		MasterInformationBlock,

sourceSystemInformationBlockType1	SystemInformationBlockType1(WITH COMPONENTS

{..., nonCriticalExtension ABSENT}),

sourceSystemInformationBlockType2	SystemInformationBlockType2,

antennaInfoCommon					AntennaInfoCommon,

sourceDl-CarrierFreq				ARFCN-ValueEUTRA,

...,

[[	sourceSystemInformationBlockType1Ext	OCTET STRING (CONTAINING

SystemInformationBlockType1-v890-IEs)	OPTIONAL,

sourceOtherConfig-r9				OtherConfig-r9

-- sourceOtherConfig-r9 should have been optional. A target eNB compliant with this transfer

-- syntax should support receiving an AS-Config not including this extension addition group

-- e.g. from a legacy source eNB

]],

[[	sourceSCellConfigList-r10			SCellToAddModList-r10			OPTIONAL

]],

[[	sourceConfigSCG-r12					SCG-Config-r12		OPTIONAL

]],

[[	as-ConfigNR-r15						AS-ConfigNR-r15					OPTIONAL

]],

[[	as-Config-v1550						AS-Config-v1550					OPTIONAL

]],

[[	as-ConfigNR-v1570					AS-ConfigNR-v1570				OPTIONAL

]],

[[	as-ConfigNR-v1620					AS-ConfigNR-v1620				OPTIONAL

]]

}

/**** text deleted ***/

AS-ConfigNR-r15 ::=				SEQUENCE {

sourceRB-ConfigNR-r15				OCTET STRING			OPTIONAL,

sourceRB-ConfigSN-NR-r15				OCTET STRING			OPTIONAL,

sourceOtherConfigSN-NR-r15			OCTET STRING			OPTIONAL

}

/**** text deleted ***/

AS-Config field descriptions

antennaInfoCommon This field provides information about the number of antenna ports in the source PCell.

p-MaxEUTRA Indicates the p-MaxEUTRA in the source PCell.

sourceOtherConfigSN-NR Other NR config set by SN (cell group, measurements) in case of (NG)EN-DC i.e. as defined by the RRCReconfiguration message in TS 38.331 [82].

sourceRB-ConfigNR NR radio bearer config, as defined by RadioBearerConfig IE in TS 38.331 [82]. The field may e.g. be set by MN in case of (NG)EN-DC, by source eNB connected to 5GCN.

sourceRB-ConfigSN-NR NR radio bearer config set by SN in case of (NG)EN-DC or of SN terminated RB without SCG, as defined by RadioBearerConfig IE in TS 38.331 [82].

/**** text deleted ***/