3GPP TSG-RAN WG3 Meeting #115-e	R3-222351

Electronic Meeting, February 21th – March 3rd, 2022

Agenda item:	24.2

Source: 	Intel Corporation

Title: 	(TP for RA-SDT BL CR for TS 38.423/463) Toward the completion of RA-SDT

Document for:	Discussion and Decision

# Introduction

In the last RAN3-114bis-e meeting, RAN3 made progresses for RA-SDT as follows [1]:

WA: Solution 2/3/4 is the basis for further work. Agree to include the SDT related UE content in TP, the details need to be further checked. SRB is FFS. Extend the XnAP: RRC TRANSFER message, to forward the UL/DL SRB PDCP PDU during SDT procedure without anchor relocation between new gNB and anchor gNB Transfer the first SRB/DRB transfer as the same method as the subsequent SRB/DRB transfer The additional SDT assistant information is needed, but it is no need to consult with RAN2. It includes either BSR information or single/multiple packets indication as optional IEs. FFS on others. It is FFS that either new Class 1 message or a new Class 2 message is used to transfer SDT configuration in case of RA-SDT without anchor relocation. Whether SDT data handling between gNBs should be per DRB (not per PDU session), which is different to the fundamental NR mobility design per PDU session basis. Whether Xn-U Address Indication should be re-used to deliver DL forwarding TNL from new gNB, or be replaced only for SDT without anchor relocation. It is FFS that the additional SDT assistant information includes either BSR information or single/multiple packets indication as optional IEs, and FFS on other IEs. The following IEs in the SDT configuration (from anchor gNB to receiving gNB) are FFS - SRB - DRB QoS (QoS Flow Level QoS Parameters 9.2.3.5) - S-NSSAI - RLC Mode - PDCP SN Length - DL Forwarding (9.2.3.34) or Packet Data Indication (Indicates downlink data available) - PDU Session ID To be continued...

In this contribution, we focus on resolving remaining FFSes and discussing necessary changes to complete RA-SDT. The corresponding TPs are also proposed.

# Discussion

## Anchor relocation

First, for SDT with anchor relocation, the following call flow has been agreed [2]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

XX.2	RA-based SDT with UE context relocation

The overall procedure for RACH based small data transmission with UE context relocation is illustrated in the figure XX.2-1.

Figure XX.2-1. RA-based SDT with UE context relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB locates the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE requested for SDT and may also provides SDT assistance information.

Editor’s Note: Details of assistance information is pending to future discussion.

3.  The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE.

4-6. The receiving gNB decides to keep UE in RRC Inactive state for SDT transmission, and initiates NG-AP Path Switch Request procedure to establish a NG UE associated signalling connection to the serving AMF. After path switch, the UL SDT data is delivered to the UPF, the UL NAS PDU is delivered to AMF.

Editor’s Note: FFS on how and when to establish forwarding tunnel for DL SDT data.

7. After SDT transmission is completed, the receiving gNB generates and sends RRCRelease including the Suspend indication to the UE to move the UE back to RRC_INACTIVE.

8. The receiving gNB indicates the last serving gNB to remove the UE context by sending UE CONTEXT RELEASE.

Editor’s Note: Details of the whole procedure is FFS.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Couple of points that needs to be corrected:

- In step 4-6, it is written that the UL SDT data is delivered to UPF after path switch. However, UL SDT data can be sent to UPF before path switch. Here, full UE context is retrieved from the last serving gNB. And the last serving gNB derives new KgNB* based on NCC that it has previously provided to the UE via RRCRelease and sends {KgNB*, NCC} to the new gNB. The last serving gNB also sends UL NG-U UP TNL at UPF for each PDU session. The receiving gNB can use the received KgNB* to decrypt the initial UL SDT data (up to SDAP SDU) and directly forward them to the UPF.

- The typical Xn-U Address Indication procedure is missing for DL data forwarding from the last serving gNB to the receiving gNB before path switch is completed.

Proposal 1: For SDT with anchor relocation, fix the call flow in 38.300 BL CR [2] so that UL SDT data can be delivered to the UPF before path switch. Also add the typical Xn-U Address Indication procedure which is missing for DL data forwarding from the last serving gNB to the new gNB before path switch is completed.

Based on discussions and the proposal above, we thus propose the following call flow for SDT with context relocation.

Proposal 2: For SDT with anchor relocation, RAN3 to endorse the following changes to RA-SDT BLCR for TS 38.300 [2]:

XX.2	RA-based SDT with UE context relocation

The overall procedure for RACH based small data transmission with UE context relocation is illustrated in the figure XX.2-1.

Figure XX.2-1. RA-based SDT with UE context relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB locates the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE requested for SDT and may also provide SDT assistance information.

3.  The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered to the UPF.

4-6. The receiving gNB decides to keep UE in RRC Inactive state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses. The receiving gNB also initiates NG-AP Path Switch Request procedure to establish a NG UE-associated signalling connection to the serving AMF. After path switch, the UL NAS PDU, if any, is delivered to the AMF.

7. After SDT is completed, the receiving gNB generates and sends RRCRelease message including the Suspend configuration to the UE to move the UE back to RRC_INACTIVE.

NOTE: In case DL non-SDT data or DL non-SDT signalling arrives, the receiving gNB may decide to send the UE to RRC_CONNECTED state by sending RRCResume message.

8. The receiving gNB indicates the last serving gNB to remove the UE context by sending the UE CONTEXT RELEASE message.

## Without anchor relocation

### PDU session for SDT without anchor relocation

Whether SDT data handling between gNBs should be per DRB (not per PDU session), which is different to the fundamental NR mobility design per PDU session basis.

Though PDU session is irrelevant to partial context transfer and Xn-U DL/UL forwarding TNL as they are per DRB basis, but in 5GS, signalling design in RAN3 has been per PDU session basis. All the basic mobility and dual connectivity related procedure messages defined in XnAP between source and target or between MN and SN have been "per PDU session basis" from day 1. Even for SN terminated MCG bearer (or MN terminated SCG bearer), the relevant XnAP messages communicated between MN and SN carry Xn-U DL/UL forwarding TNLs "per PDU session" basis.

There is no reason to design signalling for SDT without anchor relocation deviating from this fundamental 5GC principle and make signalling design complicated.

Observation 1: In 5GS, signalling design in RAN3 has been per PDU session basis. All the basic mobility and dual connectivity related procedure messages defined in XnAP between source and target or between MN and SN have been "per PDU session basis" from day 1. Even for SN terminated MCG bearer (or MN terminated SCG bearer), the relevant XnAP messages communicated between MN and SN carry Xn-U DL/UL forwarding TNLs "per PDU session" basis.

Proposal 3: For SDT without anchor relocation, RAN3 to design XnAP signalling for SDT related RLC bearer configuration and Xn-U DL/UL forwarding TNL info, per "PDU session basis", following the fundamental 5GS principle in RAN3 from day 1.

### IE design for partial SDT context from anchor to receiving gNB

Currently, we have following FFSes with some basic IEs (DRB ID, UL TNL, RLC configuration) being already agreed to be supplied.

The following IEs in the SDT configuration (from anchor gNB to receiving gNB) are FFS - SRB - DRB QoS (QoS Flow Level QoS Parameters 9.2.3.5) - S-NSSAI - RLC Mode - PDCP SN Length - DL Forwarding (9.2.3.34) or Packet Data Indication (Indicates downlink data available) - PDU Session ID

When the receiving gNB receives SDT context from the anchor gNB, it needs to properly establish UE context and the lower layer configurations for SDT DRBs into its DU. In doing so, the baseline assumption is that we are going to re-use the existing F1AP UE CONTEXT SETUP REQUEST message.

Whatever required there should be supplied from the anchor gNB.

Observation 2: For SDT without anchor relocation, the receiving gNB needs to properly establish UE context and lower layers configurations for SDT DRBs into its DU. The baseline assumption is to re-use the existing F1AP UE CONTEXT SETUP REQUEST message. Whatever required there should be supplied from the anchor gNB as part of SDT context transfer.

When CU establishes new UE context and DRBs onto DU via DRBs To Be Setup List in the F1AP UE CONTEXT SETUP REQUEST message, the following IEs are mandatorily included or required:

- DRB QoS, S-NSSAI, Flows mapped to DRB, UL TNL, RLC mode, DL PDCP SN length

Observation 3: When CU establishes DRBs onto DU via DRBs To Be Setup List, the following IEs are mandatorily included: DRB QoS, S-NSSAI, Flows mapped to DRB, UL TNL, RLC mode, DL PDCP SN length.

Moreover, CellGroupConfig (via CU to DU RRC Information IE) is required to setup UE context in DU.

Observation 4: CellGroupConfig (via CU to DU RRC Information IE) is required to setup UE context in DU. CellGroupConfig shall also be supplied from anchor to receiving gNB.

Of course, PDU session ID shall also be supplied from anchor to receiving gNB to honor the day-1 principle of per PDU session handling between NG-RAN nodes.

Observation 5: PDU session ID shall also be supplied from anchor to receiving gNB to honor the day-1 principle of per PDU session handling between NG-RAN nodes.

Proposal 4: For SDT without anchor relocation, the anchor gNB also provides the following IEs as part of SDT context transfer:

- DRB QoS, S-NSSAI, Flows mapped to DRB, UL TNL, RLC mode, DL PDCP SN length: required for the receiving gNB to establish SDT DRBs onto its DU.

- CellGroupConfig IE: required at least for SRB1 RLC and SDT DRB RLC configurations

- PDU Session ID: required to honor the day-1 principle of per PDU session handling.

Regarding DL forwarding indication (to indicate the receiving gNB that downlink data is available), we think this is not necessary and the reason is as follows. When SDT configuration is received, the receiving gNB should establish UE context and SDT DRBs on its DU. While doing so, it provides UL Xn-U TNLs and DU replies with the corresponding DL Xn-U TNLs. That is, GTP-U tunnel between CU and DU is bi-directionally setup to communicate UL/DL data. There is no reason not to supply the DL Xn-U TNLs for SDT DRBs already assigned by the receiving DU toward the anchor CU.

While it makes sense that we should follow the HO principle for INACTIVE mobility where the source (i.e. the anchor gNB) proposes DL data forwarding for which the target (i.e. the receiving gNB) provides the corresponding TNL info, but this principle is for data forwarding between CUs. Between CU and DU, GTP-U tunnel has been bi-directionally setup from day 1.

Therefore, we believe that there is no need to separately indicate to the receiving gNB that DL data is available at the anchor gNB for SDT without anchor relocation. Instead, we can just follow GTP-U tunnel setup between CU and DU that has been established bi-directionally from day 1. This indeed simplifies the design rather than supplying DL Xn-U TNLs for SDT DRBs only when DL data availability is separately indicated by the anchor gNB.

All we need is that the receiving gNB mandatorily supplies the corresponding DL Xn-U TNLs already assigned by its DU toward the anchor gNB, so that the anchor gNB can send DL data whenever available.

Observation 6: Between CU and DU, GTP-U tunnel has been bi-directionally setup from day 1. There is no reason not to supply DL Xn-U TNLs for SDT DRBs already assigned by the receiving DU toward the anchor CU. And this design is simpler compared to supplying DL Xn-U TNLs to the anchor gNB only when DL data availability is separately indicated by the anchor gNB.

Proposal 5: For SDT without anchor relocation, the anchor gNB does not need to separately indicate to the receiving gNB that DL data is available at the anchor gNB. The receiving gNB supplies DL Xn-U TNLs for SDT DRBs already assigned by its DU toward the anchor gNB, so that the anchor gNB can send SDT DL data whenever available.

Multiple Xn-U TNLs could be setup for a single SDT DRB between the receiving DU and the anchor CU in case of PDCP duplication. And when the UE initiates SDT session, the UE just restores AS context that was stored when suspended. Since SDT DRB could be configured with PDCP duplication before suspended, we think it is no harm to support multiple Xn-U TNL setup for SDT DRB as we typically did.

Proposal 6: For SDT without anchor relocation, allow multiple Xn-U TNLs setup for a SDT DRB between the receiving DU and the anchor CU in case PDCP duplication has been configured.

Based on the above observations, we proposed the following structures for SDT configuration from the anchor gNB to the receiving gNB.

Proposal 7: For SDT context transfer from anchor to receiving gNB, agree the following IE structures:

9.2.3.XX2		SDT Configuration Transfer

This IE includes necessary configuration information to process RLC PDUs and perform data forwarding related to RACH based SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SDT RLC Configuration | M |  | OCTET STRING | CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [10]. Required at least to carry SRB1 RLC configuration and DRB RLC configurations configured for SDT. |
| SDT Xn-U Context Information | M |  | 9.3.XX3 |  |

9.2.3.XX3		SDT Xn-U Context Information

This IE includes information necessary to setup Xn-U bearers related to RACH based SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>DRBs To Be Setup List |  | 1 |  |  | – |  |
| >>>DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>>>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>>>QoS Flows Mapped To DRB List |  | 1 |  |  | – |  |
| >>>>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>PDCP SN Length | M |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>>>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used. | – |  |
| >>>>UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3. 76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>>>secondary SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>>>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |

### Signalling flow design

In the last meeting, companies had heated discussions on overall signalling design for SDT without anchor relocation, and the following WA was made with FFSes.

WA: Solution 2/3/4 is the basis for further work.

It is FFS that either new Class 1 message or a new Class 2 message is used to transfer SDT configuration in case of RA-SDT without anchor relocation.

Whether Xn-U Address Indication should be re-used to deliver DL forwarding TNL from new gNB, or be replaced only for SDT without anchor relocation.

Unfortunately, those solutions in WA are all based on inserting a nested procedure while the XnAP UE Context Retrieval procedure is on-going, and also based on mixing up the initiation and end of SDT session in a weird sense. On the other hand, this design direction (which is strange) is also the reality and RAN3 really should finalize the stage-3 details (this is the last meeting indeed).

For the sake of progress, we compromise to the Solution 3 approach. There is indeed no reason to break the day-1 principle of INACTIVE framework by replacing the Xn-U Address Indication triggered from the receiving gNB. Here in SDT without anchor relocation, the receiving gNB just needs to supply DL Xn-U TNLs for SDT DRBs (in response to SDT context transfer) and all the necessary IEs are already in place in the existing Xn-U ADDRESS INDICATION message.

One thing to highlight is that, for SDT without anchor relocation, the Xn-U Address Indication procedure to providing DL Xn-U TNLs for SDT DRBs is better to be invoked always from the receiving gNB. As discussed above, a GTP-U tunnel between CU and DU has been bi-directionally setup from day 1 (which is different from the HO principle and data forwarding between CUs), and doing so between the receiving DU and the anchor CU indeed simplifies the design.

And we don't agree with the arguments from the Solution 2 proponents that the nested class-1 procedure is safe to check whether the receiving gNB can support SDT without anchor relocation, and if not, then the anchor gNB can fallback to full context relocation. Again, this is a strange ping-pong design where the anchor gNB first sends the partial context and later fallbacks to send the full context. Please note that RAN3 already agreed that the receiving gNB provides "assistance" information when requesting context retrieval, so that the anchor gNB can select the right procedure from the beginning. We really don't need such post mess clean-up by introducing a nested class-1 procedure initiated from the anchor gNB. The right design should be that the receiving gNB indicates its relocation preference from the beginning as an assistance information (which is "always" available by the way) and the anchor gNB selects the right one to go with. This indeed simplifies the design and minimizes signallings.

Regarding the new class-2, we also propose to extend the "RETREIVE UE CONTXT CONFIRM" message that was agreed for inter-MN resume without SN change in Rel-17 [3]. From functionality point of view, this new class-2 is designed to support INACTIVE mobility and well-suited for SDT framework. There, this new class-2 carries one optional IE of "UE Context Kept Indicator" so far. This message can be enhanced to carry SDT context transfer to the receiving gNB when the anchor gNB decides no anchor relocation.

Proposal 8: For SDT without anchor relocation, compromise to go with the Solution 3, with the following modifications:

Figure 1: Solution 3 [1]

- Extend the "RETRIEVE UE CONTXT CONFRIM" that was agreed for inter-MN resume without SN change in Rel-17 [3] as for the new class-2 message of step 2a that transfers SDT context.

- After step 2a, the step 2b Xn-U Address Indication that provides DL Xn-U TNLs for SDT DRBs is mandatorily invoked to the anchor gNB to complete GTP-U tunnel setup between the receiving DU and the anchor CU.

### Termination of SDT session initiated by the receiving gNB

When there is no more UL data to transmit, the UE may send an empty BSR MAC CE toward the receiving gNB as an indication that there is no more UL data to be sent. In this case, termination of SDT session may need to be triggered from the receiving gNB, so that the anchor gNB can send RRCRelease to end the SDT session.

Regarding a way to request termination from the receiving gNB, [4] proposed that the receiving DU (or gNB) should notify no more UL data to the anchor CU via user plane. On the other hand, [5] clamed that when the receiving DU detects no more UL data, then it would trigger the F1AP UE Inactivity Notification procedure and we only need to discuss on how the receiving CU can indicate the anchor CU via XnAP.

Both proposals make sense and need specification changes (either on 38.425 or 38.423). But we prefer the control plane method because control plane better suits such termination event or decision and is more versatile. In fact, termination may be requested due to other reasons (e.g. related to implementations).

Seeking control plane solution, we only need to worry about XnAP impact. For that, a class-2 procedure looks enough. We thus propose to extend the class-2 "RETREIVE UE CONTXT CONFIRM" message that was agreed for inter-MN resume without SN change in Rel-17 [3]. We can simply add an optional IE to tell the anchor gNB that the termination of SDT is requested.

Note that control plane solution has no impact for SDT with context relocation as well. Here, the receiving gNB becomes the new anchor and as mentioned above, when the receiving DU detects no more UL data, it would trigger the F1AP UE Inactivity Notification procedure toward the receiving CU. On the other hand, user plane solution still requires 38.425 update even for SDT with context relocation scenario.

Proposal 9: For SDT without anchor relocation, support termination request initiated by the receiving gNB during SDT session. For that, extend the class-2 "RETREIVE UE CONTXT CONFIRM" message that was agreed for inter-MN resume without SN change in Rel-17 [3] by adding an optional IE to tell the anchor gNB that termination of SDT is requested from the receiving gNB.

Proposal 10: No F1 impact is foreseen for termination request from the receiving DU to CU, as the receiving DU can trigger the existing F1AP UE Inactivity Notification procedure toward the receiving CU when detecting no more UL data.

### Proposed call flow for SDT without anchor relocation

Based on discussions and proposals above, we propose the following call flow:

Proposal 11: For SDT without anchor relocation, RAN3 to endorse the following changes to RA-SDT BLCR for TS 38.300 [2]:

XX.3	RA-based SDT without UE context relocation

The following figure describes the UE initiated RACH based SDT procedure when the UE is still within the configured RNA and the last serving gNB decides not to relocate the UE context.

Figure XX.3-1. RA-based SDT without UE context relocation

1-2.  	The steps 1-2 are as defined in steps 1-2 in Figure XX.2-1.

3.  	The last serving gNB decides not to relocate UE context for SDT.

4.  	The last serving gNB transfers the SDT related context via the RETRIEVE UE CONTEXT CONFIRM message. The UE context is kept at the last serving gNB and the SDT related context is established at the receiving gNB. The UL SDT data, if any, is delivered to the UPF, and the UL signalling, if any, is forwarded to the last serving gNB via the RRC TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1: The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers to the UE.

5. 	The receiving gNB sends the associated DL TNL address via the Xn-U ADDDRESS INDICATION message.

6. 	After SDT is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes suspend configuration.

NOTE 2: The receiving gNB may send the RETRIEVE UE CONTEXT CONFIRM message to request the termination of SDT session before step 6.

NOTE 3: In case DL non-SDT data or DL non-SDT signalling arrives, the last serving gNB moves the UE back to RRC_INACTIVE by sending RRCRelease message.

7.	The receiving gNB forwards the RRCRelease message to the UE.

8. 	The UE moves to RRC_INACTIVE mode.

## E1 impacts

For E1, we still need to resolve the following two FFSes [6]:

Editor’s note 1: How the gNB-CU UP knows which bearers are SDT bearers is FFS.

Editor’s note 2: whether the Bearer Context Status Change IE in BEARER CONTEXTE SETUP can be set to "ResumeforSDT" is FFS

### How CU-UP aware of SDT bearer type.

As discussed in [7], we believe that SDT capability of a certain DRB is rather static and not dynamically changed. As a result, similar to F1AP, toward CU-UP and over E1 interface, we think it is right to configure "SDT" as one type of bearer configuration. We really don't have to tell which bearers should be active as a separate list, every time "ResumeforSDT" is requested. And having "SDT" as one of bearer configuration, CU-CP, if needed, can further change SDT capability of a bearer when requesting "ResumeforSDT" via the Bearer Context Modification procedure at no problem.

Proposal 12: Configure "SDT" as one type of DRB configuration in the Bearer context. Add an optional "SDT" indicator in the DRB To Be Setup/Modified List in the PDU Session Resource To Setup/Modify List of the E1AP Bearer Context Setup/Modification Request messages to indicate whether a DRB is SDT capable or not.

### The usage of "ResumeforSDT" in Bearer Context Setup procedure

When UE context is relocated, full context is transferred to the receiving gNB. Needless to say, a bearer context should be also established in the CU-UP of the receiving gNB.

For that, one may think that the CU-CP of the receiving gNB just needs to establish bearers related to SDT only in the CU-UP, because context transfer was caused by the "SDT" initiation from the UE. However, this is in fact a wrong observation. The whole bearer context needs to be established in the CU-UP. That is, non-SDT related bearers shall also be established in the CU-UP (but suspended) so that DL non-SDT data arrival can be properly handled. As legacy, when DL non-SDT data arrives, CU-UP should be able to notify CU-CP, so that CU-CP can take actions accordingly.

From this sense, "ResumeforSDT" in the Bearer Context Setup procedure is indeed necessary. While the whole bearer context would be established anyway, CU-UP needs to know that SDT capable bearers should be active for now while other non-SDT bearers should be immediately suspended after established, so that notification mechanism for non-SDT bearers can properly work. This could not be achieved by the existing codepoint of "Resume" or "Suspend", so we need another codepoint for the Bearer Context Setup procedure.

Observation 7: When UE context is relocated, full context is transferred to the receiving gNB. In this case, the whole bearer context shall be established in the CU-UP of the receiving gNB to properly deal with DL non-SDT data arrival.

Observation 8: While the whole bearer context being established, CU-UP also needs to know that SDT capable bearers should be active for now while other non-SDT bearers should be immediately suspended after established, so that notification mechanism for non-SDT bearers can properly work, which cannot be done by the existing codepoint of "Resume" or "Suspend". The "ResumeforSDT" in the Bearer Context Setup procedure can do this job.

But the current wording in the 38.463 BLCR [6] is written in the same way for the case of the Bearer Modification procedure, where the "ResumeforSDT" codepoint is used to resume SDT capable bearers only when the whole bearer context has been suspended. But in the Bearer Context Setup case, all bearers are newly established and by default they are active after established. So, what we need for this codepoint is the opposite – suspend the established bearers that are not capable of SDT.

Proposal 13: Confirm the usage of "ResumeforSDT" codepoint in the Bearer Context Setup procedure. Fix the description so that CU-UP can suspend the non-SDT capable bearers immediately after established.

## F1 UE context establishment in the receiving DU

When the UE initiates RA-SDT, at least some F1 UE context shall need to be established in the receiving DU to support RA-SDT. Then, a question remains whether only necessary SDT related context shall be established, or full UE context shall be established in the receiving DU.

Up to how much the receiving CU shall establish UE context in the receiving DU has not been discussed well. It is better to make this design clear before we finalize the stage-3 details.

Observation 9: When the UE initiates RA-SDT, at least some F1 UE context shall need to be established in the receiving DU to support RA-SDT. A question remains whether only necessary SDT related context shall be established, or full UE context shall be established in the receiving DU, or up to how much.

In the following, we do the case-by-case analysis as follows.

When the UE initiates RA-SDT on a gNB other than the last serving gNB, two cases to consider:

- Case 1 (SDT without anchor relocation): Only partial context related to SDT is retrieved from the old anchor gNB. The receiving CU has no choice but to establish lower layer configurations of the SDT DRBs in the receiving DU.

- Case 2 (SDT with anchor relocation): Full context is transferred to the receiving CU. Then, as business as usual, the receiving CU will (1) establish full bearer context in the CU-UP and retrieves UL TNLs; (2) establish DU context and provide UL TNLs to the receiving DU and (3) the assigned DL TNLs by the receiving DU has to be tossed back to the CU-UP to complete F1-U tunnel setup.

In this case, establishing only SDT related DRBs into the receiving DU at (2) does not have much benefits, because (a) F1-U tunnels for non-SDT DRBs are left uncompleted; (b) DL/UL non-SDT data may arrive during SDT session for which the UE may be moved to CONNECTED by RRCResume and in this case full UE context has to be there anyway in the receiving DU and the F1-U tunnels for non-SDT DRBs have to be setup. We really don't have to waste energy by doing extra steps - first establish only SDT related DRBs, and then later establish other DRBs and their F1-U tunnels across DU and CU-UP before sending RRCResume.

On the other hand, when the UE initiates RA-SDT on the same gNB (i.e. the last serving gNB), the full context is already available there in the same gNB-CU and the whole bearer context at CU-UP has been suspended. Then, there are also two cases to consider:

- Case 3 (UE resumes on new DU other than the last serving DU): In this case, still the above (2)-(3) steps are required to establish F1-U tunnels with new DU.

The same argument can be applied since the anchor gNB has not been changed, and it is better to establish the whole UE context in this DU.

- Case 4 (UE resumes on the last serving DU): In this case, once CU-CP identifies the same UE, we can apply the CG-SDT fallback to RA-SDT in the same DU case (as discussed in [7]), where the existing UE context in the DU can be re-used/updated without creating new UE context anew.

Observation 10: When the UE initiates RA-SDT, in the receiving CU, full UE context is available except when SDT without anchor relocation is used where only partial context related to SDT is retrieved from the old anchor gNB.

Observation 11: When full context is available in the receiving CU, there is not much benefit to establish only SDT related context in the receiving DU, because

- Toward CU-UP, full bearer context is established (or has been established) while only non-SDT DRBs would be suspended;

- Because of (1), F1-U tunnels for non-SDT DRBs are left uncompleted;

- DL/UL non-SDT data may arrive during SDT session for which the UE may be moved to CONNECTED by RRCResume, and in this case, full UE context has to be there anyway in the receiving DU and the F1-U tunnels for non-SDT DRBs have to be setup. It is a waste of energy by doing extra steps – first establish only SDT related context in the receiving DU, and then later establish other DRBs and their F1-U tunnels across DU and CU-UP before sending RRCResume.

Based on the above observations and design simplicity, we think it is better to establish full UE context in the receiving DU whenever full UE context is available in the receiving CU. In case of SDT without anchor relocation, only partial context will be available in the receiving CU, for which it has no other choice but to establish SDT related DRBs only in the receiving DU.

Proposal 14: For design simplicity, whenever full UE context is available in the receiving gNB (either full context is transferred from anchor or the UE initiated RA-SDT on the same gNB), the receiving CU establishes full UE context in the receiving DU and setup F1-U tunnels for all DRBs as in legacy.

## Proposed call flow for RA-SDT 38.401

Based on discussions and proposals above, we propose the following call flow:

Proposal 15: RAN3 to endorse the following changes to RA-SDT BLCR for TS 38.401 [9]:

8.X.1		RACH based SDT

The procedure for RACH based small data transmission in RRC Inactive is shown in Figure 8.X.1-1.

Figure 8.X.1-1: RACH based Small Data Transmission in RRC Inactive state.

1.	The UE in RRC Inactive sends RRCResumeRequest message together with UL SDT data and/or UL SDT signalling.

2.	The gNB-DU buffers the UL SDT data and/or UL SDT signalling.

3.	The step 3 is as defined in step 4 in clause 8.6.2, with including an indication of SDT access. The gNB-DU may also provide SDT assistance information.

4-5.	The steps 4-5 are as defined in steps 6-7 in clause 8.9.6.2. The UL small data, if any, is forwarded to the gNB-CU-UP, and the UL signalling, if any, is forwarded to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1: In case that full UE context is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP establishes the UE context in the gNB-CU-UP via the Bearer Context Setup procedure and F1-U UL TEIDs are retreived. The BEARER CONTEXT SETUP REQUSET message may include an indication to suspend non-SDT bearers, and in this case, the BEARER CONTEXT MODIFICATION REQUEST message in step 6 does not include resume indication for SDT.

NOTE 2: In case that only partial UE context for SDT including F1-U UL TEIDs is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP uses those F1-U UL TEIDs for steps 4-5, and the subsequent steps 6-7 are not executed. In addition, the UL small data, if any, is forwarded from the gNB-DU to the gNB-CU-UP of the other gNB-CU-CP for which the partial context is retrieved, and the UL signalling, if any, is forwarded from the gNB-CU-CP to the other gNB-CU-CP via the XnAP RRC TRANSFER message.

6.	The gNB-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message including an resume indication for SDT. The gNB-CU-CP also includes the F1-U DL TEIDs received from the gNB-DU in step 5.

7.	The gNB-CU-CP responds with the BEARER CONTEXT MODIFICATION RESPONSE message.

# Conclusion

SDT with context relocation

Proposal 1: For SDT with anchor relocation, fix the call flow in 38.300 BL CR [2] so that UL SDT data can be delivered to the UPF before path switch. Also add the typical Xn-U Address Indication procedure which is missing for DL data forwarding from the last serving gNB to the new gNB before path switch is completed.

Proposal 2: For SDT with anchor relocation, RAN3 to endorse the following changes to RA-SDT BLCR for TS 38.300 [2]:

XX.2	RA-based SDT with UE context relocation

The overall procedure for RACH based small data transmission with UE context relocation is illustrated in the figure XX.2-1.

Figure XX.2-1. RA-based SDT with UE context relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB locates the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE requested for SDT and may also provide SDT assistance information.

3.  The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered to the UPF.

4-6. The receiving gNB decides to keep UE in RRC Inactive state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses. The receiving gNB also initiates NG-AP Path Switch Request procedure to establish a NG UE-associated signalling connection to the serving AMF. After path switch, the UL NAS PDU, if any, is delivered to the AMF.

7. After SDT is completed, the receiving gNB generates and sends RRCRelease message including the Suspend configuration to the UE to move the UE back to RRC_INACTIVE.

NOTE: In case DL non-SDT data or DL non-SDT signalling arrives, the receiving gNB may decide to send the UE to RRC_CONNECTED state by sending RRCResume message.

8. The receiving gNB indicates the last serving gNB to remove the UE context by sending the UE CONTEXT RELEASE message.

SDT without anchor relocation – (1) PDU session handling

Observation 1: In 5GS, signalling design in RAN3 has been per PDU session basis. All the basic mobility and dual connectivity related procedure messages defined in XnAP between source and target or between MN and SN have been "per PDU session basis" from day 1. Even for SN terminated MCG bearer (or MN terminated SCG bearer), the relevant XnAP messages communicated between MN and SN carry Xn-U DL/UL forwarding TNLs "per PDU session" basis.

Proposal 3: For SDT without anchor relocation, RAN3 to design XnAP signalling for SDT related RLC bearer configuration and Xn-U DL/UL forwarding TNL info, per "PDU session basis", following the fundamental 5GS principle in RAN3 from day 1.

SDT without anchor relocation – (2) IE design for partial SDT context from anchor to receiving gNB

Observation 2: For SDT without anchor relocation, the receiving gNB needs to properly establish UE context and lower layers configurations for SDT DRBs into its DU. The baseline assumption is to re-use the existing F1AP UE CONTEXT SETUP REQUEST message. Whatever required there should be supplied from the anchor gNB as part of SDT context transfer.

Observation 3: When CU establishes DRBs onto DU via DRBs To Be Setup List, the following IEs are mandatorily included: DRB QoS, S-NSSAI, Flows mapped to DRB, UL TNL, RLC mode, DL PDCP SN length.

Observation 4: CellGroupConfig (via CU to DU RRC Information IE) is required to setup UE context in DU. CellGroupConfig shall also be supplied from anchor to receiving gNB.

Observation 5: PDU session ID shall also be supplied from anchor to receiving gNB to honor the day-1 principle of per PDU session handling between NG-RAN nodes.

Proposal 4: For SDT without anchor relocation, the anchor gNB also provides the following IEs as part of SDT context transfer:

- DRB QoS, S-NSSAI, Flows mapped to DRB, UL TNL, RLC mode, DL PDCP SN length: required for the receiving gNB to establish SDT DRBs onto its DU.

- CellGroupConfig IE: required at least for SRB1 RLC and SDT DRB RLC configurations

- PDU Session ID: required to honor the day-1 principle of per PDU session handling.

Observation 6: Between CU and DU, GTP-U tunnel has been bi-directionally setup from day 1. There is no reason not to supply DL Xn-U TNLs for SDT DRBs already assigned by the receiving DU toward the anchor CU. And this design is simpler compared to supplying DL Xn-U TNLs to the anchor gNB only when DL data availability is separately indicated by the anchor gNB.

Proposal 5: For SDT without anchor relocation, the anchor gNB does not need to separately indicate to the receiving gNB that DL data is available at the anchor gNB. The receiving gNB supplies DL Xn-U TNLs for SDT DRBs already assigned by its DU toward the anchor gNB, so that the anchor gNB can send SDT DL data whenever available.

Proposal 6: For SDT without anchor relocation, allow multiple Xn-U TNLs setup for a SDT DRB between the receiving DU and the anchor CU in case PDCP duplication has been configured.

Proposal 7: For SDT context transfer from anchor to receiving gNB, agree the following IE structures:

9.2.3.XX2		SDT Configuration Transfer

This IE includes necessary configuration information to process RLC PDUs and perform data forwarding related to RACH based SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SDT RLC Configuration | M |  | OCTET STRING | CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [10]. Required at least to carry SRB1 RLC configuration and DRB RLC configurations configured for SDT. |
| SDT Xn-U Context Information | M |  | 9.3.XX3 |  |

9.2.3.XX3		SDT Xn-U Context Information

This IE includes information necessary to setup Xn-U bearers related to RACH based SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>DRBs To Be Setup List |  | 1 |  |  | – |  |
| >>>DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>>>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>>>QoS Flows Mapped To DRB List |  | 1 |  |  | – |  |
| >>>>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>PDCP SN Length | M |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>>>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used. | – |  |
| >>>>UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3. 76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>>>secondary SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>>>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |

SDT without anchor relocation – (3) Signalling flow design

Proposal 8: For SDT without anchor relocation, compromise to go with the Solution 3, with the following modifications:

Figure 1: Solution 3 [1]

- Extend the "RETRIEVE UE CONTXT CONFRIM" that was agreed for inter-MN resume without SN change in Rel-17 [3] as for the new class-2 message of step 2a that transfers SDT context.

- After step 2a, the step 2b Xn-U Address Indication that provides DL Xn-U TNLs for SDT DRBs is mandatorily invoked to the anchor gNB to complete GTP-U tunnel setup between the receiving DU and the anchor CU.

SDT without anchor relocation – (4) Termination of SDT session initiated by the receiving gNB

Proposal 9: For SDT without anchor relocation, support termination request initiated by the receiving gNB during SDT session. For that, extend the class-2 "RETREIVE UE CONTXT CONFIRM" message that was agreed for inter-MN resume without SN change in Rel-17 [3] by adding an optional IE to tell the anchor gNB that termination of SDT is requested from the receiving gNB.

Proposal 10: No F1 impact is foreseen for termination request from the receiving DU to CU, as the receiving DU can trigger the existing F1AP UE Inactivity Notification procedure toward the receiving CU when detecting no more UL data.

SDT without anchor relocation – (5) Proposed call flow

Proposal 11: For SDT without anchor relocation, RAN3 to endorse the following changes to RA-SDT BLCR for TS 38.300 [2]:

XX.3	RA-based SDT without UE context relocation

The following figure describes the UE initiated RACH based SDT procedure when the UE is still within the configured RNA and the last serving gNB decides not to relocate the UE context.

Figure XX.3-1. RA-based SDT without UE context relocation

1-2.  	The steps 1-2 are as defined in steps 1-2 in Figure XX.2-1.

3.  	The last serving gNB decides not to relocate UE context for SDT.

4.  	The last serving gNB transfers the SDT related context via the RETRIEVE UE CONTEXT CONFIRM message. The UE context is kept at the last serving gNB and the SDT related context is established at the receiving gNB. The UL SDT data, if any, is delivered to the UPF, and the UL signalling, if any, is forwarded to the last serving gNB via the RRC TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1: The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers to the UE.

5. 	The receiving gNB sends the associated DL TNL address via the Xn-U ADDDRESS INDICATION message.

6. 	After SDT is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes suspend configuration.

NOTE 2: The receiving gNB may send the RETRIEVE UE CONTEXT CONFIRM message to request the termination of SDT session before step 6.

NOTE 3: In case DL non-SDT data or DL non-SDT signalling arrives, the last serving gNB moves the UE back to RRC_INACTIVE by sending RRCRelease message.

7.	The receiving gNB forwards the RRCRelease message to the UE.

8. 	The UE moves to RRC_INACTIVE mode.

E1 impacts – (1) How CU-UP aware of SDT bearer type

Proposal 12: Configure "SDT" as one type of DRB configuration in the Bearer context. Add an optional "SDT" indicator in the DRB To Be Setup/Modified List in the PDU Session Resource To Setup/Modify List of the E1AP Bearer Context Setup/Modification Request messages to indicate whether a DRB is SDT capable or not.

E1 impacts – (2) The usage of "ResumeforSDT" in Bearer Context Setup procedure

Observation 7: When UE context is relocated, full context is transferred to the receiving gNB. In this case, the whole bearer context shall be established in the CU-UP of the receiving gNB to properly deal with DL non-SDT data arrival.

Observation 8: While the whole bearer context being established, CU-UP also needs to know that SDT capable bearers should be active for now while other non-SDT bearers should be immediately suspended after established, so that notification mechanism for non-SDT bearers can properly work, which cannot be done by the existing codepoint of "Resume" or "Suspend". The "ResumeforSDT" in the Bearer Context Setup procedure can do this job.

Proposal 13: Confirm the usage of "ResumeforSDT" codepoint in the Bearer Context Setup procedure. Fix the description so that CU-UP can suspend the non-SDT capable bearers immediately after established.

F1 UE context establishment in the receiving DU

Observation 9: When the UE initiates RA-SDT, at least some F1 UE context shall need to be established in the receiving DU to support RA-SDT. A question remains whether only necessary SDT related context shall be established, or full UE context shall be established in the receiving DU, or up to how much.

Observation 10: When the UE initiates RA-SDT, in the receiving CU, full UE context is available except when SDT without anchor relocation is used where only partial context related to SDT is retrieved from the old anchor gNB.

Observation 11: When full context is available in the receiving CU, there is not much benefit to establish only SDT related context in the receiving DU, because

- Toward CU-UP, full bearer context is established (or has been established) while only non-SDT DRBs would be suspended;

- Because of (1), F1-U tunnels for non-SDT DRBs are left uncompleted;

- DL/UL non-SDT data may arrive during SDT session for which the UE may be moved to CONNECTED by RRCResume, and in this case, full UE context has to be there anyway in the receiving DU and the F1-U tunnels for non-SDT DRBs have to be setup. It is a waste of energy by doing extra steps – first establish only SDT related context in the receiving DU, and then later establish other DRBs and their F1-U tunnels across DU and CU-UP before sending RRCResume.

Proposal 14: For design simplicity, whenever full UE context is available in the receiving gNB (either full context is transferred from anchor or the UE initiated RA-SDT on the same gNB), the receiving CU establishes full UE context in the receiving DU and setup F1-U tunnels for all DRBs as in legacy.

Proposed call flow for RA-SDT 38.401

Proposal 15: RAN3 to endorse the following changes to RA-SDT BLCR for TS 38.401 [9]:

8.X.1		RACH based SDT

The procedure for RACH based small data transmission in RRC Inactive is shown in Figure 8.X.1-1.

Figure 8.X.1-1: RACH based Small Data Transmission in RRC Inactive state.

1.	The UE in RRC Inactive sends RRCResumeRequest message together with UL SDT data and/or UL SDT signalling.

2.	The gNB-DU buffers the UL SDT data and/or UL SDT signalling.

3.	The step 3 is as defined in step 4 in clause 8.6.2, with including an indication of SDT access. The gNB-DU may also provide SDT assistance information.

4-5.	The steps 4-5 are as defined in steps 6-7 in clause 8.9.6.2. The UL small data, if any, is forwarded to the gNB-CU-UP, and the UL signalling, if any, is forwarded to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1: In case that full UE context is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP establishes the UE context in the gNB-CU-UP via the Bearer Context Setup procedure and F1-U UL TEIDs are retreived. The BEARER CONTEXT SETUP REQUSET message may include an indication to suspend non-SDT bearers, and in this case, the BEARER CONTEXT MODIFICATION REQUEST message in step 6 does not include resume indication for SDT.

NOTE 2: In case that only partial UE context for SDT including F1-U UL TEIDs is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP uses those F1-U UL TEIDs for steps 4-5, and the subsequent steps 6-7 are not executed. In addition, the UL small data, if any, is forwarded from the gNB-DU to the gNB-CU-UP of the other gNB-CU-CP for which the partial context is retrieved, and the UL signalling, if any, is forwarded from the gNB-CU-CP to the other gNB-CU-CP via the XnAP RRC TRANSFER message.

6.	The gNB-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message including an resume indication for SDT. The gNB-CU-CP also includes the F1-U DL TEIDs received from the gNB-DU in step 5.

7.	The gNB-CU-CP responds with the BEARER CONTEXT MODIFICATION RESPONSE message.

The corresponding changes to E1AP and XnAP for the above proposals can be found in Section 5 and Section 6, respectively. The corresponding changes to TS 38.300 and TS 38.401 for the above proposals can be found in [10].

# Reference

[1] R3-221452, "Summary of Offline Discussion on CB: # SDT1_RACHbased", ZTE

[2] R3-221562, "RA-SDT BLCR to TS 38.300", ZTE

[3] R3-221288, "Inter MN resume without SN change [InterMNResume]", CR0596r5 to Rel-17 TS 38.423

[4] R3-220424, "(TPs to RA-SDT BL CRs of TS 38.300, 38.420) RACH based SDT without anchor relocation", Huawei

[5] R3-220203, "(TP for TS 38.423) Support of RACH-based SDT (Nokia, Nokia Shanghai Bell)", Nokia, Nokia Shanghai Bell

[6] R3-221659, "RA-SDT BLCR to TS 38.463", China Telecom

[7] R3-222354, "(TP for CG-SDT BL CR for TS 38.401/473/470) Toward the completion of CG-SDT", Intel Corporation

[8] R3-221564, "RA-SDT BLCR to TS 38.423", Ericsson

[9] R3-221563, "RA-SDT BLCR to TS 38.401", Intel Corporation

[10] R3-222353, "(TP for RA-SDT BL CR for TS 38.300/401)", Intel Corporation.

# Text Proposal for RA-SDT BLCR to TS 38.463 [6]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2]. If the Bearer Context Status Change IE is set to "ResumeforSDT", the gNB-CU-UP shall consider that DRBs not configured with SDT are suspended after established.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, ResumeforSDT) | Indicates the status of the Bearer Context | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, ResumeforSDT) | Indicates the status of the Bearer Context | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.3.2	PDU Session Resource To Setup List

This IE contains PDU session resource related information used at Bearer Context Setup Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | Indicates that the target gNB-CU-CP requests QoS flow remapping during an intra-system lossless handover as specified in TS 38.300 [4]. | YES | reject |
| >>>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB | YES | ignore |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |
| >Redundant PDU Session Information | O |  | 9.3.1.80 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.3.10	PDU Session Resource To Setup Modification List

This IE contains PDU session resource to setup related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>DAPS Request Information | O |  | 9.3.1.91 | This IE is not used in this version of the specification. | YES | ignore |
| >>>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB | YES | ignore |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | This IE is not used in this version of the specification. | YES | reject |
| >>>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB | YES | ignore |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>Early Data Forwarding Indicator | O |  | ENUMERATED (stop, …) |  | YES | ignore |
| >>>SDT Indicator | O |  | ENUMERATED (true, false, …) | Indicates SDT DRB or not. | YES | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |
| maxnoofDataForwardingTunneltoE-UTRAN | Maximum no. of Data Forwarding Tunnels to E-UTRAN for a UE. Value is 256. |
| maxnoofQoSflows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

# Text Proposal for RA-SDT BLCR to TS 38.423 [8]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection or SDT session has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE and if the target NG-RAN Node is a gNB at least the MDT Configuration-NR IE shall be present, while if the target NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and, if supported, use it for RAN part delay reporting.

If the 5GC Mobility Restriction List Container IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

-	If the NR V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the LTE V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287[38].

In case of RRC Re-establishment, the old NG-RAN may include the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message. Upon reception of the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the SDT Support Request IE is included in the RETRIEVE UE CONTEXT REQUEST message, the old NG-RAN node shall, if supported, consider that the UE has requested for SDT as defined in TS 38.300 [6].

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the UE Context ID, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, or, if it decides not to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

If the old NG-RAN node decides to keep the UE context in case of periodic RNAU or in case of RACH based SDT, it shall store the Allocated C-RNTI IE and the Access PCI IE in the UE Context ID IE, as described in TS 38.300 [9].

If the Old NG-RAN node to New NG-RAN node Resume Container IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node should transparently forward the content of this IE to the UE as described in TS 38.300 [9].

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.2.x	Retrieve UE Context Confirm

#### 8.2.x.1	General

The Retrieve UE Context Confirm procedure is used by the new NG-RAN node to inform the old NG-RAN node whether the S-NG-RAN node associated with the old NG-RAN node for the UE that was indicated during UE context retrieval is kept or not by the new NG-RAN node during RRC resumption.

In case of RACH based SDT, the Retrieve UE Context Confirm procedure is also used to transfer part of the UE context from the old NG-RAN node to the new NG-RAN node or to request the termination of RACH based SDT session from the new NG-RAN node to the old NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.x.2	Successful Operation

Figure 8.2.X.2-1: Retrieve UE Context Confirm, successful operation

Figure 8.2.X.2-2: Retrieve UE Context Confirm, successful operation (RACH based SDT)

RRC Resume

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT CONFIRM message to the old NG-RAN node.

Upon reception of the RETRIEVE UE CONTEXT CONFIRM message, the old NG-RAN node shall release the resources related to the UE-associated signalling connection between the old NG-RAN node and the S-NG-RAN node, as specified in TS 37.340 [8].

If the UE Context Kept Indicator IE is included and set to "True", the old NG-RAN node shall consider that the S-NG-RAN node was kept by the new NG-RAN node and use this information as specified in TS 37.340 [8].

RACH based SDT

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT CONFIRM message to the old NG-RAN node or the old NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT CONFIRM message to the new NG-RAN node.

If the new NG-RAN node receives the SDT Configuration Transfer IE, the new NG-RAN node shall establish the XnAP UE association and SDT related context based on the received SDT Configuration Transfer IE and act as specified in TS 38.300 [9].

If the old NG-RAN node receives the SDT Termination Request IE and set to "true", the old NG-RAN node shall consider that the termination of the ongoing SDT session is requested from the new NG-RAN node for this UE and act as specified in TS 38.300 [9].

#### 8.2.x.3	Unsuccessful Operation

Not applicable.

#### 8.2.x.4	Abnormal Conditions

If the RETRIEVE UE CONTEXT CONFIRM message refers to a context that does not exist, the old NG-RAN node shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.9	RRC Transfer

#### 8.3.9.1	General

The purpose of the RRC Transfer procedure is to deliver a PDCP-C PDU encapsulating an LTE RRC message or NR RRC message to the S-NG-RAN-NODE that it may then be forwarded to the UE, or from the S-NG-RAN-NODE, if it was received from the UE. The delivery status may also be provided from the S-NG-RAN-NODE to the M-NG-RAN-NODE using the RRC Transfer.

The procedure is also used to enable transfer one of the following messages from the M-NG-RAN-NODE to the S-NG-RAN-NODE, when received from the UE:

-	the NR RRC message container with the NR measurements;

-	the E-UTRA RRC message container with the E-UTRA measurements;

-	the NR RRC message container with the NR failure information;

-	the NR RRC message container with the RRCReconfigurationComplete message;

-	the NR RRC message container with the UE assistance information.

This procedure is also used to deliver a PDCP-C PDU encapsulating a NR RRC message between the new NG-RAN node and the old NG-RAN node.

The procedure uses UE-associated signalling.

- 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: RRC Transfer procedure for dual connectivity, successful operation.

Figure 8.3.9.2-x: RRC Transfer procedure for SDT, successful operation.

Dual Connectivity

The M-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the S-NG-RAN-NODE or the S-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the M-NG-RAN-NODE.

If the S-NG-RAN-NODE receives an RRC TRANSFER message which does not include the RRC Container IE in the Split SRB IE, or the RRC Container IE in the NR UE Report IE, or the the RRC Container IE in the Fast MCG Recovery via SRB3 from MN to SN IE, or the the RRC Container IE in the Fast MCG Recovery via SRB3 from SN to MN IE, it shall ignore the message. If the S-NG-RAN-NODE receives an RRC TRANSFER message with the Delivery Status IE in the Split SRB IE, it shall ignore the message. If the S-NG-RAN-NODE receives the RRC Container IE in the Split SRB IE, it shall deliver the contained PDCP-C PDU encapsulating an RRC message to the UE. If the S-NG-RAN-NODE receives the RRC Container IE in the Fast MCG Recovery via SRB3 from MN to SN IE, the S-NG-RAN-NODE shall deliver the contained RRC container encapsulating an RRC message to the UE.

If the M-NG-RAN-NODE receives the Delivery Status IE in the Split SRB IE, the M-NG-RAN-NODE shall consider RRC messages up to the indicated NR PDCP SN as having been successfully delivered to UE by S-NG-RAN-NODE. If the M-NG-RAN-NODE receives the RRC Container IE in the Fast MCG Recovery via SRB3 from SN to MN IE, the M-NG-RAN-NODE shall consider MCG link failure detected at the UE as specified in TS 37.340 [8].

SDT

The new NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the old NG-RAN-NODE or the old NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the new NG-RAN-NODE.

If the new NG-RAN node receives the RRC Container IE in the SDT SRB between New NG-RAN node and Old NG-RAN node IE, it shall deliver the contained PDCP-C PDU encapsulating an RRC message to the UE. If the old NG-RAN-NODE receives the RRC Container IE in the SDT SRB between New NG-RAN node and Old NG-RAN node IE, it shall consider the contained PDCP-C PDU encapsulating an RRC message from the UE.

- 8.3.9.3	Unsuccessful Operation

Not applicable.

- 8.3.9.4	Abnormal Conditions

In case of the split SRBs, the receiving node may ignore the message, if the M-NG-RAN-NODE has not indicated possibility of RRC transfer at the bearer setup.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.1.y	RETRIEVE UE CONTEXT CONFIRM

This message is sent by the new NG-RAN node to the old NG-RAN node to inform the old NG-RAN node whether the S-NG-RAN node associated with the old NG-RAN node for the UE that was indicated during UE context retrieval is kept or not by the new NG-RAN node during RRC resumption.

In case of RACH based SDT, the Retrieve UE Context Confirm procedure is also used to transfer part of the UE context from the old NG-RAN node to the new NG-RAN node or to request the termination of RACH based SDT session from the new NG-RAN node to the old NG-RAN node.

Direction: new NG-RAN node  old NG-RAN node (RRC Resume).

Direction: old NG-RAN node  new NG-RAN node or new NG-RAN node  old NG-RAN node (RACH based SDT)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| SDT Configuration Transfer | O |  | 9.3.XX2 |  | YES | ignore |
| SDT Termination Request | O |  | ENUMERATED (true, …) |  | YES | ignore |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.3.XX2		SDT Configuration Transfer

This IE includes necessary configuration information to process RLC PDUs and perform data forwarding related to RACH based SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SDT RLC Configuration | M |  | OCTET STRING | CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [10]. Required at least to carry SRB1 RLC configuration and DRB RLC configurations configured for SDT. |
| SDT Xn-U Context Information | M |  | 9.3.XX3 |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.3.XX3		SDT Xn-U Context Information

This IE includes information necessary to setup Xn-U bearers related to RACH based SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>DRBs To Be Setup List |  | 1 |  |  | – |  |
| >>>DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>>>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>>>QoS Flows Mapped To DRB List |  | 1 |  |  | – |  |
| >>>>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>PDCP SN Length | M |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>>>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used. | – |  |
| >>>>UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3. 76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>>>secondary SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>>>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | Old NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is 64 |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional PDCP Duplication TNL. Value is 2. |