3GPP TSG-RAN WG3 Meeting #115-e	R3-222500

# Online, February 21st – March 3rd 2022

s

Agenda Item:	13.2.2

Source:	Ericsson

Title:	(TP for IAB BL CR for TS 38.423): IAB Inter-Donor Topology Redundancy

Document for:	Agreement

# Introduction

In this paper we discuss the stage-3 details of XnAP signalling for F1 transport migration for single- and dual-connected boundary nodes, and the corresponding revoking. The relevant RAN3#114bis-e agreements are:

For topology redundancy:

  - CU1 can initiate the new procedure to request setup, modification (of QoS info only), and full or partial release (e.g. for the purpose of revoking) of traffic migration.

  - FFS on which of the following two alternatives is supported (downselection expected):

    - Whether CU2 can initiate the new procedure to request modification (of L2 info only) and full/partial release (e.g. for the purpose of revoking), OR

    - Whether CU2 can initiate a new class-2 procedure to provide serving status of CU2’s topology, which can help CU1’s decision making wrt modification and release (e.g. for the purpose of revoking) of traffic migration.

For partial migration and RLF recovery:

  - CU1 can initiate the new procedure to request setup, modification (of QoS info only).

  - WA: CU1 can initiate the new procedure to request full or partial release (e.g. for the purpose of revoking) of traffic migration.

  - CU2 can initiate the new procedure to request modification of traffic migration (modification of L2 info only).

  - CU2 can initiate the XnAP HO for the boundary MT to realize full release (e.g. for the purpose of revoking) of traffic migration.

After partial migration, the non-F1-terminating CU can use the Xn HO procedure to migrate all offloaded traffic back to the F1-terminating-CU’s topology.

The non-F1-terminating CU of a dual-connected boundary node can request partial release of offloaded traffic back to the F1-terminating-CU’s topology. FFS on Stage 3.

# Procedure details for inter-donor F1 transport migration

In this section we discuss the remaining stage-3 issues related to inter-donor traffic offloading.

## Boundary node ID

At the RAN3#114bis-e meeting, the following was agreed:

The new Xn procedure can be used after the boundary-IAB-MT’s Xn HO or SN Addition procedure.

The new Xn procedure to include a boundary-node ID, which is retained after reception of the UE Context Release message. This identifier can be the XnAP UE ID or the BAP address. Further selection expected in next meeting.

TBC:

- If ID = BAP address, the boundary node ID needs to be included in the Xn HO/SN Add/UE Context Retrieval procedures.

- If ID = Xn AP UE ID, the retention of this ID after UE Context Release needs to be specified.

- Should the new Xn message be UA if the XnAP UE ID is used as the boundary node ID?

- Can the XnAP UE ID be used for the boundary node if the new Xn procedure is NUA? Why is it not a UA message in this case?

In partial migration, if the source donor sends the XnAP HANDOVER REQUEST for the boundary IAB-MT (BMT) before initiating the new XnAP procedure towards the target donor, the target donor is already in possession of the UE XnAP ID of the BMT, which can be used to reference the BMT in the subsequent new XnAP procedure.

One drawback of using BAP address as the BMT identifier is that it impacts 3 existing XnAP procedures (i.e., Xn Handover Preparation, SN Addition and UE Context Retrieval procedures). On the other hand, the standard impact incurred by selecting the UE XnAP ID of the BMT is to specify that it is retained after context release. Moreover, both the CU1-generated and the CU2-generated XnAP ID for the BMT should be indicated in the request and response messages of the XnAP IAB Transport Migration Management procedure

Proposal 1: RAN3 agrees the following:

- The boundary IAB-MT’s UE XnAP ID is retained by the F1-terminating CU after the non-F1-terminating CU has sent the UE Context Release message to the F1-terminating CU.

- Both the UE XnAP ID generated by the F1-terminating CU and the UE XnAP ID generated by the non-F1-terminating CU the boundary IAB-MT’s are used to reference the boundary IAB-MT in the new XnAP IAB Transport Migration Management procedure.

## UA vs NUA procedure

The new XnAP procedure pertains to all scenarios for inter-donor transport migration. As discussed above, CU1 should retain the UE XnAP ID of the BMT. In partial migration and RLF recovery, after the HO of the BMT, it is the target donor that serves the BMT and, formally speaking, a NUA procedure should be used. Moreover, there already exist NUA procedures that convey UE XnAP IDs.

Proposal 2: The new XnAP IAB Transport Migration procedure is NUA.

# Revoking of inter-donor traffic offloading

In this section we discuss the remaining issues related to revoking of traffic offloading.

## CU1-triggered revoking in partial migration and RLF recovery

At the RAN3#114bis-e meeting, the following was agreed:

For partial migration and RLF recovery:

  - CU1 can initiate the new procedure to request setup, modification (of QoS info only).

  - WA: CU1 can initiate the new procedure to request full or partial release (e.g. for the purpose of revoking) of traffic migration.

  - CU2 can initiate the new procedure to request modification of traffic migration (modification of L2 info only).

  - CU2 can initiate the XnAP HO for the boundary MT to realize full release (e.g. for the purpose of revoking) of traffic migration.

RAN3 has previously agreed that CU1 can trigger the revoking in topology redundancy scenario, by initiating the new XnAP procedure towards CU2. In our view, CU1 should also be able to trigger the revoking of traffic offloading in partial migration and inter-donor RLF recovery. Note that inter-donor topology adaptation, including both partial migration and traffic offloading by using inter-donor topology redundancy, may be triggered for various reasons:

- Traffic load under source CU (“load balancing”).

- Link deterioration between the boundary node (BN) and the old parent (“robustness”).

- Link failure between the BN and the old parent (“RLF recovery”).

In partial migration and inter-donor RLF recovery the BN is single-connected. In this case, after the traffic load under CU1 has become acceptable, CU1 should be able to notify CU2 that it is now able to serve the traffic of the boundary node and its descendants, resulting in the return of offloaded traffic back to its original path under CU1.

The above can be enabled as follows:

- CU1 initiates the new XnAP procedure, by indicating “All Traffic” in the Traffic To Be Released Information IE.

- CU2 initiates the XnAP Handover Preparation procedure for the boundary IAB-MT towards CU1.

- After receiving an affirmative HANDOVER REQUEST ACKNOWLEDGE, CU2 may release the resources used for traffic offloading.

Proposal 3: For partial migration and RLF recovery, CU1 can initiate the new XnAP procedure towards CU2, to request full release (e.g., for the purpose of revoking) of traffic offloading.

## CU2-triggered revoking in topology redundancy

At the RAN3#114bis-e meeting, the following was also agreed:

For topology redundancy:

  - CU1 can initiate the new procedure to request setup, modification (of QoS info only), and full or partial release (e.g. for the purpose of revoking) of traffic migration.

  - FFS on which of the following two alternatives is supported (downselection expected):

    - Whether CU2 can initiate the new procedure to request modification (of L2 info only) and full/partial release (e.g. for the purpose of revoking), OR

    - Whether CU2 can initiate a new class-2 procedure to provide serving status of CU2’s topology, which can help CU1’s decision making wrt modification and release (e.g. for the purpose of revoking) of traffic migration.

According to the above agreement, RAN3 should downselect between the following two alternatives:

- Option 1: CU2 triggers the revoking by initiating the new XnAP procedure towards CU1.

- Option 2: CU2 initiates a new class-2 XnAP procedure towards CU1 to inform CU1 about its load status, after which CU1 decides whether it should trigger the revoking of traffic offload.

We somewhat prefer Option 1, given the smaller standard impact and the fact that the new XnAP procedure is inherently bi-directional, enabling both peer donor-CUs to trigger it.

Proposal 4: In inter-donor topology redundancy, CU2 can initiate the revoking of traffic offloading by initiating the new XnAP procedure towards CU1.

The above proposals are reflected in a TP for IAB BL CR for TS 38.423, presented in the Annex. The following is proposed:

- The new XnAP procedure is bidirectional and NUA.

- Normative text is added to enable both CUs to trigger the revoking.

Proposal 5: Agree the TP for IAB BL CR for TS 38.423, presented in the Annex.

# Conclusion

In this paper we discuss the stage-3 details of XnAP signalling for inter-donor transport migration for single- and dual-connected boundary nodes, and the corresponding revoking. The following is proposed:

Proposal 1: RAN3 agrees the following:

- The boundary IAB-MT’s UE XnAP ID is retained by the F1-terminating CU after the non-F1-terminating CU has sent the UE Context Release message to the F1-terminating CU.

- Both the UE XnAP ID generated by the F1-terminating CU and the UE XnAP ID generated by the non-F1-terminating CU the boundary IAB-MT’s are used to reference the boundary IAB-MT in the new XnAP IAB Transport Migration Management procedure.

Proposal 2: The new XnAP IAB Transport Migration procedure is NUA.

Proposal 3: For partial migration and RLF recovery, CU1 can initiate the new XnAP procedure towards CU2, to request full release (e.g., for the purpose of revoking) of traffic offloading.

Proposal 4: In inter-donor topology redundancy, CU2 can initiate the revoking of traffic offloading by initiating the new XnAP procedure towards CU1.

Proposal 5: Agree the TP for IAB BL CR for TS 38.423, presented in the Annex.

# Annex: TP for IAB BL CR for TS 38.423

-------------------------------------------Start of changes-------------------------------------------

## 8.x	IAB Procedures

>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<

### 8.x.a	IAB Transport Migration Management

#### 8.x.a.1	General

The purpose of the IAB Transport Migration Management procedure is to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of managing the migration of the boundary and descendant IAB-node traffic between the topologies managed by the two IAB-donor-CUs.

The procedure is applicable to inter-donor partial migration, inter-donor RLF recovery and inter-donor topology redundancy cases.

The procedure can be initiated by the F1-terminating IAB-donor-CU of the boundary IAB-node, to set up, modify, and release (e.g., for the purpose of revoking) the resources under the non-F1-terminating IAB-donor-CU used for serving the offloaded traffic. The procedure can be initiated by the non-F1-terminating IAB-donor-CU to modify, and release (e.g., for the purpose of revoking) the resources under the non-F1-terminating IAB-donor-CU used for serving the offloaded traffic.

The procedure uses non-UE-associated signalling.

NOTE: For a single-connected boundary IAB-node, the terms “F1-terminating donor CU” and “non-F1-terminating donor CU” refer to the former source IAB-donor-CU and the former target IAB-donor-CU, respectively. For a single-connected boundary IAB-node, the terms “F1-terminating donor CU” and “non-F1-terminating donor CU” refer to the Master NG-RAN node and the Secondary NG-RAN node serving the boundary IAB-node, respectively.

#### 8.x.a.2	Successful Operation

The procedure initiated by the F1-terminating donor

Figure 8.x.a.2-1: IAB Transport Migration Management triggered by the F1-terminating donor, successful operation

The F1-terminating donor initiates the procedure by sending the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor.

The non-F1-terminating donor may respond with the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message by indicating:

-	Traffic accepted for offloading, within the Traffic Added List IE.

-	Already offloaded traffic accepted for modification, within the Traffic Modified List IE.

-	Traffic not accepted for offloading, within the Traffic Not Added List IE.

-	Already offloaded traffic not accepted for modification, within the Traffic Not Modified List IE.

If the Traffic to Be Released Information IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, the non-F1-terminating donor should release all offloaded traffic if the All Traffic Indication IE in the Traffic to Be Released Information IE is set to “true”, or release only the offloaded traffic indicated by the Traffic to Be Released Item IE in the Traffic to Be Released Information IE.

If the IAB TNL Address Request IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, the non-F1-terminating donor should provide the allocated TNL address via the IAB TNL Address Response IE in the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message.

The procedure initiated by the non-F1-terminating donor

Figure 8.x.a.2-2: IAB Transport Migration Management triggered by the non-F1-terminating donor, successful operation

The non-F1-terminating donor initiates the procedure by sending the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the F1-terminating IAB-donor.

The F1-terminating donor may respond with the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message by indicating:

-	Already offloaded traffic accepted for modification, within the Traffic Modified List IE.

-	Already offloaded traffic not accepted for modification, within the Traffic Not Modified List IE.

If the Traffic to Be Released Information IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, the F1-terminating donor should return all offloaded traffic if the All Traffic Indication IE in the Traffic to Be Released Information IE is set to “true” back to its original path under the F1-terminating donor, or return only the offloaded traffic indicated by the Traffic to Be Released Item IE in the Traffic to Be Released Information IE back to its original path under the F1-terminating donor.

#### 8.x.a.3	Unsuccessful Operation

Not applicable.

#### 8.x.a.4	Abnormal Conditions

Not applicable.

-------------------------------------------Next Change-------------------------------------------

### 9.1.x	Messages for IAB Procedures

>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<

#### 9.1.x.a	IAB TRANSPORT MIGRATION MANAGEMENT REQUEST

This message is sent by the F1-terminating IAB-donor-CU to the non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of setting up, modifying, or releasing (e.g., for the purpose of revoking) the configuration for the migration of boundary and descendant node traffic between two IAB-donor-CUs.

This message is sent by the non-F1-terminating IAB-donor-CU to the non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of modifying, or releasing (e.g., for the purpose of revoking) the configuration for the migration of boundary and descendant node traffic between two IAB-donor-CUs.

Direction: F1-terminating donor CU  non-F1-terminating donor CU.

Direction: Non-F1-terminating donor CU  F1-terminating donor CU.

NOTE: For a single-connected boundary IAB-node, the terms “F1-terminating donor CU” and “non-F1-terminating donor CU” refer to the former source IAB-donor-CU and the former target IAB-donor-CU, respectively. For a single-connected boundary IAB-node, the terms “F1-terminating donor CU” and “non-F1-terminating donor CU” refer to the Master NG-RAN node and the Secondary NG-RAN node serving the boundary IAB-node, respectively.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-Terminating Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID reference or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Non-F1-Terminating Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID reference or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Traffic To Be Added List |  | 0..1 |  | Present only if the sender is the F1-terminating IAB-donor-CU, when applicable. | YES | reject |
| >Traffic To Be Added Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>Traffic Profile | M |  | 9.2.2.x1 |  | – |  |
| >>F1-terminating Topology BH Information | M |  | 9.2.2.x2 |  | – |  |
| Traffic To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic To Be Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>Traffic Profile | O |  | 9.2.2.x1 | Present only if the sender is the F1-terminating IAB-donor-CU, when applicable. | – |  |
| >>F1-terminating Topology BH Information | O |  | 9.2.2.x2 | Present only if the sender is the F1-terminating IAB-donor-CU, when applicable. | – |  |
| >>Non-F1-Terminating Donor Transport Information | O |  | 9.2.2.x3 | Present only if the sender is the non-F1-terminating IAB-donor-CU, when applicable. | – |  |
| Traffic To Be Released Information | O |  | 9.2.2.x4 |  | YES | reject |
| IAB TNL Address Request | O |  | 9.2.2.x5 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor-CU. The value is FFS. |

#### 9.1.x.b	IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE

This message is sent by the non-F1-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU, or by the F1-terminating IAB-donor-CU to the non-F1-terminating IAB-donor-CU of a boundary IAB-node to provide inter-donor transport related configurations for the offloaded traffic.

Direction: Non-F1-terminating donor CU  F1-terminating donor CU.

Direction: F1-terminating donor CU  non-F1-terminating donor CU.

NOTE: For a single-connected boundary IAB-node, the terms “F1-terminating donor CU” and “non-F1-terminating donor CU” refer to the former source IAB-donor-CU and the former target IAB-donor-CU, respectively. For a single-connected boundary IAB-node, the terms “F1-terminating donor CU” and “non-F1-terminating donor CU” refer to the Master NG-RAN node and the Secondary NG-RAN node serving the boundary IAB-node, respectively.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-Terminating Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | reject |
| Non-F1-Terminating Donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | reject |
| Traffic Added List |  | 0..1 |  | Present only if the sender is the non-F1-terminating IAB-donor-CU, when applicable. | YES | reject |
| >Traffic Added Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>Non-F1-terminating Topology BH Information | M |  | 9.2.2.x3 |  | – |  |
| Traffic Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>F1-terminating Topology BH Information | O |  | 9.2.2.x2 |  | – |  |
| >>Non-F1-terminating Topology BH Information | O |  | 9.2.2.x3 |  | – |  |
| Traffic Not Added List |  | 0..1 |  |  | YES | reject |
| >Traffic Not Added Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>Cause | O |  | 9.2.2.2 |  | – |  |
| Traffic Not Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic Not Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>Cause | O |  | 9.2.3.2 |  | – |  |
| Traffic Released Information | O |  | 9.2.2.x16 |  | YES | reject |
| IAB TNL Address Response | O |  | 9.2.2.x6 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor-CU. The value is FFS. |

-------------------------------------------Next change-------------------------------------------

#### 9.2.1.13	UE Context Information – Retrieve UE Context Response

This IE contains the UE context information within the RETRIEVE UE CONTEXT RESPONSE message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the old NG-C connection. | – |  |
| Signalling TNL Association Address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| AS Security Information | M |  | 9.2.3.50 |  | – |  |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| PDU Session Resources To Be Setup List | M |  | 9.2.1.1 |  | – |  |
| RRC Context | M |  | OCTET STRING | Includes the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331[10] if the old and new serving NG-RAN nodes are gNBs. Includes either the HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [14] or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the old and new serving NG-RAN nodes are ng-eNBs. | – |  |
| Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| 5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | Ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | Applicable to IAB only. | YES | ignore |

-------------------------------------------Next change-------------------------------------------

### 9.2.2	NG-RAN Node and Cell Configuration related IE definitions

>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<

#### 9.2.2.x0 Traffic Index

This IE is used to identify the traffic offloaded to the topology of non-F1-terminating IAB-donor-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Traffic Index | M | FFS |  |  |

#### 9.2.2.x1 Traffic Profile

This IE is used to indication the Traffic QoS parameters for F1-U traffic or non-UP traffic type.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Traffic type | M |  |  |  |
| >UP Traffic |  |  |  |  |
| >>QoS Parameters | M |  | 9.2.3.5 |  |
| >Non-UP UL Boundary Node Traffic |  |  |  |  |
| >>Non-UP Traffic Type | M |  | 9.2.2.x14 |  |
| >Non-UP DL traffic |  |  |  |  |
| >>Control Plane Traffic Type | M |  | INTEGER (1..3, ...) | Control plane traffic types with different priorities are identified by the different codepoints in this IE, where 1 has the highest priority. |

#### 9.2.2.x2	 F1-Terminating Topology BH Information

This IE provides the BH information of the traffic used in F1-terminating donor’s topology.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BH information Request list |  | 1 |  |  |
| >BH Information Request item IEs |  | 1..<maxnoofBHInfo> |  |  |
| >>BH Info Index | M |  | 9.2.2.x15 |  |
| >>DL TNL Address | M |  | 9.2.2.x12 |  |
| >>CHOICE Traffic Direction |  |  |  |  |
| >>>DL |  |  |  |  |
| >>>>Egress BAP Routing ID | M |  | 9.2.2.x7 |  |
| >>>> Egress BH RLC CH ID | M |  | 9.2.2.x8 |  |
| >>>UL |  |  |  |  |
| >>>>Ingress BAP Routing ID( | M |  | 9.2.2.x7 |  |
| >>>>Ingress BH RLC CH ID | M |  | 9.2.2.x8 |  |
| >>>Both |  |  |  |  |
| >>>>BAP Routing ID | M |  | 9.2.2.x7 |  |
| >>>>BH RLC CH ID | M |  | 9.2.2.x8 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBHInfo | Maximum no. of BH information for one traffic offloaded to the non-F1-terminating IAB-donor-CU. The value is FFS. |

#### 9.2.2.x3	 Non-F1-terminating Topology BH Information

This IE provides the BH information of the traffic used in non-F1-terminating donor’s topology

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BH Information Response list |  | 1 |  |  |
| >BH Information Response item IEs |  | 1..<maxnoofBHInfo> |  |  |
| >>BH Info Index | M |  | 9.2.2.x15 |  |
| >>CHOICE Traffic Direction | M |  |  |  |
| >>>DL |  |  |  |  |
| >>>>Ingress BAP routing ID | M |  | 9.2.2.x7 |  |
| >>>>Ingress BH RLC CH | M |  | 9.2.2.x8 |  |
| >>>>Prior-hop BAP address | M |  | 9.2.2.x9 |  |
| >>>> QoS mapping information | M |  | 9.2.2.x11 |  |
| >>>UL |  |  |  |  |
| >>>>Egress BAP Routing ID | M |  | 9.2.2.x7 |  |
| >>>>Egress BH RLC CH ID | M |  | 9.2.2.x8 |  |
| >>>> Next-hop BAP address | M |  | 9.2.2.x9 |  |
| >>>Both |  |  |  |  |
| >>>>Ingress BAP Routing ID | M |  | 9.2.2.x7 |  |
| >>>>Egress BAP Routing ID | M |  | 9.2.2.x7 |  |
| >>>>non-F1-Terminating Topology BH RLC CH ID | M |  | 9.2.2.x8 | This IE indicates the egress BH RLC CH for UL direction, and ingress BH RLC CH for DL direction when both DL and UL traffic share the same BH RLC CH in non-F1-terminating IAB-donor-CU’s topology |
| >>>>non-F1-Terminating Topology BAP Address | M |  | 9.2.2.x9 | This IE indicates the next-hop BAP address for UL direction, and Prior-hop BAP address for DL direction when the routing path of both DL and UL traffic share the same parent node in non-F1-terminating IAB-donor-CU’s topology |

| Range bound | Explanation |
| --- | --- |
| maxnoofBHInfo | Maximum no. of BH information corresponding to one Traffic Index assigned to traffic offloaded to the non-F1-terminating IAB-donor-CU. The value is FFS. |

#### 9.2.2.x4	 Traffic To Be Released Information

This IE is used to indicate the request to release of the offloaded traffic.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Release Type | M |  |  |  |
| >Full Release |  |  |  |  |
| >>All Traffic Indication | M |  | ENUMERATED(true, ..) |  |
| >Partial Release |  |  |  |  |
| >>Traffic To Be Released List |  | 1 |  |  |
| >>>Traffic To Be Released Item IE |  | 1 .. <maxnoofTrafficIndexEntries> |  |  |
| >>>>Traffic Index | M |  | 9.2.2.x0 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor-CU. The value is FFS. |

#### 9.2.2.x5	 IAB TNL Address Request

This IE indicates the request of IP address assignment.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB IPv4 Addresses Requested | O |  | IAB TNL Addresses Requested 9.2.2.x13 |  |
| CHOICE IAB IPv6 Request Type | O |  |  |  |
| >IPv6 Address |  |  |  |  |
| >>IAB IPv6 Addresses Requested | M |  | IAB TNL Addresses Requested 9.2.2.x13 |  |
| >IPv6 Prefix |  |  |  |  |
| >>IAB IPv6 Address Prefixes Requested | M |  | IAB TNL Addresses Requested 9.2.2.x13 |  |
| IAB TNL Address To Remove List |  | 0..1 |  |  |
| >IAB TNL Address To Remove Item |  | 1..<maxnoofTLAsIAB> |  |  |
| >>IAB TNL Address | M |  | 9.2.2.x12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum no. of individual IPv4/IPv6 addresses or IPv6 address prefixes that can be allocated in one procedure execution. The value is 1024. |

#### 9.2.2.x6	 IAB TNL Address Response

This IE indicates the assigned IP address.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB Allocated TNL Address List |  | 1 |  |  |
| >IAB Allocated TNL Address Item |  | 1..<maxnoofTLAsIAB> |  |  |
| >>IAB TNL Address | M |  | 9.2.2.x12 |  |
| >>IAB TNL Address Usage | O |  | ENUMERATED (F1-C, F1-U, Non-F1, All, …) | The usage of the allocated IPv4 or IPv6 address or IPv6 address prefix. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum no. of IPv6 addresses or IPv6 address prefixes and/or individual IPv4 addresses that can be allocated in one procedure execution. The value is 1024. |

#### 9.2.2.x7	 BAP Routing ID

This IE indicates the BAP Routing ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BAP Address | M |  | 9.2.3.x9 |  |
| Path ID | M |  | BAP Path ID 9.2.3.x10 |  |

#### 9.2.2.x8	 BH RLC Channel ID

This IE uniquely identifies a BH RLC channel in the link between IAB-MT of the IAB-node and IAB-DU of the parent IAB-node or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BH RLC CH ID | M |  | BIT STRING (SIZE(16)) |  |

#### 9.2.2.x9	 BAP Address

This IE indicates the BAP address of an IAB-node or of an IAB-donor-DU, and it is part of the BAP Routing ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BAP Address | M |  | BIT STRING (SIZE(10)) | Corresponds to the bap-Address-r16, defined in subclause 6.2.2 or subclause 6.3.2 of TS 38.331 [10], or the iab-donor-DU-BAP-address-r16 defined in subclause 6.2.2 of TS 38.331[10]. |

#### 9.2.2.x10	BAP Path ID

This IE indicates the BAP path ID, which is part of the BAP Routing ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BAP Path ID | M |  | BIT STRING (SIZE(10)) | Corresponds to the Bap-Pathid-r16 defined in subclause 6.3.2 of TS 38.331 [10]. |

#### 9.2.2.x11	QoS mapping information

This IE indicates the DSCP and/or IPv6 Flow Label field(s) of IP packet which is sent through the GTP-U tunnel of a requested DRB. This IE is only used for IAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DSCP | O |  | BIT STRING (SIZE(6)) |  |
| Flow Label | O |  | BIT STRING (SIZE(20)) |  |

#### 9.2.2.x12	IAB TNL Address

The IAB TNL Address IE indicates an IPv4 or IPv6 address or an IPv6 address prefix assigned to an IAB-node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE IAB TNL Address | M |  |  |  |
| >IPv4 |  |  |  |  |
| >>IPv4 Address | M |  | BIT STRING (SIZE(32)) | The IPv4 address allocated to an IAB-node. |
| >IPv6 |  |  |  |  |
| >>IPv6 Address | M |  | BIT STRING (SIZE(128)) | The IPv6 address allocated to an IAB-node. |
| >IPv6prefix |  |  |  |  |
| >>IPv6 Prefix | M |  | BIT STRING (SIZE(64)) | The IPv6 address prefix allocated to an IAB-node. |

#### 9.2.2.x13	IAB TNL Addresses Requested

The IAB TNL Addresses Requested IE indicates the number of IPv4 or IPv6 addresses or IPv6 address prefixes requested for the indicated usage.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TNL Addresses or Prefixes Requested - All Traffic | O |  | INTEGER (1..256) | The number of TNL addresses/ IPv6 prefixes requested for all traffic. |
| TNL Addresses or Prefixes Requested - F1-C traffic | O |  | INTEGER (1..256) | The number of TNL addresses/IPv6 prefixes requested for F1-C traffic. |
| TNL Addresses or Prefixes Requested - F1-U traffic | O |  | INTEGER (1..256) | The number of TNL addresses/ IPv6 prefixes requested for F1-U traffic. |
| TNL Addresses or Prefixes Requested - Non-F1 traffic | O |  | INTEGER (1..256) | The number of TNL addresses/ IPv6 prefixes requested for non-F1 traffic. |

#### 9.2.2.x14	Non-UP Traffic Type

This IE indicates the type of non-UP traffic.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Non-UP Traffic Type | M |  | ENUMERATED(UE-associated F1AP, non-UE-associated F1AP, non-F1, BAP control PDU, ...) |  |

#### 9.2.2.x15	BH Info Index

This IE indicates the index of BH information entry.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BH Info Index | M |  | BIT STRING (SIZE(4)) |  |

#### 9.2.2.x16	Traffic Released Information

This IE is used to indicate the status of the traffic for which the release is requested.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Release TSype | M |  |  |  |
| >Full Release |  |  |  |  |
| >>All Traffic Released | M |  | ENUMERATED(true, ..) |  |
| >Partial Release |  |  |  |  |
| >>Traffic To Be Released List |  | 1 |  |  |
| >>>Traffic To Be Released Item IE |  | 1 .. <maxnoofTrafficIndexEntries> |  |  |
| >>>>Traffic Index | M |  | 9.2.2.x0 |  |
| >>Traffic Failed To Be Released List |  | 1 |  | Only the F1-terminating donor-CU can reject the release request. |
| >>>Traffic Failed To Be Released Item IE |  | 1 .. <maxnoofTrafficIndexEntries> |  |  |
| >>>>Traffic Index | M |  | 9.2.2.x0 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor-CU. The value is FFS. |

-------------------------------------------End of changes-------------------------------------------