3GPP TSG-RAN WG3 #115-e	R3-222419

Online, 21.02 – 3.03 2022

Agenda Item:	31.2.5

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on MLB enhancements 
(CB #SONMDT3)

Document for:	Approval

# Introduction

CB: # SONMDT3_LoadBalance

- Whether to introduce per-slice MSC?

- Check the details on per-SSB MSC

- Remove the Editor’s note, capture agreements and provide the TPs if agreeable

(Nok - moderator)

Summary of offline disc R3-222419

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion (1st round)

## Per-slice mobility setting change

Two companies propose to discuss further and enable per-slice MSC ([1],[2]). Other two comment that it shall not be considered ([6],[7]). This has not changed much since the last meeting. However, in [2], it is suggested that operators shall provide their view.

Question 1: A question to operators mainly: considering the limitations discussed in [2], is the per-slice MSC possibly valuable?

| Company | Answer (Yes/No) | If the answer is negative, please, explain why not. |
| --- | --- | --- |

## Per-beam mobility setting change

The solution has been agreed at the last meeting, but details remain still undecided. The first issue is the parameter to negotiate. In [2],[4] and [7], it is propose to change the current definition in the BL CR to the HO Trigger.

Question 2: Considering support, can RAN3 agree to use the HO Trigger for the per-beam MSC, instead of elected parameters currently listed in the BL CR?

| Company | Answer (Yes/No) | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | Yes | There may be an issue with the own HO Trigger change – please, see Q3 below. |
| Ericsson | Yes |  |

In [4], there is a possible problem mentioned: in disaggregated CU-DU architecture, the CU is not aware of serving beam for own UEs. It can’t therefore apply a modified HO triggering to own UEs in the overloaded beam. To address this issue, the serving beam information is proposed in [5] to be added to F1. Alternatively, a beam may be switched off completely.

Question 3: Do you acknowledge the issue mentioned in [4]?

| Company | Answer (Yes/No) | If the answer is ‘yes’, do you agree for the solution proposed in [5], or do you prefer switching off the overloaded beam? If the answer is ‘no’, how the overloaded node can select UEs to be applied beam-specific HO trigger? |
| --- | --- | --- |
| Nokia | Yes | We propose both, the information on serving beam or the beam switching off. The latter is more “total” solution, so the information on serving beam, as proposed in [5], may be better. |
| Ericsson | No | It is still unclear why per-SSB MSC needs to be connected to individual UEs. For beam-level MSC we should follow the same principle adopted for cell-specific MSC, that is the proposed SSB offset should not be associated to any specific UE but the changes negotiated among nodes should relate to RAN information (in this case SSB offset). Therefore, the negotiation of per SSB offsets should be disconnected from specific UEs served by the CU/DU. Our understanding is that the CU has enough knowledge and therefore has all the information it needs |

Also in [4], it is further proposed to enable a flag indicating that a beam is to be switched off. This can help to handle CHO UEs (but in [6], it is proposed not to consider CHO), but also to handle beam overload, if it is not known which UEs use the overloaded beam.

Question 4: Do you agree to enable a flag or value indicating that a beam is proposed to be switched off to address the issues listed above?

| Company | Answer (Yes/No) | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | Yes | It does not need to be used, while it offers bigger robustness and helps address CHO users. |
| Ericsson | No | Beam deactivation is not a way to balance load, but to save energy when a beam is not loaded, or to change beam shape when necessary for CCO. We do not see the link to use MOBILITY CHANGE REQEST and MOBILITY CHANGE ACKNOLEDGE to negotiate beam deactivation. This is part of CCO at best. |

## Completion of the reporting of the load in cells that may be aggregated with the reporting cell

At RAN3 #114-bis, a solution for reporting load (CAC) in cells that may be aggregated with the reporting cell was agreed. Some details were still marked as “editor’s note”. At this meeting, there are proposals for removing it:

- In [3] and [6], the note is removed. In [3], ale some other edits are made.

- In [7], it is also proposed to enable indicating a minimum level of CAC for a cell to be included.

Question 5: Do you agree for the above changes that will enable completing the solution?

| Company | Suported solution | Comments, if any is not acceptable |
| --- | --- | --- |
| Nokia | (1) | Regarding (2), the thresholds do not seem cell-specific. Therefore, it seems more like something that an operator may set globally in its network – signalling is not needed. |
| Ericsson | (1) partially, (2) | (1) In order to limit the signalling, the number of reported cells which can be aggregated should be limited. One possibility would be to lower the maximum number of items, which is actually too high in the TP (2) This information will help the reporting node to select the PSCells/SCells which fulfil the load requirements from the requesting node. This is not static (e.g. depends on the requesting node load and overload requirements) and therefore cannot be configured by OAM |

## Other proposals

There are some further proposals:

- In [7], it is proposed to enable partial stop of resource reporting in NR.

- In [8] and [9], it is proposed to add the per-MIMO PRB reporting to X2 and F1.

Question 6: Do you agree for the above proposals?

| Company | Suported solution | Comments, if any is not acceptable |
| --- | --- | --- |
| Nokia | (2), possibly (1) | We understand that (2) is just completion of the agreement made at RAN3 #114-bis. Regarding (1), it is all right, as a copy of the LTE solution – but the TP in (1) does not seem to include the proposal – so how is it supposed to look like? |
| Ericsson | (1) and (2) partially | (1) Solution is to add Cell Reporting Indicator IE into RESOURCE STATUS UPDATE message, at cell level. Please check the revised TP in R3-222508 in the inbox. (2) Per-S-NSSAI PRB usage is optional in the Radio Resource Status IE. Shouldn’t be the case for all rel-17 PRB usage enhancement (e.g. PRB usage for MIMO)? Also, in order to clarify that the PRB usage for MIMO is per cell, we propose to change the name to “Cell DL GBR PRB usage” keeping the definition in the semantics description only |

Since RAN3 is closing the discussion on Rel.17 MLB enhancement, is there anything else that requires attention at this meeting?

Question 7: Please, provide any comments, if there are any issues that require attention before the work on Rel.17 MLB enhancements is closed?

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-221777, Per-Slice Mobility Settings Change (NEC)

- R3-221831, (TP for SON BLCR for 38.423) Load Balancing Enhancements (Huawei)

- R3-221913, (TP for 36.423) Support of PSCell MLB (CATT)

- R3-221935, [TP to SON BL CR for TS 38.423]  Per-beam mobility setting change (Nokia, Nokia Shanghai Bell)

- R3-221936, [TP to SON BL CR for TS 38.473]  Per-beam mobility setting change (Nokia, Nokia Shanghai Bell)

- R3-222066, Further Discussion on Load Balancing Enhancements in NR (ZTE, China Telecom)

- R3-222068, (TP for SON BL CR for TS 38.423, TS 36.423) MLB enhancements (Ericsson)

- R3-222258, (TP for SON BLCR to 38.473) PRB usage for MIMO (CMCC)

- R3-222259, (TP for SON BLCR to 36.423) PRB usage for MIMO (CMCC)