3GPP TSG-RAN WG3 #115-e	R3-222420

21th Feb – 3rd Mar, 2022

Agenda Item:	10.2.1.6

Source:	Samsung (moderator)

Title:	Summary of Offline Discussion on MRO for SN Change Failure

Document for:	Approval

# Introduction

CB: # SONMDT4_SNChangeFailure

- d) Suitable PSCell CGI, e) Mobility Information in the XnAP message from MN to the source SN?

- RAN3 to review if and how to avoid duplication with the Rel.15 S-RLF signalling. Enhance R15 signaling? Define two new class-2 procedures?

- IEs in the message from the MN to the last serving SN?

- IEs from the last serving SN to the MN?

- Let SN keep the UE context for some time by introducing MN implementation? e.g. MN sends the UE CONTEXT RELEASE message to the source SN after T310 expiry

- Deal with the WA about the information in the S-NODE CHANGE REQUIRED message? Turn into agreement?

- How to capture the MN behavior in stage 2?

- Focus on key issues, capture agreements and provide TPs if agreeable

(Samsung - moderator)

Summary of offline disc R3-222420

# For the Chairman’s Notes

Proposal 1: Include e) Mobility Information as optional IE in the new message from the MN to the source SN.

Proposal 2: Remove the following editor’s note in XnAP BLCR.

Editor’s note: UE AP IDs presence and non UE-associated vs UE-associated signaling are FFS.

Proposal 3: Include 1) SCGFailureInformation and 2) UE AP IDs in the message from the MN to the last serving SN.

Proposal 4: Include i) UE AP IDs in the message from the last serving SN MN to the MN.

Proposal 5: Include Mobility Information and Source PSCell CGI as optional IE in the S-NODE Change Required message.

Proposal 6: Capture the MN behavior in stage 2.

# Discussion (Second round)

In the first round, we mainly focused on the IEs to be transmitted from MN to the source SN, from the MN to the last serving SN and from the last serving SN to the MN. We have no explicit discussion on how many class 2 procedures need to be defined in stage 3 for the information transmission from MN to the source SN, from MN to the last serving SN and from the last serving SN to the MN.

In order to decide how to define the class 2 procedure in stage 3, let’s reiterate the agreed IEs for the information transfer over each interface.

The IEs agreed in the message from MN to the source SN:

Source PSCell CGI, Failed PSCell CGI, SCG Failure Report Container, UE AP IDs, Mobility Information

The agreed IEs in the message from the MN to the last serving SN:

UE AP IDs, SCGFailureInformation

The agreed IEs in the message from the last serving SN to the MN:

UE AP IDs

Considering the agreed IEs, the moderator observed that similar IEs are included in the message from MN to the last serving SN and from the last serving SN to the MN. It seems better to define two class 2 procedures:

One class 2 procedure is from MN to the source SN

The second class 2 procedure is from MN to the last serving SN or from the last serving SN to the MN. In the second class 2 procedure, the UE AP IDs could be defined as Mandatory because the MN and the last serving SN still have the UE context. SCGFailureInformation could be defined as optional. It is needed from MN to the last serving SN.

Question 1: Do you agree to define two class 2 procedures, one is from MN to the source SN, the second one is from MN to the last serving SN or from the last serving SN to the MN?

| Company | Comment |
| --- | --- |

Question 2: If the answer to Question 1 is no, what’s your proposal to define the class 2 procedures in stage 3? Pls fill it in the table below.

| Company | Comment |
| --- | --- |

# Discussion (First round)

## The IEs in the message from the MN to the source SN

Based on the conclusion at last RAN3#114bis-e meeting, two IEs are left on the table as open as follow:

Continue to discuss d) Suitable PSCell CGI, e) Mobility Information in the XnAP message from MN to the source SN.

[4][5][7][9] proposed to include d) Suitable PSCell CGI in the message from the MN to the source SN. The reasons include:

- When the MN decides the node which bring the problem, the MN needs to decide a suitable PScell. Since the MN has such information, it is beneficial to send the information to the source SN [7]

- If the MN decides to continue the DC for the UE and it can decode the measurement results, it is highly anticipated that the MN selects a new target PSCell based on the received measurement results. This can quickly recover the SCG configuration and ensure the UE experience compared to the SN decision solution.[4]

- e) Suitable PSCell CGI and j)  Indicator for Whether to add SN shall be used together to make final MRO analysis. We do not support source SN to select the next suitable PSCell as UE context may have been removed [5]

- In the ‘change to wrong PSCell’ case, only MN can provide the Suitable PSCell after SCG failure [9].

[1][7][9] proposed to include e) Mobility Information in the XnAP message from MN to the source SN. The reasons include:

- If source SN has already released the UE context, the Mobility Information IE is needed to enable SN to perform optimization, which is similar as legacy MRO solution. It is possible to have new requirement to let the Source SN save the UE context. But the Rel-15 and Rel-16 gNB may have released the UE context based on the specification in TS37.340. This new requirement will bring non-backward compatible change. [1][7][9]

[5] has no strong view on the Mobility Information.

[5] proposed again to include j)  Indicator for Whether to add SN in the message from MN to the source SN. The reason is that Suitable PSCell CGI and j)  Indicator for Whether to add SN shall be used together to make final MRO analysis by the source SN.

Q1: Could you accept to include  d) Suitable PSCell CGI, e) Mobility Information and j)  Indicator for Whether to add SN in the message from MN to the source SN?

| Company | Option | Comment |
| --- | --- | --- |
| Samsung | Fine for d) and e). | For d), to identify the node which bring the problem, the MN needs to know the suitable PSCell. So it’s beneficial to transmit the information to the source SN for information. Then the source SN has freedom whether to use it or not. For e), in case of SCG failure just after successful SN1->SN2 change, it’s possible that SN1 has released the UE context based on TS37.340 indicated by several companies. Similar as HO case, mobility information is useful. So e) is needed. There is proposal to have new requirement to let source SN keep the UE context for some time. But Rel-15 and Rel-16 gNB may have released the UE context in implementation. This new requirement bring NBC functional change. There is new proposal in this meeting to let MN delay sending UE Context Release message to the source SN. This bring NBC function change for Rel-15 and Rel-16 MN. For j), we need to understand more why suitable cell is not enough. If suitable cell is included, it means a SN can be added. |
| Nokia | (e), if provided to the MN | Even if (d) has to be tentatively identified in the MN, signalling it to the source SN is not needed – the SN has to evaluate the situation on its own anyway, based on the information from the UE, so providing this offers no gain at all. Providing (e) may help, indeed, but it has to be provided from the source. So, we must first agree on the information from the source in the SN CHANGE REQUIRED. |
| ZTE | d), e) | d) is necessary. We think MN has a better view about the suitable PSCell. Even if SN can evaluate the UE measurements, sometimes the UE itself cannot know which cell is the suitable cell. e) is also needed. We have already had enough discussion about the fact that source SN might have released UE context when it receives the SCG failure report. |
| Lenovo | e) | e) is beneficial for the case that the source SN has released the UE context. |
| CATT | OK for d), j) | For d), we propose MN to select the next suitable PSCell as MN has overall information. so d) shall be included. For j), if MN decide not to add SN after SCG failure, only d) is not enough and an indicator j) is needed. For e), we do not have strong view, but considering it is not a long time between SN change and SCG failure, it is also OK to let source SN to keep UE context. For NBC issue, MRO for MR-DC is not supported by Rel-15 and Rel-16 MN and shall be supported only in Rel-17, so we believe it is not an issue. |
| Huawei | d) | If the MN decides to continue DC for the UE, it is preferred that the MN can decide the suitable PSCell to fast recover the DC. The MN can send the suitable PSCell CGI to the node responsible for the SCG failure. |
| China Telecom | e) | The Mobility Information IE can help SN to perform root cause analysis in case that UE context has already been removed. For d), we think it is source SN’s responsibility to perform further analysis, and adjust/optimize SCG configurations itself, there is no need for MN to analysis the suitable PScell information. |
| Qualcomm | d) – No e) – Depends whether we include XnAP ID f) - No | d) – No. If there is a SCG failure, it is the responsibility of MN to add a new PSCell based on PSCell measurements reported by UE, we don’t understand how sending the Suitable PSCell to the source PSCell would even help. Also, in the case where only SN has configured PSCell measurements, MN can’t even understand the PSCell measurements reported by UE; then how can it even indicate a Suitable PSCell to source SN? e) – If we agree on XnAP IDs (next question), we probably don’t need this j) – It is up to MN whether it wants to add a new PSCell or not after SCG Failure. How would sending this indicator help; Source SN can’t initiate PSCell addition anyway |
| Ericsson | No | For d) SN has to perform further analysis and has all the information needed to do so e) is not needed because this message is sent right after the failure |

Moderator Summary

7 companies are fine for e)

4 companies are fine for d)

1 company is fine for j).

Majority companies can accept e). The moderator propose to include e) as optional IE in the new message from the MN to the source SN.

For d) and j), seems it is very difficult to have an agreement at this meeting.

Proposal 1: Include e) Mobility Information as optional IE in the new message from the MN to the source SN.

h)  S-NG-RAN node UE X2AP ID and i)  M-NG-RAN node UE XnAP ID were defined as Optional in the BLCR. An editor’s node was captured as below:

Editor’s note: UE AP IDs presence and non UE-associated vs UE-associated signaling are FFS.

In [7], it was proposed to keep the UE AP IDs as optional and the class 2 procedure is non-UE associated. The reason includes:

- For the SCG failure just after successful SN change, the MN may has released the UE context related with the source SN. In this case, the MN may not have the UE AP IDs.

- It’s future proof to define the SON related procedure as non-UE associated.

In [6], it was proposed to define the UE AP IDs as Mandatory and the class 2 procedure is UE associated procedure. To assure the MN and the source SN always has the UE context, it is proposed that the MN sends the UE CONTEXT RELEASE message to the source SN after T310 expiry.

From moderator point of view, at the last meeting discussion, some companies think the UE AP IDs are not needed. Some companies think only one UE AP ID is needed. Some companies think the UE AP IDs are needed. In order to move forwarding, the compromise is to include the two AP IDs as optional. It’s better to keep this compromised way forward.

Q2: Are you ok to keep UE AP IDs are optional?

| Company | Option | Comment |
| --- | --- | --- |
| Samsung | OK | For the SCG failure just after successful SN change, the MN may has released the UE context related with the source SN. In this case, the MN may not have the UE AP IDs The new proposal to let MN send UE Context Release message to the source SN after T310 expiry is a new requirement for the MN. This is a NBC functional change for Rel-15 and Rel-16 MN. |
| Nokia | OK |  |
| ZTE | OK |  |
| Lenovo | OK |  |
| CATT | ok | If we decide to let source SN to keep UE context tentatively, it is needed. |
| Huawei | No | We are still not convinced about this. Any implementation supporting this feature can store the relevant information and use it for MRO if he wants to. This is very different from mandating the storage of the full context. In LTE we added the mobility info because the RLF report may be delayed 48 hours. But in this case we will receive the failure message almost immediately. |
| China Telecom | OK |  |
| Qualcomm | OK, but.. | The implementation option i.e., either MN delaying sending the UE CONTEXT RELEASE or source SN to store the UE context till the T_SN_Change_Failure Timer expires – is this really NBC? In our view, can’t this be left to implementation to ensure this is done (and not mention in the spec) for this feature? Anyway, the inclusion of UE AP IDs are only needed if the source SN can’t store the UE context till the reception of SCGFailureInformation and we are OK to include this if implementation option is not preferred. Wouldn’t this need be to be mandatory in such a case? |
| Ericsson | No | Agree with Huawei. The requirement cannot be compared to RLF report, as the failure message will be sent right after the MN receives SCGFailureInformation |

Moderator Summary

Majority companies are fine to keep UE AP IDs are optional. Considering this is the compromise from last meeting, the moderator propose keep UE AP IDs as optional.

Proposal 2: Remove the following editor’s note in XnAP BLCR.

Editor’s note: UE AP IDs presence and non UE-associated vs UE-associated signaling are FFS.

## The IEs in the message from the MN to the last serving SN

The following IEs were proposed to be included in the message from the MN to the last serving SN:

- SCGFailureInformation [9]

- UE AP IDs [1][5][7][9]

- Suitable PSCell CGI [4][5][7]

- Mobility Information [1]

- Indicator for Whether to add SN, UE history information [5]

- Intra-SN change query [2][9]

Majority companies didn’t discuss whether SCGFailureInformation needs to be included in the message from the MN to the last serving SN in the submitted contributions. The moderators guess that the assumption is that SCGFailureInformation is needed. They just focus on other IEs.

If a company has different interpretation as the moderator, pls indicate in the table below.

| Company | Comment |
| --- | --- |

Q3: Among 1) – 6), which IEs from the MN to the last serving SN are acceptable for your company?

| Company | Comment |
| --- | --- |
| Samsung | We are fine for 1) 2) 3). 3) Suitable PSCell CGI can be included if the MN has such information. The last SN takes this for reference e.g. decide whether the last serving SN should have triggered a PSCell change to this suitable PScell. |
| Nokia | Only (1), (2) and possibly (6) The last serving SN must have the UE context when it receives the information from the MN about the S-RLF, so once the UE is identified with (2), everything else is known at the last serving SN! (3) can be read from the SCGFailureInformation. (6) is needed only if the Rel.15 signalling is reused; otherwise, the new class-2 message may be interpreted as the query implicitly. |
| ZTE | - 2) 6) We think 6) is needed not only for Rel.15 signalling. We understand that at current stage, a new class-2 message itself would represent a query about whether intra-SN change happened. However, it is possible that the new message can be reused for other features in future release, although it is only used for query the intra-SN change. So, in case that there might be other uses in the future, we think an explicit query in the new class-2 message is necessary. |
| Lenovo | 1) and 2), 2) can be used to relate SCG Failure Information with the UE context. Agree with Nokia that 6) is needed only when R15 signalling is reused. |
| CATT | OK for 1) 2) 3) 5) For 2), there may be UE context in last serving SN, so UE AP ID is needed. For 3) and 5), we propose MN to select the next suitable PSCell as MN has overall information. So 3) shall be included. If MN decides not to add SN after SCG failure, only 3) is not enough and an indicator 5) is needed. |
| Huawei | - is mandatory. - is mandatory (3) can be provided if the MN configures DC to the UE. If the last serving SN is the node responsible for the SCG failure, it can take this information as the potential optimization. |
| China Telecom | We are fine with (1) and (2): (1) is mandatory, (2) can help SN identify the UE. |
| Qualcomm | OK with (1) and (2). Regarding (6), it might be good to have an explicit query as ZTE mentioned for future proofing. |
| Ericsson | 1) 2) as mandatory 6) is needed even in case a new message is used, because this new message could be used between MN&S-SN or MN&T-SN |

Moderator Summary

All companies are fine for 1) and 2).

3 companies are fine for 3)

1 company is fine for 5)

2 company are fine for 6). 2 other companies think 6) is need in case Rel-15 class 1 procedure is used.

Proposal 3: Include 1)SCGFailureInformation and 2) UE AP IDs in the message from the MN to the last serving SN.

In [2], it is proposed to use the existing SN Modification procedure instead of two class 2 procedures between the MN and the last serving SN to save some signaling exchange over Xn.

In [5][7][9], class 2 procedure is preferred. The reason in [9] is that the MN may decide to release the SN when SCGFailureInformation is received. In [7], the reason is that: if reusing this class 2 procedure, we don’t need to enhanced SN Modification procedure for SON purpose e.g. add flag in request and response message.

Q4: Between MN and the last serving SN, do you prefer to use SN Modification procedure or class 2 procedure, and the reasons?

| Company | Comment |
| --- | --- |
| Samsung | Both solutions work. We slightly prefer new class 2 procedure. In this case, RAN3 doesn’t need to consider how to enhance SN Modification procedure e.g. add a flag in the request and response message (maybe suitable PSCell ID as well). When the SN receives SN Modification Request message with S-RLF, the legacy behavior is performed. When the SN receives new class 2 procedure, SCG MRO function is performed. There is no problem. It’s possible to add more IEs for further SON enhancement. |
| Nokia | Indeed, both options work. However, reusing the Rel.15 procedure reduces the overall signalling significantly (comparison of the scenarios in [2]), while the cost is one extra flag IE in the MOD REQ and one in the MOD REQ ACK. Also, if the existing procedure is reused, there is no problem about the order of signalling: if the Rel.15 procedure is sent first, the last serving SN may release the UE context before Rel.17 procedure arrives. Therefore, if RAN3 eventually decides that a new procedure is to be used for Rel.17 signalling, the order of signalling with the Rel.15 procedures (MN-initiated Modification or MN-initiated Release) for the scenarios where both are to be used has to be specified. |
| ZTE | We prefer new class-2 message. As mentioned in our paper, MN would not always send the MOD REQ message to SN, sometimes it may decide to release SN and send Release message instead. As explained by Samsung, we can simply separate the Rel.15 signalling and new class-2 message — Rel.15 message for legacy behavior, and new class-2 message for MRO. To Nokia: We are not clear about why the order of the two procedures should be specified, they are different in logic and the order of them might depend on actual cases. |
| Lenovo | Agree with ZTE. We slightly prefer new class 2 procedure even both solutions can work. |
| CATT | We do not have a strong view on this issue. We think either introducing a new class 2 message or reusing Rel.15 S-RLF signalling can work well. but considering Rel.15 S-RLF signalling is a class 1 procedures, we slightly prefer introducing a new class 2 message. |
| Huawei | Both can work, but we slightly prefer new class 2 procedure. For MN supporting the SCG MRO, it can know which node is responsible for the SCG failure based on the Rel17 SCGFailureInformation message. If it is the source node, the MN cannot reuse Rel. 15 procedures but trigger the new class 2 procedure. Consequently, we prefer to consider a common procedure for both MN&S-SN and MN&T-SN(last serving SN). |
| China Telecom | We slightly prefer to introduce the new class-2 message, it is beneficial for further SON enhancement. |
| Qualcomm | Seeking clarification on ZTE’s comment: The following is from TS 37.340: The UE includes in the SCGFailureInformation message the measurement results available according to current measurement configuration of both the MN and the SN. The MN handles the SCGFailureInformation message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN. - Doesn’t this mean MN would always send MOD REQ to SN? Anyway, we also don’t have a strong view – perhaps a new class-2 message could be useful for future releases too, say when we deal with MRO for CPAC. But OK to reuse SN MOD REQ as well if we just have to include Source PSCell CGI and Failed PSCell CGI in addition to SCGFailureInformation. |
| Ericsson | New class-2 is preferred. Agree with Huawei that the same message could then be used between MN&S-SN or MN&T-SN. |

Moderator Summary

Majority companies still prefer class 2 procedure. This is the existing agreement. So no new agreement on this point.

## The IEs in the message from the last serving SN to the MN

The following IEs were proposed to be included in the message from the last serving SN to the SN:

- XnAP IDs [1][5][7][9]

- Failed PSCell CGI,  suitable PSCell CGI [5][7]

- Target PSCell CGI, Indicator for Whether to add SN [5]

Q5: Among i) – iii), which one do you think is needed from the last serving SN to the MN?

| Company | Option | Comment |
| --- | --- | --- |
| Samsung | Fine for i and ii | The MN may not know Failed PScell ID. It’s better to transmit the information from the last serving SN to the MN. For suitable PSCell CGI, the last serving SN can forward it back to the MN if the last serving SN has received such information. |
| Nokia | (i) only | The (ii) and (iii) do not seem needed, as the only information that the MN requires is if there was any intra-SN PSCell change in the last serving SN after the last SN change. - If there was no (most likely case), all the MRO analysis shall be done in the source SN, so the last serving does not need to provide anything back to the MN. - If there was PSCell change, the MN stops further processing of the case. |
| ZTE | (i) | We share the view with Nokia. We would prefer only (i) to keep it simple. At current stage, the use of the new class-2 message is to inform SN that there is NO intra-SN change. |
| Lenovo | i | Agree with Nokia. Not clear why the serving SN needs to transmit failed/suitable/target PSCell CGI or the indicator for adding SN to the MN. |
| CATT | OK for (i) (ii) and (iii) | (i) is needed for MN to identify the UE. For Pre-Rel-17 UE, Failed PSCell CGI and Target PSCell CGI are only known by last serving SN, so they shall be provided to MN. |
| Huawei | i | Agree with Nokia |
| China Telecom | i | Agree with Nokia |
| Qualcomm | i | Agree with Nokia. Also, MN should know the source PSCell CGI from its UE context. Also we would need a yes/no reply if (6) intra-SN change query is agreed in Q3 |
| Ericsson | Only (i) | Analysis is performed in the SN, therefore Failed PSCell CGI and Target PSCell CGI are not needed in MN |

Moderator Summary

All companies are fine for i).

2 companies are fine for ii)

1 company is fine for iii)

Proposal 4: Include i) UE AP IDs in the message from the last serving SN MN to the MN.

## The IEs in the message in the SN Modification Required message

At last RAN3#114bis-e meeting, the following working assumption was agreed.

WA: Including the following IEs in in the S-NODE CHANGE REQUIRED message

- Mobility Information

- Source PSCell CGI.

Q3 are you ok to change the working assumption to the agreement?

| Company | Opinion | Comment |
| --- | --- | --- |
| Samsung | Agree | Mobility Information: as explained by several companies, the source SN may have released the UE context when the source SN receives UE Context Release message based on TS37.340. The mandate the source SN keep the UE context when receiving UE Context Release message is a NBC change from function point of view. The source SN needs the mobility information for MRO function. Source PScell CGI, the MN may not know Source PScell CGI so this should be transmitted to the MN. |
| Nokia | Agree |  |
| ZTE | Agree |  |
| Lenovo | Agree |  |
| CATT | Agree |  |
| Huawei | No | - Mobility information: As discussed in Q2 in section 3.1, we can rely on MN implementation to delay the UE Context Release message. In this way, we can ensure the source SN to keep the UE context. - Source PSCell CGI: The MN has UE context and can know the source SN node. This is sufficient for MN to transfer the SCGFailureInformation to the source SN. |
| China Telecom | Agree |  |
| Qualcomm | See comments | Mobility Information – Depends on Q2 Source PSCell CGI – Similar view as HW. When would MN not know the Source PSCell CGI? (MN can still have the UE context even if UE context is released in Source PSCell) |
| Ericsson | No | As discussed in other sections, RAN2 reusing SCGFailureInformation means that the SN will be aware of SCG failure right away and will be able to retrieve these information from the UE Context |

Moderator Summary

Majority companies agree to change this working assumption to agreement. Considering the issue has been discussed for several meetings and this is the last meeting to close this WI, the moderator propose to include the two IEs as optional.

Proposal 5: Include Mobility Information and Source PSCell CGI as optional IE in the S-NODE Change Required message.

## Stage-2 clarification

[4] proposed stage-2 clarification to capture that the MN behavior.

Q8 what’s your view on the proposal?

| Company | Comment |
| --- | --- |
| Samsung | In general, the TP is [4] is fine. The last sentence may need to be updated. |
| ZTE | We are generally fine. Some update is needed. And maybe 38.331 can be added as a reference for SCG failure Information? |
| Lenovo | Fine to capture MN behavior in stage-2. |
| CATT | No strong opinion |
| Huawei | We prefer to capture the TP in [4]. If needed, some wording can be updated. |
| China Telecom | No strong view. |
| Qualcomm | Having a TP is fine; but need updates. The highlighted part in the TP is not consistent with our agreements. MN always sends SCG Failure Information to last serving SN. If there was intra-SN PSCell change, it would transfer to the source SN For analysis of PSCell change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN which is responsible for the PSCell change failures (see the section 13.x in TS 37.340 [21]). Otherwise, the MN transfers the SCG Failure Information to the last serving SN. If needed, the MN transfer the SCG Failure Information to the source SN (see the section 13.x in TS 37.340 [21]) |
| Ericsson | Ok to discuss some wording for stage-2 |

Moderator Summary

All companies can accept to have stage 2 TP to capture the MN behavior. Some companies think some rewording in [4] are needed.

Proposal 6: Capture the MN behavior in stage 2.

Discuss the wording in the second round.

# Conclusion, Recommendations [if needed]

If needed

# References

| [1] R3-221713 | MRO for SN Change Failure (China Telecommunication) |
| --- | --- |
| [2] R3-221765 | [TP to SON BL CRs to 38.423, NR_ENDC_SON_MDT_enh] MRO for PSCell change failure (Nokia, Nokia Shanghai Bell) |
| [3] R3-221766 | [TP to SON BL CRs to 36.423, NR_ENDC_SON_MDT_enh] MRO for PSCell change failure (Nokia, Nokia Shanghai Bell) |
| [4] R3-221832 | (TP for SON BLCR for 38.423, 38.300) MRO for SN Change Failure (Huawei) |
| [5] R3-222010 | (TP on SON for 38.423)Consideration on support of SN change failure in case of MR-DC (CATT) |
| [6] R3-222069 | (TP for SON BL CR for TS 38.300, TS 38.423) MRO for SN change failure (Ericsson) |
| [7] R3-222304 | MRO for SN Change Failure (Samsung) |
| [8] R3-222305 | TP for SON BLCR for TS 38.423: Support of SON for SN change failure (Samsung) |
| [9] R3-222364 | Further consideration on MRO SN change failure (ZTE) |