3GPP TSG-RAN WG3 #115-e						                        R3-222423

21th Feb – 3rd Mar 2022

Online

Agenda Item:	10.2.4

Source:	CMCC (moderator)

Title:	Summary of Discussion on Inter-System Load Balancing

Document for:	Discussion

# Introduction

This document provides the email discussion on the following CB,

CB: # SONMDT7_InterSystemLB

- Check details on Stage 2 and Stage 3 TPs

(CMCC - moderator)

Summary of offline disc R3-222423

This CB is planned to be carried out with two phases,

Phase 1: Till 1100UTC before the Friday online session. Collect comments and achieve consensus if any.

Phase 2: From Friday online session to CB deadline, prepare agreeable TPs according to agreements.

# For the Chairman’s Notes

[To be completed later]

# Discussion

## PRB usage

Last meeting we agreed the following,

Introduce PRB usage (i.e. the ratio of the utilised PRBS to the total number of PRBs) as a load metric and no further discussion on introducing additional parameters related to PRB usage.

WA: PRB usage is reported per cell

However, the stg3 details on PRB usage has not been decided yet.

Regarding the open issue above,

- CMCC [2]:

Proposal 1: Turn the WA into agreement, i.e. PRB usage is reported per cell for inter-system load balancing.

Proposal 2: Reuse the definition of Radio Resource Status as specified in TS 36.423, section 9.2.37 for inter-system load reporting from E-UTRAN to NG-RAN.

Proposal 3: Reuse the PRB usage for MIMO per cell for inter-system load reporting from NG-RAN to E-UTRAN.

Q1: Do you agree the above three proposals? Please provide your comments in the table below if any one of these proposals is unacceptable.

| Company | Comment |
| --- | --- |
| Nokia | P1 and P2 are all right. However, we are not sure if PRB per MIMO shall be reported in inter-RAT scenarios. |
| Samsung | Fine for P1 and P2. We are also not sure about PRB per MIMO. |

## Number of Active NR capable UEs

Whether to introduce Number of Active NR capable UEs has been discussed for several meetings. Regarding the open issue above,

- Ericsson [1]:

Proposal 1: No need to introduce Number of NR capable active UES for inter-system load balancing

It seems the controversy still remains, so we’d like to further check if new arguments could be provided.

Q2: Do you agree the above proposal? Please provide your comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia | Yes, we agree with Ericsson’s proposal. |
| Samsung | We agree the proposal. |

## Stop/Pause indication

According to the open issue in RAN3#114-e,

Issue 2: Stop indicator - is this beneficial to have? Are there any technical questions on the proposed solution?

Regarding the open issue above,

- Ericsson [1]:

Proposal 2: Introduce indications of measurements stop, pause and resume for periodic inter-system resource status reporting.
Proposal 3: Introduce a Cause Value in periodic inter-system resource status reporting to indicate the reason for measurements stop or pause.

The moderator also notices that the similar discussion is also carried out in CB: SONMDT3 at both last and this meeting. And [1] does not provide stg3 details on how to enable Proposal 2 and 3 above.

Q3: Do we need to introduce stop/pause mechanism for periodic reporting? If yes, do we need to introduce Cause Value to indicate the reason for measurement stop/pause? Please provide your comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia | Pause mechanism is not needed and overly complicated as compared to the benefits. Stop may be considered, if there is wide support for it in RAN3. |
| Samsung | We are fine for stop. Pause/resume is not needed. |

## Stg2 detail

[1] proposes in TPs for 36.300 and 38.300 to capture call flows for the initiation and reporting for the load reporting.

Q4: Do you prefer to capture call flows in stg2 descriptions for the load reporting? Please provide your comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia | If we remember all right, in LTE, inter-RAT load reporting was not captured in stage-2, was it? If it was not, we prefer to skip adding it now; if it was, the existing description could be extended to take NR into account. |
| Samsung | Not needed. The same reason as Nokia. Inter-RAT load reporting in LTE is not captured in stage 2. Also in [1], some exchanged parameters are listed. The drawback is that once a new IE is added, the stage 2 need to be updated. The parameters are clear in stage 3. Therefore we think listing those are not needed in stage 2. |

For other issues which are identified but not provided by the moderator, please describe the question and provide comments in the table below.

| Company | Potential question | Comment |
| --- | --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

| [1] | R3-222072 | (TP for SON for TS 38.413, TS 38.300, TS 36.300) Inter-System Load Balancing (Ericsson) |
| --- | --- | --- |
| [2] | R3-222260 | (TP for SON BLCR to 38.413) Leftover issues for inter-system load balancing (CMCC) |
| [3] | R3-222361 | (TP for SON BL CR 36.300) Inter-system Load Balancing (ZTE,China Telecom, China Unicom) |
| [4] | R3-222362 | (TP for SON BL CR 38.300) Inter-system Load Balancing (ZTE,China Telecom, China Unicom) |
| [5] | R3-222363 | (TP for SON BL CR 38.413) Inter-system Load Balancing (ZTE,China Telecom, China Unicom) |