3GPP TSG-RAN WG3 Meeting #115-e	R3-222502

# Online, February 21st – March 3rd 2022

Agenda Item:	13.3.1

Source:	Ericsson

Title:	(TP for IAB BL CR for TS 38.473): Congestion Mitigation in IAB Networks

Document for:	Agreement

# TP for IAB BL CR for TS 38.473

-------------------------------------------Start of changes-------------------------------------------

## 8.10	IAB Procedures

### 8.10.1	BAP Mapping Configuration

#### 8.10.1.1	General

The BAP Mapping Configuration Procedure is initiated by the gNB-CU in order to configure the DL/UL routing information and/or traffic mapping information needed for the gNB-DU. The procedure uses non-UE associated signalling.

NOTE:	Implementation shall ensure the avoidance of potential race conditions, i.e. it shall ensure that conflicting traffic mapping configurations are not concurrently performed using the non-UE-associated BAP Mapping Configuration procedure and the UE-associated UE Context Management procedures.

#### 8.10.1.2	Successful Operation

Figure 8.10.1.2-1: BAP Mapping Configuration procedure: Successful Operation

The gNB-CU initiates the procedure by sending BAP MAPPING CONFIGURATION message to the gNB-DU. The gNB-DU replies to the gNB-CU with BAP MAPPING CONFIGURATION ACKNOWLEDGE.

If BH Routing Information Added List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, store the BH routing information from this IE and use it for DL/UL traffic forwarding. If BH Routing Information Added List IE contains information for an existing BAP Routing ID, the gNB-DU shall, if supported, replace the previously stored routing information for this BAP Routing ID with the corresponding information in the BH Routing Information Added List IE.

If BH Routing Information Removed List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, remove the BH routing information according to such IE.

If the Traffic Mapping Information IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, process the Traffic Mapping Information IE as follows:

-	if the IP to layer2 Traffic Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the IP to layer2 Mapping Info To Add IE, if present, and remove the previously stored mapping information as indicated by the IP to layer2 Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored for the mapping of IP traffic to layer 2, as specified in TS 38.340 [30].

-	if the BAP layer BH RLC channel Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the BAP layer BH RLC channel Mapping Info To Add IE, if present, and remove the previously stored mapping information as indicated by the BAP layer BH RLC channel Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored when forwarding traffic on BAP-layer, as specified in TS 38.340 [30].

If the Buffer Size Threshold IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, use it to trigger local re-routing based on the flow control feedback from child IAB-nodes.

-------------------------------------------Next change-------------------------------------------

### 9.2.9	IAB messages

#### 9.2.9.1	BAP MAPPING CONFIGURATION

This message is sent by the gNB-CU to provide the backhaul routing information and/or traffic mapping information to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| BH Routing Information Added List |  | 0...1 |  |  | YES | ignore |
| >BH Routing Information Added List Item |  | 1.. <maxnoofRoutingEntries> |  |  | EACH | ignore |
| >>BAP Routing ID | M |  | 9.3.1.110 |  | - |  |
| >>Next-Hop BAP Address | M |  | 9.3.1.111 | Indicates the BAP address of the next hop IAB-node or IAB-donor-DU. | - |  |
| BH Routing Information Removed List |  | 0...1 |  |  | YES | ignore |
| >BH Routing Information Removed List Item |  | 1.. <maxnoofRoutingEntries> |  |  | EACH | ignore |
| >>BAP Routing ID | M |  | 9.3.1.110 |  | - |  |
| Traffic Mapping Information | O |  | 9.3.1.95 |  | YES | ignore |
| Buffer Size Threshold | O | FFS – to be aligned with RAN2 decision. | FFS | The configured threshold of available buffer size feedback is used to determine the congestion, for the purpose of local re-routing | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRoutingEntries | Maximum no. of routing entries, the maximum value is 1024. |

-------------------------------------------Next change-------------------------------------------

#### 9.3.1.x	IAB Congestion Indication

This IE contains the IAB downlink congestion indication.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB Congestion Indication List |  | 1 |  |  |
| >IAB Congestion Indication List Item |  | 1.. <maxnoofIABCongInd> |  |  |
| >>Child Node Identifier | M |  | 9.3.1.111 | This IE identifies the child node, the link to which is congested. |
| >>BH RLC CH List |  | 0..1 |  |  |
| >>>BH RLC CH List Item |  | 1.. <maxnoofBHRLCChannels> |  |  |
| >>>>BH RLC CH ID | M |  | 9.3.1.113 | This IE identifies the congested BH RLC channel over the link towards the node identified by the Child Node Identifier IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofIABCongInd | Maximum no. of congestion indications, the maximum value is 1024. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |

-------------------------------------------End of changes-------------------------------------------