3GPP TSG-RAN WG3 Meeting #115-e	R3-222774

Electronic Meeting, February 21th – March 3rd, 2022

Agenda item:	24.2

Source: 	Intel Corporation, LG Electronics, ZTE, CATT, Ericsson, NEC, Samsung

Title: 	(TP for RA-SDT BL CR for TS 38.401)

Document for:	Other

# 1		Introduction

This contribution proposes text proposal for RA-SDT BL CR for TS 38.401 [1] based on 38.401 TP proposed in [2] and online discussions from CB: # SDT2_RACHbased by [3].

# 2		Reference

[1] R3-221563, “RA-SDT BLCR to TS 38.401", Intel Corporation

[2] R3-222353, "(TP for RA-SDT BL CR for TS 38.300/401)", Intel Corporation

[3] R3-222644, "Summary of Offline Discussion on RACH based SDT", Ericsson

# 3		Text Proposal for RA-SDT BLCR to TS 38.401 [1]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.X	Overall procedure for Small Data Transmission during RRC Inactive

#### 8.X.1	RACH based SDT

The procedure for RACH based small data transmission in RRC Inactive is shown in Figure 8.X.1-1.

Figure 8.X.1-1: RACH based Small Data Transmission in RRC Inactive state.

1.	The UE in RRC Inactive sends RRCResumeRequest message together with UL SDT data and/or UL SDT signalling.

2.	The gNB-DU buffers the UL SDT data and/or UL SDT signalling.

3.	The step 3 is as defined in step 4 in clause 8.6.2, with including an indication of SDT access. The gNB-DU may also provide SDT assistance information.

4-5.	The steps 4-5 are as defined in steps 6-7 in clause 8.9.6.2. The UL SDT data, if any, is forwarded to the gNB-CU-UP, and the UL signalling, if any, is forwarded to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1: In case that full UE context is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP first establishes the UE context in the gNB-CU-UP via the Bearer Context Setup procedure and F1-U UL TEIDs are retreived before step 4. The BEARER CONTEXT SETUP REQUSET message may include an indication to suspend non-SDT bearers, and in this case, the BEARER CONTEXT MODIFICATION REQUEST message in step 6 does not include resume indication for SDT DRBs.

NOTE 2: In case that only partial UE context for SDT including F1-U UL TEIDs is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP uses those F1-U UL TEIDs for steps 4-5, and the subsequent steps 6-7 are not executed. In addition, the UL SDT data, if any, is forwarded from the gNB-DU to the gNB-CU-UP of the other gNB-CU-CP for which the partial context is retrieved, and the UL signalling, if any, is forwarded from the gNB-CU-CP to the other gNB-CU-CP via the XnAP RRC TRANSFER message.

6.	The gNB-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message including an resume indication for SDT DRBs. The gNB-CU-CP also includes the F1-U DL TEIDs received from the gNB-DU in step 5.

7.	The gNB-CU-CP responds with the BEARER CONTEXT MODIFICATION RESPONSE message.